3GPP TSG RAN WG1 #99			R1-1913484 Reno, USA, November 18th – 22nd, 2019

Agenda item:	6.2.4

Source: 	Qualcomm Incorporated

Title: 	Proposals for terrestrial broadcast (v3)

Document for:	Discussion and Decision

# CAS

Proposal 3.1: PBCH repetition is specified as following,

- For downlink bandwidth configuration NRBDL≥25, PBCH is repeated in subframe #0 of SFN satisfying SFN mod 4 = 0 (already agreed)

- For downlink bandwidth configuration NRBDL<25, PBCH is repeated in subframe #0 of SFN satisfying SFN mod 8 = 4.

- For downlink bandwidth configuration NRBDL=6, PBCH is not repeated.

The copied PBCH symbols are multiplied with  θl,k at OFDM symbol l slot ns and subcarrier k, where the length-144 Gold sequence at each OFDM symbol l within slot nsused to determine θl,k is initialized by cinit,l=213(NIDcell+1)×(1+l+NsymbDLns)+24NIDcell+(l+NsymbDLns)  , and θl,k is given by:

θl,k= 1,            cl2k=0, cl2k+1=0-1,            cl2k=0, cl2k+1=1j,           cl2k=1, cl2k+1=0-j,           cl2k=1, cl2k+1=1

NOTE: l is the symbol index within one slot, which can take values from 0 to 6 for normal CP, and from 0 to 5 for ECP