3GPP TSG RAN WG1 Meeting #99	                                                                          R1-1913599

Reno, USA

Nov 18th – Nov 22nd, 2019

Agenda item:    7.2.2

Source:             Qualcomm Incorporated

Title:                  Summary of NR-U agreements till RAN1 #99

Document for:  FYI

# Introduction

This paper summarizes the agreements reached during NR-U work item phase up to RAN1 #99.

# 7.2.2	NR-based Access to Unlicensed Spectrum

### RAN1 #AH-1901, Jan 2019

### RAN1 #96, Feb 2019

R1-1903576	[DRAFT] LS to ETSI BRAN on EN 301 893	Qualcomm Incorporated

Final LS is agreed in R1-1903757 with the following modifications:

- Replace the last sentence in the third paragraph with “Hence, it is RAN1’s understanding that for the case where more than one grant is provided to a responding device, 3GPP Rel-14/15 LTE-LAA specifications allow behaviour that is consistent with the provided clarifications and this is also expected to be the case for NR-U specifications currently being developed.”

- Replace the last two sentences of the second last paragraph with, “There is no consensus on the proposal to (1) ban the use of no LBT transmissions and (2) to restrict the use of short LBT transmissions so that it can only be used 1% of time rather than 5% as currently defined in the clause on Short Control Signalling Transmissions. There is also no consensus on limiting NR-U DRS transmissions to a 1% limit. Furthermore, reducing 5% transmission limit to 1% transmission limit would not be consistent with the behaviour of a device implementing LTE-LAA and is not preferred.”

- Modify the action as “RAN1 kindly requests ETSI TC BRAN to take the above information and requests for clarifications into account.”

### RAN1 #96b, April 2019

R1-1905748	Draft Response LS on short control signaling in NR-U	Qualcomm Incorporated

Final LS approved in R1-1905860

### RAN1 #97, May 2019

### RAN1 #98, August 2019

### RAN1 #98bis, October 2019

Agreement:

Include the following in a reply LS to RAN2 for R1-1909954:

- There are other signals/channels that can be detected with similar reliability to a DCI with CRC scrambled with a P-RNTI (e.g., PDCCHs with CRC scrambled with other RNTIs)

- There is no consensus in RAN1 on using these other signals/channels for the purpose of the UE stopping the monitoring of the additional PDCCH monitoring occasions at a given paging occasion

## 7.2.2.1	 Physical Layer Signals and Channels

### 7.2.2.1.1 Initial access signals/channels

#### RAN1 #AH-1901, Jan 2019

Agreement:

- UE assumes 30KHz SCS for SS/PBCH block for 5GHz band and 6GHz band if the SCS is not indicated by higher layers.

- Support configuration by higher layers of 15KHz or 30KHz SCS for SS/PBCH block

- Include this agreement in a LS to RAN4 (cc RAN2) for inclusion in specs managed by RAN4

Conclusion:

No changes are required to the time and frequency position of the PSS/SSS/PBCH relative to each other in one PSS/SSS/PBCH block.

Agreement:

The Type0-PDCCH monitoring configuration for NR-U should satisfy at least the following properties:

- TDM of Type0-PDCCH and SSB similar to existing pattern 1 (already agreed)

- Support the monitoring of Type0 PDCCH of the 2nd SSB position in a slot in the gap between 1st and 2nd SSB within the slot

  - FFS start at symbol #6 of #7 or both

- FFS: The Type0-PDCCH candidates associated with an SSB are confined within a slot carrying the associated SSB (with the same QCL assumptions)

Agreement:

Companies are encouraged to provide results comparing the different alternatives using the following simulation assumptions to select between alternative PRACH designs.

- The Rel-15 PRACH design should be simulated as a baseline

| Property | Value |
| --- | --- |
| Carrier frequency | 5 GHz |
| Channel model | TDL-C |
| Delay scaling | 10ns, 100 ns |
| Antenna configuration at BS(1) | (M,N,P) = (1,1,2) with omni-directional antenna element |
| Antenna configuration at UE | Single omni-directional antenna element |
| Antenna port virtualization | No beamforming and no beam selection |
| Frequency offset | 0.05ppm (fixed) at TRP, and 0.1 ppm (fixed) at UE |
| UE speed | 3 km/h |
| Initial timing offset | Uniformly distributed in [0, 1.2 µs (corresponding to 300 m ISD)] Optional: Uniformly distributed in [0, 2 µs (corresponding to 500 m ISD)] |
| PRACH format | A1 with other formats optional |
| Subcarrier spacing | 15/30 kHz. (with other SCS optional) |
| PRACH sequence and frequency resource allocation | For evaluation purpose, the Rel-15 PRACH ZC sequence (with possible length change) should be simulated. Additional/new sequences can be simulated. Each company should provide details on the sequence (type and length) and the resource allocation (e.g., Alt1~Alt4 and detailed mapping). |
| Total number of preambles per cell | 64, each company should provide details on how these 64 preambles are generated |
| Preamble detector | Each company should provide details on used algorithm |
| Interference assumption | No interference. Optional: -3/0/3dB interference power compared with target PRACH |
| Detection Criteria | 1% maximum mis-detection probability(2) |
|  | 0.1% maximum false alarm probability(3) |
|  | maximum timing estimation error being 50% of the normal CP length |
| Formatting of results (please also reference Section 8 of R1-1704144 for reporting formats) | Mis-detection probability vs. SNR |
|  | False alarm probability vs. SNR(4) |
|  | CDF of timing estimation error |
|  | PRACH capacity (maximum number of preambles) |
|  | Peak-to-average power ratio and cubic metric |
|  | MCL(5) |
| (1) See Table 7-1 of R1-1704144 (2) The missed detection probability is defined as the ratio between the total number of transmitted preambles that are either not detected, or detected as a different preamble, or detected but with timing error greater than the maximum value (i.e., 50% of normal CP length), and the total number of transmitted preambles within an observation interval. (3) Maximum false alarm probability refers to the case when input at receiver is noise only (considering 64 preamble detectors as in 3GPP TS 36.104, section 8.4.1). (4) False alarm probability is defined as the ratio of total number detected but not transmitted preambles, and the total number of possible detection occurrences, where each occurrence (occurrence refers to 64 detections, one for each of the 64 preambles in a cell) is one potential preamble transmission in a RO. (5) In the MCL calculation, needs to consider the maximum transmit power supported by the PRACH design under PSD limitation and PAPR/EVM characteristic of the design. Note: Assumptions on the following should be stated - use of a guard band (if any) - definition of SNR - signal bandwidth used |  |

#### RAN1 #96, Feb 2019

Agreement:

- Down-select from the following options for SSB pattern (symbol index starts at 0)

  - Option 1: SSBs are at symbols (2,3,4,5) and (8,9,10,11) in the slot

  - Option 2: SSBs are at symbols (2,3,4,5) and (9,10,11,12) in the slot

- The down-selected pattern applies no matter if SSB SCS is indicated by higher layer or not, and no matter if RMSI is transmitted or not.

Agreement:

- The SCS for all SSBs and Coreset #0 on a carrier is always the same for operation of NR in unlicensed spectrum.

- CORESET #0 frequency domain resource configuration should be 48 RBs for 30KHz SCS and 96 RBs for 15KHz SCS.

Agreement:

For PRACH evaluations, following metrics should be provided:

| Parameter | Value | Notes |
| --- | --- | --- |
| Scheme |  | Eg. Alt4-ZC139x2 |
| SCS |  | 15KHz or 30KHz |
| PRACH sequence length (L_RA) |  | Eg. 139, |
| # of repetition (R) |  | If repetition of sequence is used in freq domain |
| N_cs |  | Eg. 11 |
| # of RBs used for one RO (N_RB) |  | # of RBs occupied by PRACH. Eg. 12 for ZC139 design |
| # of interlaces used by one RO (N_interlace) |  | # of uniform interlaces (M=5 for 30KHz and M=10 for 15KHz) with RBs used for one PRACH RO |
| RACH frequency occupancy (MHz) |  | The actually used bandwidth with one RO, SCS*L_RA*R |
| Noise level, Np (dBm) |  | Np= -174+10*log10(SCS*L_RA*R)+NF NF=-5dB |
| SNR (dB) |  | SNR needed at 1% misdetection, read from simulation curve |
| P_max (dBm) |  | Maximum allowed transmit power under PSD limit of 10dBm/MHz measured in any 1MHz chunk and considers the RBs used by the proposed scheme |
| Backoff (dB) |  | Backoff is computed as 95% percentile of CCDF of [cubic metric] over the preambles in the RO. Note: If cubic metric is not used, information on the backoff metric used should be provided. |
| P_TX (dBm) |  | P_TX=min(P_max, 23- Backoff) is maximum allowed transmit power for the waveform considering backoff |
| MCL (dB) |  | MCL = P_TX-SNR-Np |
| N_FDM |  | # of ROs in 20MHz |
| Capacity |  | Across all ROs in 20MHz. Should report any constraints on ISD for the scheme evaluated. |

#### RAN1 #96bis, April 2019

Agreement:

PRB/RE-interlaced PRACH is not considered further. Consider the following alternatives should be studied further as options.

- Alt 1: Legacy NR PRACH sequence of length 139 mapped to contiguous subcarriers, with repetitions in frequency

  - FFS: Guard bands between repetitions

  - FFS: Number of repetitions

  - FFS: Whether repetitions are constrained to be contiguous in frequency or not

- Alt 2: A single PRACH sequence mapped to contiguous PRBs according to one of the following alternatives

  - Alt 2.1: ZC sequence with longer length than 139

  - Alt 2.2: New sequence with longer length than 139

Agreement:

Only coreset #0 lengths of 1 and 2 symbols are supported for NR-U

Agreement:

Select one of the following options in RAN1 #97:

- Alt 1: Legacy SSB positions in a slot

  - Support Type0-PDCCH in symbol (#0, #1) for length-2 coreset 0 and symbol (#0) for length-1 coreset 0 at least for the first SSB in a slot

  - Support Type0-PDCCH in symbol (#6, #7) for length-2 coreset 0 and symbol #6 or symbol #7 for length-1 coreset 0 for the second SSB in a slot

    - FFS: configurable between symbols #6 and #7 for the length-1 coreset 0 for the second SSB in a slot

- Alt2: New SSB positions in a slot

  - Support Type0-PDCCH in symbol (#0, #1) for length-2 coreset 0 and symbol (#0) for length-1 coreset 0 for the first SSB in a slot

  - Support Type0-PDCCH in symbol (#7, #8) for length-2 coreset 0 and symbol (#7) for length-1 coreset 0 for the second SSB in a slot

- Alt 3: Legacy SSB positions in a slot

  - Support Type0-PDCCH in symbol (#0, #1) for length-2 coreset 0 and symbol (#0) for length-1 coreset 0 for the first SSB in a slot

  - Support Type0-PDCCH in symbol (#7) for coreset 0 for the second SSB in a slot

#### RAN1 #97, May 2019

Agreement:

For SSB positions and type-0 PDCCH monitoring in a slot, only Alternatives 1 and 2 (from the previous agreement in RAN1#96bis) are considered further.

Note: As per the previous agreement one of Alternative 1 and 2 is to be selected at RAN1#97

Conclusion:

There is no consensus between Alternative 1 and Alternative 2 on the topic of SSB positions and type-0 PDCCH monitoring in a slot. No further online or scheduled offline discussions will be conducted on this topic. If there is consensus on such enhancements, the topic can be revisited in the Rel-16 NR-U work item.

R1-1907884	[DRAFT] LS to RAN4 on impact of sync raster placement for NR-U in RAN1	Qualcomm

The final LS is approved in R1-1907896 with the main body of the LS changed to the following:

“RAN1 understands RAN4 is discussing the sync raster design for NR-U band in 5GHz with the options of placing the sync raster either in approximately the middle of each 20MHz channel, or at approximately the edge of each 20MHz channel. RAN1 evaluated the impact to RAN1 if either of the options are adopted in RAN4.

From a RAN1 perspective, in unlicensed bands, where the DL initial BWP is nominally 20 MHz, RAN4 decisions on sync raster points have consequences on RAN1 design. RAN1 has discussed these consequences.

- If a SSB is not close to the edge, the number of RBs available to carry SIB1 would be limited although further enhancement to introduce rate matching around the SSBs to address this limitation are being discussed in RAN1

- If a SSB is close to the edge, the number of RBs available to carry SIB1 would be less limited although whether or not further enhancement to introduce rate matching around the SSBs is needed is being discussed in RAN1”

Agreement:

For a new enhanced design of NR-U PRACH in addition to the Rel-15 design (sequence length of 139) further discussion is limited to the following options

- ZC sequence of the following lengths

- 15 kHz: Choose one of L_RA=[571, 1151]

- 30 kHz: Choose one of L_RA=[283, 571]

- Repetition of Rel-15 PRACH sequences in frequency domain with potentially some mechanisms to improve the cubic metric

  - Consider one of 2 and 4 repetitions for 30 kHz and one of 4 and 8 repetitions for 15 kHz

- Note: Decision will be based on previously agreed evaluation metrics, capacity per cell (i.e., number of preambles per RACH occasion and number of RACH occasions) for the same time and frequency resources, specification impact and implementation complexity.

- Note: Companies should state any deviations in assumptions from the agreed evaluation assumptions.

#### RAN1 #98, August 2019

Agreement:

No new PRACH formats are specified for operation in unlicensed spectrum.

Agreement:

To support RMSI transmission for ANR purpose on a carrier with an SSB not on a sync raster consider the following alternatives:

- Alt 1: gNB configures the UE to report the CGI for a PCI on a given SSB frequency not on a sync raster point. After the UE detects the SSB it will proceed with RMSI decoding whereby the frequency location of the CORESET #0 scheduling the PDSCH carrying the RMSI is implicitly known.

- Alt 2: gNB configures the UE to report the CGI for a PCI on a given SSB frequency not on a sync raster point and the MIB in SSB will point to the frequency location of the coreset #0.

  - FFS: number of CORESET 0 offsets configurable in MIB when SSB is not on sync raster entry

  - Note: Signalling of offset only necessary if more than one off-sync-raster point offset is agreed

#### RAN1 #98bis, October 2019

Conclusion:

For the PDSCH Default A SLIV Table, Type-B PDSCH lengths other than the 2/4/7 lengths already supported in Rel-15, are not introduced for NR-U.

Working assumption:

For RMSI transmission for ANR purpose on a carrier with an SSB not on a sync raster, the PBCH in SSB not on a sync raster does not directly provide the location of the CORESET 0 for RMSI reception.

- The frequency domain difference between an off-sync SS/PBCH block and its associated CORESET #0 is determined at least based on

- The offset between the frequency location of the off-sync SS/PBCH block configured by gNB (high layer parameter ssbFrequency) and the frequency location corresponding to the GSCN of the synchronization raster entry within the same LBT bandwidth.

- Also based on the offsets signaled in PBCH payload (including MIB).

  - FFS: How many offsets

- Note: For ANR purpose, the SSB and and the associated CORESET0 are expected to be in the same LBT bandwidth

- Note: This working assumption assumes that there is only one sync raster point defined per 20 MHz. If RAN4 decides that there is more than one sync raster point per 20 MHz, then this working assumption is not valid and will be revisited

#### RAN1 #99, November 2019

Agreement:

For default A table for PDSCH SLIV for normal CP, replacing row 9 with an entry with (S=6, L=7)

- FFS if additional row can be replaced for (7,7)

Conclusion:

RMSI PDSCH rate matching around SSB will not be supported in Rel.16 for NR-U

Agreement:

Support CSI-RS multiplexing in DRS following channel access procedures for DRS

- Note: This address the FFS in 37.213 on if CSI-RS multiplexing is allowed in DRS

Agreement:

In addition to the Rel-15 design for NR short PRACH (sequence length of 139), support an enhanced PRACH design for NR-U by adopting a single long ZC sequence of the following lengths

- For 15 kHz SCS L_RA= 1151, For 30 kHz SCS L_RA= 571

- Introduce signalling in SIB1 to indicate to UE whether Rel-15 PRACH or enhance PRACH sequences above are used

- Logical root indices, cyclic shifts and frequency position are determined as give in Tables in Appendx B provided in R1-1911863

Agreement:

- For Coreset #0 frequency domain location determination

- For 30KHz SCS case, the pdcch-ConfigSIB1 indicates RB level offsets from {0, 1, 2, 3}

- For 15KHz SCS case, the pdcch-ConfigSIB1 indicates RB level offsets from {10, 12, 14, 16}

### 7.2.2.1.2	DL signals and channels

#### RAN1 #AH-1901, Jan 2019

Agreement:

- The UE may assume the presence of a signal, such as the DMRS in any [PDCCH or GC-PDCCH] transmission, to detect transmission bursts by the serving gNB, to enable power saving by not necessitating performing blind decodes to detect the transmission burst (Note: The power saving possibility by not necessitating blind decodes assumes performance relaxation for PDCCH decoding is not needed. Also, this does not mandate a two-step PDCCH decoding process for the UE with respect to DMRS detection).

  - If a preamble transmitted at the start of a burst is agreed to be specified (this does not preclude usage of preambles by implementation), it may be used in addition to the DMRS of PDCCH/GC-PDCCH or any other signals in the gNB transmission to detect the start of transmission bursts by the serving gNB and potentially for power saving

    - Note: Whether a preamble, if defined, can be used for power saving in all cases depends on the details of the design.

  - Note: Other signals present in the transmission burst may also be used for the purpose of detection of the transmission burst

  - FFS: Potential enhancements to DMRS design to address issues with detection probability

- The payload of a PDCCH and/or GC-PDCCH transmission can contain information regarding COT structure that may be used by the UE for power saving

#### RAN1 #96, Feb 2019

#### RAN1 #96bis, April 2019

Agreement:

Only further discuss/consider Option 3 (PDSCH mapping type B with durations other than 2/4/7 symbols) as potential enhancements to PDSCH transmissions.

#### RAN1 #97, May 2019

Agreement:

- Additional durations from 2 to 13 (other than 2, 4 and 7 which are already supported in Rel-15) for PDSCH mapping type B are supported.

  - FFS: A duration of 14

  - FFS: DMRS aspects for the additional durations (Reuse Rel-15 patterns or define new patterns for these additional durations)

- Capability signalling will be defined for UEs to indicate which specific subset of durations are supported by the UE.

- FFS: Some duration(s) are mandatory for the UE to support for initial access

- FFS: PDSCH processing times for the new durations (reuse the values for durations of 2/4/7 or define new ones)

- Note: PDCCH monitoring occasions will be discussed separately as part of the NR-U WI

- Note: This agreement does not by itself imply an increased requirement in PDSCH processing and decoding capabilities compared to Rel-15

#### RAN1 #98, August 2019

Agreement:

- The following is agreed for DM-RS positions for PDSCH mapping Type B of length 9 and 10:

| ld in symbols | DM-RS positions |  |  |  |
| --- | --- | --- | --- | --- |
|  | PDSCH mapping type B |  |  |  |
|  | dmrs-AdditionalPosition |  |  |  |
|  | 0 | 1 | 2 | 3 |
| 9 |  | , 7 | ,4,7 | ,4,7 |
| 10 |  | , 7 | ,4,7 | ,4,7 |

- If the PDSCH duration ld is 10 OFDM symbols for normal cyclic prefix, and if at least one of the DM-RS symbols corresponding to the PDSCH allocation collides with resources reserved for at least one of the symbols of a LTE CRS transmission, shall be incremented by 1

  - FFS: Whether the increment is applicable to all slots or a subset of slots

Conclusion:

- PDSCH type B durations 9,10 will not be optimised for double symbol DM-RS for avoiding LTE-CRS collisions

Agreement:

Support bit field corresponding to available LBT bandwidths in GC-PDCCH (add a bitmap in the GC-PDCCH DCI)

#### RAN1 #98bis, October 2019

Agreement:

Add a COT duration bit-field per serving cell in GC-PDCCH, i.e., DCI format 2_0

- The following are configurable by RRC:

  - Presence of this bit-field

  - Location of this bit-field in the DCI

  - Length of this bit-field in the DCI

    - FFS: Whether a single value will suffice in which case, the length is not configurable

  - Encoding of the bit field value, i.e., what COT duration corresponds to which bit-field value

- If a UE receives this bit-field, it applies the knowledge about end-of-COT at least for the purpose of UL transmission LBT category switching in a gNB-acquired COT

- If this field is not present, UE should use SFI indication to determine end-of-COT (if SFI is available)

  - FFS: details for this SFI-based mechanism

- FFS: Whether the duration is encoded as e.g., total length or remaining length.

- FFS: Granularity the signalled duration

Agreement:

- A UE can be provided with at least two groups (FFS: more than two groups) of search space sets for PDCCH. The UE can be configured to switch between the groups, indicated based on at least the following alternatives.

  - Alt 1: implicitly e.g. after detection of [FFS: DL burst, (WB-)DM-RS, GC-PDCCH and/or PDCCH] and/or e.g., based on information on COT structure.

  - Alt 2: explicitly in GC-PDCCH and/or PDCCH

- Search space sets that are not part of the configured groups (e.g., a common search space set) will always be monitored by the UE regardless of the search space set indication

- A single search space set can be part of more than one group.

- It is up to RAN2 to optimize the signalling to minimize overhead.

#### RAN1 #99, November 2019

Agreement:

Adopt DM-RS patterns from the table below (for single-symbol DM-RS and intra-slot frequency hopping disabled) for durations up to 11 symbols.

- FFS (in RAN1#99): DM-RS patterns for durations 12-14.

- Note: The cells in green have already been agreed or are part of Rel-15.

| ld in symbols | DM-RS positions PDSCH mapping type B |  |  |  |
| --- | --- | --- | --- | --- |
|  | dmrs-AdditionalPosition |  |  |  |
|  | 0 | 1 | 2 | 3 |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  | , 4 | , 4 | , 4 |
| 6 |  | , 4 | , 4 | , 4 |
| 7 |  | , 4 | , 4 | , 4 |
| 8 |  | , 6 | , 3, 6 | , 3, 6 |
| 9 |  | , 7 | , 4, 7 | , 4, 7 |
| 10 |  | , 7 | , 4, 7 | , 4, 7 |
| 11 |  | , 8 | , 4, 8 | , 3, 6, 9 |

Agreement:

If a DMRS symbol falls in the last symbol of the type-B allocation, the DMRS symbol is dropped.

- Note: This is same as specified in Rel-15 for durations 2/4/7.

Agreement:

Adopt DM-RS patterns from the table below (for double-symbol DM-RS and intra-slot frequency hopping disabled) for durations up to 11 symbols. Come back to durations 12-14 during this week.

- FFS (in RAN1#99): DM-RS patterns for durations 12-14.

- Note: The cells in green have already been agreed or are part of Rel-15.

| ld in symbols | DM-RS positions |  |  |  |
| --- | --- | --- | --- | --- |
|  | PDSCH mapping type B |  |  |  |
|  | dmrs-AdditionalPosition |  |  |  |
|  | 0 | 1 | 2 | 3 |
| <4 | - | - |  |  |
| 4 | - | - |  |  |
| 5 |  |  |  |  |
| 6 |  |  |  |  |
| 7 |  |  |  |  |
| 8 |  | , 5 |  |  |
| 9 |  | , 5 |  |  |
| 10 |  | , 7 |  |  |
| 11 |  | , 7 |  |  |

Agreement:

- For UE processing capability 1,

  - if L>=7 then d1,1 = 0

  - if L>=4 and L<=6, then d1,1 = 7-L

  - if L=3, then d1,1 = 3+min(d, 1), where d is the number of overlapping symbols of the scheduling PDCCH and the scheduled PDSCH.

- For UE processing capability 2

  - if L>=7 then d1,1 = 0

  - if L>=3 and L<=6, then d1,1 is the number of overlapping symbols of the scheduling PDCCH and the scheduled PDSCH

- Note: processing times for L=2/4/7 are included for sake of completeness and are unmodified from Rel-15.

Agreement:

A UE can be provided with not more than two groups of search space sets (as per previous agreement on search space switching) for PDCCH.

- Note: This resolves the FFS from the previous agreement

Agreement:

A UE shall not average CSI-RS measurements for channel estimation across different transmission bursts from the UE's perspective.

- FFS: Potential issues due to AGC

Agreement:

COT duration indicates remaining length from the beginning of the slot where the information is received

- When a UE receives a COT duration indication with a given symbol being within the COT duration, the UE is not expected to receive a subsequent COT duration indication that indicates that symbol to not be within the COT duration.

Agreement:

The UE can be configured with a set of up to [64] values for the COT duration. The COT duration bit field in DCI format 2_0 indicates the COT duration as an index to this set of values.

- The granularity of the duration is in units of OFDM symbols where the symbol duration is according to a configured reference sub-carrier spacing.

Agreement:

When the COT duration field is not configured to the UE in DCI format 2_0, the UE may assume that the duration of the COT is the same as the duration for which SFI is provided in DCI format 2_0.

Agreement:

- UE determines if a Cat 4 UL transmission can be switched to Cat 2 LBT with 25 us duration within a gNB initiated COT in indicated available LBT bandwidths, based on the indicated remaining channel occupancy duration.

- Note: If UEs with configured grant UL transmissions may apply the switch to Cat. 2, the gNB ensures that the COT is acquired using the highest CAPC

Agreement:

The indication of available LBT bandwidth is valid until the end of the determined channel occupancy

Agreement:

Resolve the FFS from the agreement in RAN1#98 as follows

- If the PDSCH duration ld is 10 OFDM symbols for normal cyclic prefix, and if at least one of the DM-RS symbols corresponding to the PDSCH allocation collides with resources reserved for at least one of the symbols of a LTE CRS transmission, shall be incremented by 1

  - The increment is applicable to all slots

Agreement:

Adopt the following single DM-RS positions for durations 12-13

| ld in symbols | DM-RS positions PDSCH mapping type B |  |  |  |
| --- | --- | --- | --- | --- |
|  | dmrs-AdditionalPosition |  |  |  |
|  | 0 | 1 | 2 | 3 |
| 12 |  | , 9 | , 5, 9 | , 3, 6, 9 |
| 13 |  | , 9 | , 5, 9 | , 3, 6, 9 |

Agreement:

Adopt the following double DM-RS positions for durations 12-13

| ld in symbols | DM-RS positions |  |  |  |
| --- | --- | --- | --- | --- |
|  | PDSCH mapping type B |  |  |  |
|  | dmrs-AdditionalPosition |  |  |  |
|  | 0 | 1 | 2 | 3 |
| 12 |  | , 8 |  |  |
| 13 |  | , 8 |  |  |

Agreement:

For PDSCH type B durations of 12 and 13, if a DMRS symbol falls in the last symbol of the type-B allocation or on or after symbol 12 (indexing starts from 0) of the slot, the DMRS symbol is dropped.

Agreement:

When a UE is configured with two groups of search space sets for PDCCH

- The presence of a monitoring group flag in DCI format 2_0 is configurable

- A timer is configured as a number of slots, which is decremented at the end of each slot.

- If the monitoring group flag is present in DCI format 2_0 (Note: Explicit switching between Group1 and Group2 and vice versa, plus implicit switching from Group2 to Group1)

  - If the flag = 1

    - Switch to (or keep) monitoring Group2 at the next applicable slot boundary with respect to the detected DCI format 2_0

    - The UE starts (or restarts) the configurable timer

  - If the flag = 0

    - Switch to (or keep) monitoring Group1 at the next applicable slot boundary with respect to the detected DCI format 2_0

  - The UE switches from Group2 to Group1 at the earliest slot boundary that is at least P1 symbols after

    - the end of the slot in which the timer expires, or

    - an indicated COT duration is exceeded, whichever comes first

- If the monitoring group flag is not present in DCI format 2_0 (Note: Implicit switching between Group1 and Group2 and vice versa)

  - If any PDCCH in Group1 is successfully detected

    - Switch from Group1 to Group2 at the next applicable slot boundary with respect to the detected PDCCH

  - The UE starts (or restarts) the configurable timer if it receives any PDCCH

  - The UE switches from Group2 to Group1 at the earliest slot boundary that is at least P2 symbols after

    - the end of the slot in which the timer expires, or

    - an indicated COT duration is exceeded, whichever comes first

- If the UE does not monitor for DCI format 2_0 (Note: Implicit switching between Group1 and Group2 and vice versa)

  - If any PDCCH in Group1 is successfully detected

    - Switch from Group1 to Group2 at the next applicable slot boundary with respect to the detected PDCCH

  - The UE starts (or restarts) the configurable timer if it receives any PDCCH

  - The UE switches from Group2 to Group1 at the earliest slot boundary that is at least P2 symbols after the end of the slot in which the timer expires

- The "next applicable slot boundary" is the earliest start of a slot that is at least P1/P2 symbols later than the last symbol of the corresponding PDCCH. P1 and P2 should be no less than the processing time required by the UE for performing search space switching.

- Search space switching is a new UE capability

  - Processing time is fixed in the specification

- FFS: Whether P1 and P2 need to be different

- The groups of serving cells to which the above behaviour is applicable can be configured via RRC

Agreement:

Search space set groups for the purpose of search space set group switching are configurable per BWP

### 7.2.2.1.3	UL signals and channels

#### RAN1 #AH-1901, Jan 2019

Agreement:

For interlace transmission of at least PUSCH and PUCCH, the following PRB-based interlace design is supported for the case of 20 MHz carrier bandwidth:

a.	15 kHz SCS: M = 10 interlaces with N = 10 or 11 PRBs / interlace

b.	30 kHz SCS: M = 5 interlaces with N = 10 or 11 PRBs / interlace

Note: PRACH design to be considered separately, including multiplexing aspects with PUSCH and PUCCH

Working assumption:

- For a given SCS, the following interlace design is supported at least for PUSCH:

  - Same spacing (M) between consecutive PRBs in an interlace for all interlaces regardless of carrier BW, i.e., the number of PRBs per interlace is dependent on the carrier bandwidth

  - Point A is the reference for the interlace definition

- For 15 kHz SCS, M = 10 interlaces and for 30 kHz SCS, M = 5 interlaces for all bandwidths

- FFS: Interlace design for PUCCH for bandwidths greater than 20 MHz

- FFS: Whether and how partial interlace allocation is supported

#### RAN1 #96, Feb 2019

Agreement:

- Support short and long PUCCH durations based on enhancements of at least Rel-15 PUCCH formats PF2 and PF3. The enhancements include at least the following aspects:

  - For a 20 MHz carrier bandwidth, support mapping to physical resources of at least one full interlace

  - Mechanism to support user multiplexing for both data and reference symbols of PUCCH

  - The following aspects are FFS:

    - Support for small payloads (1 and 2 bits)

      - Alt-1: Support both small payloads and larger payloads (> 2 bits) for enhanced PF2 and enhanced PF3

      - Alt-2: Small payloads are supported by enhanced PF0 and/or enhanced PF1

    - Whether or not to replace DFT-s-OFDM with CP-OFDM for the enhanced PF3

Agreement:

Suppport configuration of an SRS resource in additional OFDM symbol locations other than the last 6 symbols of a slot with PUSCH and SRS time division multiplexed as in Rel-15.

- FFS: which symbols locations.

Agreement:

Sub-PRB interlace design for PUSCH and PUCCH is not supported.

#### RAN1 #96bis, April 2019

Agreement:

For a 20 MHz carrier bandwidth, if enhancements to PF0 and PF1 are supported, a mapping to physical resources of at least one full interlace is supported

- FFS: Whether or not to support enhancements to PF0/1.

- Companies are encouraged to provide user multiplexing capacity and UCI payload analysis for enabling the decision for relevant use cases

Agreement:

Support RRC configuration of an SRS resource to start at any OFDM symbol within a slot by extending the RRC parameter startPosition of resourceMapping of SRS-Config for Rel-16 to have a value range 0..13.

Agreement:

Decisions on which additional PUCCH formats (enhanced or combination of legacy and enhanced) are supported should be at least based on the following.

- Which PUCCH format(s) are to be used at least for the following use cases:

  - HARQ ACK prior to dedicated PUCCH resource configuration

  - HARQ ACK, SR, CSI and combinations thereof after dedicated PUCCH resource configuration

- Specification impact, e.g., UE procedures in 38.213 and 38.212, for all proposed PUCCH formats to be supported

- User multiplexing capacity and UCI payload analysis for all proposed PUCCH formats to be supported

- In-band and out-of-band emissions

Agreement:

The following common set of evaluation assumptions and reporting metrics are used for enhanced PUCCH design

Evaluation assumptions

| Property | Value |
| --- | --- |
| Carrier frequency | 5 GHz |
| Channel bandwidth | 20 MHz |
| Channel model | TDL-C |
| Delay scaling | At least 10ns, 100ns with 300 ns optional |
| Antenna configuration at BS* | (M,N,P) = (1,1,2) with omni-directional antenna element |
| Antenna configuration at UE | Single omni-directional antenna element |
| Antenna port virtualization | No beamforming and no beam selection |
| Frequency offset | 0 ppm |
| UE speed | 3 km/h |
| Subcarrier spacing | 15/30 kHz (with other SCS optional) |
| Number of code-division multiplexed users if applicable | 1 user, with other cases optional |
| Interference assumption | No inter-cell interference |
| * See Table 7-1 of R1-1704144 |  |

Reporting metrics

| Parameter | Value | Notes |
| --- | --- | --- |
| Enhanced PUCCH Format |  | e.g., E-PF0, E-PF1, E-PF2, E-PF3 |
| Number of OFDM symbols used for PUCCH resource |  | e.g., 1, 2, 4, 14 |
| Number of RBs used for PUCCH resource (N_RB) |  | At least 1 full interlace assumed (Assume interlace design for 20 MHz carrier bandwidth as agreed in RAN1AH#1901 for 15kH and 30 kHz is used) |
| Frequency domain OCC configuration details (if applicable) |  | Include length and type of OCC, mapping of OCCs to control symbols and reference (DMRS) symbols, OCC cycling (if applicable) |
| Time domain OCC configuration details (if applicable) |  | Include length and type of OCC, mapping of OCCs to control symbols and reference (DMRS) symbols |
| Number of multiplexed users, e.g., by code division if applicable |  | 1 user is assumed as baseline Companies are to report other cases if evaluated |
| Waveform |  | e.g., CP-OFDM or DFT-s-OFDM |
| PUCCH encoder type |  | e.g., Reed Muller or Polar |
| SCS |  | 15KHz, 30KHz |
| Noise level, Np (dBm) |  | Np = -174 + 10*log10(SCS*12*N_RB) + NF NF = 5dB |
| Required SNR (dB) |  | Required SNR needed to fulfil detection criterion(1)(2), read from simulation curve |
| Cubic Metric |  | Note: Single value reported in dB |
| P_max (dBm) |  | Maximum allowed transmit power under PSD limit of 10dBm/MHz measured in any 1MHz chunk and considers the RBs used for the PUCCH resource |
| Backoff (dB) |  | Backoff is computed as the cubic metric. Note: If cubic metric is not used, information on the backoff metric used should be provided. |
| P_TX (dBm) |  | P_TX = min(P_max, 23- Backoff) is maximum allowed transmit power for the waveform considering backoff |
| MCL (dB) |  | MCL = P_TX – SNR – Np |
| PUCCH payload size(s) (bits) |  | If multiple payload sizes evaluated, then MCL to be plotted vs. PUCCH payload size |
| PUCCH encoding rate(s) |  | If multiple payload sizes evaluated, then multiple encoding rates to be reported (if applicable) |
| - For PUCCH payloads of 1 or 2 bits, detection criterion assumes that the PUCCH payload consists of randomly drawn HARQ ACK/NACK bits and is defined as P(ACK to Error) ≤ 1% and P(NACK to ACK) ≤ 0.1%. Error is defined as NACK or DTX where the decision region for DTX is determined to ensure that the maximum P(DTX to ACK) ≤ 1% for the case when the input to the receiver is noise only. - For PUCCH payloads greater than 2 bits follow the requirements in TS38.104 Section 8.3 Note: - Companies should provide details of RE mapping in frequency and time of enhanced PUCCH formats that are evaluated - Companies should provide details about channel estimation assumptions, e.g. practical, ideal - Definition of SNR to be reported with evaluation metrics |  |  |

#### RAN1 #97, May 2019

Agreement:

Support enhancement of Rel-15 PUCCH formats PF0 and PF1 as follows:

- Mapping to physical resources of one full interlace in 20 MHz.

  - FFS: Sequence type and mapping considering the following alternatives:

    - Alt-1: Repetition of the length-12 Rel-15 PF0 and PF1 sequence in each PRB of an interlace with mechanism to control PAPR/CM considering the following alternatives

      - Alt-1a: Cycling of cyclic shifts across PRBs

      - Alt-1b: Phase rotation across PRBs of an interlace where the phase rotation is can be per RE or per PRB

    - Alt-2: Mapping of different length-12 Rel-15 PF0 and PF1 sequences to the PRBs of an interlace based on different group number u (range is 0 .. 29)

    - Alt-3: Mapping of a single long sequence to the PRBs of an interlace

  - FFS: Impact due to guardbands

- Note: Decisions on the above should be based on at least performance using the agreed MCL metric and specification impact

- Note: Interlaced PF2 and 3 are not enhanced to support 1-2 bit payloads

Agreement:

For enhanced Rel-15 PF3 supporting interlaced mapping, do not replace DFT-s-OFDM with CP-OFDM

#### RAN1 #98, August 2019

Agreement:

The working assumption from RAN1 AH1901 is converted to an agreement with the following modifications:

- For a given SCS, the following PRB-based interlace design is supported at least for PUSCH and PUCCH:

  - Same spacing (M) between consecutive PRBs in an interlace for all interlaces regardless of carrier BW, i.e., the number of PRBs per interlace is dependent on the carrier bandwidth

  - Point A is the reference for the interlace definition

- For 15 kHz SCS, M = 10 interlaces and for 30 kHz SCS, M = 5 interlaces for all bandwidths

- FFS: Interlace design for PUCCH for bandwidths greater than 20 MHz

- FFS: Whether and how partial interlace allocation is supported considering mechanisms specific to PUSCH and PUCCH

- FFS: PUCCH bandwidth

- FFS: Whether or how an interlace design for PUSCH and/or PUCCH is supported on 10 MHz according to the revised WID objective

Agreement:

Alt-1a (Cycling of cyclic shifts across PRBs of the interlace) is selected from the four alternatives in the RAN1#97 agreement on enhanced Rel-15 PUCCH formats PF0 and PF1

- FFS: Cyclic shift ordering

Agreement:

A bandwidth occupied by a PUCCH resource does not exceed the bandwidth corresponding to a 20 MHz carrier/LBT bandwidth

Agreement:

- For interlaced PUSCH transmission in a BWP, X bits of the PUSCH frequency domain resource allocation field are used for indicating which combination of M interlaces is allocated to the UE.

- This applies to PUSCH of the following types:

  - Msg3 PUSCH

  - PUSCH Scheduled by fallback and non-fallback DCI

  - Type 1 and Type 2 Configured Grant PUSCH

- For 30 kHz SCS

  - Support X = 5 (5-bit bitmap to indicate all possible interlace combinations)

- For 15 kHz SCS

  - Down-select between the following two alternatives:

  - Alt-1: Support X = 10 (10-bit bitmap to indicate all possible interlace combinations)

  - Alt-2: Support X = 6 bits to indicate start interlace index and number of contiguous interlace indices (RIV) and using remaining up to 9 RIV values to indicate specific pre-defined interlace combinations

Conclusion:

For 10 MHz carrier bandwidth, enhancements to Rel-15 UL signals and channels are not necessary.

Agreement:

A PUCCH resource configured with interleaved mapping occupies consecutive PRBs within at least one interlace within a BWP. The PUCCH resource configuration includes the following:

- An indication of the allocated interlace

- An indication of the location of the PUCCH resource within the allocated interlace

  - Note: This may not be needed for a bandwidth part of 20 MHz or less

- The number of PRBs NPUCCH within the allocated interlace given by the following:

  - For Interlaced PF0/1/2:

    - NPUCCH = 10 or 11 depending on the allocated interlace

  - For Interlaced PF3:

    - NPUCCH = 10

- FFS: Whether/how an interlaced PF2/3 resource can be configured on 2 interlaces to increase the number of allocated PRBs to 20, 21, or 22 depending on the allocated interlaces

- FFS: Whether or not the BWP can be configured such that NPUCCH is less than 10 or 11

- FFS: Potential impact due to in-carrier guard bands

- Note: The UE is not expected to be configured with PUCCH transmissions spanning multiple LBT bandwidths

#### RAN1 #98bis, October 2019

Agreement:

Update the RRC parameter InterlaceAllocation-r16 (defined for a dedicated PUCCH resource) with the following:

- Index of 1st interlace

- Indication of the 2nd interlace index (if 2nd interlace is configured)

  - FFS: How the indication is achieved

- Indication of the LBT bandwidth location in which the PUCCH resource is configured

Agreement:

For PUCCH Formats 2 and 3 configured with interlace mapping, the number of configured interlaces is 1 or 2

- For Interlaced PF3:

  - NPUCCH = 10 if one interlaced is configured (as previously agreed)

  - NPUCCH = 20 if two interlaces are configured

- UE should use either one full interlace or two full interlaces according to configured maximum code rate and actual UCI payload size (subject to FFS below on the case of a BWP possible less than full carrier BW)

  - FFS: In case one interlace is used, which interlace is used

- FFS: If two interlaces are configured, whether or not there are configuration restrictions on the spacing between the two interlaces

- FFS: For a 20 MHz carrier bandwidth, whether a BWP can be configured to be less than the carrier bandwidth.

  - If allowed, NPUCCH can be less than 10 (for 1 interlace) or can be less than 20 (for 2 interlaces)

- Note: This agreement refers to configured interlaces, not actually used interlaces.

- Note: User multiplexing is to be further discussed. This agreement does not imply that user multiplexing is supported or not supported.

Agreement:

- For PUCCH formats 0 and 1 configured with an interlace mapping, the formula for cyclic shift hopping in Section 6.3.2.2.2 of 38.211 is given by

αl=2πNscRBm0+Δ*i+mcs+ncsns,fμ,l+l'modNscRB

where i∈0,1,…,N-1 indexes consecutive PRBs within an interlace starting with the lowest indexed PRB of the PUCCH resource within the BWP. N is the number of PRBs in an interlace (10 or 11).

- FFS: The step size Δ is down-selected to one value amongst {1,5,7,11}

  - The decision on which value to select is based on minimizing the 95th percentile CM

- FFS: For PF0, whether or not the above formula Δ includes an additional term 6*SR in order to increase user multiplexing from 1 to 3 in case of 2-bit ACK/NACK + SR (SR = 1 for positive SR and 0 for negative SR)

- FFS: In order to increase user multiplexing capacity for PF0, PF1, introduction of Δ*(i+i0) into the formula instead of Δ*i  where i+i0 is modulo N, and i0 is configurable in the range {0, 1, …, N-1}.

  - This mechanism may create collisions when the user multiplexing capacity is increased (i0 is not 0)

Agreement:

- Support configurability between interlace and contiguous (Rel-15) mappings for cell-specific PF0/1 resources, i.e., PUCCH resources configured prior to dedicated configuration

  - If interlace mapping is configured, the UE assumes there is no frequency hopping

- RAN2 to decide how broadcast signaling (SIB1) is modified to support this configurability

- Enhancements of Rel-15 PF0/1 when interlacing is not used will not be considered as part of the NR-U work in Rel-16

Agreement:

- Support configurability between interlace and contiguous (Rel-15) mappings for cell-specific PUSCH (e.g., msg3), i.e., PUSCH transmitted prior to dedicated configuration

  - If interlace mapping is configured, the UE assumes there is no frequency hopping

- RAN2 to decide how broadcast signaling (SIB1) is modified to support this

- Cell specific PUSCH and PUCCH can only be configured to both have interlaced mapping or to both have non-interlaced mapping

- FFS: The UE does not expect the configuration of interlaced or contiguous mapping of PUSCH/PUCCH to be different before and after dedicated RRC configuration

Agreement:

Support an RRC parameter to configure a UE with either interlace or contiguous (Rel-15) mappings for dedicated PUCCH resources, i.e., PUCCH resources configured by dedicated signalling

- If interlace mapping is configured, frequency hopping is not configured

- FFS: The UE does not expect configuration of interlace or contiguous mapping for different PUCCH resources to be different

Agreement:

For PUSCH transmissions after dedicated configuration, support an RRC parameter to enable configurability between interlace resource allocation and Rel-15 (Type 0/1) resource allocation

- If interlace mapping is configured, frequency hopping is not configured

- FFS: Whether this RRC configuration applies to fallback DCI

- FFS: Whether/how dynamic (non-fallback DCI based) switching is supported between interlaced resource allocation and Rel-15 RA (Type0/1) resource allocation

- FFS: The UE does not expect configuration of interlace or contiguous mapping for different PUSCH transmissions to be different

Agreement:

- Support a user-multiplexing mechanism for both UCI and reference symbols for interlaced PF3 when one interlace is configured as follows:

  - User multiplexing for the UCI is based on the application of pre-DFT OCC with

    - Alt. 1: Block-wise repetition in time domain followed by mapping over whole interlace in frequency (analogous to (Rel-15) PF4)

    - Alt. 2: Symbol-wise repetition in time domain followed by mapping over whole interlace in frequency

  - User multiplexing for the reference symbols is based on the use of different cyclic shifts of the same base sequence for all multiplexed users. The base sequence is a Z-C sequence (as for legacy Rel-15 PF3) mapped to the PRBs of the interlace

    - Length of ZC sequence = number of tones in the interlace

    - FFS: Mapping between OCC index applied on UCI symbols and cyclic shift applied to DMRS symbols

  - Support 1, 2, and 4 users for all PF3 durations (4 – 14 OFDM symbols)

- Support an RRC parameter for indicating OCC configuration for interlaced PF3

- If two interlaces are configured, user multiplexing is not supported for interlaced PF3

Agreement:

- Support a user-multiplexing mechanism for both UCI and reference symbols for interlaced PF2 when one interlace is configured as follows:

  - User multiplexing is based on OCCs applied in at least the frequency domain to the UCI and reference signal REs separately

    - User multiplexing for the UCI is based on RE-wise repetition within a PRB

  - Support only frequency domain OCC lengths of 1, 2 and 4 to multiplex users on both 1-symbol and 2-symbol PF2

  - Consider the following for controlling CM/PAPR. If supported, select from the following alternatives using CM and MCL as criteria

    - OCC cycling across PRBs of an interlace

    - Scrambling

    - No CM/PAPR controlling mechanism

- Support an RRC parameter for indicating OCC configuration for interlaced PF2

- If two interlaces are configured, user multiplexing is not supported for interlaced PF2

Agreement:

- Allow configured UL (CUL) transmissions in a set of symbols of a slot when the UE is configured with DCI 2_0 format monitoring and does not detect a DCI 2_0 format providing a slot format for the set of symbols. This is applicable when operating with LBE for the following cases.

  - When the set of symbols are indicated as ‘F’ with a semi-static TDD pattern or

  - When the UE is not configured with a semi-static TDD pattern

- Note: Configured UL (CUL) transmissions are UL signals/channels configured by higher layers:

  - PUCCH

  - Configured Grant PUSCH

  - Periodic SRS, [semi-persistent SRS]

    - Unclear if semi-persistent SRS covered by the current Rel-15 behaviour

  - [PRACH]

    - Unclear if PRACH covered by the current Rel-15 behaviour

- FFS: How to allow the CUL transmissions as per the above

#### RAN1 #99, November 2019

Agreement:

- For interlaced PF2, the following mechanism is supported for controlling CM/PAPR when OCCs are configured

  - OCC-cycling across PRBs of an interlace, where

    - OCC index for the ith PRB in an interlace is given by ni=n0+imodNSFPUCCH,2 where n0 is the initial OCC index configured by the RRC parameter OCC-Index-r16

- Note: This applies only for the case when a single interlace is configured (as previously agreed)

Agreement:

For Interlaced PF3, use the following mapping between the OCC index applied on UCI REs and cyclic shift index applied on DMRS symbols:

| Orthogonal sequence index configured by OCC-Index-r16 | Cyclic shift index m0 |  |
| --- | --- | --- |
|  | NSFPUCCH,3=2 | NSFPUCCH,3=4 |
| 0 | 0 | 0 |
| 1 | 6 | 6 |
| 2 | - | 3 |
| 3 | - | 9 |

- If OCC-Index-r16 is not configured, the cyclic shift index is m0=0.

Agreement:

For Interlaced PF2/3, if two interlaces are configured and the UE uses only one, the used interlaced is Interlace0 as configured in InterlaceAllocation-r16

Agreement:

At least for 30 kHz SCS, for interlaced PF2/3, if two interlaces are configured, there are no configuration restrictions on the spacing between the two interlaces

- The 1st and 2nd interlaces are configured by the parameter Interlace0 and Interlace1, respectively, each with value range 0 .. M – 1 where M is the number of interlaces

Agreement:

For 15 kHz SCS, for interlaced PF2/3, if two interlaces are configured,

- The 1st and 2nd interlaces are configured by the parameter Interlace0 and Interlace1, respectively, each with value range 0 .. M – 1 where M is the number of interlaces

- The interval between the two interlaces can be 1 or 5, i.e., Interlace1 can be derived as mod(Interlace0 + X, M), where M is the number of interlaces, where X is 1, -1 or 5

Agreement:

From the RAN1#98 agreement on interlace indication for PUSCH for 15 kHz SCS, Alt-2 is selected:

- Support X = 6 bits to indicate start interlace index and number of contiguous interlace indices (RIV) and using remaining up to 9 RIV values to indicate specific pre-defined interlace combinations

- RIV values from 0..54 indicate start interlace index and number of consecutive interlace indices

- RIV values from 55 ..63 indicate the following interlace combinations (from 36.213):

| RIV | Interlace Indexes |
| --- | --- |
| 55 | 0,5 |
| 56 | 0,1,5,6 |
| 57 | 1,6 |
| 58 | 1,2,3,4,6,7,8,9 |
| 59 | 2,7 |
| 60 | 2,3,4,7,8,9 |
| 61 | 3,8 |
| 62 | 4,9 |
| 63 | Reserved |

Agreement:

- For interlaced PUSCH transmission in a BWP, Y bits of the frequency domain resource allocation (FDRA) field indicate which RB sets (corresponding to LBT bandwidths) are allocated to the UE

- This applies to PUSCH of the following types

  - PUSCH scheduled by at least non-fallback DCI

    - FFS: applicability to fallback DCI

  - Configured Grant PUSCH Type 2 (FDRA indicated by DCI)

  - Configured Grant PUSCH Type 1 (FDRA configured by RRC)

- The UE determines the overall PUSCH frequency domain resource allocation by the intersection of the following:

  - Allocated interlaces (indicated by X bits of the FDRA field, as previously agreed)

  - Available PRBs derived at least from the allocated RB sets (indicated by Y bits of the FDRA field) and intra-carrier guard bands between RB sets corresponding to contiguous LBT bandwidths

    - Note: An RB set contains PRBs within an LBT bandwidth and does not include any inter or intra carrier guard PRBs

    - Note: The PRBs between adjacent RB sets comprise an intra-carrier guard

- Y is determined by the number of RB sets contained in the BWP

- The Y bits indicate a first RB set and a number of RB sets corresponding to contiguous LBT bandwidths

- Note: The maximum possible value of Y is thus log2NN+12 where N is the number of RB sets contained in the BWP

Agreement:

For Interlaced PF3, user multiplexing for the UCI is based on the application of pre-DFT OCC with block-wise repetition in the time domain followed by mapping over the whole interlace in frequency (analogous to Rel-15 PF4)

Agreement:

The OCC used for interlaced PF3 is a DFT sequence (as in Rel-15)

Agreement:

Support RRC parameter for enabling CUL transmissions according to new behavior agreed in RAN1#98b

- If the parameters is not configured, the UE assumes Rel-15 behavior

Agreement:

For PUCCH resource sets before dedicated PUCCH resource configuration when useInterlacePUCCH-Common-r16 is configured, Table 9.2.1-1 in 38.213 is used to determine the PUCCH resource configuration as follows:

- The UE interprets the PRB offset as a Starting Interlace Index Offset

  - The starting interlace is used in the mapping between r_PUCCH and the allocated PUCCH resouce

- The UE ignores row 15

- Rows 0,1, and 2: Support a 2nd PUCCH resource starting at symbol 9

- Rows 3, 7, and 11: Support a 2nd OCC code

- Feature lead to draft text proposal for 38.213 Section 9.2.1 on procedure text associated with modified table

- For PF0 resource index ordering the UE considers the cyclic shift first, the interlace index second, and the starting symbol index last.

- For PF1 resource index ordering the UE considers the cyclic shift first, the interlace index second, and the OCC index last.

Agreement:

In Section 6.3.2.2.2 of 38.211 (draft CR R1-1913178) where the formula for cyclic shift hopping includes the term mint=Δ∙nIRBμ, the step size Δ is fixed at 5

Agreement:

In Rel-16, for a cell, the UE can expect that cell-specifically configured PUCCH resources and UE-specifically configured PUCCH resources either all have interlaced mapping or all have non-interlaced mapping as per Rel-15.

- Note: RRC parameters that are made redundant due to this agreement can be eliminated

Agreement:

In Rel-16, for a cell, the UE can expect that cell-specifically configured PUSCH resources and UE-specifically configured PUSCH resources either all have interlaced mapping or all have non-interlaced mapping as per Rel-15.

- Note: RRC parameters that are made redundant due to this agreement can be eliminated

Agreement:

- The UE transmits PUSCH scheduled by fallback DCI in CSS within the initial BWP on the interlaces indicated by the X bits of the FDRA field

- Note: The FDRA field for fallback DCI in CSS does not include Y bits

Agreement:

In Rel-16, for a cell, the UE can expect that UE-specifically configured PUCCH resources and all PUSCH transmissions (scheduled and configured) after dedicated configurations either all have interlaced mapping or all have non-interlaced mapping as per Rel-15.

## 7.2.2.2	Physical Layer Procedure

### 7.2.2.2.1	Channel access procedures

#### RAN1 #AH-1901, Jan 2019

Agreement:

- A gap (DLUL, ULUL, or UL DL) of a specific duration is created using one or more of:

  - Timing Advance

  - CP extension

    - max value of not more than one OFDM symbol

  - Shortening of DL or UL transmission duration by one or more OFDM-symbol(s) by puncturing or rate matching

  - Note: the mechanisms applied in each case may be different for different SCSs

- FFS: how to signal the way of creating the gap to the UEs

#### RAN1 #96, Feb 2019

Conclusion:

A common preamble and a common energy detection threshold between NR-U and other technologies for 5 and 6 GHz was discussed. There is no consensus on these aspects at this stage.

Conclusion:

Cat 2 is not used for initiating a UE transmission outside of a gNB COT for the following channels/signals (or any combination of them):

- PUSCH (with or without UCI),

- SRS-only,

- PUCCH-only

Note:

- Cat 4 for these channels was already agreed during the study item

- This does not preclude the use of Cat 2 for transmission on a LBT bandwidth if it is allowed for the case of transmission on multiple LBT bandwidths

Agreement:

For initiation of a gNB transmission:

- LBT other than Cat 4 is not used for DRS multiplexed with unicast data

- LBT other than Cat 4 is not used for PDCCH and/or PDSCH transmission outside of DRS.

Note:

- This does not preclude the use of Cat 2 for transmission on a LBT bandwidth if it is allowed for the case of transmission on multiple LBT bandwidths

Agreement:

LBT other than Cat4 is not considered for UL transmissions that are part of a RACH procedure that initiate a channel occupancy

- Note: This does not preclude the use of Cat 2 for transmission on a LBT bandwidth if it is allowed for the case of transmission on multiple LBT bandwidths

#### RAN1 #96bis, April 2019

Conclusion:

If a category 2 LBT with a duration of 25 microseconds is required, no changes to the baseline method as followed in LAA for such LBT are needed

Agreement:

The following agreement from the SI is updated as shown:

Table 7.2.1.3.1-1: Channel access schemes for initiating a COT by gNB as LBE device

|  | Cat 2 LBT | Cat 4 LBT |
| --- | --- | --- |
| DRS alone or multiplexed with non-unicast data (e.g. OSI, paging, RAR) | When the DRS duty cycle ≤1/20, and the total duration is up to 1 ms: 25 µs Cat 2 LBT is used (as in LAA) | When DRS duty cycle is > 1/20, or total duration > 1 ms Cat4 with any channel access priority class value can be used |
| DRS multiplexed with unicast data | N/A except for the cases discussed in the Note below | Channel access priority class is selected according to the multiplexed data |
| PDCCH and PDSCH | N/A except for the cases discussed in the Note below | Channel access priority class is selected according to the multiplexed data |

Agreement:

For a UCI-only transmission on PUSCH in a channel occupancy initiated by the UE, Cat4 with lowest channel access priority class value can be used by the UE

Agreement:

For LBT by a UE prior to transmission of a UL burst within a gNB-initiated channel occupancy as LBE device, for gap durations shorter than 25 microseconds, choose one of the following alternatives

- Alt 1: Cat 2 LBT can be indicated (FFS: explicit and/or implicit) to the UE if the gap is 16 microseconds (allowing for implementation tolerances)

- Alt 2: Cat 2 LBT is not indicated to the UE for gaps less than 25 microseconds

- Notes (applicable to both alternatives):

  - This means that the gNB ensures that gaps between 16 and 25 microseconds do not occur

  - This doesn’t change the previous agreement for Cat 1 and Cat 2 LBT for gaps of 16 microseconds or less

- FFS: Conditions on channel occupancy after a Cat. 1 or Cat. 2 LBT after a gap of 16 microseconds or less

#### RAN1 #97, May 2019

Agreement:

For LBT by a UE prior to transmission of a UL burst within a gNB-initiated channel occupancy as an LBE device, for gap durations shorter than 25 microseconds, Cat 2 LBT can be indicated (FFS: explicit and/or implicit) to the UE if the gap is 16 microseconds (allowing for implementation tolerances)

Note: this is the Alt 1 identified in RAN1#96bis

Agreement:

Select one of the following alternatives for Cat2 LBT in a 16 us gap.

- Alt 1: The 16us measurement period is split into two slots with the first slot having a duration 7us and second slot having a duration of 9us.

  - Energy measurement is done in the 9us slot with the measurement including averaging for at least 4 us in any portion of the slot. LBT is said to be successful if the measured energy is lower than the ED threshold.

- Alt 2: The 16us measurement period is split into two slots with the first slot having a duration 7us and second slot having a duration of 9us.

  - Energy measurement is done in both the 7us and 9us slot with the measurement including averaging for at least 4 us in any portion of each slot. LBT is said to be successful if the measured energy is lower than the ED threshold in both slots.

- Alt 3: Energy measurement is done in any portion of the 16 us duration including averaging for at least 4 us. LBT is said to be successful if the measured energy is lower than the ED threshold.

Agreement:

Multi-carrier channel access schemes that are applicable to multiple carriers with a single LBT sub-band per carrier that is the same as the carrier bandwidth are also applicable to multiple LBT sub-bands within a carrier when such carriers are used either as a single wideband carrier or as part of carrier aggregation of a set of carriers.

#### RAN1 #98, August 2019

Agreement:

For a gNB initiated channel occupancy the reference duration for CWS adjustment is defined as follows.

- For a CO with unicast PDSCH(s) and for each set of LBT bandwidths for which a single contention window is maintained, the reference duration for CWS adjustment is from the beginning of the CO until the end of the first slot where at least one unicast PDSCH is transmitted over all the resources allocated for the PDSCH, or until the end of the first transmission burst by the gNB that contains unicast PDSCH(s) transmitted over all the resources allocated for the PDSCH, whichever occurs earlier.

  - If the CO has a unicast PDSCH, but doesn’t have any unicast PDSCH transmitted over all the resources allocated for that PDSCH, then, the duration of the first transmission burst by the gNB within the CO that contains unicast PDSCH(s) is the reference duration for CWS adjustment.

Agreement:

For a UE initiated channel occupancy the reference duration for CWS adjustment is defined as follows.

- For a CO with PUSCH(s) and for each set of LBT bandwidths for which a single contention window is maintained, the reference duration for CWS adjustment is from the beginning of the CO until the end of the first slot where at least one PUSCH is transmitted over all the resources allocated for the PUSCH, or until the end of the first transmission burst by the UE that contains PUSCH(s) transmitted over all the resources allocated for the PUSCH, whichever occurs earlier.

  - If the CO has a PUSCH, but doesn’t have any PUSCH transmitted over all the resources allocated for that PUSCH, then, the duration of the first transmission burst by the UE within the CO that contains PUSCH(s) is the reference duration for CWS adjustment.

Agreement:

For a gNB initiated channel occupancy, for a DL burst without unicast PDSCH and with one or multiple UL grants, and for each set of LBT bandwidths for which a single contention window is maintained, CWS adjustment is based on the success or failure (FFS: CB, CBG, or TB) of reception of PUSCH transmissions in the granted resources

- FFS: Details of CWS adjustment based on the reception of a successful transmission

- FFS: Handling of PUCCH/PRACH/SRS and PUSCH without UL-SCH

#### RAN1 #98bis, October 2019

Agreement:

Sharing of a UE-initiated channel occupancy (either CG-PUSCH or scheduled UL) with gNB is supported, such that the gNB is allowed to transmit control/broadcast signals/channels for any UEs as long as the transmission contains transmissions for the UE that initiated the channel occupancy and/or DL signals/channels (PDSCH, PDCCH, reference signals) meant for the UE that initiated the channel occupancy.

- The ED threshold that the UE applies when initiating a channel occupancy to be shared with the gNB is configured by gNB (RRC signaling)

  - if ED threshold that the UE applies when initiating a channel occupancy to be shared with the gNB is not configured, the transmission of the gNB in UE initiated COT may include only control/broadcast signals/channels transmissions of up to 2/4/8 OFDM symbols in duration for 15/30/60 kHz SCS

  - When absence of WiFi cannot be assumed based on e.g. regulation, the ED threshold that the gNB configures to the UE to apply when initiating the channel occupancy is determined based on the max gNB TX power

- Cat. 2 LBT can be used (for gaps of 16us and 25us).

- Cat. 1 LBT can be used under the following conditions.

  - Gap duration <= 16us

  - For the transmission of the gNB after the first switch between the UE and the gNB if the gNB transmission contains only control/broadcast signals/channels

  - For the transmission of the gNB after the first switch between the UE and the gNB if the gNB transmission has a duration below X ms (X >= 0).

    - FFS: X

  - FFS: For transmissions after the second and subsequent switches between UE and gNB

Agreement:

- For the CP extension prior to at least a dynamically scheduled PUSCH transmission, the CP extension is located in the symbol(s) immediately preceding the PUSCH allocation indicated by SLIV. The supported durations for CP extension at the UE are:

  - 0 (i.e. no CP extension)

  - C1*symbol length – 25 us

  - C2*symbol length – 16 us - TA

  - C3*symbol length – 25 us – TA

- C1=1 for 15 and 30 kHz SCS, C1=2 for 60 kHz SCS

- FFS: Whether C2/C3 is fixed or implicitly derived based on TA for each subcarrier spacing

- The N2 timeline (UL grant to PUSCH delay) needs to be relaxed to take the CP extension into account

- FFS: Whether the limit as per the previous agreement bounding the resulting CP extension to be less than or equal to one symbol for the given subcarrier spacing should be relaxed

- FFS: Applicability of this to other UL transmissions

- FFS: Whether the number of durations for CP extension that the UE can be signalled dynamically can be configured

Agreement:

- For FBE operation, when the gNB operates as an initiating device

- The UE is provided information on the gNB fixed frame period and the starting positions of the fixed frame periods

- For the provision of the above information the following is signalled

  - Indication of the fixed frame period and the starting positions of the fixed frame period (SIBx)

    - FFS: Whether Rel-15 signaling can be reused

- When the UE is not initiating a channel occupancy, UE transmissions within a fixed frame period can occur if DL signals/channels (e.g., PDCCH, SSB, PBCH, RMSI, GC-PDCCH, …) within the fixed frame period are detected.

  - FFS: Extension of GC-PDCCH configuration to idle UEs

- FFS: Signaling for FBE operation, when the UE operates as an initiating device

#### RAN1 #99, November 2019

Agreement:

C2 &C3 (from prior agreement on CP extension) are UE specifically RRC configured

- Note: this does not restrict the maximum TA that can be used in a cell

- FFS: Agree the values of C2 and C3 in RAN1#99

Agreement:

For FBE operation

- FFP configuration is included in SIB-1

- FFP configuration can be signaled for a UE with UE-specific RRC signaling

Agreement:

For a Cat2 LBT in a 16 us gap, energy measurement is done for a total of at least 5 us with at least 4 us of sensing falling within the 9 us slot immediately before the transmission.

- LBT is said to be successful if the measured energy is lower than the ED threshold

Agreement:

Channel grouping restrictions applicable for multi-carrier channel access procedures in the uplink for LTE-LAA are also applicable for Type B multi-channel channel access procedures in the downlink.

Agreement:

- The fixed frame period is restricted to values of {1ms, 2ms, 2.5ms, 4ms, 5ms, 10ms} (this is including the idle period)

- The starting positions of the FFPs within every two radio frames starts from an even radio frame and are given by i*P where i={0,1,.., 20/P-1} where P is the fixed frame period in ms

- The idle period for a given SCS = ceil( Minimum idle period allowed by regulations / Ts) where

  - Minimum idle period allowed = max(5% of FFP, 100us)

  - Ts is the symbol duration for the given SCS

- Note: Offset does not need to be signaled via RRC

Agreement:

The supported range of values of both C2 and C3 (from prior agreement on CP extension) that can be configured by RRC is

- 1, 2, …, 28 for 15 and 30 kHz SCS, and

- 2, 3, …, 28 for 60 kHz SCS

Note: Other agreements are not superseded by this agreement

Agreement:

- The maximum duration of the transmission by a UE after Cat. 1 LBT is restricted to 584 microseconds

- The parameter X (from prior agreement) for DL transmission after Cat. 1 LBT is equal to 584 microseconds

Agreement:

For non-fallback DCI UL grant:

- LBT type, length of the CP extension, and CAPC are jointly encoded in the UL grant.

- The combinations of LBT type, length of the CP extension, and CAPC that can be dynamically signaled are RRC configured for the UE with UE-specific RRC signaling

- RRC configuration supports indication of all the combinations of the rows in the three tables below except:

  - combination of “C2*symbol length – 16 us – TA” and “Cat2 25 µs”

  - combination of “C3*symbol length – 25 us – TA” and “Cat1 16 µs”

  - combination of “C3*symbol length – 25 us – TA” and “Cat2 16 µs”

  - combination of “C1*symbol length – 25 us” and “Cat1 16 µs” or “Cat2 16 µs”

- The bitfield in the DCI has up to [6] bits, depending on how many combinations the RRC signaling indicates for the UE

LBT Type

Cat116 µs

Cat216 µs

Cat2 25 µs

Cat4

CP extension

0 (i.e. no CP extension)

C1*symbol length – 25 us

C2*symbol length – 16 us – TA

C3*symbol length – 25 us – TA

CAPC

1

2

3

4

Agreement:

For non-fallback DL assignments scheduling UL transmissions (e.g. PUCCH):

- LBT type, and the length of the CP extension, are jointly encoded in the DL assignment.

- Highest CAPC is always assumed as per prior agreements

- The combinations of LBT type and the length of the CP extension, that can be dynamically signaled are RRC configured for the UE with UE-specific RRC signaling

- RRC configuration supports indication of all combinations of the rows in the in the two tables below except:

  - combination of “C2*symbol length – 16 us – TA” and “Cat2 25 µs”

  - combination of “C3*symbol length – 25 us – TA” and “Cat1 16 µs”

  - combination of “C3*symbol length – 25 us – TA” and “Cat2 16 µs”

  - combination of “C1*symbol length – 25 us” and “Cat1 16 µs” or “Cat2 16 µs”

- The bitfield in the DCI has up to 4 bits, depending on how many combinations the RRC signaling indicates for the UE

LBT Type

Cat116 µs

Cat216 µs

Cat2 25 µs

Cat4

CP extension

0 (i.e. no CP extension)

C1*symbol length – 25 us

C2*symbol length – 16 us – TA

C3*symbol length – 25 us – TA

Agreement:

For fallback DCI UL grant:

- LBT type, length of the CP extension, and CAPC are jointly encoded in the UL grant and signaled with 2 bits.

- The supported combinations of LBT type, and length of the CP extension and CAPC are hardcoded in RAN1 specs

Agreement:

For fallback DL assignments scheduling UL transmissions (e.g. PUCCH):

- LBT type, and the length of the CP extension are jointly encoded in the DL assignment and signaled with 2 bits.

- The supported combinations of LBT type and length of the CP extension are hardcoded in RAN1 specs

Agreement:

At least for LBE operation:

- For signaling of LBT type & CP extension for both Fallback DL assignment and Fallback UL Grant, the following table is used:

| LBT Type | CP extension |
| --- | --- |
| Cat1 16 µs | C2*symbol length – 16 us – TA |
| Cat2 25 µs | C3*symbol length – 25 us – TA |
| Cat2 25 µs | C1*symbol length – 25 us |
| Cat4 | 0 |

  - CAPC is not indicated explicitly:

    - For the UL grants

      - The UE assumes CAPC=4 was used by the gNB to acquire the CO,

      - For UE initiated COTs (Cat4 case) the UE may select the CAPC by itself.

        - Note: The mapping between priority classes and traffic classes follows the same mechanism as defined for UL CG transmissions.

    - Note: For PUCCH associated with DL assignments may use the highest priority CAPC when CAT4 LBT is used, as agreed earlier.

- If the network indicates FBE operation, for an indication of LBT type of Cat 2 25 us or Cat4 the UE follows the mechanism whereby one 9 microsecond slot is measured within a 25 microsecond interval as in 37.213.

Agreement:

- For RAR, we use the same LBT type and CP extension tables and CAPC selection mechanism as used for the Fallback DCI UL grant.

  - This requires 2 bits to be signaled in RAR (the PDSCH)

  - If the UE multiplexes user plane data with PUSCH

    - The UE assumes CAPC=4 was used by the gNB to acquire the CO,

    - For UE initiated COTs (Cat4 case) the UE may select the CAPC by itself.

      - Note: The mapping between priority classes and traffic classes follows the same mechanism as defined for UL CG transmissions.

  - The frequency domain resource allocation field can be reduced to accommodate 2 bits in the RAR

- Include this agreement in an LS to RAN2

Agreement:

For CW adjustment for transmissions including PDSCH and PUSCH:

- if new HARQ feedback is available relative to the prior CW update, the feedback for the latest COT for which new feedback is received shall be used:

- if the HARQ feedback is ACK, CW shall be set to CWmin.

- if the HARQ feedback is NACK (or if in absence of feedback within a window as defined below, the gNB or the UE retransmits the TB), CW shall be set to min(CW×2 + 1, CWmax).

  - Window starts at the end of the reference duration and has a duration of max (X ms, the duration of the transmission burst from start of the reference duration + 1 ms)

  - If the absence of other technologies cannot be guaranteed (same condition as in existing specs for other cases), X = 5. Otherwise, X = 10.

- Otherwise (if new HARQ feedback is not available), CW shall remain the same.

- Note: HARQ feedback includes any implicit methods of HARQ feedback determination.

Agreement:

For TB based HARQ feedback within a single LBT subband, CW is reset if at least one “ACK” is received, or at least one NDI is toggled for the TB(s) transmitted in the reference duration

- Note: HARQ feedback includes any implicit methods of HARQ feedback determination.

Agreement:

For CBG based HARQ feedback within a single LBT subband, and when all CBGs are confined within the LBT subband, CW is reset if “ACK” is received for at least 10 % of the CBGs in the reference duration

- For purpose of CWS adjustment, a CBG TI set to 0 is assumed to be an ACK

- Note: HARQ feedback includes any implicit methods of HARQ feedback determination.

Agreement:

Channels without explicit feedback use the CWS last updated by channels with explicit feedback and using the same CAPC if such channels exist; otherwise they use the minimum CWS corresponding to the CAPC.

Agreement:

For CWS adjustment for an LBT sub-band when a single contention window is maintained per LBT subband, all CBGs (if any are present) and TBs that partially or fully overlap with that LBT sub-band are taken into account.

- CW is reset if “ACK” is received for at least 10 % of the CBGs or for at least one TB in the reference duration

- Note: Other procedures for contention window adjustment within an LBT subband are also applicable

- A UE can choose to apply feedback only based on TBs for CW adjustment

Agreement:

For CWS adjustment for DL, when a single contention window is maintained for multiple LBT subbands, all CBGs (if any are present) and TBs that partially or fully overlap with those multiple LBT sub-bands are taken into account.

- CW is reset if “ACK” is received for at least 10 % of the CBGs or for at least one TB in the reference duration

- Note: Other procedures for contention window adjustment within an LBT subband are also applicable.

### 7.2.2.2.2	Enhancements to initial access procedure

#### RAN1 #AH-1901, Jan 2019

#### RAN1 #96, Feb 2019

Agreement:

- For a given cell, UE may assume SS/PBCH blocks in the same candidate position within the DRS transmission window are QCL across DRS transmission windows

  - Alt1: The PBCH DMRS sequence index is also the same

  - Alt2: The PBCH DMRS sequence index may be different

  - Note: The first candidate position of the DRS transmission window is located at the first half slot of a half frame

- FFS: QCL assumption for SSBs in different candidate positions within a DRS transmission window and across DRS transmission windows

Agreement:

- At least the functionalities of Rel-13 LTE-LAA RSSI and channel occupancy reporting as a baseline should be supported

- FFS:

  - Enhanced RSSI metrics, for e.g., sub-band-level interference measurements in a wideband operation scenario

  - Reporting of a new medium contention/load metric other than channel occupancy

  - Any modification of the parameters of the Rel-15 SMTC for operation in unlicensed spectrum

Agreement:

- An RLM measurement window for serving cell RLM measurements based on SSBs in the DRS is supported for in-sync and out-of-sync evaluations.

  - FFS: How RLM measurement window is indicated or determined and relation to DRS transmission window

  - FFS: Whether or not an SSB can fall outside the measurement window and, if so, whether it can be used for in-sync and out-of-sync evaluations.

  - FFS: Any relationship of RLM measurements based on CSI-RS to the measurement window.

- FFS: Mechanism to handle missing RLM-RS due to LBT failure

#### RAN1 #96bis, April 2019

Agreement:

The maximum DRS transmission window duration is 5 ms.

- The maximum number of candidate SSB positions within a DRS transmission window, Y, is selected as Y = 10 for 15 kHz SCS and Y = 20 for 30 kHz SCS.

  - Note: The number of starting points for DRS transmissions with the 5 ms window that can use a Cat. 2 LBT is to be discussed further as part of channel access discussions.

- FFS: If the DRS transmission window is configurable, and if yes, how to configure and indicate the window, including the range of configurable values.

Agreement:

For a given cell, the UE may assume that the PBCH DMRS sequence index is the same for SS/PBCH blocks that are transmitted at the same candidate positions across DRS transmission windows.

Agreement:

UE determines serving cell timing from the detected SSB candidate position, where the SSB candidate positions within the DRS transmission window are indexed from 0,…,Y-1 (Y = 10 for 15 kHz SCS and Y = 20 for 30 kHz SCS).

#### RAN1 #97, May 2019

Agreement:

The mechanism to determine serving cell timing is as follows:

- The SS/PBCH block position index within a DRS transmission window is detected using a combination of PBCH DMRS sequence index and 1 bit/2 bits for 15 kHz SCS/30 kHz SCS of the 3 available bits in the PBCH payload (not in MIB) originally used in Rel-15 FR2 for MSB SSB index

- 10-bits SFN and half-frame indicator are indicated as in Rel-15

- PBCH payload size is not increased compared to Rel-15

  - FFS: Whether reuse of other available bits in PBCH payload is also required for timing determination

- If the UE is required to perform PBCH decoding of neighbor cell(s) (e.g., in asynchronous deployments), an explicit time allowance for acquisition of SSB index is provided to the UE

Agreement:

LBT category for msg 3 initial transmission is provided to the UE in RAR

Agreement:

Reply to the RAN2 LS informing them of the following:

- RAN1 has made the following agreement which facilitates COT sharing between Msg2 and Msg3:

  - LBT category for msg 3 initial transmission is provided to the UE in RAR

- Multiple msg3 tx opportunities with a single or multiple RARs in the time domain is feasible from a RAN1 perspective but there is no consensus at this time in RAN1 to support this. RAN1 will continue discussions on the support of multiple msg3 tx opportunities.

R1-1907858 	Reply LS on RACH agreements for NR-U	Charter

Final LS approved in R1-1907902 with Observation 2 modified as follows:

Extending the maximum RAR window size was concluded to be beneficial during the study item but there has been no discussion in RAN1 during the work item on multiple msg1 transmissions and extending the RAR window size.

Agreement:

For a serving cell, UE may assume a QCL relation between SS/PBCH blocks which are detected across DRS transmission windows and have the same value of modulo(A, Q), once Q is known to the UE

- FFS: A is the SSB candidate position index and/or PBCH DMRS sequence index

- FFS: How Q is indicated or determined

- FFS: Restriction on the range of Q.

Note: Neighbor cell RRM measurements will be addressed separately

Agreement:

For SSB-based RLM, UE may assume the RLM measurement window to be the same as the DRS transmission window.

- Note: This implies that the SSB-based RLM-RS cannot fall outside the measurement window

- FFS: Whether and how DRS transmission window is configured to the UE

#### RAN1 #98, August 2019

Agreement:

For purposes of SSB QCL derivation, the following values of Q are supported: {1, 2, 4, 8}.

- FFS: Further down-selection of allowed values.

Conclusion:

The maximum number of PBCH DMRS sequences used in a cell is unchanged from Rel-15 (=8).

- The number of PBCH DMRS sequences used in a cell is independent of Q.

Agreement:

The 3 LSB bits of the SSB candidate position index are represented by the PBCH DMRS sequence index.

Agreement:

For a cell (either serving or a neighbour cell), UE may assume a QCL relation between SS/PBCH blocks within or across DRS transmission or measurement windows that have the same value of modulo(A, Q), once Q is known to the UE

- A is the PBCH DMRS sequence index.

- Note: This agreement extends a prior agreement for serving cells on QCL relation between SS/PBCH blocks to neighbour cells

Agreement:

The following bits of the PBCH payload (not MIB) are used for serving cell timing determination within a frame in addition to the PBCH DMRS sequence index as agreed earlier:

- aA+7aA+6 for 15 kHz SCS, and  aA+6 and aA+7 for 30 kHz SCS

- aA+4 (the half-frame bit as in Rel-15)

Agreement:

If Q is known, candidate monitoring slots for Type0 PDCCH search space are the PDCCH monitoring slots associated with SS/PBCH blocks that are QCL with the SS/PBCH block from which the UE determines that a CORESET for Type0-PDCCH CSS set is present

- Note: Q may be always known depending on where Q is signalled. This aspect is to be discussed further.

#### RAN1 #98bis, October 2019

Agreement:

If 2 bits can be found to not be necessary in the current MIB, then Parameter Q is indicated in the MIB by a serving cell. Otherwise, Q is indicated in the RMSI.

Agreement:

For RRM measurements of a neighbor cell in IDLE, INACTIVE and CONNECTED UE states,

- Support signaling of a common Q value per frequency by broadcast RRC signaling (SIBx) and/or dedicated RRC signaling (measObjectNR) from the serving cell.

- Support signaling from the serving cell of a Q value for a listed neighbour cell.

  - If Q is signaled for a listed cell, it overrides any common Q value per frequency

Agreement:

- No further down selection is made for set of Q values, i.e., Q = {1,2,4,8} (resolves FFS of prior agreement on Q values)

- UE does not expect to be configured with two Type-0 PDCCH monitoring occasions in a slot with Q=1.

- Note: This does not imply a change in Rel-15 behavior for Type-0 PDCCH monitoring in two consecutive slots.

- Note: This agreement does not imply any changes to Rel-15 PDCCH monitoring requirements on BD/CCE limits per slot

- RAN4 requirements for SSB detection during initial cell search should be defined assuming Q = 8. This assumption does not have any other specification impact.

  - Include this agreement in an LS to RAN4 asking them to take this into account. The LS should also provide an overview on the definition of Q.

Agreement:

- The RSSI measurement timing configuration (RMTC) includes the following:

- Time-domain parameters:

- Periodicity (e.g., 40/80/160/320/640 ms)

- Measurement duration in terms of OFDM symbols with a configured reference subcarrier spacing

- Offset of RMTC measurement duration

- Frequency-domain parameters:

- Measurement bandwidth at least in units of LBT bandwidths

  - FFS: Units other than LBT bandwidths

  - Note: RAN4 can determine if the bandwidth used for the measurement within an LBT bandwidth can be less than the signaled measurement bandwidth.

- Measurement ARFCN for inter-frequency measurements

- Configurable L3 filtering as in RSSI for LTE-LAA

Conclusion:

No new medium contention/load metrics other than channel occupancy are introduced

Agreement:

Support configuration of the DRS transmission window duration of a UE’s serving cell(s)

- DRS transmission window periodicity is the same as the configuration of SSB burst periodicity

- Note: RAN4 performance requirements depending on the DRS transmission window size, e.g., RRM, will assume a DRS transmission window size of 5 ms

Agreement:

The ssb-PositionsInBurst IE is configured as in Rel-15 NR

- FFS: Interpretation of the field for NR-U

#### RAN1 #99, November 2019

R1-1913571	[Draft] Response LS to RAN2 LS on SFN LSB indication in msg2/msgB

The LS is approved in R1-1913582 with the following modifications:

- Change “in granting DCI to carry the 2-bit LSB of SFN” to “in DCI format 1_0 scheduling msg2/msgB reception”.

- Fix typo for RACH in the first paragraph

Agreement:

RAN1 recommends the following terminology to be used consistently across RAN1 (and RAN2/RAN4) specifications for operation with shared spectrum channel access:

- “SS/PBCH block index” can be expressed as modulo(PBCH DMRS sequence index, Q) or modulo(Candidate SS/PBCH block index, Q)

  - Note: These two modulo operations yield the same result and which index should be applied depends on the specific scenario

  - Note: This is applicable for cases in the specification where “SS/PBCH block” instead of “SS/PBCH block index”

- “Candidate SS/PBCH block index” within a DRS transmission window was previously agreed in RAN1

- Include this agreement in an LS to RAN2/RAN4

Agreement:

- DRS transmission window duration can be configured as 0.5 1, 2, 3, 4, or 5 ms

- If the DRS transmission window duration is not known, a UE may assume the DRS transmission window has a duration of 5 ms

Conclusion:

CSI-RS-based RLM-RS both within and outside the SSB-based RLM measurement window (i.e., DRS transmission window) can be used for in-sync and out-of-sync evaluations (resolves FFS from previous agreement).

Agreement:

Units other than a single LBT bandwidth are not supported for RSSI measurement bandwidth configuration

Conclusion:

The Rel-15 SMTC window can be used without any modifications for configuring RRM measurement windows

Agreement:

For RSSI measurements:

- Measurement ARFCN is also provided for intra-frequency measurements (provision of ARFCN for inter-frequency measurements already agreed earlier)

- The measured bandwidth is centered on the provided ARFCN (for both intra- and inter-frequency).

Agreement:

- If 2 bits are available for DCI format 1_0 for 2-step RACH, include 2 bits of SFN LSB information using the reserved bits in DCI format 1_0 for both 2-step and 4-step RACH for operation in unlicensed spectrum

Agreement:

A PRACH resource is considered invalid if:

- it overlaps with the IDLE period of a fixed frame period when FBE operation is indicated

Agreement:

The interpretation of ssb-PositionsInBurst in SIB1 or ServingCellConfigCommon is as follows:

- A bit set to 1 at position k (indexing starts at 1) indicates SS/PBCH block index k-1.

- The UE expects that a bit at position k > Q is set to 0

Agreement:

From a UE’s perspective, the number of transmitted SSBs within a DRS transmission window is not larger than Q

Agreement:

The Q value signalled in system information for a serving cell is also used for RRM measurements on the serving cell.

Conclusion:

There is no consensus in RAN1 to introduce a mechanism to handle missing RLM-RS

Agreement:

For type-0 PDCCH monitoring, a UE does not expect to be configured with M=2 when Q is configured to be 1.

Agreement:

A serving cell can indicate Q via dedicated RRC signaling (e.g., for SCell or SCG addition).

Agreement:

For signaling of Q for a serving cell with possible values {1,2,4,8}, the following is supported:

- If RAN2 agrees to use the spare bit and still allow release independent introduction of the 6 GHz band, then Alt 1-4 is supported, otherwise Alt 1-2 is supported:

  - Alt 1-2: For operation with shared spectrum channel access, the UE interprets the following 2 bits of the Rel-15 MIB for providing the value of Q

    - ssbSubcarrierSpacingCommon (1 bit)

    - LSB of ssb-SubcarrierOffset (1 bit)

  - Alt 1-4: For operation with shared spectrum channel access, the UE interprets the 2 bits in the following two fields of the Rel-15 MIB for providing the value of Q

    - ssbSubcarrierSpacingCommon (1 bit)

    - spare (1 bit)

R1-1913590	Draft LS on signaling of Q for a serving cell in NR-U		Nokia, Nokia Shanghai Bell

Final LS is approved in R1-1913592

### 7.2.2.2.3	HARQ enhancement

#### RAN1 #AH-1901, Jan 2019

Agreement:

- RRC parameter dl-DataToUL-ACK supports a value that can be signaled by PDSCH-to-HARQ-timing-indicator, which indicates that the UE needs to store the HARQ A/N feedback result for the corresponding PDSCH, and which does not provide any timing for the transmission of this HARQ A/N feedback result

Conclusion:

- No additional value of K1 signaled by PDSCH-to-HARQ-timing-indicator needs to be introduced for the purpose of requesting feedback outside the COT (i.e. Alt3 in TR38.889 section 7.2.1.3.3)

  - Further discuss the details for determining the LBT type for the UE transmission of HARQ A/N feedback for this case

  - Further discuss whether additional values need to be introduced for the purpose of requesting feedback inside the COT

    - If additional values are introduced these can also be used for requesting feedback outside the COT

  - Note: This does not necessitate that the UE needs to know whether the feedback is inside or outside the COT at the time when the feedback is requested

Agreement:

For enabling multiple opportunities for HARQ A/N transmission and for cross-COT HARQ-ACK feedback, at least the following is supported:

gNB requests/triggers feedback for PDSCH from earlier COT(s) or additional reporting of earlier HARQ feedback, where the exact HARQ feedback timing and resource is provided to the UE in another DCI (in the same or in another COT)

Agreement:

Cross-carrier HARQ re-transmissions will not be discussed for NR-U in Rel-16.

#### RAN1 #96, Feb 2019

Agreement:

Scheduling PUSCH over multiple slots/mini-slots by single DCI supports at least multiple continuous PUSCHs with separate TBs

- Each TB is mapped to at most one slot or one mini-slot

#### RAN1 #96bis, April 2019

Agreement:

A non-numerical value is added to the possible range of PDSCH-to-HARQ-timing-indicator values defined in Rel-15, and is used to indicate to the UE that the HARQ-ACK feedback for the corresponding PDSCH is postponed until the timing and resource for the HARQ-ACK feedback is provided by the gNB.

Agreement:

Restrict further discussion on HARQ codebook to the following:

- For dynamic HARQ codebook:

  - PDSCH grouping by explicitly signalling a group index in DCI scheduling the PDSCH

  - gNB can request HARQ-ACK feedback in the same PUCCH for all PDSCHs in the same group

  - Option 1:

    - One PUCCH can carry HARQ-ACK feedback for one or more PDSCH groups

    - DCI can request HARQ-ACK feedback for one or more PDSCH groups

    - FFS one of the two options below

      - C-DAI/T-DAI can be accumulated across multiple PDSCH groups for which feedback is requested in the same PUCCH

      - C-DAI/T-DAI is accumulated only within one PDSCH group

    - FFS: New ACK-Feedback Group Indicator for each PDSCH Group

    - The number of HARQ-ACK bits for one PDSCH group is constant between a first HARQ-ACK feedback transmission and a HARQ-ACK feedback re-transmission, i.e. the PDSCH group cannot be enlarged after the first feedback transmission

  - Option 2:

    - One PUCCH can carry HARQ-ACK feedback for a single PDSCH group

      - FFS: Feedback for more than one PDSCH group

    - DCI can request HARQ-ACK feedback for a single PDSCH group

      - FFS: Requests for more than one PDSCH group

    - C-DAI/T-DAI is accumulated within one PDSCH group

    - A reset indicator signals new HARQ-ACK feedback for a PDSCH group

    - The number of HARQ-ACK bits for one PDSCH group may not be constant between a first HARQ-ACK feedback transmission and a HARQ-ACK feedback re-transmission

- Semi-static codebook. Options FFS.

- If request/trigger for one-shot group HARQ ACK feedback for all configured HARQ processes is introduced (at least for non-CBG HARQ), select one or more of the following candidate schemes:

  - The request is carried in a UE-specific DCI carrying a PUSCH grant

  - The request is carried in a UE-specific DCI carrying a PDSCH assignment

  - The request is carried in a UE-specific DCI not scheduling PDSCH nor PUSCH

  - The request is carried in a UE-common DCI

  - The request can be used for UE configured with dynamic or semi-static HARQ codebook

- Note: The discussion on preconfigured/pre-indicated multiple opportunities in frequency domain in different LBT subbands is a separate discussion

#### RAN1 #97, May 2019

Agreement:

For multiple PUSCHs scheduled by a single DCI

- The following fields in the DCI are signalled per PUSCH

  - NDI

  - RV (FFS: compression scheme for RV)

- FFS if CBG-based re-transmission is supported for multi-TTI scheduling, at least the following options are considered for signalling the CBGTI field in the DCI

  - Option 1: per re-transmitted PUSCH

    - FFS: limitations on number of re-transmitted PUSCH for which CBGTI field is signalled

  - Option 2: per PUSCH

  - Option 3: only for a fixed number of PUSCHs

- HARQ process ID signalled in the DCI applies to the first scheduled PUSCH. HARQ process ID is then incremented by 1 for subsequent PUSCHs in the scheduled order (with modulo operation as needed)

- Time domain resource assignment mechanism is enhanced for indicating the number of scheduled mapping Type A and Type B PUSCHs, and their starting and ending symbols

  - At least continuous time domain resource assignment is supported

  - FFS: whether multiple mapping Type B PUSCHs is allowed within the first slot

  - FFS: whether multiple starting positions is allowed for UE-initiated COT (discussed as part of agenda 7.2.2.1.3)

  - FFS: configuration/signalling details

- CSI request field in the DCI applies to a single PUSCH

  - FFS: relation between the timing of the triggered CSI-RS and the PUSCH carrying the CSI feedback, and how to determine which PUSCH carries the CSI feedback

- FFS: Whether existing DCI formats can be extended or new formats are necessary and associated details for supporting scheduling multiple PUSCHs

- FFS: Potential impact on MCS signalling for re-transmission

- Note: Indication of the LBT type and priority class should be possible with the DCI

- Note: SRS request field in the DCI applies to a single slot with the applicable slot signalled as in Rel-15

- Note: The number of DAI fields is not increased as compared to the single PUSCH scheduling DCI in Rel-16. Applicability of the DAI field is the same as multi-slot PUSCH scheduling in Rel-15.

Agreement:

For operation with dynamic HARQ codebook (type-2 codebook):

- PDSCH grouping by explicitly signalling a group index in DCI scheduling the PDSCH

  - For any PDSCH scheduled with numerical or non-numerical value of K1

- The number of HARQ-ACK bits for one PDSCH group can change between successive requests for HARQ-ACK feedback for the same PDSCH group

- HARQ-ACK feedback for all PDSCHs in the same group is carried in the same PUCCH

- One DCI can request HARQ-ACK feedback for one or more PDSCH groups in the same PUCCH

- C-DAI/T-DAI is accumulated only within each PDSCH group

  - FFS: Choose between the following options:

    - T-DAI is included only for the scheduled group

    - T-DAI is included for each group

- New ACK-Feedback Group Indicator for each PDSCH Group operates as a toggle bit

- Maximum number of PDSCH groups: 2 (FFS: maximum number of groups 4 and maximum number of groups for which feedback is requested in the same PUCCH)

- A UE can signal support of this feature as part of capability signaling

#### RAN1 #98, August 2019

Conclusion:

In Rel-16, the PDSCH-to-HARQ-timing-indicator field is not enhanced to indicate larger values of K1 compared to Rel-15.

Agreement:

When configured for operation with enhanced dynamic HARQ codebook (type-2 codebook), a UE shall support 2 PDSCH groups.

- The number of PDSCH groups that the UE shall support is not RRC configurable

- FFS: More groups, depending on the resolution of the FFS on up to 4 groups from a prior agreement

Agreement:

- For enhanced dynamic HARQ codebook, choose one of the alternatives below:

  - Alt-1: NFI and T-DAI are indicated in non-fallback DCI scheduling PDSCH only for the scheduled group

  - Alt-2: NFI and T-DAI are indicated in non-fallback DCI scheduling PDSCH for both groups

- FFS: Extension to fallback DCI

Agreement:

For enhanced dynamic HARQ-ACK codebook when the feedback is triggered by a DL DCI scheduling a PDSCH, a UE is not expected to be requested in that DL DCI to provide feedback only for a PDSCH group not scheduled in that DL DCI

Agreement:

For enhanced dynamic codebook operation, HARQ-ACK timing for a PDSCH scheduled with a non-numerical value for K1 is derived by the next DL DCI scheduling PDSCH with a numerical value for K1 triggering feedback for PDSCH group(s) including the PDSCH group of this PDSCH.

- FFS: additional methods including

  - based on an earlier DCI

  - one-shot HARQ-ACK feedback

- FFS: dependency on NFI

Conclusion:

There is no consensus to support a 2-stage uplink grant for NR-U in Rel-16.

Conclusion:

For multi-TTI scheduling and for single TTI scheduling for PUSCH, the UE shall only attempt transmitting a PUSCH at the single starting position indicated in the UL grant in PDCCH for this PUSCH.

- FFS: for msg3

#### RAN1 #98bis, October 2019

Agreement:

- For enhanced dynamic HARQ codebook with single or multiple configured DL cells,

  - NFI is indicated in non-fallback DCI scheduling PDSCH for the scheduled group

  - T-DAI is indicated in non-fallback DCI scheduling PDSCH for the scheduled group if more than one DL cell is configured

- Indication of NFI and T-DAI in non-fallback DCI for the non-scheduled group with single or multiple configured DL cells can be configured by RRC.

Agreement:

If the DCI is received that toggles the NFI for a PDSCH group, then the UE

- Discards the HARQ-ACK feedback for the PDSCH(s) in that PDSCH group including the PDSCH(s) that were associated with a non-numerical K1 value.

- Note: HARQ-ACK feedback for PDSCH(s) scheduled in this DCI are not discarded.

- The UE expects the DAI values for the PDSCH group to be reset.

Agreement:

If enhanced dynamic codebook is configured, for a PDSCH scheduled by DL DCI 1_0:

- UE reports HARQ-ACK feedback for the corresponding PDSCH as part of the PDSCH group #0

  - FFS: whether/how to define a rule for UE to determine NFI if NFI is not explicitly signalled

- The fallback DCI does not support signalling a non-numerical value of K1

- FFS: Whether NFI is introduced in DCI 1_0 for the purpose of enhanced dynamic HARQ codebook operation

Agreement:

Support requesting feedback of a HARQ-ACK codebook containing all DL HARQ processes (one-shot feedback) for all CCs configured for a UE in the PUCCH group.

- One-shot feedback should be configurable separately from the configuration of semi-static(including any potential enhancements)/non-enhanced dynamic HARQ codebook

  - FFS: Simultaneous configuration of enhanced dynamic codebook and one-shot feedback

Agreement:

- The last PDSCH for which A/N decoding result is reported in the HARQ-ACK codebook containing all DL HARQ processes (one-shot feedback) is determined as the last PDSCH within the UE N1 processing time capability

  - UE also reports HARQ-ACK feedback for earlier PDSCHs scheduled with non-numerical K1

  - For NDI, choose one of the following alternatives:

    - Alt. A: NDI value is not reported along with HARQ-ACK for the corresponding PDSCH

    - Alt. B: Latest NDI value is reported along with HARQ-ACK for the corresponding PDSCH

      - UE assumes NDI=0 if there is no prior NDI value for the HARQ process

    - Alt. C: UE reports XOR of latest NDI value and HARQ-ACK for the corresponding PDSCH

      - UE assumes NDI=0 if there is no prior NDI value for the HARQ process

- For any HARQ ID that is scheduled after the last determined PDSCH for which A/N decoding result is reported, choose one of the following alternatives (applicability dependent on decision on alternatives for the latest NDI value):

  - Alt1: the HARQ A/N result of the previous PDSCH with that HARQ ID is reported, even if it had already been reported

  - Alt2: The HARQ A/N value is set to the default value of NACK

- FFS: applicability with CBG-based HARQ

Agreement:

If a UE is configured to monitor feedback request of a HARQ-ACK codebook containing all DL HARQ processes (one-shot feedback):

- The feedback can only be requested in a UE-specific DCI

- The feedback can be requested for reporting in PUCCH (As per Rel-15 behaviour, the feedback can be piggybacked on PUSCH)

Agreement:

If enhanced dynamic codebook is configured, the presence of UL DAI for an additional PDSCH group in the non-fallback DCI can be configured by RRC signalling.

- FFS: If there is UL DAI configured to be present in the non-fallback DCI for only one group, to which group this UL DAI applies

Agreement:

For signaling the number of scheduled PUSCHs and TDRA in one DCI scheduling multiple PUSCHs, the TDRA table is extended such that each row indicates multiple PUSCHs (continuous in time-domain)

- Alt. 1: Each PUSCH has a separate SLIV and mapping type. The number of scheduled PUSCHs is signalled by the number of indicated valid SLIVs in the row of the TDRA table signalled in DCI.

  - FFS: Separate k2

- Alt. 2:

  - A separate SLIV for

    - the starting slot,

    - the ending slot

  - The slots in between have a SLIV of S=0, L=14

  - A separate mapping type for

    - the starting slot,

    - the ending slot,

    - the slots in between (a single mapping type for all of them)

  - The total number of PUSCHs for the row.

- FFS: Extension to non-contiguous PUSCHs

Agreement:

Scheduling multiple PUSCHs is not supported in UL fallback DCI

Agreement:

Consider the following alternatives for signalling the MCS index for PUSCHs scheduled by a single DCI.

- Alt. 1: A single MCS index is signalled for all the PUSCHs scheduled by a single DCI

  - UE applies the same behaviour for the PUSCHs scheduled for initial transmission or re-transmission (Rel-15 behaviour):

    - If MCS row 0-27 is indicated: the UE determines the TB size based on the explicitly indicated MCS

    - If MCS row 28-31 is indicated: the UE refers to the DCI scheduling the initial PUSCH to determine the TB size

- Alt. 2: A single normal MCS (row 0-27) is indicated in the scheduling DCI for uplink multi-TTI scheduling

  - For a PUSCH scheduled for initial transmission: the UE determines the TB size based on the explicitly indicated MCS

  - For a PUSCH scheduled for re-transmission: the UE refers to the DCI scheduling the initial PUSCH to determine the TB size

  - FFS: The UE does not expect to receive signalling of an implicit MCS row 28-31 in a DCI scheduling multiple PUSCHs

- Alt. 3: Use a bitmap to determine for which of the PUSCHs the TBS determination should not be based on MCS in the multi-TTI DCI, and should instead be based on the previous DCI scheduling the same TB for the same HARQ process ID.

  - The bitmap is included in the UL DCI only if the RRC-configured TDRA table includes at least one row that allows indicating more than one PUSCH

#### RAN1 #99, November 2019

Agreement:

For enhanced dynamic HARQ-ACK codebook, when more than one PDSCH group exists in a HARQ-ACK feedback report, the placement of HARQ-ACK feedback for the two groups is ordered based on increasing group index.

- FFS: Further enhancement for the case when only T-DAI is configured only for one group.

Agreement:

For enhanced dynamic HARQ-ACK codebook, the number of requested groups is indicated in DCI 1_1 by one bit, indicating “0” for only the scheduled group, and “1” for both groups.

Agreement:

C-DAI and T-DAI are accumulated within a PDSCH group until NFI for the PDSCH group is toggled. The C-DAI and T-DAI are reset when the NFI is toggled. C-DAI and T-DAI are not reset by PUCCH transmission occasions.

Agreement:

If enhanced dynamic codebook is configured, for a PDSCH scheduled by DL DCI 1_0:

- NFI for group #0 is not signalled in DCI 1_0

- If the UE detects a DCI that indicates an NFI corresponding to group #0 since the last scheduled PUCCH that includes feedback for group #0 and before the PUCCH occasion that includes feedback corresponding to PDSCH scheduled with the DCI 1_0

  - UE follows the indicated NFI (in DCI 1_1) to determine the codebook.

- If the UE does not detect a DCI that indicates an NFI corresponding to group #0 since the last scheduled PUCCH that includes feedback for group #0 and before the PUCCH occasion that includes feedback corresponding to PDSCH scheduled with the DCI 1_0

  - UE assumes that the PDSCHs scheduled by DCI 1_0 since the latest PUCCH occasion do not belong to any group, and the UE only reports HARQ-ACK feedback for the PDSCHs scheduled by DCI 1_0 since the latest PUCCH occasion (as in Rel-15)

  - If the C-DAI received in the first DCI format 1_0 received after the latest PUCCH occasion is not equal to 1, and if the UE didn’t detect a DCI format 1_1 since the latest PUCCH occasion, the UE assumes that missed DCIs are DCI formats 1_0

Agreement:

If UL DAI is configured to be present in the non-fallback DCI for two groups, these UL DAI fields apply separately to each group

Agreement:

- If UL DAI is configured to be present in the non-fallback DCI for only one group, then for enhanced dynamic HARQ-ACK codebook feedback in PUSCH:

  - If feedback is requested for just one group, then this UL DAI applies to the reported group

  - Working assumption: If feedback is requested for both groups, the UL DAI applies to group #0 and the UE derives the DAI for group #1 by the latest DCI scheduling PDSCH reception for group #1 for this HARQ feedback occasion

Agreement:

For non-enhanced dynamic HARQ-ACK codebook, the HARQ-ACK timing for PDSCH scheduled with non-numerical value for K1 is derived by the next DCI scheduling PDSCH with a numerical K1 value.

- Note: The C-DAI should account for the PDSCHs allocated with a non-numerical K1 value

Conclusion:

Further enhancements and optimizations for the case where one-shot HARQ ACK feedback and enhanced dynamic HARQ codebook are configured together are not considered.

Agreement:

For one-shot HARQ feedback:

- NDI can be configured to be part of one-shot HARQ feedback.

  - When NDI is configured

    - The latest NDI value detected by the UE is reported along with HARQ-ACK for the corresponding HARQ process ID. The UE assumes NDI=0 if there is no prior NDI value for the HARQ process

    - NDI is included for each TB

  - When NDI is not configured:

    - NDI value is not reported along with HARQ-ACK for the corresponding PDSCH

    - UE is expected to reset HARQ-ACK state (as DTX or NACK) for a HARQ process ID once ACK is reported for the same HARQ process ID in the previous feedback

- CBG-based HARQ-ACK or TB-based HARQ-ACK can be configured to be part of the one-shot HARQ feedback for the CCs configured with CBG.

- Note: For any HARQ ID that is scheduled after the last determined PDSCH for which A/N decoding result is reported, the UE does not consider this PDSCH for one-shot HARQ codebook composition

Agreement:

For scheduling multiple PUSCHs by a single DCI Format 0_1:

- The same DCI Format 0_1 can schedule a single PUSCH or multiple PUSCHs

- Maximum number of PUSCHs that can be configured in a row of the TDRA table: 8

- The number of NDI bits and RV bits in DCI format 0_1 is determined based on the configured TDRA table

  - 1 RV bit per PUSCH in case multiple PUSCHs are scheduled

  - 2 RV bits for the PUSCH in case only a single PUSCH is scheduled

Agreement:

When a DCI schedules M PUSCHs:

- The PUSCH that carries the aperiodic CSI feedback is:

  - When M <= 2: the (M)-th scheduled PUSCH

  - When M > 2: the (M-1)-th scheduled PUSCH

- The timing relation between A-CSI-RS and UL grant is same as Rel-15 NR.

Agreement:

When UL DCI 0_1 schedules more than one PUSCH:

- A single MCS is indicated

  - For MCS row 0-27/28

    - For a PUSCH scheduled for initial transmission: the UE determines the TB size based on the explicitly indicated MCS

    - For a PUSCH scheduled for re-transmission, the UE determines the TB size based on the explicitly indicated MCS

  - For MCS row 28/29-31

    - The UE refers to the DCI scheduling the initial PUSCH to determine the TB size

  - Note: the modulation order is determined based on the MCS field

Agreement:

The payload size of DCI format 0_1 is determined based on the largest between the DCI size scheduling one PUSCH and the DCI size scheduling multiple PUSCHs, based on the configured TDRA table.

- When UL DCI 0_1 schedules more than one PUSCH:

  - UL-SCH indicator field is not present

  - CBGTI field is not present

- When UL DCI 0_1 schedules a single PUSCH:

  - UL-SCH indicator field is present

  - CBGTI field is present

Agreement:

- The TDRA table configuration allows indicating single or multiple continuous PUSCHs in any slot of the multiple scheduled slots

- For signaling the number of scheduled PUSCHs and TDRA in one DCI format 0_1 scheduling multiple PUSCHs, the TDRA table is extended such that each row indicates multiple PUSCHs (continuous in time-domain)

  - Each PUSCH has a separate SLIV and mapping type. The number of scheduled PUSCHs is signalled by the number of indicated valid SLIVs in the row of the TDRA table signalled in DCI.

- Note: For the fallback DCI, Rel-15 TDRA table is used

Agreement:

The single bit for RV values chooses from RV {0, [2 or 3]}

Agreement:

If a UE is configured to monitor feedback request for one-shot HARQ-ACK codebook feedback:

- The feedback can be requested in DL DCI 1_1 with the introduction of 1 bit (present if one-shot HARQ-ACK feedback is configured to the UE) for triggering the feedback

  - UE determines the PUCCH for one-shot HARQ-ACK feedback from K1, PRI, TPC

    - FFS: Use of NFI for determining PUCCH

    - Note: as per Rel-15 behaviour, the feedback can be piggybacked on PUSCH

Agreement:

If a UE is configured to monitor feedback request for one-shot HARQ-ACK codebook feedback and the feedback is requested in DL DCI 1_1

- This DL DCI can either schedule or not schedule a PDSCH

  - Working assumption: One value of the frequency domain resource assignment field indicates that this DCI does not schedule a PDSCH

  - If the DL DCI does not schedule a PDSCH, the HARQ process ID and NDI fields are ignored by the UE

- If UE is triggered to report both one-shot and other HARQ-ACK feedback in the same slot, the UE reports only the one-shot feedback.

Agreement:

- In the one-shot codebook, the NDI follows the HARQ-ACK information for each TB

- In the one-shot codebook, the ordering of information for HARQ-ACK and NDI is as follows:

- CBG index

- TB index

- HARQ process ID

- Serving cell index

### 7.2.2.2.4	Configured grant enhancement

#### RAN1 #AH-1901, Jan 2019

Agreement:

For configured grant resource configuration in time domain, the following alternatives are to be studied with more detailed proposal and analysis, strive to down-select in RAN1#96:

- Alt. 1: Bitmap based approach as baseline with potential enhancement

  - Companies are encouraged to provide detailed design in next meeting

- Alt. 2: NR Rel-15 based time domain resource allocation approach as baseline with potential enhancement

  - Companies are encouraged to provide detailed design in next meeting

Agreement:

- Support multiple UE starting time offsets with sub-symbol granularity with FeLAA AUL approach as the baseline

  - FFS: Enhancements specific to NRU

- Companies are encouraged to provide views and analysis on the following issues:

  - Whether to support allowing the UE to start transmission later than the starting symbol as indicated in configured grant based on LBT outcome

    - If yes, multiple starting positions within a slot for a configured grant configuration;

      - Alt. 1: subset of symbols

      - Alt. 2: any symbol

      - FFS: gNB knowledge of starting symbol, whether UE indicates to gNB

      - FFS signaling details

      - FFS: whether similar design for scheduled grant and configured grant

  - Whether the ending symbol can be punctured

  - Whether the position of the ending symbol can be shifted depending on the starting position due to LBT procedures

Conclusion:

The following aspects should be discussed further as part of the channel access discussions

- Contention window adjustment

- Details of COT sharing related to NRU configured grant including details and limitations on UE-initiated COT sharing with gNB and configured grant UL transmissions within gNB acquired COT

Agreement:

CG-UCI should at least include the following information:

- HARQ ID

- NDI

- RV

- COT sharing information, FFS details

- FFS: other information including UE ID

#### RAN1 #96, Feb 2019

Agreement:

For PUSCH transmitted using CG, CBG-based retransmission is supported at least by using dedicated scheduled resource allocated by an UL grant.

- FFS: CBG-based retransmission using a configured grant

- Note: Include this agreement in an LS to RAN2 informing them of relevant RAN1 agreements

Agreement:

For initial transmission on configured grant resource, HARQ retransmission on configured grant resource upon configured grant timer expiration (assume NACK if no ACK is received) is supported

- Note: Include this agreement in an LS to RAN2 informing them of relevant RAN1 agreements

Agreement:

When a UE initiates a channel occupancy with a transmission using a configured grant, it can signal at least the following

- The duration that the gNB is allowed to transmit in the channel occupancy initiated by the UE

- FFS:

  - How the duration is signalled

  - Whether the UE should signal continued use of the COT for its own transmissions

  - LBT priority class

#### RAN1 #96bis, April 2019

Agreement:

Select from the following additional options for type 1 and type 2 configured grant time domain resource allocation mechanism in NR by RAN1#97

- Option 1: A bitmap to selectively enable or disable configured UL transmission opportunities as per NR Rel-15 configurations.

  - FFS: A bit in the bitmap can correspond to a slot or sub-slot or group of slots

  - FFS: duration of bitmap in time, e.g. 40ms

- Option 2: A mechanism based on multiple NR Rel-15 configurations

  - FFS: Whether any further enhancement is needed to Rel-16 beyond what is being considered in the URLLC WI

- Option 3: Configuration in addition to the Rel-15 baseline of one or more of the following aspects:

  - Multiple offsets within an active configuration

  - Duration of transmission for an offset

- Option 4: A bitmap to configure UL transmission opportunies to replace current time domain resource configuration

  - FFS: A bit in the bitmap can correspond to a slot or sub-slot or group of slots

  - FFS: duration of bitmap in time, e.g. 40ms

  - Note: This is importing LAA AUL functionality into NR

#### RAN1 #97, May 2019

Agreement:

For configured grant time domain resource allocation, the mechanisms in Rel-15 (both Type 1 and Type 2) are extended so that the number of allocated slots following the time instance corresponding to the indicated offset can be configured

- FFS: How to indicate multiple PUSCHs within a slot.

Agreement:

UE can only start transmissions accessing transmission opportunities provided by a configured grant at the configured/indicated starting position.

#### RAN1 #98, August 2019

Agreement:

- For DFI design for configured grants, support at least the following

- DFI including at least TB level HARQ-ACK bitmap for all UL HARQ processes

- Note: Total number of HARQ processes is as defined in Rel-15

- FFS: CBG level HARQ-ACK feedback, if supported

- RRC configured minimum duration, D, from the ending symbol of the PUSCH to the starting symbol of the DFI carrying HARQ-ACK for that PUSCH

  - Note: UE assumes HARQ-ACK is valid only for PUSCH transmissions ending before n-D, where n is the time corresponding to the beginning of the start symbol of the DFI.

  - FFS: the definition of minimum duration for the case of slot aggregation

- UE blind decoding complexity shall not be increased due to DFI size

Agreement:

The number of separately encoded UCIs multiplexed in a PUSCH transmitted using a configured grant is not changed from Rel-15 (maximum of 3).

Working assumption:

CG-UCI is included in every CG-PUSCH transmission

#### RAN1 #98bis, October 2019

Agreement:

The starting time offset applied by a UE at the beginning of a transmitted burst with a CG resource at the start of the transmission burst, is RRC configured and defined as the length of a CP extension of the first symbol that is located before the configured resource

- Regardless of SCS, the CP extension is up to 72 micro seconds with a granularity of 9 micro seconds

Agreement:

CG-UCI is mapped as per Rel-15 rules with CG-UCI having the highest priority (CG-UCI is mapped on the symbols starting after first DMRS symbol)

Agreement:

To determine the number of REs used for CG-UCI, the mechanism of beta-offset in Rel-15 NR for HARQ-ACK on CG-PUSCH is reused.

- A new RRC parameter to configure the beta-offset for CG-UCI is defined. FFS: Value range

Agreement:

CG-UCI is included in every CG-PUSCH transmission (confirms working assumption from RAN1#98)

Agreement:

- CG-UCI, CSI-part1, CSI-part 2 can be sent on CG-PUSCH at least when CG-UCI and HARQ-ACK feedback is not multiplexed on a CG-PUSCH

#### RAN1 #99, November 2019

R1-1913531	Draft reply to LS on PHR reporting for NR-U	Lenovo

Final LS approved in R1-1913584

Agreement:

- DFI is transmitted using PDCCH scrambled with CS-RNTI

- DFI size is similar to UL DCI size

- Size is aligned with UL grant DCI format 0_1 size

- To distinguish DCI for activation/deactivation CG transmission and DFI, a 1 bit flag (explicit indication) is used, when type 1 and/or type 2 CG PUSCH is configured

- Content of DFI includes

- UL/DL flag

- CIF in the case of cross carrier scheduled is configured

- 1 bit flag

- HARQ Bitmap

- TPC command – 2 bits

- For type 1 configured grant, UE assumes DFI is only present when CG is configured.

- For type 2 configured grant, UE assumes DFI is only present when CG is configured, and UE is in activated state for configured grant transmissions

Agreement:

For UE initiated COT sharing indication

- When ED threshold is configured

  - The CAPC value is indicated in CG-UCI.

Agreement:

RRC configuration can be provided to the UE indicating whether to multiplex CG-UCI and HARQ-ACK

- When configured for such multiplexing: In the case of PUCCH overlapping with CG-PUSCH(s) within a PUCCH group, the CG-UCI and HARQ-ACK are jointly encoded (CG-UCI is treated as the same type as a HARQ-ACK)

- When not configured for such multiplexing: In the case of PUCCH overlapping with CG-PUSCH(s) within a PUCCH group and PUCCH carries HARQ ACK feedback, configured grant PUSCH is skipped

Agreement:

For configured PUSCHs within a single configuration, length of all PUSCHs is the same.

- Note: This may not have a specification impact

Agreement:

- The time domain resource assignment in configured grant repeats over the multiple slots within the CG-allocated slots. The same symbol allocation and mapping type is used for the first PUSCH in every slot of the allocated CG-slots.

- The SLIV indicates the information about the first PUSCH in a slot

- A parameter N indicates how many consecutive PUSCHs are allocated within a slot

  - Note: If the parameter has already been introduced in another WI, it can be reused

Agreement:

When the UE is configured with repK > 1, repetition of a TB is mapped within a configuration in the case when an UE is configured with multiple active configurations

- The UE repeats the TB in the earliest consecutive transmission occasion candidates within the same configuration instead of consecutive slots

  - The UE may drop repetition transmissions that fall into a subsequent configured period.

- The UE terminates the repetitions if an explicit feedback indicating ACK in the DFI is received for the HARQ process.

Agreement:

When ED threshold is configured, UCI indication for purpose of COT sharing:

  - UE provides a row index in a RRC configured table where D, O and CAPC are jointly encoded.

    - D: number of slots where DL transmissions can be assumed within UE initiated COT.

    - O: DL offset indicates starting slot of DL transmission indicated in number of slots from the end of the slot where the indicated D>0.

    - one row indicates no COT sharing information.

When ED threshold is not configured,

- COT sharing indication: 1 bit indicating if slot/symbol  n+X is an applicable slot for UL to DL sharing). X is configured by the gNB as part of RRC configuration.

  - X is number of symbols from the end of the slot where the indication is enabled

Agreement:

Definition of the minimum duration, D, in the case of slot aggregation/CG repetition

- For the case of CG repetitions, the minimum duration D is calculated from the ending symbol of each PUSCH of the K repetitions to the starting symbol of the DFI carrying HARQ-ACK for the associated HARQ process ID

- For the case of slot aggregation for scheduled UL transmission, the minimum duration, D, refers to the time between ending symbol of the PUSCH in the first slot of aggregated slots to the starting symbol of the DFI carrying HARQ-ACK for the associated HARQ process ID

  - For ACK, the minimum duration, D, from the ending symbol of the first PUSCH of the TB repetitions to the starting symbol of the DFI

  - For NACK, the minimum duration, D, from the ending symbol of the last available PUSCH of the TB repetitions to the starting symbol of the DFI

### 7.2.2.2.5	Wide-band operation

#### RAN1 #AH-1901, Jan 2019

Agreement:

- For wideband operation in DL with a single serving cell operation within a carrier with bandwidth larger than 20 MHz

  - Multiple BWPs can be configured, single BWP activated, gNB may transmit PDSCH on parts or whole of single active BWP where CCA is successful at gNB (i.e., option 2 and 3 from previous agreement)

    - FFS: Restrictions on supportable gaps and combinations of gaps between discontiguous blocks where

      - each block spans contiguous (one or) multiple successful LBT sub-bands

      - each gap spans one or multiple contiguous unsuccessful LBT sub-bands

    - FFS: Transmission bandwidth adaptation delay, potentially different delay for e.g., different number of supported gaps, different transmission bandwidths and different positions of the LBT sub-bands where transmissions occur

    - FFS: Limit on the occupied LBT sub-bands due to regulation and coexistence considerations (not intended to imply that regulation and coexistence considerations will not be addressed)

    - FFS: Whether/how to indicate gNB’s transmitted LBT sub-bands

    - FFS: Enhancements to PDCCH/PDSCH configuration/transmission for the parts of BWP where gNB does not transmit due to CCA failure

- Send LS to RAN4 to inform above decision with the description that RAN1 requires RAN4’s feedback on the first three FFS parts in addition to what was requested in earlier LSs.

Agreement:

Operation with multiple active BWPs for a carrier on unlicensed bands is not supported for DL or UL at least in Rel-16 NR-U WI.

- Inform RAN2 of this decision

R1-1901446	[DRAFT] Reply to reply LS on wideband carrier operation for NR-U	LG Electronics

Final LS is agreed with the following changes in R1-1901460

- Include RAN WG2 in the “To” field

- Add an action to RAN WG2: “RAN1 respectfully asks RAN2 to take the above RAN1 agreements into account in their further work”

- Delete the corresponding statement requesting action from RAN2 above in the main body.

- Modify a statement in the main body as follows: “RAN1 respectfully asks RAN4 for a response to FFS parts in the above agreement, in addition to any pending input to questions as requested in [1] not yet answered.”

Conclusion:

The channel access aspects of wideband operation should be discussed further as part of the channel access discussions

#### RAN1 #96, Feb 2019

#### RAN1 #96bis, April 2019

Agreement:

For UL transmissions in a serving cell with carrier bandwidth greater than LBT bandwidth, for the case where UE performs CCA before UL transmission, support at least Alt. 1 among the following alternatives

- Alt. 1: UE transmits the PUSCH only if CCA is successful at UE in all LBT bandwidths of the scheduled PUSCH.

- Alt. 2: UE transmits the PUSCH in all or a subset of LBT bandwidths of the scheduled PUSCH for which CCA is successful at the UE.

  - Decision on whether this alternative is supported will depend on feedback from RAN4

  - FFS on restrictions to the subset of LBT bandwidths, e.g., only contiguous LBT bandwidths allowed, based on feedback from RAN4

- Necessity of guard bands within the scheduled PUSCH should be determined by RAN4

- FFS: Whether this applies also to configured grant PUSCH

- FFS: Whether this applies also to PUCCH

LS to RAN4 on the above agreement captured in R1-1905892.

Agreement:

- Support a mechanism for a UE to detect gNB is transmitting across

  - Multiple carriers

  - Multiple LBT bandwidths in a carrier.

- The following mechanisms are to be considered:

  - Option 1: Explicit indication via PDCCH

    - FFS: The type of PDCCH (e.g., group common PDCCH or UE-specific PDCCH)

    - FFS: Signaling details of the indication

  - Option 2: Explicit indication via selection of a PDCCH DM-RS sequence from a set of PDCCH DM-RS sequences

    - FFS: Details of the indication

  - Option 3: Via UE implementation, i.e., implicit method based on NR-based signal such as DM-RS and/or corresponding PDCCH detection

    - FFS: Which signals/channels or combination of signals/channels could be used by the UE

  - Note: Above options are not mutually exclusive

#### RAN1 #97, May 2019

Agreement:

When GC-PDCCH is configured, explicit indication via GC-PDCCH is supported as a mechanism to inform the UE that one or more carriers and/or LBT bandwidths are not available or available for DL reception, at least for slot(s) that are not at the beginning of DL transmission burst.

- FFS: Signalling details of the indication, including e.g., the time domain validity of the indication

- FFS: Whether and how to support the mechanism at the beginning of DL transmission burst

- FFS: Whether and how to handle the case when GC-PDCCH is not configured or not received by the UE

Conclusion:

A UE can receive a PDSCH scheduled within an LBT bandwidth or over multiple LBT bandwidths as per Rel-15 and current agreements in Rel-16.

#### RAN1 #98, August 2019

Conclusion:

The following are unchanged from Rel-15 for PDCCH.

- The maximum number of monitored PDCCH candidates per slot and per serving cell.

- The maximum number of non-overlapped CCEs per slot and per serving cell.

- CCE-to-REG mapping rule and hashing function.

Agreement:

For CORESET configuration in a serving cell with carrier bandwidth greater than LBT bandwidth,

- For the case where a CORESET is confined within a LBT bandwidth, the search space set configuration associated with the CORESET can have multiple monitoring locations in the frequency domain (per LBT bandwidth)

  - Send an LS to RAN2 informing them of this agreement and providing clarifications on the above if necessary

- Note: For scenarios in which gNB transmits PDCCH/PDSCH on a single BWP if CCA is successful at gNB for the whole BWP, CORESET(s) need not all be confined within an LBT bandwidth, and no specification impact is foreseen

#### RAN1 #98bis, October 2019

Agreement:

For a search space set configuration associated with multiple monitoring locations in the frequency domain (as per the previous agreement defining such a search space set associated with a CORESET confined within an LBT bandwidth):

- PRBs allocated by frequencyDomainResources in the CORESET configuration are confined within one of LBT bandwidths within the BWP corresponding to the CORESET.

- Within the search space set configuration associated with the CORESET, each of the one or more monitoring locations in the frequency domain corresponds to (and is confined within) an LBT bandwidth and has a frequency domain resource allocation pattern that is replicated from the pattern configured in the CORESET.

- CORESET parameters other than frequency domain resource allocation pattern are identical for each of the one or more monitoring locations in the frequency domain.

- Include this and the prior agreement on this issue in an LS to RAN2

Agreement:

The intra-carrier guard bands on a carrier can be semi-statically adjusted with an RB level granularity. The RAN4 minimum guard band requirements are used as the guard bands when no semi-static adjustment is applied.

- The guard bands adjustments do not affect the already agreed restrictions on PUCCH resource allocation.

- FFS: Whether and how to handle the case where the intra-carrier guard bands are part of a resource allocation

Agreement:

- For UL transmissions in a serving cell with carrier bandwidth greater than the LBT bandwidth, for the case where UE performs CCA before UL transmission, UE transmits on the UL only if CCA is successful at UE in all LBT bandwidths that overlap with the resource allocation for the UL transmission

- The UE is not expected to receive resource allocations in discontiguous LBT bandwidths within a wideband carrier

  - This does not preclude such resource allocation in discontiguous LBT bandwidths being supported by specifications managed by RAN1 in Rel-16.

#### RAN1 #99, November 2019

Agreement:

UE determines the number of RB sets (corresponding to LBT bandwidths) and the available PRBs in each RB set, both for DL and UL, based on,

- If configured, the RRC parameters intraCellGuardBandDL-r16 and intraCellGuardBandUL-r16 (if UL is configured) configure the lists of intra-carrier guard-bands per cell, e.g.,

  - If intraCellGuardBandDL-r16 or intraCellGuardBandUL-r16 are given as {GB1low, GB1high}, …, {GBN-1low, GBN-1high} where GB is given by CRB index, the UE determines

    - The number of RB sets for a cell: N

    - The available PRBs in each RB set: [RBstart, GB1low-1] for RB set #1, [GB1high+1, GB2low-1] for RB set #2,…, [GBN-1high+1, RBend] for RB set#N, where RBstart and RBend corresponds to starting and ending RB index of cell, respectively.

    - Note: {GB1low, GB1high}, …, {GBN-1low, GBN-1high} may be provided separately for DL and UL

- If intraCellGuardBandDL-r16 is not configured, then {GB1low, GB1high}, …, {GBN-1low, GBN-1high} is derived from the RAN4 specifications

  - Note: This supersedes a previous agreement

- If intraCellGuardBandUL-r16 is not configured, then {GB1low, GB1high}, …, {GBN-1low, GBN-1high} is derived from the RAN4 specifications

  - Note: This supersedes a previous agreement

- Note: This addresses the FFS in sections 5.1.2.2 and 6.1.2.2 in 38.214.

Agreement:

For the frequency domain resource allocation that is provided with frequencyDomainResources in CORESET configuration,

- Introduce a new RRC parameter rb-Offset (with the value range of 0,1,…,5) in ControlResoureSet IE.

  - If rb-Offset is not configured, rb-Offset is 0

- The bits of the 45-bit bitmap frequencyDomainResources have a one-to-one mapping with non-overlapping groups of 6 consecutive PRBs, in ascending order of the PRB index in the BWP with the starting PRB position as {the first PRB index in the BWP + rb-Offset} for a CORESET.

- FFS: For multi-cluster CORESET configuration, rb-Offset also applies to the RB offset between the starting PRB index of the first 6 PRB group and the first PRB index in each RB set. Full 6 PRB groups are counted till the end of the RB set. The bits in frequencyDomainResources sequentially maps to the 6 RB groups in all RB sets in the BWP.

- Note: Cluster above implies a group of resource blocks that are not contiguous in frequency

Conclusion:

For a legacy CORESET configuration, the UE can expect to process PDCCH as per Rel-15 behaviour

Agreement:

For a search space set configuration with multiple monitoring locations in the frequency domain,

- Within the SearchSpace IE, the agreed RRC parameter freqMonitorLocations-r16 provides a bitmap (where the first bit in the bitmap corresponds to the first RB set in the BWP, and the second bit corresponds to the second RB set, and so on). For a RB set indicated in the bitmap, the first PRB of the frequency domain monitoring location confined within the RB set is aligned with {the first PRB of the RB set + rb-Offset provided by the associated CORESET configuration}.

- The frequency domain resource allocation pattern for each monitoring location is determined based on the first A bits in frequencyDomainResources provided by the associated CORESET configuration, where A = floor({the number of available PRBs in the first RB set (accounting for rb-Offset) for the BWP}/6).

Agreement:

- The RRC parameters intraCellGuardBandDL-r16 and intraCellGuardBandUL-r16 include a mechanism to indicate that no intra-carrier guard-bands are configured

  - Note: This configuration may be used for the case where transmission only occurs in a BWP if LBT is successful in all RB sets within the BWP

- For a carrier with intra-carrier guard bands, the UE does not expect that the dedicated BWP is configured to include parts of a RB set.

Agreement:

If a UE is configured with a CSI-RS spanning over multiple LBT bandwidths,

- The UE assumes that the CSI-RS is not transmitted if the UE is monitoring DCI format 2_0 carrying an LBT BW indication and detects the DCI format 2_0 indicating any of corresponding LBT bandwidths is not available for DL reception.

Agreement:

For CSI-RS for tracking in unlicensed spectrum,

- Text proposal for section 5.1.6.1.1 in TS 38.214:

  - The bandwidth of the CSI-RS resource, as given by the higher layer parameter freqBand configured by CSI-RS-ResourceMapping, is the minimum of 48 and NBWP,i size resource blocks, or is equal to NBWP,i size resource blocks.

## 7.2.2.3	Others

#### RAN1 #AH-1901, Jan 2019

#### RAN1 #96, Feb 2019

#### RAN1 #96bis, April 2019

#### RAN1 #97, May 2019

#### RAN1 #98, August 2019