3GPP TSG RAN WG1 Meeting #100-e	R1-2000578

e-Meeting, February 24th-March 6th, 2020

Agenda Item:	7.2.8.4

Source:	Futurewei

Title:	TP on Measurement Procedures

Document for:	Discussion and decision

# Introduction

At RAN1#99, RAN1 completed the standardization work for NR Positioning Support [1]. Therefore, essential corrections or clarifications will be handled through change requests. In this contribution, some potential clarifications on the PRS reception procedures in TS38.214 are presented.

# Discussions

In Section 5.1.6.5 of TS38.214 [2], the following description can be found:

“…The UE expects that it will be configured with [IDs] each of which is defined such that it is associated with multiple DL PRS Resource Sets from the same cell. The UE expects that one of these [IDs] along with a DL-PRS-ResourceSetId and a DL-PRS-ResourceId can be used to uniquely identify a DL PRS Resource.”

In RAN1#98bis, the following was agreed:

Agreement:

An ID is defined that can be associated with multiple DL PRS Resource Sets associated with a single TRP.

  - This ID can be used along with a DL PRS Resource Set ID and a DL PRS Resources ID to uniquely identify a DL PRS Resource

  - Name can be defined by RAN2

Each TRP should only be associated with one such ID

Per the agreement in RAN1#98bis quoted above, the intended behaviour is such that the UE can be configured with an ID that can be associated with multiple DL PRS Resource Sets associated with a single TRP. It is not per the agreement that the UE expects that this ID will always be configured. Therefore, the correct specification should be corrected as follows:

Proposal 1:

“…The UE expects that it will can be configured with [IDs] each of which is defined such that it is associated with multiple DL PRS Resource Sets from the same cell. The UE expects that one of these [IDs] along with a DL-PRS-ResourceSetId and a DL-PRS-ResourceId can be used to uniquely identify a DL PRS Resource.”

# Conclusions

# In this contribution, we propose the following text clarifications for TS38.214, Section 5.1.6.5:

# Proposal 1:

“…The UE expects that it will can be configured with [IDs] each of which is defined such that it is associated with multiple DL PRS Resource Sets from the same cell. The UE expects that one of these [IDs] along with a DL-PRS-ResourceSetId and a DL-PRS-ResourceId can be used to uniquely identify a DL PRS Resource.”

# References

R1-193201, Status report of WI: NR positioning support, RAN1#86.

TS38.214, “Physical layer procedures for data,” v16.0.0.