3GPP TSG RAN WG1 #100-e			R1-2000913

e-Meeting, February 24th – March 6th, 2020

Source:	NTT DOCOMO, INC.

Title:	Configured grant enhancement for NR-U

Agenda Item:	7.2.2.2.4

Document for: 	Discussion and Decision

- Introduction

In this contribution, remaining aspects of configured grant enhancement are discussed.

- CG-DFI

At the RAN1#99 meeting, following agreements have been made [1]:

Agreement: - DFI is transmitted using PDCCH scrambled with CS-RNTI - DFI size is similar to UL DCI size - Size is aligned with UL grant DCI format 0_1 size - To distinguish DCI for activation/deactivation CG transmission and DFI, a 1 bit flag (explicit indication) is used, when type 1 and/or type 2 CG PUSCH is configured - Content of DFI includes - UL/DL flag - CIF in the case of cross carrier scheduled is configured - 1 bit flag - HARQ Bitmap - TPC command – 2 bits - For type 1 configured grant, UE assumes DFI is only present when CG is configured. - For type 2 configured grant, UE assumes DFI is only present when CG is configured, and UE is in activated state for configured grant transmissions Agreement: When the UE is configured with repK > 1, repetition of a TB is mapped within a configuration in the case when an UE is configured with multiple active configurations - The UE repeats the TB in the earliest consecutive transmission occasion candidates within the same configuration instead of consecutive slots - The UE may drop repetition transmissions that fall into a subsequent configured period. - The UE terminates the repetitions if an explicit feedback indicating ACK in the DFI is received for the HARQ process. Agreement: Definition of the minimum duration, D, in the case of slot aggregation/CG repetition - For the case of CG repetitions, the minimum duration D is calculated from the ending symbol of each PUSCH of the K repetitions to the starting symbol of the DFI carrying HARQ-ACK for the associated HARQ process ID - For the case of slot aggregation for scheduled UL transmission, the minimum duration, D, refers to the time between ending symbol of the PUSCH in the first slot of aggregated slots to the starting symbol of the DFI carrying HARQ-ACK for the associated HARQ process ID - For ACK, the minimum duration, D, from the ending symbol of the first PUSCH of the TB repetitions to the starting symbol of the DFI - For NACK, the minimum duration, D, from the ending symbol of the last available PUSCH of the TB repetitions to the starting symbol of the DFI

As stated above, CG-DFI includes TPC command with 2 bits. However, it is unclear which PUSCH the TPC command is applied to. Since the HARQ-ACK information in CG-DFI is used by the UE to initiate autonomous retransmission of CG-PUSCH, the TPC command should be applied to the autonomous retransmission of CG-PUSCH for which the HARQ-ACK in CG-DFI is NACK (i.e., CG retransmission timer keeps running). For other PUSCHs (i.e., DG-PUSCH (re)transmission, CG-PUSCH retransmission scheduled by UL grant and CG-PUSCH initial transmission), UL grant or CG activation DCI can be used for indicating TPC command.

Proposal 1: TPC command in CG-DFI is applied to the autonomous retransmission of CG-PUSCH for which HARQ-ACK in CG-DFI is NACK

In addition, CG-PUSCH repetitions can be terminated by CG-DFI. However, there is no explicit agreement on the termination of DG-PUSCH repetitions by CG-DFI. Since the timeline (i.e., minimum duration, D) between DG-PUSCH repetitions and valid ACK/NACK in CG-DFI has been agreed as well as CG-PUSCH repetitions, the termination of DG-PUSCH repetitions by CG-DFI should be supported.

Proposal 2: UE terminates the repetitions if an explicit feedback indicating ACK in the DFI is received for the HARQ process not only for CG-PUSCH but also for DG-PUSCH

Another unclear point is the timeline determination for UE to terminate the PUSCH repetitions for a given HARQ process when UE receives ‘ACK’ in CG-DFI for the same HARQ process. In Rel.15, for CG-PUSCH repetitions, the timeline for UE to terminate the repetitions by the UL grant scheduling a PUSCH with the same HARQ process is defined in TS 38.214. For simplicity and feasibility, the same rule can be reused for the termination of DG/CG-PUSCH repetitions by CG-DFI as:

Proposal 3: A UE is not expected to receive a ACK in CG-DFI in a PDCCH ending in symbol i for a given HARQ process to terminate a PUSCH repetition on a given serving cell with the same HARQ process starting in symbol j after symbol i, and if the gap between the end of PDCCH and the beginning of symbol j is less than N2 symbols

- Text proposal

Based on above proposals, following corrections would be necessary;

Proposal 4: Adapt following text proposal for TS 38.213 and TS 38.214 which reflects proposals 1-3 in this contribution

TS38.213

== Start ==

### 7.1.1	UE behaviour

If a UE transmits a PUSCH on active UL BWP  of carrier  of serving cell  using parameter set configuration with index  and PUSCH power control adjustment state with index , the UE determines the PUSCH transmission power  in PUSCH transmission occasion  as

[dBm]

where,

<omitted text>

-	For the PUSCH power control adjustment state  for active UL BWP  of carrier  of serving cell  in PUSCH transmission occasion

-	 is a TPC command value included in a DCI format that schedules the PUSCH transmission occasion  on active UL BWP  of carrier  of serving cell  or jointly coded with other TPC commands in a DCI format 2_2 with CRC scrambled by TPC-PUSCH-RNTI, as described in Clause 11.3 or included in a CG-DFI that initiates the PUSCH retransmission occasion  on active UL BWP  of carrier  of serving cell

-	 if the UE is configured with twoPUSCH-PC-AdjustmentStates and  if the UE is not configured with twoPUSCH-PC-AdjustmentStates or if the PUSCH transmission is scheduled by a RAR UL grant as described in Clause 8.3

-	For a PUSCH (re)transmission configured by ConfiguredGrantConfig, the value of  is provided to the UE by powerControlLoopToUse

-	If the UE is provided SRI-PUSCH-PowerControl, the UE obtains a mapping between a set of values for the SRI field in a DCI format scheduling the PUSCH transmission and the  value(s) provided by sri-PUSCH-ClosedLoopIndex and determines the  value that is mapped to the SRI field value

-	If the PUSCH transmission is scheduled by a DCI format that does not include an SRI field, or if an SRI-PUSCH-PowerControl is not provided to the UE,

-	If the UE obtains one TPC command from a DCI format 2_2 with CRC scrambled by a TPC-PUSCH-RNTI, the  value is provided by the closed loop indicator field in DCI format 2_2

-	 is the PUSCH power control adjustment state  for active UL BWP  of carrier  of serving cell  and PUSCH transmission occasion  if the UE is not provided tpc-Accumulation, where

-	The  values are given in Table 7.1.1-1

-	 is a sum of TPC command values in a set  of TPC command values with cardinality  that the UE receives between  symbols before PUSCH transmission occasion  and  symbols before PUSCH transmission occasion  on active UL BWP  of carrier  of serving cell  for PUSCH power control adjustment state , where  is the smallest integer for which  symbols before PUSCH transmission occasion  is earlier than  symbols before PUSCH transmission occasion

-	If a PUSCH transmission is scheduled by a DCI format,  is a number of symbols for active UL BWP  of carrier  of serving cell  after a last symbol of a corresponding PDCCH reception and before a first symbol of the PUSCH transmission

-	If a PUSCH transmission is configured by ConfiguredGrantConfig,  is a number of  symbols equal to the product of a number of symbols per slot, , and the minimum of the values provided by k2 in PUSCH-ConfigCommon for active UL BWP  of carrier  of serving cell

-	If the UE has reached maximum power for active UL BWP of carrier  of serving cell  at PUSCH transmission occasion  and , then

-	If UE has reached minimum power for active UL BWP of carrier  of serving cell  at PUSCH transmission occasion  and , then

-	A UE resets accumulation of a PUSCH power control adjustment state  for active UL BWP  of carrier  of serving cell  to

-	If a configuration for a corresponding  value is provided by higher layers

-	If a configuration for a corresponding  value is provided by higher layers

where  is determined from the value of  as

-	If  and the UE is provided higher SRI-PUSCH-PowerControl,  is any of the sri-PUSCH-ClosedLoopIndex value(s) configured in any SRI-PUSCH-PowerControl with the sri-P0-PUSCH-AlphaSetId value corresponding to

-	If  and the UE is not provided SRI-PUSCH-PowerControl or j=0,

-	If ,  is provided by the value of powerControlLoopToUse

-	 is the PUSCH power control adjustment state for active UL BWP  of carrier  of serving cell  and PUSCH transmission occasion  if the UE is provided tpc-Accumulation, where

-	 absolute values are given in Table 7.1.1-1

<omitted text>

== End ==

TS38.214

== Start ==

## 6.1	UE procedure for transmitting the physical uplink shared channel

PUSCH transmission(s) can be dynamically scheduled by an UL grant in a DCI, or the transmission can correspond to a configured grant Type 1 or Type 2. The configured grant Type 1 PUSCH transmission is semi-statically configured to operate upon the reception of higher layer parameter of configuredGrantConfig including rrc-ConfiguredUplinkGrant without the detection of an UL grant in a DCI. The configured grant Type 2 PUSCH transmission is semi-persistently scheduled by an UL grant in a valid activation DCI according to Clause 10.2 of [6, TS 38.213] after the reception of higher layer parameter configuredGrantConfig not including rrc-ConfiguredUplinkGrant. If Configuredgrantconfig-ToAddModList-r16 is configured, more than one configured grant configuration of configured grant Type 1 and/or configured grant Type 2 may be active at the same time on an active BWP of a serving cell.

<omitted text>

A UE is not expected to be scheduled by a PDCCH ending in symbol i to transmit a PUSCH on a given serving cell for a given HARQ process, if there is a transmission occasion where the UE is allowed to transmit a PUSCH with configured grant according to [10, TS38.321] with the same HARQ process on the same serving cell starting in a symbol j after symbol i, and if the gap between the end of PDCCH and the beginning of symbol j is less than N2 symbols. The value N2 in symbols is determined according to the UE processing capability defined in Clause 6.4, and N2 and the symbol duration are based on the minimum of the subcarrier spacing corresponding to the PUSCH with configured grant and the subcarrier spacing of the PDCCH scheduling the PUSCH.

A UE is not expected to receive a ACK in CG-DFI in a PDCCH ending in symbol i for a given HARQ process to terminate a PUSCH repetition on a given serving cell with the same HARQ process starting in a symbol j after symbol i, and if the gap between the end of PDCCH and the beginning of symbol j is less than N2 symbols. The value N2 in symbols is determined according to the UE processing capability defined in Clause 6.4, and N2 and the symbol duration are based on the minimum of the subcarrier spacing corresponding to the PUSCH and the subcarrier spacing of the PDCCH indicating CG-DFI.

For PUSCH scheduled by DCI format 0_0 on a cell, the UE shall transmit PUSCH according to the spatial relation, if applicable, corresponding to the dedicated PUCCH resource with the lowest ID within the active UL BWP of the cell, as described in Clause 9.2.1 of [6, TS 38.213].

<omitted text>

#### 6.1.2.1	Resource allocation in time domain

When the UE is scheduled to transmit a transport block and no CSI report, or the UE is scheduled to transmit a transport block and a CSI report(s) on PUSCH by a DCI, the Time domain resource assignment field value m of the DCI provides a row index m + 1 to an allocated table. The determination of the used resource allocation table is defined in Clause 6.1.2.1.1. The indexed row defines the slot offset K2, the start and length indicator SLIV, or directly the start symbol S and the allocation length L, the PUSCH mapping type, and the number of repetitions (if numberofrepetitions is present in the resource allocation table) to be applied in the PUSCH transmission.

<omitted text>

For PUSCH repetition Type A, in case K>1, the same symbol allocation is applied across the K consecutive slots and the PUSCH is limited to a single transmission layer. The UE shall repeat the TB across the K consecutive slots applying the same symbol allocation in each slot. The redundancy version to be applied on the nth transmission occasion of the TB, where n = 0, 1, … K-1, is determined according to table 6.1.2.1-2.

For any RV sequence, the repetitions shall be terminated after transmitting K repetitions, or from the starting symbol of the repetition that was terminated by ACK for the HARQ process in CG-DFI, whichever is reached first.

Table 6.1.2.1-2: Redundancy version for PUSCH transmission

| - rvid indicated by the DCI scheduling the PUSCH | - rvid to be applied to nth transmission occasion (repetition Type A) or nth actual repetition (repetition Type B) |  |  |  |
| --- | --- | --- | --- | --- |
|  | - n mod 4 = 0 | - n mod 4 = 1 | - n mod 4 = 2 | - n mod 4 = 3 |
| - 0 | - 0 | - 2 | - 3 | - 1 |
| - 2 | - 2 | - 3 | - 1 | - 0 |
| - 3 | - 3 | - 1 | - 0 | - 2 |
| - 1 | - 1 | - 0 | - 2 | - 3 |

<omitted text>

##### 6.1.2.3.1	Transport Block repetition for uplink transmissions of PUSCH repetition Type A with a configured grant

The procedures described in this clause apply to PUSCH transmissions of PUSCH repetition Type A with a Type 1 or Type 2 configured grant.

The higher layer parameter repK-RV defines the redundancy version pattern to be applied to the repetitions. If the parameter repK-RV is not provided in the configuredGrantConfig, the redundancy version for uplink transmissions with a configured grant shall be set to 0. Otherwise, for the nth transmission occasion among K repetitions, n=1, 2, …, K, it is associated with (mod(n-1,4)+1)th value in the configured RV sequence. If a configured grant configuration is configured with Configuredgrantconfig-StartingfromRV0 set to ‘off’, the initial transmission of a transport block may only start at the first transmission occasion of the K repetitions. Otherwise, the initial transmission of a transport block may start at

-	the first transmission occasion of the K repetitions if the configured RV sequence is {0,2,3,1},

-	any of the transmission occasions of the K repetitions that are associated with RV=0 if the configured RV sequence is {0,3,0,3},

-	any of the transmission occasions of the K repetitions if the configured RV sequence is {0,0,0,0}, except the last transmission occasion when K≥8.

For any RV sequence, the repetitions shall be terminated after transmitting K repetitions, or at the last transmission occasion among the K repetitions within the period P, or from the starting symbol of the repetition that overlaps with a PUSCH with the same HARQ process scheduled by DCI format 0_0, 0_1 or 0_2, or from the starting symbol of the repetition that was terminated by ACK for the HARQ process in CG-DFI, whichever is reached first. The UE is not expected to be configured with the time duration for the transmission of K repetitions larger than the time duration derived by the periodicity P. If the UE determines that, for a transmission occasion, the number of symbols available for the PUSCH transmission in a slot is smaller than transmission duration L, the UE does not transmit the PUSCH in the transmission occasion.

For both Type 1 and Type 2 PUSCH transmissions with a configured grant, when K > 1, the UE shall repeat the TB across the K consecutive slots applying the same symbol allocation in each slot, except if the UE is provided with higher layer parameters cg-nrofSlots-r16 and cg-nrofPUSCH-InSlot-r16, in which case the UE repeats the TB in the repK earliest consecutive transmission occasion candidates within the same configuration. A Type 1 or Type 2 PUSCH transmission with a configured grant in a slot is omitted according to the conditions in Clause 11.1 of [6, TS38.213].

== End ==

- Conclusion

In this contribution, we discussed on the channel access procedure for NR-U. Based on the discussion, we made following proposals.

Proposal 1: TPC command in CG-DFI is applied to the retransmission of CG-PUSCH for which HARQ-ACK in CG-DFI is NACK

Proposal 2: UE terminates the repetitions if an explicit feedback indicating ACK in the DFI is received for the HARQ process not only for CG-PUSCH but also for DG-PUSCH

Proposal 3: UE is not expected to receive a ACK in CG-DFI in a PDCCH ending in symbol i for a given HARQ process to terminate a PUSCH repetition on a given serving cell for the HARQ process starting in symbol j, where the gap between the ending of PDCCH symbol i and the beginning of PDSCH symbol j is less than N2 symbols

Proposal 4: Adapt following text proposal for TS 38.213 and TS 38.214 which reflects proposals 1-3 in this contribution

References

- 3GPP, RAN1#99, RAN1 Chairman’s Notes, November 2019.