3GPP TSG RAN WG1 #100bis		R1-2001736

e-Meeting, April 20th – 30th, 2020

Source:	OPPO

Title:	Text proposals for UL Power Sharing for NR-DC

Agenda Item:	7.2.10.1

Document for:	Discussion and Decision

Introduction

In this contribution, we present our views on the remaining issues of UL power control for NR-DC and propose some text proposals to update the specification.

Discussion

RRC parameter alignment

There are some misalignments of RRC parameters in TS 38.213 and in TS 38.331. Thus we have the following proposal:

Proposal 1: Align the following RRC parameters in TS 38.213 with those in TS 38.331

- NR-DC-PC-mode   ->  nrdc-PCmode-FR1-r16, nrdc-PCmode-FR2-r16

- Semi-static-mode1 -> semi-static-mode1

- Semi-static-mode2 ->  semi-static-mode2

- Dynamic -> dynamic

The following gives the corresponding text proposal for Proposal 1

In TS 38.213 Section 7.6 Dual Connectivity <omitted text> ### 7.6.2 NR-DC If a UE is configured with an MCG using NR radio access in FR1 or in FR2 and with a SCG using NR radio access in FR2 or in FR1, respectively, the UE performs transmission power control independently per cell group as described in Clauses 7.1 through 7.5. If a UE is configured with an MCG and a SCG using NR radio access in FR1 and/or in FR2, the UE is configured a maximum power PMCG for transmissions on the MCG by p-NR-FR1 and/or by p-NR-FR2 and a maximum power PSCG for transmissions on the SCG by p-NR-FR1 and/or by p-NR-FR2 and with an inter-CG power sharing mode by nrdc-PCmode-FR1-r16 for FR1 and/or by nrdc-PCmode-FR2-r16 for FR2. The UE determines a transmission power on the MCG and a transmission power on the SCG per frequency range. If a UE is provided with an inter-CG power sharing mode of semi-static-mode1 or semi-static-mode2, the UE does not expect PMCG and PSCG to be configured such that PMCG+PSCG>PTotalNR-DC, where PMCG is the linear value of PMCG, PSCG is the linear value of PSCG, and PTotalNR-DC is the linear value of a configured maximum transmission power for NR-DC operation in FR1 as defined in [8-3, TS 38.101-3]. If a UE is provided with an inter-CG power sharing mode of semi-static-mode1, the UE determines a transmission power for the MCG or for the SCG as described in Clauses 7.1 through 7.5 using PMCG or PSCG as the maximum transmission power, respectively. If a UE is provided with an inter-CG power sharing mode of semi-static-mode2 - if at least one symbol of slot i1 of the MCG or of the SCG that is indicated as uplink or flexible to a UE by tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated, if provided, overlaps with a symbol for any ongoing transmission overlapping with slot i2 of the SCG or of the MCG, respectively, the UE determines a power for the transmission on the SCG or the MCG overlapping with slot i2 as described in Clauses 7.1 through 7.5 using PSCG or PMCG, respectively, as the maximum transmission power - otherwise, the UE determines a power for the transmission on MCG or the SCG overlapping with slot i2, as described in [8-3, TS 38.101-3] and in Clauses 7.1 through 7.5 without considering PMCG or PSCG, respectively The UE expects to be provided with an inter-CG power sharing mode of semi-static-mode2 only for synchronous NR-DC operation [10, TS 38.133]. If the UE indicates a capability for dynamic power sharing between the MCG and the SCG and is provided with an inter-CG power sharing mode of dynamic - if UE transmission(s) in slot i1 of the MCG overlap in time with UE transmission(s) in slot i2 of the SCG and if PMCGactuali1+PSCGactuali2>PTotalNR-DC in any portion of slot i2 of the SCG, the UE reduces transmission power in any portion of slot i2 of the SCG so that PMCGactuali1+PSCGactuali2≤PTotalNR-DC in all portions of slot i2, where PMCGactuali1 and PSCGactuali2 are the UE transmission powers in slot i1 of the MCG and in slot i2 of the SCG, respectively, that the UE determines according to Clauses 7.1 through 7.5 using PMCG and PSCG, respectively, as the maximum transmission powers on the MCG and the SCG and PTotalNR-DC is the linear value of a configured maximum transmission power for NR-DC operation in FR1 as defined in [8-3, TS 38.101-3] - if UE transmission(s) in slot i1 of the MCG or in slot i2 of the SCG do not overlap in time with any UE transmission(s) on the SCG or the MCG, respectively, the UE determines a transmission power in slot i1 of the MCG or in slot i2 of the SCG as described in [8-3, TS 38.101-3] and in Clauses 7.1 through 7.5 without considering PMCG or PSCG, respectively If a UE - is provided with an inter-CG power sharing mode of dynamic, and - indicates a capability to determine a total transmission power on the SCG at a first symbol of a transmission occasion on the SCG by determining transmissions on the MCG that - are scheduled by DCI formats in PDCCH receptions with a last symbol that is earlier by more than Toffset from the first symbol of the transmission occasion on the SCG, and - overlap with the transmission occasion on the SCG <omitted text>

Correcting the timing of maximum transmission power determination

For the dynamic power sharing between SCG and MCG, the current version of TS 38.213 has the following descriptions

If a UE - is provided NR-DC-PC-mode = Dynamic, and - indicates a capability to determine a total transmission power on the SCG at a first symbol of a transmission occasion on the SCG by determining transmissions on the MCG that - are scheduled by DCI formats in PDCCH receptions with a last symbol that is earlier by more than Toffset from the first symbol of the transmission occasion on the SCG, and - overlap with the transmission occasion on the SCG the UE determines a maximum transmission power on the SCG at the beginning of the transmission occasion on the SCG as - minPSCG,PTotalNR-DC- PMCGactual, if the UE determines transmissions on the MCG with a PMCGactual total power - PTotalNR-DC, if the UE does not determine any transmissions on the MCG

The highlighted parts are quite confusing. There may be two interpretations

- The determination of transmission power on the SCG is done at a first symbol or the beginning of a transmission occasion on the SCG

- The maximum or total transmission power is determined for the first symbol of a transmission occasion on the SCG

The first one is not correct since UE should determine the transmission before the first symbol or the beginning of the transmission occasion. The second one is not right either since the maximum transmission is for the whole transmission occasion rather than only the first symbol.

In order to address the above the issue, we have the following proposal

Proposal 2: In TS 38.213, clarify determination of the total/maximum transmission power on the SCG to avoid the wrong UE behaviors.  .

The following gives the corresponding text proposal for Proposal 2.

In TS 38.213 Section 7.6 Dual Connectivity ### 7.6.2 NR-DC If a UE is configured with an MCG using NR radio access in FR1 or in FR2 and with a SCG using NR radio access in FR2 or in FR1, respectively, the UE performs transmission power control independently per cell group as described in Clauses 7.1 through 7.5. <omitted text> If a UE - is provided NR-DC-PC-mode = Dynamic, and - indicates a capability to determine a total transmission power on the SCG for a transmission occasion on the SCG by determining transmissions on the MCG that - are scheduled by DCI formats in PDCCH receptions with a last symbol that is earlier by more than Toffset from the first symbol of the transmission occasion on the SCG, and - overlap with the transmission occasion on the SCG the UE determines a maximum transmission power on the SCG for the transmission occasion on the SCG as - minPSCG,PTotalNR-DC- PMCGactual, if the UE determines transmissions on the MCG with a PMCGactual total power - PTotalNR-DC, if the UE does not determine any transmissions on the MCG where - Toffset=max⁡{Tproc,MCGmax,Tproc,SCGmax}, - Tproc,MCGmax and Tproc,SCGmax is the maximum of Tproc,2, Tproc,CSI, Tproc,releasemux, Tproc,2mux, and Tproc,CSImux based on the configurations on the MCG and the SCG, respectively, when the UE indicates a first value for the capability, and - Tproc,MCGmax and Tproc,SCGmax is the maximum of Tproc,2, Tproc,CSI, Tproc,releasemux, Tproc,2mux based on the configurations on the MCG and the SCG, respectively, when the UE indicates a second value for the capability. The UE does not expect to have transmissions on the MCG that - are scheduled by DCI formats in PDCCH receptions with a last symbol that is earlier by less than or equal to Toffset from the first symbol of the transmission occasion on the SCG, and - overlap with the transmission occasion on the SCG <omitted text>

Remaining Issues

For dynamic UL grant, the corresponding PDCCH is compared with the cut-off time to determine whether this dynamic UL grant shall be considered into power allocation. Some contributions pointed out for some cases, no individual DCI is available. Feature lead summary [3] captures the following corresponding cases

- Case 1: CG-PUSCH

- Case 2: Periodic PUCCH (e.g. SR, P-/SP-CSI) and P-/SP-SRS

  - For this case, there is no DCI associated with periodic PUCCH e.g. for SR and SP-SRS.

- Case 3: Group-TPC command by DCI format 2_2, or 2_3 or 2_0 for MCG transmission

  - These Group-TPC command DCI formats do not schedule uplink transmissions but would impact on the value of PMCGactual. Therefore, how to handle it needs to be discussed.  DCI format 2_0 does not schedule uplink transmissions but would impact on the value of PMCGactual, since it may cancel or trigger semi-static uplink transmissions.

- Case 4: Uplink transmission skip for MCG dynamic grant PUSCH/configured grant PUSCH due to 5.4.3.1.3 of TS38.321 or by DCI format 2_4

  - This DCI format 2-4 does not schedule uplink transmissions but would impact on the value of PMCGactual. Similarly, for configured grant PUSCH transmission occasion, the UE may or may not transmit the PUSCH. For dynamic grant PUSCH, the UE may skip the transmission if some conditions are met. These are specified in 5.4.3.1.3 of TS38.321.

The transmissions in Case 1 and Case 2 are predictable. Thus UE should take into account them when determining the transmission power of MCG.  In contrast, the transmissions / cancelation of transmission / TPC in Case 3 and Case 4 are indicated by DCI. UE needs time to decode the corresponding DCI and do some preparations. Following the similar principle addressing DCI scheduled PUSCH in dynamic power sharing, the cut-off time of Toffset should be used to determine whether or not to consider the DCIs of Case 3 and Case 4.

Proposal 3: When UE determines the transmission power of MCG with dynamic power sharing mode,

- CG-PUSCH, Periodic PUCCH, P/SP-SRS should be taken into account

- Only DCI format 2_0, 2_2, 2_3 and 2_4 before T0 – Toffset should be taken into account

Conclusion

In this contribution, we present our view on the current specification regarding NR-DC power sharing. The following proposals and the corresponding text proposals are provided.

Proposal 1: Align the following RRC parameters in TS 38.213 with those in TS 38.331

- NR-DC-PC-mode   ->  nrdc-PCmode-FR1-r16, nrdc-PCmode-FR2-r16

- Semi-static-mode1 -> semi-static-mode1

- Semi-static-mode2 ->  semi-static-mode2

- Dynamic -> dynamic

Proposal 2: In TS 38.213, clarify determination of the total/maximum transmission power on the SCG to avoid the wrong UE behaviors.

Proposal 3: When UE determines the transmission power of MCG with dynamic power sharing mode,

- CG-PUSCH, Periodic PUCCH, P/SP-SRS should be taken into account

- Only DCI format 2_0, 2_2, 2_3 and 2_4 before T0 – Toffset should be taken into account

References

- TS 38.213, NR; Physical layer procedures for control, V16.1.0

- TS 38.331, NR; Radio Resource Control (RRC) protocol specification, v16.0.0

- R1-2001208, FL summary#2 on UL Power Control for NN-DC, Apple