3GPP TSG RAN WG1 #100bis			R1-2001906

e-Meeting, April 20th – 30th, 2020

Agenda item:	7.2.4.2.1

Title:	Sidelink mode-1 resource allocation

Source:	MediaTek Inc.

Document for:	Discussion and Decision

# Introduction

In this contribution we discuss the remaining issues on Rel-16 NR V2X mode-1 resource allocation. We discuss minimum processing/preparation times, DCI 3_0 content and payload size, SL-HARQ reporting to gNB.

# Processing times

## PSCCH/PSSCH preparation time

The minimum PSCCH/PSSCH preparation time in LTE V2X is 4 subframes (i.e. 4ms). Similarly in NR, the minimum slot offset indication in DCI 3_0 should not be shorter than a UE minimum processing time. The minimum value should also depend on sidelink numerology.

We propose the following:

Proposal 1: The minimum preparation time from DCI to PSCCH/PSSCH transmission should not be shorter than NR PUSCH preparation time values according to active sidelink numerology.

## PSFCH-to-PUCCH HARQ preparation time

DCI format 3_0 includes a PSFCH-to-HARQ feedback timing indication field which points to an uplink slot in reference from the last PSFCH slot. For example, if the indicated timing value is k = 0 (see section 16.5, TS 38.213 v16.1 [1]), the last slot for PUCCH has to overlap with the last PSFCH reception occasion.

The SL-HARQ preparation time on PUCCH should depend on PSFCH decoding time, UCI preparation time, as well as uplink numerology. Unless a sufficient PSFCH-to-HARQ timing is indicated in DCI 3_0, UE may not be guaranteed to prepare HARQ feedback on the indicated uplink slot.

Proposal 2: The minimum SL-HARQ reporting time from PSFCH decoding to PUCCH/PUSCH transmission should not be shorter than the minimum NR Uu-HARQ timing values according to each corresponding uplink numerology.

# DCI 3_0 content and payload size

In RAN1-99 [2], the following agreement was made on DCI format 3_0 payload size:

Agreements: - Existing DCI size budget is maintained when the UE is configured with SL - (Working assumption): The size of the new DCI format and the size of one of the existing NR DCI formats are aligned.

According to 38.212 v16.1 [3], the size of each DCI format 3_0 field is listed in Table 1. As the existing DCI format 0_0/1_0 have a payload of at least 34 bits or more depending on active BWP (excluding padding bits), the alignment of payload sizes between DCI format 3_0 and DCI format 0_0/1_0 appears to be feasible by zero padding.

We have the following proposal:

Proposal 3: Confirm working assumption. DCI format 3_0 payload size is aligned with DCI format 0_0/1_0 by zero padding.

Table 1: DCI format 3_0 field size.

| DCI field | Number of bits |
| --- | --- |
| Time gap | [x] bits ( e.g., x is 3-4 bits) |
| HARQ process ID | [x] bits ( e.g., x is 3-4 bits) |
| NDI | 1 bit |
| Lowest index of the subchannel allocation to initial transmission | log2(N subChannel SL) bits |
| SCI frequency resource assignment | log2(N subChannel SLN subChannel SL + 12) or log2(N subChannel SLN subChannel SL + 12N subChannel SL + 16) bits |
| SCI time resource assignment | 5 or 9 bits |
| PSFCH-to-HARQ feedback timing indicator | 3 bits |
| PUCCH resource indicator | 3 bits |
| Configuration index | 0 or [x] bits (e.g., x is 3 bits) |

In addition, the size of DCI 3_0 should be the same for dynamic grants and configured grant type-2 activation/deactivation. Size alignment can be achieved by using zero padding.

Proposal 4: DCI format 3_0 size is aligned using zero padding between dynamic grants and configured grants type-2 activation/deactivation.

# SL-HARQ reporting to gNB

It has been agreed that multiplexing of Uu UCI and SL-HARQ feedback is not supported in Rel-16. The SL-HARQ codebook can follow similar design to existing Uu HARQ codebook design with some simplifications.

We propose the following:

Proposal 5: SL-HARQ codebook design should follow existing Uu-HARQ codebook design with necessary simplifications.

# Conclusions

We have the following proposals:

Proposal 1: The minimum preparation time from DCI to PSCCH/PSSCH transmission should not be shorter than NR PUSCH preparation time values according to active sidelink numerology.

Proposal 2: The minimum SL-HARQ reporting time from PSFCH decoding to PUCCH/PUSCH transmission should not be shorter than the minimum NR Uu-HARQ timing values according to each corresponding uplink numerology.

Proposal 3: Confirm working assumption. DCI format 3_0 payload size is aligned with DCI format 0_0/1_0 by zero padding.

Proposal 4: DCI format 3_0 size is aligned using zero padding between dynamic grants and configured grants type-2 activation/deactivation.

Proposal 5: SL-HARQ codebook design should follow existing Uu-HARQ codebook design with necessary simplifications.

# References

- TS 38.213 version 16.1.0, “Physical layer procedures for control”, Release 16, March 2020.

- Chairman’s notes, RAN1#99, Reno, USA, November 2019.

- TS 38.212 version 16.1.0, “Multiplexing and channel coding”, Release 16, March 2020.