3GPP TSG RAN WG1 Meeting #100bis-e 		 		R1-2001949

e-Meeting, April 20th – 30th, 2020

Agenda Item:	7.2.1.1

Source: 	LG Electronics

Title: 	Remaining details of Channel Structure for 2-step RACH

Document for:	Discussion and Decision

- Introduction

In this contribution, we discuss remaining details of channel structure for 2-step RACH.

- Multiplexing of msgA PUSCH and msgA PUSCH in same slot

- Multiplexing of msgA PUSCH and PUSCH/PUCCH/SRS

Because PRACH is transmitted without adjusting TA values and TA values commonly is applied for other UL signals (i.e., PUSCH/PUCCH/SRS), RAN1 agreed that UE cannot transmit both PRACH and PUSCH in same slot or within certain gap for 2-step RACH, as shown in current specification [TS 38.213]

In 2-step RACH, msgA PUSCH can be transmitted without any adjustment of TA which is the same with msgA PRACH. Hence, it is needed to apply the principle to msgA PUSCH in 2-step RACH.

Proposal 1:

- UE cannot transmit both magA PUSCH and PUSCH/PUCCH/SRS in same slot or within certain gap for 2-step RACH.

Conclusion

In this contribution, we discuss on channel structure for two-step RACH. We proposed as follows:

msgA PUSCH and PUSCH

Proposal 1:

- UE cannot transmit both magA PUSCH and PUSCH/PUCCH/SRS in same slot or within certain gap for 2-step RACH.

- Adopt the TP#1 to capture the random access response in TS 38.213.

============ Start of Text Proposal for TS38.213 [1] ==================

8.1A	PUSCH for Type-2 random access procedure

<--------------------------------------Other parts are omitted---------------------------------------------------------->

A PUSCH occasion is valid if it does not overlap in time and frequency with any PRACH occasion associated with either a Type-1 random access procedure or a Type-2 random access procedure. Additionally, if a UE is provided tdd-UL-DL-ConfigurationCommon, a PUSCH occasion is valid if

-	it is within UL symbols, or

-	it does not precede a SS/PBCH block in the PUSCH slot and starts at least Ngap symbols after a last downlink symbol and at least Ngap symbols after a last SS/PBCH block symbol, where Ngap is provided in Table 8.1-2.

For single cell operation or for operation with carrier aggregation in a same frequency band, a UE does not transmit PUSCH for type-2 random access procedure and PUSCH/PUCCH/SRS in a same slot or when a gap between the first or last symbol of a PRACH transmission in a first slot is separated by less than  symbols from the last or first symbol, respectively, of a PUSCH/PUCCH/SRS transmission in a second slot where  for  or ,  for  or , and  is the SCS configuration for the active UL BWP.

<--------------------------------------Other parts are omitted---------------------------------------------------------->

============ End of Text Proposal for TS38.213 ==================

Reference

[1] 3GPP TS 38.213 V16.1.0 (2020-3): "NR; Physical layer procedures for control"