3GPP TSG RAN WG1 Meeting #100bis-e                                                         	   R1-2002016

E-meeting, April 20th – 30th, 2020

Agenda Item:	7.2.11.2

Source: 	Intel Corporation

Title:	UE features for NR-U

Document for: 	Discussion/Decisions

# Introduction

This contribution discusses the UE features related to NR unlicensed [1].

# Discussion

This section discusses the UE features that need to be modified/removed from the one drafted based on the past email discussions [1]. In particular, the UE features that need further discussion are listed below with some columns deleted.

## Basic feature groups

It is proposed to clarify that the basic feature groups including 10-1, 10-1a, 10-2, and 10-2a are targeting unlicensed band in the FR1 regime in order to avoid unnecessary capability signaling to licensed band or FR2. Therefore, it is proposed to put the restriction of FR1 unlicensed band in the note of the basic feature groups as shown below:

Proposal 1: Put the restriction of FR1 unlicensed band in the note of 10-1, 10-1a, 10-2, and 10-2a.

| Index | Feature group | Components | Prerequisite feature groups | Type | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- |
| 10-1 | UE stand-alone (DL and UL) operation in shared spectrum under dynamic channel access mode | 1. Type 1 channel access 2. Type 2A channel access 3. Type 2B channel access (FFS if move this to separate feature) 4. Type 2C channel access 5. 20MHz LBT bandwidth 6. Contention window adjustment 7. CP extension up to 1 symbol for PUSCH/PUCCH transmission 8. SSB/MIB/RMSI reception with Q 9. SSB RRM with Q in DMTC 10. SSB-RLM with Q in DMTC window 11. Support of RAR extension from 10ms to [40ms] by decoding of the 2-bit SFN indication in DCI 1_0 |  | Per band | This can be a basic feature group for operating only in FR1 unlicensed band with both DL and UL transmission support under dynamic channel access | Optional with capability signalling |
| 10-1a | UE DL only operation in shared spectrum under dynamic channel access mode | 1. SSB RRM with Q in DMTC | 6-5 | Per band | This can be a basic feature group for operating only in FR1 unlicensed band with DL only operation | Optional with capability signalling |
| 10-2 | UE stand-alone (DL and UL) operation in shared spectrum under semi-static channel access mode | 1. Type 2C channel access 2. Single sensing slot of 9us channel access 3. 20MHz LBT bandwidth 4. SSB/MIB/RMSI reception with Q 5. SSB RRM with Q in DMTC 6. SSB-RLM with Q in DMTC window 7. Support of RAR extension from 10ms to [40ms] by decoding of the 2-bit SFN indication in DCI 1_0 8. Support fixed frame period of 5ms and 10ms |  | Per band | This can be a basic feature group for operating only in FR1 unlicensed band with DL only operation | Optional with capability signalling |
| 10-2a | UE DL only operation in shared spectrum under semi-static channel access mode | 1. SSB RRM with Q in DMTC 2. Support fixed frame period of 5ms and 10ms | 6-5 | Per band | This can be a basic feature group for operating only in FR1 unlicensed band with DL only operation | Optional with capability signalling |

## Semi-static channel access mode (10-2/10-2a/10-2b)

It is OK to have 10-2 and 10-2a for both stand-alone operation and DL only operation. However, current table has different feature groups for stand-alone operation depending on the fixed frame period length. We do not have clear motivation to define additional feature groups for supporting small fixed frame period length. And this differentiation is only applied for stand-alone operation and not applied for DL only operation, which does not look desirable either.

Proposal 2: Remove 10-2b and change the 8th bullet of 10-2.

| Index | Feature group | Components | Prerequisite feature groups | Type | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- |
| 10-2 | UE stand-alone (DL and UL) operation in shared spectrum under semi-static channel access mode | 1. Type 2C channel access 2. Single sensing slot of 9us channel access 3. 20MHz LBT bandwidth 4. SSB/MIB/RMSI reception with Q 5. SSB RRM with Q in DMTC 6. SSB-RLM with Q in DMTC window 7. Support of RAR extension from 10ms to [40ms] by decoding of the 2-bit SFN indication in DCI 1_0 |  | Per band | This can be a basic feature group for operating in unlicensed band. Support of channel access mechanism for FBE operation, including fixed frame period, Cat 2 LBT, Cat 1 LBT | Optional with capability signalling |
| 10-2a | UE DL only operation in shared spectrum under semi-static channel access mode | 1. SSB RRM with Q in DMTC 2. Support fixed frame period of 5ms and 10ms | 6-5 | Per band | This can be a basic feature group for operating in unlicensed band with DL only operation | Optional with capability signalling |

## PRB interlace frequency domain resource allocation (10-3/10-3a/10-3b/10-3c)

Interlace design is a fundamental feature for uplink transmission in order to be complaint with  the unlicensed band regulation. Also based on the agreement below (first two agreements), a gNB can configure the interlaced mapping for the whole cell both for PUCCH and PUSCH. From this perspective, it is proposed to merge the “PRB interlace frequency domain resource allocation” into 10-1 and 10-2.

Even though it is separately defined, it is proposed to combine 10-3/10-3a/10-3b/10-3c into a single feature group. This is because if interlace mapping is used for one PUCCH format, then other PUCCH formats also have to use interlace mapping based on the 3rd agreement below. However, we do not need to use the interlace mapping for licensed use since there is no reason for it.

Agreement: In Rel-16, for a cell, the UE can expect that cell-specifically configured PUCCH resources and UE-specifically configured PUCCH resources either all have interlaced mapping or all have non-interlaced mapping as per Rel-15. - Note: RRC parameters that are made redundant due to this agreement can be eliminated Agreement: In Rel-16, for a cell, the UE can expect that cell-specifically configured PUSCH resources and UE-specifically configured PUSCH resources either all have interlaced mapping or all have non-interlaced mapping as per Rel-15. - Note: RRC parameters that are made redundant due to this agreement can be eliminated Agreement: In Rel-16, for a cell, the UE can expect that UE-specifically configured PUCCH resources and all PUSCH transmissions (scheduled and configured) after dedicated configurations either all have interlaced mapping or all have non-interlaced mapping as per Rel-15.

Proposal 3:

- Merge 10-3/10-3a/10-3b/10-3c into 10-1 and 10-2.

- If not merged, combine 10-3/10-3a/10-3b/10-3c into a single feature group and remove “Need discussion for licensed use”.

| Index | Feature group | Components | Prerequisite feature groups | Type | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- |
| 10-1 | UE stand-alone (DL and UL) operation in shared spectrum under dynamic channel access mode | 1. Type 1 channel access 2. Type 2A channel access 3. Type 2B channel access (FFS if move this to separate feature) 4. Type 2C channel access 5. 20MHz LBT bandwidth 6. Contention window adjustment 7. CP extension up to 1 symbol for PUSCH/PUCCH transmission 8. SSB/MIB/RMSI reception with Q 9. SSB RRM with Q in DMTC 10. SSB-RLM with Q in DMTC window 11. Support of RAR extension from 10ms to [40ms] by decoding of the 2-bit SFN indication in DCI 1_0 12. PRB interlace mapping for PUCCH and PUSCH |  | Per band | This can be a basic feature group for operating in unlicensed band with both DL and UL transmission support under dynamic channel access | Optional with capability signalling |
| 10-2 | UE stand-alone (DL and UL) operation in shared spectrum under semi-static channel access mode | 1. Type 2C channel access 2. Single sensing slot of 9us channel access 3. 20MHz LBT bandwidth 4. SSB/MIB/RMSI reception with Q 5. SSB RRM with Q in DMTC 6. SSB-RLM with Q in DMTC window 7. Support of RAR extension from 10ms to [40ms] by decoding of the 2-bit SFN indication in DCI 1_0 8. Support fixed frame period of 5ms and 10ms 9. PRB interlace mapping for PUCCH and PUSCH |  | Per band | This can be a basic feature group for operating in unlicensed band with DL only operation | Optional with capability signalling |

## Wideband operation (10-19/10-19a/10-19b/10-19c)

In the context of 10-19b, in previous meetings RAN1 has made the agreements listed below on the UL wideband operation. From them, it is straightforward that a UE will transmit UL channels using the RB sets that overlap with the resource allocation for the UL transmission. From this perspective, the UE only perform the transmission based on the LBT outcomes of the RB sets of interest. Since LBT operation is performed per RB set (LBT BW), we do not see any motivation to define the feature group of 10-19b.

Agreement: - For UL transmissions in a serving cell with carrier bandwidth greater than the LBT bandwidth, for the case where UE performs CCA before UL transmission, UE transmits on the UL only if CCA is successful at UE in all LBT bandwidths that overlap with the resource allocation for the UL transmission - The UE is not expected to receive resource allocations in discontiguous LBT bandwidths within a wideband carrier - This does not preclude such resource allocation in discontiguous LBT bandwidths being supported by specifications managed by RAN1 in Rel-16.

As for 0-19c, we do not have enough justification to support it due to the lack of discussion. Therefore, there is need to discuss further whether this is really needed. Also, it is not clear on the reason that the prerequisite is only 10-1a (only for DL-only operation under dynamic channel access mode).

Proposal 4:

- Remove 10-19b.

- Discuss more on the necessity of 10-19c.

| Index | Feature group | Components | Prerequisite feature groups | Type | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- |
| 10-19 | Number of LBT bandwidth FFS if this is needed | Number of ED measurements the UE is able to perform simultaneously [Whether the UE in WB operating mode can support nx20MHz measurement | 10-1 or 10-2 | Per band | This is the number of LBT bandwidth a UE can perform separate ED check on simultaneously | Optional with capability signalling |
| 10-19a | Support DL reception with subset of RB sets | 1. When DL BWP has multiple RB sets, support using the available RB set bitmap in DCI 2_0 to validate the periodic CSI-RS transmission if the CSI-RS is over multiple RB-sets | 10-1 or 10-2 | Per band | Without this capability, UE will assume all RB sets in the DL BWP are all transmitted or none of them are transmitted | Optional with capability signalling |
| [10-19c] | Suppport intra-cell guard band(s) for DL carrier BW > 20MHz | Support DL frequency domain resource allocation type 0 with PRG partially overlapped with Type-2 intra-cell gard band between two adjacent LBT RB sets where CCA is clear on only one of RB sets as defined by RAN4, and rate matching around the guard bands either at RB level (in case of wideband precoding) or at PRG level (in case of precoding granularity of 2 or 4 RBs) according to the information provided by DCI Format 2_0 available RB set bitmap and RRC intra-cell guard band configurations | 10-1a | [Per band] |  | Optional with capability signalling |

## Features for licensed use

We support that some of feature groups are used for licensed use, including 10-8 and 10-11. And also some of HARQ features seem beneficial for licensed use as well including enhanced dynamic HARQ codebook, one-shot HARQ ACK, and multi-PUSCH UL grant.

Proposal 5:

- Let 10-8/10-11/10-14/10-15/10-16/10-16a/10-17 be used for licensed band

| Index | Feature group | Components | Prerequisite feature groups | Type | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- |
| 10-8 | Type B PDSCH length | Length 3, 5, 6, 8, 11, 12, 13 FFS the capability are separate for each length or some groups are formed to signal the capability together |  | Per UE | Support of additional length (other than 2/4/7) for type B PDSCH. Note length 9/10 are already covered by 14-3 | Optional with capability signalling |
| 10-11 | SRS starting position at any OFDM symbol in a slot |  |  | Per UE | Support transmitting SRS starting in all symbols (0,…,13) of a slot | Optional with capability signalling |
| 10-14 | Non-numerical PDSCH to HARQ-ACK timing | 1. Support configuration of a value for dl-DataToUL-ACK indicating an imapplicable time to report HARQ ACK |  | Per UE | If non-numerical K1 value is supported | Optional with capability signalling |
| 10-15 | Enhanced dynamic HARQ codebook | 1. Support of bit fields signalling PDSCH HARQ group index and NFI in DCI 1_1 2. Support of bit field in DCI 0_1 for other group total DAI if configured. 3. Support the retransmission of HARQ ACK FFS if need to further split under other group DAI/NFI configured or not |  | Per UE | Enhanced dynamic HARQ codebook supporting grouping of HARQ ACK and triggering the retransmission of HARQ ACK in each groups | Optional with capability signalling |
| 10-16 | One-shot HARQ ACK feedback | 1. Support feedback of HARQ-ACK codebook containing all configured HARQ processes for all configured CCs, triggered by a DCI 1_1 scheduling a PDSCH |  | Per UE | Upon triggering, UE reports A/N for all HARQ processes and all CCs in a PUCCH group. | Optional with capability signalling |
| 10-16a | One-shot HARQ ACK feedback trigger with empty DCI 1_1 | 1. Support feedback of HARQ-ACK codebook containing all configured HARQ processes for all configured CCs with a DCI 1_1 without scheduling a PDSCH using a reserved FDRA value |  | Per UE |  | Optional with capability signalling |
| 10-17 | Multi-PUSCH UL grant | 1. Support of scheduling up to 8 PUSCH with a single DCI 0_1 |  | Per UE |  | Optional with capability signalling |

However, do not see any motivation to let the features for search space set group switching and search space/CORESET configuration in wideband to be used for licensed band. Those features were introduced to overcome the limitation of unlicensed band and we do not see any benefits when used for licensed operation.

Proposal 6:

- Do not open 10-9/10-9a/10-9b/10-20/10-20a for licensed use.

| Index | Feature group | Components | Prerequisite feature groups | Type | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- |
| 10-9 | Search space set group switching with explicit DCI 2_0 bit field trigger | 1. Two groups of search space sets 2. Monitor DCI 2_0 with a search space set switching field 3. Support a timer to switch back to original search space set group 4. Monitor DCI 2_0 for channel occupancy time and use the end of channel occupancy time to switch back to the original search space set group | 10-1, 10-1a, 10-2, or 10-2a | Per band | Being configured with two groups of search spaces, and switch between them. Some search space sets can be configured in both groups. | Optional with capability signalling |
| 10-9a | Search space set group switching with implicit PDCCH decoding with DCI 2_0 monitoring | 1. Two groups of search space sets 2. Support switching the search space set group with PDCCH decoding in group 1 3. Support a timer to switch back to original search space set group 4. Monitor DCI 2_0 for channel occupancy time and use the end of channel occupancy time to switch back to the original search space set group | 10-1, 10-1a, 10-2, or 10-2a | Per band | Being configured with two groups of search spaces, and switch between them. Some search space sets can be configured in both groups. | Optional with capability signalling |
| 10-9b | Search space set group switching with implicit PDCCH decoding without DCI 2_0 monitoring | 1. Two groups of search space sets 2. Support switching the search space set group with PDCCH decoding in group 1 3. Support a timer to switch back to original search space set group | 10-1, 10-1a, 10-2, or 10-2a | Per band | Being configured with two groups of search spaces, and switch between them. Some search space sets can be configured in both groups. | Optional with capability signalling |
| 10-20 | Support search space set configuration with freqMonitorLocation-r16 | 1. Support search space set configuration with freqMonitorLocations-r16 | 10-1, 10-1a, 10-2, or 10-2a | Per band |  | Optional with capability signalling |
| 10-20a | Support coreset configuration with rb-Offset | 1. Support coreset configuration with rb-Offset | 10-1, 10-1a, 10-2, or 10-2a | Per band |  | Optional with capability signalling |

# Conclusions

It is proposed to reflect the following proposals for NR-U UE features.

Proposal 1: Put the restriction of FR1 unlicensed band in the note of 10-1, 10-1a, 10-2, and 10-2a.

Proposal 2: remove 10-2b and change the component 8 of 10-2.

Proposal 3:

- Merge 10-3/10-3a/10-3b/10-3c into 10-1 and 10-2.

- If not merged, combine 10-3/10-3a/10-3b/10-3c into a single feature group and remove “Need discussion for licensed use”.

Proposal 4:

- Remove 10-19b.

- Discuss more on the necessity of 10-19c.

Proposal 5:

- Let 10-8/10-11/10-14/10-15/10-16/10-16a/10-17 be used for licensed band

Proposal 6:

- Do not open 10-9/10-9a/10-9b/10-20/10-20a for licensed use.

# References

[1] R1-2001484, “RAN1 UE features list for Rel-16 NR after RAN1#100-E” AT&T, NTT DOCOMO, inc.