3GPP TSG-RAN WG1 Meeting #100bis-e	R1-2002031

Online, 20th – 30th April, 2020

Agenda Item:	7.2.2.2.1

Source:	Ericsson

Title:	Channel Access Procedures

Document for:	Discussion, Decision

# 1	Introduction

This document is for the purposes of NR-U maintenance under the channel access procedures agenda item.

# 2	Discussion

## 2.1	CP extension before dedicated RRC

In RAR grant the 2-bit ChannelAccess-CPext field indicates channel access parameters. It should be clarified that the corresponding parameters are defined in Table 7.3.1.1.1-4 in TS 38.212.

Table 8.2-1: Random Access Response Grant Content field size

| RAR grant field | Number of bits |
| --- | --- |
| Frequency hopping flag | 1 |
| PUSCH frequency resource allocation | 14, for operation without shared spectrum channel access 12, for operation with shared spectrum channel access |
| PUSCH time resource allocation | 4 |
| MCS | 4 |
| TPC command for PUSCH | 3 |
| CSI request | 1 |
| ChannelAccess-CPext | 0, for operation without shared spectrum channel access 2, for operation with shared spectrum channel access |

Therefore, we propose the following:

Adopt the following TP for Table 8.2-1 in TS 38.213 to define channel access parameters indicated in RAR grant:

## 8.2 Random access response ## *** Unchanged text is omitted *** The ChannelAccess-CPext field indicates a channel access type and CP extension for operation with shared spectrum channel access [15, TS 37.213]. Table 8.2-1: Random Access Response Grant Content field size RAR grant field; Number of bits. Frequency hopping flag; 1. PUSCH frequency resource allocation; 14, for operation without shared spectrum channel access 12, for operation with shared spectrum channel access. PUSCH time resource allocation; 4. MCS; 4. TPC command for PUSCH; 3. CSI request; 1. ChannelAccess-CPext; 0, for operation without shared spectrum channel access 2, to indicate an entry in Table 7.3.1.1.1-4 in [5, TS 38.212] for operation with shared spectrum channel access. ## *** Unchanged text is omitted ***

## 2.2	UL Type 1 channel access

When a UE performs Type 1 channel access procedures for configured grant UL transmissions, the corresponding channel access priority class (CAPC) is determined based on the traffic class that the corresponding transmitted data as specified in TS 38.300. Based on the following agreements, the same mapping is applicable to UL transmissions scheduled by fallback DCI or RAR grant when the UE initiates a channel occupancy.

Agreement RAN1#99:

At least for LBE operation:

- For signaling of LBT type & CP extension for both Fallback DL assignment and Fallback UL Grant, the following table is used:

| LBT Type | CP extension |
| --- | --- |
| Cat1 16 µs |  | C2*symbol length – 16 us – TA |
| Cat2 25 µs |  | C3*symbol length – 25 us – TA |
| Cat2 25 µs |  | C1*symbol length – 25 us |
| Cat4 |  | 0 |

  - CAPC is not indicated explicitly:

    - For the UL grants

      - The UE assumes CAPC=4 was used by the gNB to acquire the CO,

      - For UE initiated COTs (Cat4 case) the UE may select the CAPC by itself.

        - Note: The mapping between priority classes and traffic classes follows the same mechanism as defined for UL CG transmissions.

    - Note: For PUCCH associated with DL assignments may use the highest priority CAPC when CAT4 LBT is used, as agreed earlier.

- If the network indicates FBE operation, for an indication of LBT type of Cat 2 25 us or Cat4 the UE follows the mechanism whereby one 9 microsecond slot is measured within a 25 microsecond interval as in 37.213.

Agreement RAN1#99:

- For RAR, we use the same LBT type and CP extension tables and CAPC selection mechanism as used for the Fallback DCI UL grant.

  - This requires 2 bits to be signaled in RAR (the PDSCH)

  - If the UE multiplexes user plane data with PUSCH

    - The UE assumes CAPC=4 was used by the gNB to acquire the CO,

    - For UE initiated COTs (Cat4 case) the UE may select the CAPC by itself.

      - Note: The mapping between priority classes and traffic classes follows the same mechanism as defined for UL CG transmissions.

  - The frequency domain resource allocation field can be reduced to accommodate 2 bits in the RAR

- Include this agreement in an LS to RAN2

Determination of the CAPC for the cases mentioned above is not specified in 37.213. The following text proposal aims to address this issue:

Adopt the following TP in TS 37.213 in subclause 4.2.1 for determination of CACP when a UE initiates a channel occupancy for UL configured grant transmissions or UL transmissions scheduled by fallback DCI or RAR grant:

4.2.1 Channel access procedures for uplink transmission(s) ## *** Unchanged text is omitted *** A UE shall use Type 1 channel access procedure for RACH only transmissions related to random access procedure that initiate a channel occupancy with UL channel access priority class p=1 in Table 4.2.1. When a UE uses Type 1 channel access procedures for PUSCH transmissions on configured resource, the UE determines the corresponding UL channel access priority p in Table 4.2.1 following the procedures described in Subclause X.X.:X in [TS 38.300]. When a UE uses Type 1 channel access procedures for PUSCH transmissions with user plane data indicated by fallback UL grant or related to random access procedure where the corresponding UL channel access priority p is not indicated, the UE determines p in Table 4.2.1 following the same procedures as for PUSCH transmission on configured resources using Type 1 channel access procedures. ## *** Unchanged text is omitted ***

# 3	Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	Adopt the following TP for Table 8.2-1 in TS 38.213 to define channel access parameters indicated in RAR grant:

## 8.2 Random access response ## *** Unchanged text is omitted *** The ChannelAccess-CPext field indicates a channel access type and CP extension for operation with shared spectrum channel access [15, TS 37.213]. Table 8.2-1: Random Access Response Grant Content field size RAR grant field; Number of bits. Frequency hopping flag; 1. PUSCH frequency resource allocation; 14, for operation without shared spectrum channel access 12, for operation with shared spectrum channel access. PUSCH time resource allocation; 4. MCS; 4. TPC command for PUSCH; 3. CSI request; 1. ChannelAccess-CPext; 0, for operation without shared spectrum channel access 2, to indicate an entry in Table 7.3.1.1.1-4 in [5, TS 38.212] for operation with shared spectrum channel access. ## *** Unchanged text is omitted ***

Proposal 2	Adopt the following TP in TS 37.213 in subclause 4.2.1 for determination of CACP when a UE initiates a channel occupancy for UL CG transmissions or UL transmissions scheduled by fallback DCI or RAR grant:

4.2.1 Channel access procedures for uplink transmission(s) ## *** Unchanged text is omitted *** A UE shall use Type 1 channel access procedure for RACH only transmissions related to random access procedure that initiate a channel occupancy with UL channel access priority class p=1 in Table 4.2.1. When a UE uses Type 1 channel access procedures for PUSCH transmissions on configured resource, the UE determines the corresponding UL channel access priority p in Table 4.2.1 following the procedures described in Subclause X.X.:X in [TS 38.300]. When a UE uses Type 1 channel access procedures for PUSCH transmissions with user plane data indicated by fallback UL grant or related to random access procedure where the corresponding UL channel access priority p is not indicated, the UE determines p in Table 4.2.1 following the same procedures as for PUSCH transmission on configured resources using Type 1 channel access procedures. ## *** Unchanged text is omitted ***