3GPP TSG RAN WG1 Meeting #100bis-e	R1-2002041

e-Meeting, April 20th-30th, 2020

Agenda Item:	7.2.4.2.2

Source:	FUTUREWEI

Title:	Remaining details on mode-2 resource allocation

Document for:	Discussion and decision

# Introduction

At RAN1#99, RAN1 completed the standardization work for NR V2X. Some details were left for the CR phase, with a list of open issues identified at RAN#86[1].  Some issues were addressed at RAN1#100-e, but a few remain to be corrected. In this contribution, we address the remaining issues on mode-2 resource allocation. More specifically, we discuss the time parameters in the sensing procedure. Note also that in addition, some agreements from RAN1#100-e need TP. Those are not addressed in this contribution since we except the Feature Lead to provide them.

# Discussion

## Details of T1, T2 T3 in the sensing and resource (re-)selection window

At RAN1#99, the broad strokes of the sensing procedure were agreed, but a lot of details still need to be ironed out. In particular, the timing parameters for the sensing: T0, T1, T2 and T3 are not fully defined yet, although decisions for T0 and T2 were taken:

- T2min is (pre-)configured per priority indicated in SCI from the following set of values:

  - {1, 5, 10, 20}*2µ, where µ = 0,1,2,3 for SCS 15,30,60,120 respectively

- T0 is (pre)-configured between: 1000+[100]ms and [100]ms

T3 was introduced in RAN1#98b:

- The resource selection window starts at time instance (n + T1), T1 ≥ 0 and ends at time instance (n + T2)

  - The start of selection window T1 is up to UE implementation subject to T1 ≤ Tproc,1

  - T2 is up to UE implementation with the following details as a working assumption:

    - T2 ≥ T2min

    - If T2min > Remaining PDB, then T2min is modified to be equal to Remaining PDB

    - FFS other details of T2min including whether the minimum window duration T2min - T1 is a function of priority

  - UE selection of T2 shall fulfil the latency requirement, i.e. T2 ≤ Remaining PDB

- A sensing window is defined by time interval [n – T0, n – Tproc,0)

  - T0 is (pre-)configured, T0 > Tproc,0 FFS further details

- FFS, if Tproc,0 and Tproc,1 are defined separately or as a sum

- FFS relation of T3, Tproc,0, Tproc,1

- Time instances n, T0, T1, T2, T2min are measured in slots, FFS Tproc,0 and Tproc,1

The illustration of timing is shown in Figure 1 (extracted from [3]).

Figure 1. Sensing timings

For the relationship between T3, Tproc,0, Tproc,1, the UE should be able to reselect as late as possible. The latest possible moment when this can be achieved is obtained for T3=Tproc,0+Tproc,1.

Next, we discuss the values of these parameters. Tproc,0 is related to the time it takes to decode the first, and possibly the second stage SCIs. This time is bounded by the PDSCH decoding time, thus we propose to use 1 slot for 15 and 30kHz, and 2 slots for 60 and 120kHz, as proposed in [4]. Similarly, Tproc,0 is related to the time it takes to prepare to send the PSSCH/PSCCH, and is bounded by the time it takes to prepare to send the PDSCH, thus the values of [3] can be used: 1 slot for 15kHz and 30kHz, 2 slots for 60kHz and 3 slots for 120 kHz. Note that [3] proposes to use one additional slot to account for the edge effects. While mathematically true, we do not think that it is necessary since the processing times intervals are defined by the chipset capabilities and are not tied to the slot duration. However, if the group wants to add one slot to Tproc,1, we would have no issue with that.

Proposal 1: the values for T3, Tproc,0, Tproc,1 are as follows:

| SCS, kHz | Tproc,0, slots | Tproc,1, slots | T3, slots |
| --- | --- | --- | --- |
| 15 | 1 | 1 | 2 |
| 30 | 1 | 1 | 2 |
| 60 | 2 | 2 | 4 |
| 120 | 2 | 3 | 5 |

# Conclusion

The remaining details of mode-2 were discussed. We propose the following:

Proposal 1: the values for T3, Tproc,0, Tproc,1 are as follows:

| SCS, kHz | Tproc,0, slots | Tproc,1, slots | T3, slots |
| --- | --- | --- | --- |
| 15 | 1 | 1 | 2 |
| 30 | 1 | 1 | 2 |
| 60 | 2 | 2 | 4 |
| 120 | 2 | 3 | 5 |

# References

[1] RP-193198, Task list for 5G V2X in RAN1#100, LGE, RAN#86, Sitges, Spain, Dec 9-12, 2019

[2] R1-2001170, FL summary of critical issues for 7.2.4.2.2 - V2X Mode 2, RAN1#100-e, Feb 24-Mar 6, 2020

[3] R1-1912205, Design of Resource Allocation Mode-2 for NR V2X Sidelink, RAN1#99, Reno, US, Nov 24-28, 2019

[4] R1-1912599, Resource allocation for Mode-2 transmissions, RAN1#99, Reno, US, Nov 24-28, 2019