3GPP TSG RAN WG1#100-e-Bis			R1-2002155

e-Meeting, 20th – 30th April, 2020

Agenda item:	7.2.11.6

Source: 	Samsung

Title: 	On UE features for eMIMO

Document for:	Discussion and Decision

Introduction

This contribution provides Samsung’s views on Rel.16 UE features for eNR-MIMO.

Multi-TRP

The UE capability signaling for Rel. 16 multi-TRP is discussed in the companion contribution [1]. The summary of the proposal is as follows.

Proposal 1: Support UE capability signaling of multi-DCI based multi-TRP per FSPC level.

Proposal 2: Support to have all the out-of-HARQ operations {PDCCH-to-PDSCH, PDSCH-to-HARQ-ACK, PDCCH-to-PUSCH} (basic components #4-#6) as basic components for FG 16-2a.

Proposal 3: Remove the basic component #8 from FG 16-2a.

Proposal 4: Remove the basic component #9 from FG 16-2a.

Proposal 5: Change optional component #7 in FG 16-2a as ‘Support of separate HARQ-ACK with intra-slot TDM’.

Proposal 6: Remain FG 16-2b-1 to FG-2b-5 as separate FGs, all of which are optional.

Proposal 7: Support to have component #1 in FG 16-2b-1.

MU-CSI

The UE capability signalling for Rel. 16 eTypeII and eTypeII port selection codebooks is discussed in the companion contribution [2]. The summary of the proposal is as follows.

Proposal 8: Regarding UE capability for Rel. 16 eTypeII and eTypeII port selection codebooks,

- The following is mandatory, hence doesn’t require additional capacity signalling.

  - Rank restriction

  - UCI omission

- The following (sub-)features requires additional capability signalling.

  - R=2 and N3<=2

  - L=6 for eTypeII codebook

  - Rank 3-4

  - Amplitude restriction – reuse Rel.15 parameter amplitudeSubsetRestriction

- The following and corresponding UE capability signalling is not supported.

  - UE capability signalling for concurrent codebooks

  - Maximum number of configured aperiodic CSI Report Settings

Full-power UL Tx

The UE capability signalling for Rel. 16 full power UL transmission is discussed in the companion contribution [3]. The summary of the proposal is as follows.

Proposal 9: Regarding UE capability for full power UL transmission,

- The following is mandatory, hence doesn’t require additional capacity signalling.

  - new UL codebook set(s) when the UE reports mode1 as its capability

  - TPMI group signalling when the UE reports mode2 as its capability

- The following (sub-)features requires additional capability signalling.

  - Separate capability signalling for mode0, mode1, and mode2

- The following and corresponding UE capability signalling is not supported.

  - Multiple modes, e.g., mode1AndMode2

Low-PAPR RS

In Rel-15, applicability of pi/2-BPSK for PUSCH or PUCCH was not mandated without capability signalling, i.e. a set of UE capability signalling on pi/2-BPSK has been supported from Rel-15. Given that efficiency of UL low PAPR sequences and/or modulation can be maximized when they are applied for both data and RS transmissions, we do not see a strong motivation to make Rel-16 low-PAPR RS as mandatory scheme for Rel-16 UEs.

Proposal 10: FG 16-6a and FG 16-6b should be an optional feature with capability signaling in Rel-16.

Conclusion

In this contribution, the following proposals are given:

Proposal 1: Support UE capability signaling of multi-DCI based multi-TRP per FSPC level.

Proposal 2: Support to have all the out-of-HARQ operations {PDCCH-to-PDSCH, PDSCH-to-HARQ-ACK, PDCCH-to-PUSCH} (basic components #4-#6) as basic components for FG 16-2a.

Proposal 3: Remove the basic component #8 from FG 16-2a.

Proposal 4: Remove the basic component #9 from FG 16-2a.

Proposal 5: Change optional component #7 in FG 16-2a as ‘Support of separate HARQ-ACK with intra-slot TDM’.

Proposal 6: Remain FG 16-2b-1 to FG-2b-5 as separate FGs, all of which are optional.

Proposal 7: Support to have component #1 in FG 16-2b-1.

Proposal 8: Regarding UE capability for Rel. 16 eTypeII and eTypeII port selection codebooks,

- The following is mandatory, hence doesn’t require additional capacity signalling.

  - Rank restriction

  - UCI omission

- The following (sub-)features requires additional capability signalling.

  - R=2 and N3<=2

  - L=6 for eTypeII codebook

  - Rank 3-4

  - Amplitude restriction – reuse Rel.15 parameter amplitudeSubsetRestriction

- The following and corresponding UE capability signalling is not supported.

  - UE capability signalling for concurrent codebooks

  - Maximum number of configured aperiodic CSI Report Settings

Proposal 9: Regarding UE capability for full power UL transmission,

- The following is mandatory, hence doesn’t require additional capacity signalling.

  - new UL codebook set(s) when the UE reports mode1 as its capability

  - TPMI group signalling when the UE reports mode2 as its capability

- The following (sub-)features requires additional capability signalling.

  - Separate capability signalling for mode0, mode1, and mode2

- The following and corresponding UE capability signalling is not supported.

  - Multiple modes, e.g., mode1AndMode2

Proposal 10: FG 16-6a and FG 16-6b should be an optional feature with capability signaling in Rel-16.

References

- R1-2002139, “On Rel.16 multi-TRP/panel transmission,” Samsung

- R1-2002136, “On maintenance of Rel.16 MU CSI enhancements”, Samsung

- R1-2002141, “On UL full power transmission”, Samsung