3GPP TSG RAN WG1 #100bis-e	R1-2002235

e-Meeting, April 20th – 30th, 2020

Agenda Item:	7.2.4.2.3

Source:	Ericsson

Title:	Other outstanding resource allocation issues

Document for:	Discussion, Decision

# 1	Introduction

In this paper we discuss issues related to the system-level coexistence between Mode-1 and Mode-2.

# 2	System-level coexistence of Mode 1 and Mode 2

In RAN1#99, the following was agreed:

Agreements: - For dynamic grant, DCI indicates the time-frequency resource allocation with the signalling format used for SCI. - In addition, the starting sub-channel for initial transmission is signalled in DCI. Agreements: - At least the following parameters are part of a SL configured grant configuration: - Configuration index of the CG - Time offset (for type-1 only) - Time-frequency allocation (for type-1 only) - Using the same format as in DCI. - Periodicity - The configured grant is associated with a single transmit resource pool. - RAN2 can add other parameters if deemed necessary by RAN2 - A UE in mode 1 is configured at least with one transmit resource pool - For type-2 CG, the time-frequency allocation and the configuration index of the CG are indicated in DCI. - All parameters for CG type 2 for activation DCI re-use the same respective parameters configured for CG type 1, when applicable

Looking at these agreements, we observe the following regarding coexistence between Mode-1 and Mode-2 in a pool of resources:

- Multiple transmissions of the same TB can be reserved in the same way for Mode-1 and Mode-2. That is, the same level of protection can be expected in both cases.

- Transmissions of multiple TBs cannot be reserved in the same way for Mode-1 and Mode-2. More specifically, the use of periodic reservations is not yet supported for Mode-1.

Unlike LTE Rel-15, the use of reservations for transmissions of multiple TBs is not supported for Mode-1.

In principle, the configuration options for the signalling in SCI (i.e., reservation field) and for the periodicity of a CG may be different. Nonetheless, for many configurations of interest the UE can make use of the reservation flag.

For configured grant (type-1, type-2), the UE sets the reservation period in SCI equal to the CG periodicity if the value is among the reservation periods that are configured for the pool.

# 3	Conclusion

In this document, we have observed the following:

Observation 1	Unlike LTE Rel-15, the use of reservations for transmissions of multiple TBs is not supported for Mode-1.

Based on the discussion in the previous sections we propose the following:

Proposal 1	For configured grant (type-1, type-2), the UE sets the reservation period in SCI equal to the CG periodicity if the value is among the reservation periods that are configured for the pool.