3GPP TSG-RAN WG1 Meeting #100bis-e	R1- 2002239

e-Meeting, April 20th - April 30th, 2020

Agenda Item:	7.2.4.6

Source:	Ericsson

Title:	QoS management for NR sidelink

Document for:	Discussion, Decision

# 1	Introduction

This contribution addresses the remaining issues related to QoS management and congestion control for NR SL Rel-16.

# 2	Discussion

During RAN1#98bis and RAN1#99 the following was agreed:

Agreements: - Congestion control can restrict the values of at least the following PSSCH/PSCCH TX parameters per resource pool: - Range of MCS for a given MCS table supported within the resource pool - Range of number of sub-channels - Upper bound of number of (re)transmissions – already agreed in mode 2 AI - Upper bound of TX power (including zero TX power) - Congestion control can set an upper bound on channel occupancy ratio (CR), CRlimit. - Ranges/bounds of the transmission parameters and CRlimit are functions of QoS and CBR. - In addition to congestion control (in use or not in use), the above parameters can be restricted by reusing the same mechanism as in LTE - For speed, further discussion on absolute vs. relative speed - FFS other parameter(s) that can be restricted - FFS whether or not to tie the speed with a UE capability Agreements: (from [99-NR-11]) - Only TX parameter restriction based on absolute speed can be (pre)configured in Rel-16.

Regarding the list of Tx parameters, since LTE V2X does not support limiting the Tx power based on UE speed, we believe the same principle applies in NR SL.

NR SL do not support limiting Tx power based on UE speed.

# 3	Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	NR SL does not support limiting Tx power based on UE speed.