3GPP TSG RAN WG1 Meeting #100bis-e	   R1-2002273

20th – 30th April, 2020

Agenda Item:     7.2.6.5

Source:	Spreadtrum Communications

Title:	Remaining issues on Low PAPR DMRS sequence in R16

Document for:	Discussion and decision

Introduction

In this contribution, we will present our opinion on the remaining issues for the low PAPR RS for PUCCH in TS 38.211[1]:

Discussion

In RAN1#96 meeting, there was agreement on PUCCH multiplexing capacity as following:

Agreement The PUCCH multiplexing capacity when Rel-16 DMRS is configured for pi/2 BPSK PUCCH is a single port

In Rel-15，cyclic shift in Rel-15 is used to generate multiple orthogonal DMRS port. As the agreement above, In Rel-16，PUCCH multiplexing capacity is single port for Rel-16 sequence, so there is no need to introduce cyclic shift in Rel-16 sequence.

In current spec, the low PAPR DMRS in Rel-16 as shown:

The low-PAPR sequence ru,v(α,δ)n is defined by a base sequence ru,vn according to ru,v(α,δ)n=ru,vn, 0≤n<M

the sequence generation function for Rel-16 DMRS does not introduce cyclic shift α, we think current description and agreement is aligned well. The statement of cyclic shift α in Section 6.4.1.1.3 of TS38.211 for PUCCH format 3 and 4 is really redundant. So we have the following text proposal:

Text Proposal: for Section 6.4.1.3.3.1 of 38.211:

------------------------------------------------Unchanged text omitted------------------------------------------------

- 6.4.1.3.3	Demodulation reference signal for PUCCH formats 3 and 4

- 6.4.1.3.3.1	Sequence generation

The reference-signal sequence rlm shall be generated according to

where MscPUCCH,s is given by clause 6.3.2.6.3 and ru,vα,δ(m) depends on the configuration:

-	if the higher-layer parameter DMRSuplinkTransformPrecodingPUCCH-r16 is configured, ru,vα,δ(m) is given by clause 5.2.3 with cinit given by clause 6.4.1.3.2.1. The sequence group u and the sequence number v depend on the sequence hopping in clause 6.3.2.2.1 and the cyclic shift α depends on the cyclic shift hopping in clause 6.3.2.2.2.

-	otherwise, ru,vα,δ(m) is given by clause 6.3.2.2.

The cyclic shift α varies with the symbol number and slot number according to clause 6.3.2.2.2 with  for PUCCH format 3 without interlaced mapping and obtained from Table 6.4.1.3.3.1-1 with the orthogonal sequence index n given by clause 6.3.2.6.3 for PUCCH format 3 with interlaced mapping and PUCCH format 4.

------------------------------------------------Unchanged text omitted------------------------------------------------

Conclusion

We have the following text proposals regarding to Rel-16 Low PAPR RS:

Proposal 1: Based on the above, we have the following text proposal for Section 6.4.1.3.3.1 of 38.211:

------------------------------------------------Unchanged text omitted------------------------------------------------

- 6.4.1.3.3	Demodulation reference signal for PUCCH formats 3 and 4

- 6.4.1.3.3.1	Sequence generation

The reference-signal sequence rlm shall be generated according to

where MscPUCCH,s is given by clause 6.3.2.6.3 and ru,vα,δ(m) depends on the configuration:

-	if the higher-layer parameter DMRSuplinkTransformPrecodingPUCCH-r16 is configured, ru,vα,δ(m) is given by clause 5.2.3 with cinit given by clause 6.4.1.3.2.1. The sequence group u and the sequence number v depend on the sequence hopping in clause 6.3.2.2.1 and the cyclic shift α depends on the cyclic shift hopping in clause 6.3.2.2.2.

-	otherwise, ru,vα,δ(m) is given by clause 6.3.2.2.

The cyclic shift α varies with the symbol number and slot number according to clause 6.3.2.2.2 with  for PUCCH format 3 without interlaced mapping and obtained from Table 6.4.1.3.3.1-1 with the orthogonal sequence index n given by clause 6.3.2.6.3 for PUCCH format 3 with interlaced mapping and PUCCH format 4.

------------------------------------------------Unchanged text omitted------------------------------------------------

References

3GPP TS 38.211: "NR; Physical channels and modulation", g10