3GPP TSG-RAN WG1 Meeting #100bis-e	Tdoc R1-2002279

e-Meeting, April 20th – 30th 2020

Agenda Item:	7.2.11.11

Source:	Ericsson

Title:	UE features for CLI/RIM

Document for:	Discussion, Decision

# 1	Introduction

In this contribution we discuss UE features for CLI/RIM and the discussion is based on the features as defined in R1-2001484[1]. The following feature groups were defined for CLI-RIM

| 17. NR_CLI_RIM | 17-1 | CLI-RSSI measurement | 1) Support CLI-RSSI measurement, The max number of resources across all CCs configured to measure RSSI simultaneously shall not exceed 64 2) Up to 64 of measurement resource for CLI-RSSI measurement 2) Subcarrier spacing for CLI-RSSI measurement is same as subcarrier spacing for active BWP. |  |  | N/A |  | [Per UE] | TDD only | Yes |  | UE operates CLI-RSSI measurement. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 17-2 | SRS-RSRP measurement | 1) Support SRS-RSRP measurement, The max number of SRS resources across all CCs configured to measure SRS-RSRP simultaneously shall not exceed 32 2) Up to 32 of SRS resources to be monitored by a UE |  |  | N/A |  | [Per UE] | TDD only | Yes |  | UE operates SRS-RSRP measurement. | Optional with capability signalling |
|  | 17-3 | Simultaneous reception of PDSCH and CLI-RSSI measurement resource | Support simultaneousRxPDSCHCLIRSSIMeasResource if not supported that UE only receives CLI-RSSI resources’ | 17-1 |  | N/A |  | Per UE | TDD only | Yes |  | UE can assume that PDSCH is FDMed with CLI-RSSI measurement resource(s) | Optional with capability signalling |
|  | 17-4 | Simultaneous reception of PDSCH and SRS-RSRP measurement resource | Support simultaneousRxPDSCHSRSRSRPMeasResource if not supported that UE only receives SRS-RSRP resources | 17-2 |  | N/A |  | Per UE | TDD only | Yes |  | UE can assume that PDSCH is FDMed with SRS RSRP measurement resource(s). | Optional with capability signalling |

# 2	Discussion

During the email discussion of the UE features, there were proposals to allow UEs to indicate the maximum supported number of resources across all CCs configured to measure PDSCH CLI-RSSI(17-1) and SRS-RSRP(17-2) simultaneously.  This we consider an unnecessary fragmentation of the UE population. Similarly, there were proposals to change the indication from “Per UE” to “Per band”. The motivation for only supporting this function in certain bands are unclear. Hence, we propose:

The definitions of 17-1 and 17-2 to are kept as they are with no indication of the number of measurements supported and “Per UE” indication

According to our understanding, 17-3 and 17-4 have already been defined by RAN2. Hence, it should be clearly indicated to RAN2 that these are not additional capabilities but contain definitions by RAN1.

Clarify for RAN2 that 17-3 and 17-4 are the same capabilities as already defined by RAN2 and that the RAN1 definitions should be included when finalizing these.

# 3	Conclusion

In this contribution we discuss UE features for CLI/RIM and propose the following:

Proposal 1	The definitions of 17-1 and 17-2 to are kept as they are with no indication of the number of measurements supported and “Per UE” indication

Proposal 2	Clarify with RAN2 that 17-3 and 17-4 are the same capabilities are already defined by RAN2.

# References

R1-2001484, RAN1 UE features list for Rel-16 NR after RAN1#100-E, AT&T, NTT DOCOMO, INC.