3GPP TSG RAN WG1 #100bis			R1-2002328

e-Meeting, April 20th – 30th, 2020

Agenda Item:	7.2.4.6

Source:	Apple

Title:	On Remaining Details of Sidelink Channel Occupancy Ratio

Document for:	Discussion/Decision

# Introduction

The Release-16 NR V2X specifications were approved in Dec. 2019 [1]. There are still some remaining tasks for NR V2X, which were identified in [2], where some of these tasks were discussed in RAN1 meeting #100-e [3].

In this contribution, we discuss one remaining issue of sidelink channel occupancy ratio (CR).

# Discussion

The details of sidelink channel occupancy ratio (CR) have been discussed in RAN1 meeting #100-e [3]. The constraints on past/future window in CR evaluation and CR processing time were agreed. One remaining issue is whether the reserved and then released resources, due to HARQ-ACK feedback, are counted as granted or used resources in CR evaluation. In our view, since these resources are (or will be) released by UE, they should not be counted as granted or used in sidelink CR evaluation. Furthermore, the reserved resources, which are preempted by another UE, should also not be counted as granted/used in sidelink CR calculation. Hence, we have the following proposal.

Proposal 1: In evaluating sidelink CR, the reserved but released resources, due to HARQ ACK feedback or pre-emption, are not counted as granted or used.

# Conclusion

One remaining issue on sidelink CR was discussed. Our proposal is as follows:

Proposal 1: In evaluating sidelink CR, the reserved but released resources, due to HARQ ACK feedback or pre-emption, are not counted as granted or used.

# References

- Chairman’s Notes, 3GPP TSG RAN #86 Meeting, Sitges, ES, Dec. 2019.

- RP-193198, Task list for 5G V2X in RAN1 #100, Sitges, ES, Dec. 2019.

- Chairman’s Notes, 3GPP TSG RAN1 WG1 #100-e Meeting, Feb. 2020.