3GPP TSG-RAN WG1 Meeting #100bis	Tdoc R1-2002495

e-Meeting, April 20th – 30th 2020

Agenda Item:	7.2.11.9

Source:	Ericsson

Title:	Mobility UE features

Document for:	Decision

# 1	Introduction

In this contribution, we discuss the UE features for mobility enhancements.

# 2	Discussion

In [1], the outcome of the email discussion for the UE features for Release-16 are captured. Here we provide input to the discussion on the UE features related to mobility enhancements.

The currently proposed features for mobility enhancements are listed below. Our proposed changes are introduced directly in Table 1, and commented below.

Table 1: Proposed updates for UE features related to multi-beam enhancements.

| Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know | Applicable to V2X | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 21-1 | DAPS HO | For the Indicated support of intra-frequency DAPS-HO for a given [band] and indicated support of inter-frequency DAPS-HO for a given [band combination]. 1) Indicates support of simultaneous DL reception of PDCCH and PDSCH from source and target cell in DAPS-HO 2) Indicates support of PDCCH blind decoding capability in the first MCG and second MCG. | DAPS (Note: RAN2 feature) | Yes | N/A | The network cannot configure UE with DAPS HO | [Per BC] | No | N/A | N/A |  | [Optional with capability signalling] |
| 21-2 | UE power sharing for DAPS HO | ALT 1) Indicates support of dynamic UL power sharing during DAPS-HO operation. ALT 2) Indicates support of UL power sharing mode during DAPS-HO operation. | DAPS (Note: RAN2 feature) | Yes | N/A | ALT 1) The UE is only able to perform semi-static power allocation for source and target cell, or to drop the transmission to the source. ALT 2) The UE is only able to to drop the transmission to the source. | [Per BC] | No | N/A | N/A |  | ALT 1) Optional with capability signalling ALT 2) Optional with capability signalling {values: Semi-static-mode1, Semi-static-mode2, Dynamic} |
| 21-2a | UL transmission cancellation | Indicates support of cancelling UL transmission to the source cell | DAPS (Note: RAN2 feature) | Yes | N/A | The UE does not expect UL transmissions to source cell and target cell overlap. The indication is signalled per pair of bands per band combination. | [Per UE] | No | Yes | N/A |  | Optional with capability signalling |
| 21-3 | PDCCH blind detection for MCG1 and MCG2 | Indicates PDCCH blind decoding capability supported in the first MCG (intended for target cell in HO) and second MCG (intended for source cell in HO) when in DAPS-HO. Value range for both cells should be {1, …, 15}. | DAPS (Note: RAN2 feature) | Yes | N/A | The network may not able to configure UE with DAPS HO, as parameters required for PDCCH monitoring is essential for defining UE behaviour | [Per UE] | No | Yes | N/A | This capability (although optional) parameter value must be conveyed to the network in order to correctly define UE behaviour | Optional with capability signalling For first MCG {values: 1, 3, … 15} For second MCG {values: 1, 3, … 15} |
| 21-4 | PDCCH blind detection for MCG2 | Indicates PDCCH blind decoding capability supported in the second MCG (intended for source cell in HO) when in DAPS-HO | DAPS (Note: RAN2 feature) | Yes | N/A | Same as for 21-3 | [Per UE] | No | Yes | N/A | Same as for 21-3 | Optional with capability signalling {values: 1, 3, … 15} |

UE power sharing for DAPS HO: Of the two options, we prefer Alt2: this is more in line with how the RAN1 specification is currently formulated: 38.213 already describes how to interpret the values {Semi-static-mode1, Semi-static-mode2, Dynamic} The specification also describes what would happen if the UE does not report any value for the feature: the UE would drop the transmission to the source.

UL transmission cancellation: This feature is not needed, RAN1 has already agreed that the UE should be able to drop in case there is an overlap.

PDCCH blind detection: Our understanding is that the BD capabilities should be signalled separately for MCG1 and MCG2. Hence, two feature groups are needed.

For UE features related to mobility enhancements, agree on the updates captured in Table 1.

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	For UE features related to mobility enhancements, agree on the updates captured in Table 1.

# References

R1-2001484, RAN1 UE features list for Rel-16 NR after RAN1#100-E, AT&T, NTT DOCOMO INC., RAN1 #100-e, February 2020