3GPP TSG RAN WG1 Meeting #100bis-e                                                                  		R1-2002595

E-meeting, April 20 - April 30, 2020

Agenda Item:	7.2.11.10

Source:	Huawei, HiSilicon

Title:	Rel-16 UE features for MR-DC/CA

Document for:	Discussion and Decision

Introduction

In this contribution, we will provide some views towards the MR-DC/CA related part.

Discussion

- FG 18-1

The new added FG [18-1] with bracket should be removed. FG [18-1] is about Synchronous NR-DC operation, which should not be separated from FG18-1a since it is a special case of FG18-1a (non-SFN sync NR-DC), and RAN plenary decision RP-192345 mandates all Rel-16 UEs to support non-SFN sync NR-DC.

Proposal 1: The new added FG 18-1 with bracket should be removed

- FG 18-2

The “intra-band” in the name of FG 18-2 should be removed, because it is not aligned with WID nor agreements. In the WID or agreements of RAN1 for single TX enhancement, there is no limitations that the MCG and the SCG should be in the same band.

“3) HARQ subframe offset” should be modified to “HARQ subframe offset (Optional feature with Rel-15 FG 8-1 as prerequisite feature)”. This is exactly shown in the agreement, since it should be a separate capability signaling for HARQ subframe offset. One alternative is to take this component out and create a new FG for it.”

There should be a space between “single” and “UL” as captured in TS 38.306. Also its Italic format should be removed. This comment is also applied to FG 18-2a.

“Component 5” about association with 38.101-3 should be removed because it causes big confusion by overlapping with component 4, and not sustained by any agreement. This comment is also applied to FG 18-2a.

Proposal 2: For FG 18-2 the following should be accepted.

- The “intra-band” in the name of FG 18-2 should be removed

- “3) HARQ subframe offset” should be modified to “HARQ subframe offset (Optional feature with Rel-15 FG 8-1 as prerequisite feature)”

- There should be a space between “single” and “UL”

- “Component 5” about association with 38.101-3 should be removed, as well as in FG 18-2a

- FG 18-2a

“TDD” should be removed from “Applicable to in FDD-LTE TDD-NR EN-DC”, which is not aligned with agreements.

Proposal 3: Remove “TDD” from “Applicable to in FDD-LTE TDD-NR EN-DC” in FG 18-2a.

- FG 18-3a

Add 18-3 as prerequisite according to the RAN1#99 agreement

Proposal 4: Add 18-3 as prerequisite in FG 18-3a according to the RAN1#99 agreement.

- FG 18-4/4a

FG 6-5/6-6 should be as pre-requisite

Change the reporting type from ‘per UE’ to ‘per BC’, and at this stage please put per UE in bracket.

Proposal 5: FG 6-5/6-6 should be as pre-requisite and Change the reporting type from ‘per UE’ to ‘per BC’ in FG 18-4/4a

- FG 18-8

There is no need to restrict to same SCS case only with only 6-7 as the pre-requisite, so 6-8 can also be pre-requisite FG and/or the condition for mandatory support.

Proposal 6: There is no need to restrict to same SCS case only with only 6-7 as the pre-requisite in FG 18-8

In summary, all proposed changes above are incorporated into Table 1.

Proposal 7: In summary, adopt all changes in Table 1.

Table 1 Aggregated changes

| Index | Feature group | Components | Prerequisite feature groups | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 18-2 | Single UL TX operation for TDD PCell in EN-DC | TDM restriction to LTE TDD PCell in EN-DC for singleUL-Transmission associated functionality when tdm-patternConfig-r16 is configured 1) TDD UL/DL configuration#2, #4, #5 configured as DL-reference UL/DL configuration 2) PRACH transmission in non- designated UL subframes given by the DL-reference configuration (for type 1 UE) 3) HARQ subframe offset (Optional feature with Rel-15 FG 8-1 as prerequisite feature) 4) LTE UL transmissions scheduled/triggered by a DCI in any UL subframe of the TDM pattern |  | Per band combination | Applicable to TDD-TDD EN-DC only | Applicable to FR1 only | Extension of the R15 capability tdm-Pattern to TDD PCell 3) {not supported, supported} | Optional with capability signalling |
| 18-2a | Enhanced single UL TX operation for FDD Pcell EN-DC | TDM restriction to LTE FDD Pcell in EN-DC for singleUL-Transmission associated functionality when tdm-patternConfig-r16 is configured 1) DL-reference UL/DL configuration defined for LTE-FDD-SCell in LTE-TDD-FDD CA with LTE-TDD-PCell 2) PRACH transmission in non- designated UL subframes given by the DL-reference configuration (for type 1 UE) 3) HARQ sub-frame offset 4) LTE UL transmissions scheduled/triggered by a DCI in any UL subframe of the TDM pattern (for type 1 UE) |  | Per band combination | Applicable to in FDD-LTE NR EN-DC | Applicable to FR1 only | Enhancement to the R15 capability tdm-Pattern | Optional with capability signalling |
| 18-3a | Semi-statically configured LTE UL transmissions in all UL subframes not limited to tdm-pattern | UE configured with tdm-patternConfig-r16 can be semi-statically configured with LTE UL transmissions in all UL subframes not limited to the reference tdm-pattern | 18-2, 18-2a, 18-3 | Per UE |  |  |  | Optional with capability signaling |
| 18-4 | SCell dormancy within active time | Support for SCell dormancy indication sent within the active time on PCell with DCI format 0_1/1_1 | FG 6-5/6-6 | Per BC | No | No |  | Optional with capability signalling |
| 18-4a | SCell dormancy outside active time | Support for SCell dormancy indication sent outside the active time on PCell with DCI format 2_6 | [19-1] , FG 6-5/6-6 | Per BC | No | No |  | Optional with capability signalling |
| 18-8 | HARQ-ACK codebook type and HARQ-ACK spatial bundling configuration per PUCCH group | HARQ-ACK codebook type and HARQ-ACK spatial bundling configuration per PUCCH group | 6-7 | Per UE | No | No | Support HARQ-ACK codebook type and HARQ-ACK spatial bundling configuration per PUCCH group. Rel-15 had this per cell group | [Optional with capability signalling or Mandatory conditioned to support for multiple PUCCH groups 6-7] |

Conclusions

In this contribution, we discuss the issues for Rel-16 UE feature groups corresponding to EN-DC NR-DC, and CA with the following proposals.

Proposal 1: The new added FG 18-1 with bracket should be removed

Proposal 2: For FG 18-2 the following should be accepted.

- The “intra-band” in the name of FG 18-2 should be removed

- “3) HARQ subframe offset” should be modified to “HARQ subframe offset (Optional feature with Rel-15 FG 8-1 as prerequisite feature)”

- There should be a space between “single” and “UL”

- “Component 5” about association with 38.101-3 should be removed, as well as in FG 18-2a

Proposal 3: Remove “TDD” from “Applicable to in FDD-LTE TDD-NR EN-DC” in FG 18-2a.

Proposal 4: Add 18-3 as prerequisite in FG 18-3a according to the RAN1#99 agreement.

Proposal 5: FG 6-5/6-6 should be as pre-requisite and Change the reporting type from ‘per UE’ to ‘per BC’ in FG 18-4/4a

Proposal 6: There is no need to restrict to same SCS case only with only 6-7 as the pre-requisite in FG 18-8

Proposal 7: In summary, adopt all changes in Table 1.

- References

R1-2001484, “RAN1 UE features list for Rel-16 NR after RAN1#100-E ”, 3GPP, RAN1#100-e