3GPP TSG-RAN WG1 Meeting #100b-e	R1-2002621

e-Meeting, April 20th – April 30th , 2020

Agenda Item:	7.2.8.2

Source:	Ericsson

Title:	maintenance of Rel-16 UL reference signals for NR positioning

Document for:	Discussion, Decision

# Introduction

This contribution address  maintenance issues pertaining to UL reference signals for NR positioning.

# SRS maintenance for release 16

## SRS performance evaluation and potential improvements

The positioning error performance of the SRS was evaluated with a comb 2 and 8 cyclic shifts using the release 16 baseline. Each drop of UEs is done such that all UEs in the same cell are allocated an orthogonal resource.

The baseline result was compared with two potential improvements. First the allocation of cyclic shift is done so that the distance between cyclic shifts in time is maximized over the SRS resource (compared to over the srs symbol in rel16). Second the sequence generation is done per resource instead of per symbol in rel16. The proposed solutions are given in section 2.2 and 2.3. in Figure 1, we show that the performance improves greatly by changing the way cyclic shifts are assigned to maximize the distance between cyclic shifts. The reason for this improvement is a reduction of the amount of ISI generated by the delay spread of the channel overlapping between users on the same comb and only separated by cyclic shift. The gain in performance is around 12 meters in the 90th percentile.  Moreover, additional improvement can be obtained by extending the SRS ZC sequence to span the whole resource instead of a single symbol.

The separation between cyclic shifts is a limiting factor in performance of the rel16 SRS for positioning

When the gnodeB receives SRS for UL RTOA measurement, it will utilise the whole SRS resource to maximize the TOA range, i.e. it will collapse all symbol in the resource into a single symbol with TOA Range T*min⁡(1,NsymbSRSKTC).  Currently, each of the symbols in the SRS resource are generated independently with distinct ZC low-PAPR sequences. When using each symbol with the same comb offset (rel15 legacy behavior), this allows some diversity between symbol and is desirable. However, when the different symbols in the resource have a different comb offset, the whole resource will be used to measure the TOA. In that case, the whole resource cross correlation should be optimized.

The cross correlation  between SRS resources is a limiting factor in performance of the rel16 SRS for positioning

Figure 1 Performance of release 16 SRS for positioning baseline (yellow curve) and proposed enhancements to the cyclic shift implementation (blue curve) and to the ZC sequence length (red line)

## Cyclic Shift allocation for staggered SRS

The allocation of cyclic shifts over a combed signal such as the SRS folds the cyclic shift range within the comb duration, so that the time-domain representation of the cyclic shift is periodic with period TKTC. In the legacy rel-15 SRS, the cyclic shift is applied to each symbol equally. Figure 2 illustrates how the cylic shift is applied on the frequency grid

Figure 2 Cyclic shift allocation for SRS. In this example, the resource spans 4 symbols.

While this is the correct behavior for the non-staggered SRS comb, it should be modified when a fully or partially staggered pattern is used, to utilize the full range of the comb-1 equivalent symbol obtained by the combination of all symbols in the SRS resource.  One way to resolve this is to have the sequence distributed over the full SRS resource so that each symbol in the pattern features a part of the cyclic shift sequence.  Doing so would space each SRS resource every T/(nSRScs,max)  instead of T/(KTCnSRScs,max)  in rel15.

The current implementation of cyclic shift for SRS is as follow [38.211]:

ru,v(α,δ)(n)=ejαnru,v(n),0≤n<MZC,  with MZC=mNscRBKTC

For SRS bandwidth covering more than 36 active subcarriers (based on the size of the comb, the corresponding bandwidth is (6PRBs for comb 2,12 PRB for comb 4, 24PRBs for comb 8), the SRS is using a  Zadoff Chu (ZC) base sequence. Shorter base sequences use computer generated sequences (CGS).  The sequence is then mapped onto to the time-frequency grid, accounting for the comb size with

r(pi)n,l'=ru,v(αi,δ)n0≤n≤Msc,bRS-1l'∈0,1,...,NsymbSRS-1

where  δ=log2KTC and KTC is the comb factor. So that the SRS is mapped on the time frequency grid as:

aKTCk'+k0(pi),l'+l0(pi)=1NapβSRSr(pi)(k',l')k'=0,1,…,Msc,bRS-1l'=0,1,...,NsymbSRS-10otherwise

where length of the sounding reference signal sequence Msc,bRS is given by

Msc,bRS=mSRS,bNscRBKTC

where   mSRS,b is the bandwidth allocation in number of PRBs,  NscRB is the PRB size.

As mentioned earlier, the current implementation of the CS multiplexes different shifts over a time span of TKTC, so that each cyclic shift is equaly spaced by T/(KTCnSRScs,max)  .  This is inefficient as we could either extend the space between cyclic shifts to improve separation between signals, or extend the number of multiplexed signals while keeping  the same separation between signals.

In Figure 3, we show a possible allocation that would extend the space between cyclic shifts by utilizing the whole duration of the comb-1 equivalent SRS.

Figure 3 Allocation of cyclic shifts to utilize the full duration of the SRS symbol

To utilize the extended range from the staggered SRS resource, the SRS equation for sequence generation needs to be made dependent of both the frequency index  and the symbol index, so that the generated sequence ru,v(αi,KTC,l')n spans the whole resource bandwidth KTCMsc,bSRS:

| ru,v(αi,KTC,l')n= ejα(KTC*n+ koffsetl')ru,v(n) | (1) |
| --- | --- |

0≤n≤ Msc,bSRS-1

l'∈0,1,…,NsymbSRS-1

- The cyclic shift  of  the UL SRS with staggered pattern can be configured 1) per symbol, according to REL-15 behaviour or 2)  per SRS resource, across all symbol in the SRS resource, according to equation 1 above

- Text proposal 1 is endorsed in TS 38.211

## Issue with ZC sequence

With the SRS for positioning, the SRS resource is used to form a comb-1 symbol that maximizes the range of the reference signal. With this use in mind, the construction of the RS with independent ZC sequences per symbol is sub-optimal as it yields a higher cross correlation property compared to stretching the ZC sequence over all symbols. When sequence hopping is not configured, it is beneficial to allow to extend the sequence to the whole resource length instead of the symbol length.

The ZC sequence used to create the SRS for positioning can be configured to span either a symbol bandwidth of length  Msc,bSRS or the whole comb-1 equivalent resource length, i.e.  Msc,bSRSKTC

## Priority handling for SRS for positioning

During the RAN1#100e e-meeting, it was proposed to have a dynamic handling of the priority of aperiodic SRS for positioning. This proposal mirrored a discussion happening for URLLC, where high and low priority indication for UL transmission is an open issue.

The baseline dropping rule for aperiodic SRS for positioning is that aperiodic SRS for positioning should be dropped when colliding with PUSCH. Normally, if the application for which positioning is configured is not latency sensitive, the SRS dropping rule does not degrade the positioning significantly and allows to preserve the pusch transmission. However, UEs may be configured with low priority PUSCH.  In that case the PUSCH is known to be tolerant to latency. At the same time the aperiodic SRS for positioning could also be used in a latency sensitive case, e.g. industrial solutions.  In that case it would be of interest of prioritizing the SRS for positioning. Thus indication of the priority for of the SRS can provide a benefit.

Priority can already be indicated in DCI for the transmission of PUSCH or PDSCH, using a [priority indicator] field present in the scheduling DCI. This bit currently applies to the PUSCH or PDSCH transmission, but it could be extended to apply to the SRS triggered by the same DCI (if the SRS trigger is present).  Since the priority indicator is already specified, applying this bit to SRS for positioning  would come at no cost in overhead.

- The aperiodic SRS priority follows the priority bit in DCI, if configured. Otherwise, the aperiodic SRS is considered to be low priority.

# Conclusion

In the previous sections we made the following observations:

Observation 1	The separation between cyclic shifts is a limiting factor in performance of the rel16 SRS for positioning

Observation 2	The cross correlation  between SRS resources is a limiting factor in performance of the rel16 SRS for positioning

Observation 3	The ZC sequence used to create the SRS for positioning can be configured to span either a symbol bandwidth of length  Msc,bSRS or the whole comb-1 equivalent resource length, i.e.  Msc,bSRSKTC

Based on the discussion in the previous sections we propose the following:

Proposal 1	The cyclic shift  of  the UL SRS with staggered pattern can be configured 1) per symbol, according to REL-15 behaviour or 2)  per SRS resource, across all symbol in the SRS resource, according to equation 1 above

Proposal 2	Text proposal 1 is endorsed in TS 38.211

Proposal 3	The aperiodic SRS priority follows the priority bit in DCI, if configured. Otherwise, the aperiodic SRS is considered to be low priority.

# References

RP-190752, “New WID: NR Positioning Support”, Intel Corporation, Ericsson, Shenzhen, China, March, 2019

# Text Proposals for Specification CRs

Text proposal 1 (38.211):

### 5.2.4 Low-PAPR sequence generation type 3 The low-PAPR sequence ru,vαi,δ,l'n is defined by a cyclic shift αi of a base sequence ru,vn according to ru,vα,δn= ejα2δ*n+ koffsetl'ru,vn, 0≤n≤MZC-1 where MZC=mNscRB2δ is the length of the sequence, l'is a symbol index defined in section 6.4.1.4.2 and koffsetl'is defined in section 6.4.1.4.3. the definition of the base sequence ru,vn is given in section 5.2.2. -----unchanged text omitted ------ ##### 6.4.1.4.2 Sequence generation The sounding reference signal sequence for an SRS resource shall be generated according to r(pi)n,l'=ru,v(αi,δ)n 0≤n≤Msc,bSRS-1 l'∈0,1,…,NsymbSRS-1 where Msc,bSRS is given by clause 6.4.1.4.3, ru,vα,δ(n) is given by clause 5.2.4 if SRS is configured with [SRS for positioning], clause 5.2.2 otherwise, with δ=log2KTC and the transmission comb number KTC∈2,4,8 is contained in the higher-layer parameter transmissionComb. -----unchanged text omitted ------