3GPP TSG RAN WG1 #100b-e	R1-2002686

April 20th – 30th, 2020

E-Meeting

Agenda item:	7.2.11.11

Source: 	Intel Corporation

Title: 	UE features in CLI/RIM

Document for:	Discussion and decision

# Introduction

In R1-2001484, the following table is used as baseline for CLI-related UE feature discussion in e-meeting. [1]

This contribution provides our views on CLI-related UE features.

# CLI-related UE feature list

In RAN1 #96 [2], the following agreements have been made

One or multiple resources for CLI-RSSI measurement can be configured

- Maximum number of measurement resource for CLI-RSSI measurement is 64.

For SRS-RSRP, the total number of SRS resources to be monitored by a UE should not exceed 32.

The constraint is for the maximum number of CLI measurement resources configured to a UE. It is not the constraint for the maximum number of CLI measurement resource to be measured simultaneously by a UE. As illustrated in the following figure, if a UE is configured with 64 SRS-RSRP measurement resources but there are only 4 SRS-RSRP measurement resources would be measured simultaneously, then such configuration still violates the agreement made in RAN1 #96, as the total number of SRS-RSRP resources configured exceeds the limit of 32.

Therefore, we propose to rephrase the first component in FG 17-1 and 17-2 as “the maximum number of CLI measurement resources across all CCs shall not exceed” 64 for CLI-RSSI and 32 for SRS-RSRP.

Proposal 1: Rephrase the first component in FG 17-1 and 17-2 as follows:

- FG 17-1: Support CLI-RSSI measurement. The maximum number of CLI-RSSI measurement resources configured for a UE across all CCs shall not exceed 64.

- FG 17-2: Support SRS-RSRP measurement. The maximum number of SRS-RSRP measurement resources monitored by a UE across all CCs shall not exceed 32.

In R1-1903835 [3], the value range of nrofSRS-Ports in SRS-RSRP-Measurement-ResourceConfig is listed as “1, [2], [4]”.

In TS 38.331 version 16.0.0 [4], section 6.3.2, the field description for nrofSRS-Ports in RRC IE SRS-Resource states: “For CLI SRS-RSRP measurement, the network always configures this parameter to 'port1'.”

Therefore, only single-port SRS can be configured for SRS-RSRP measurement in NR R-16. RAN1 should decide whether to support two-port or four-port SRS-RSRP measurement. If RAN1 decided to support multi-port SRS-RSRP measurement, then the constraint that only single-port SRS can be configured for SRS-RSRP measurement should be removed in TS 38.331.

Proposal 2: Discuss and decide whether NR supports multi-port SRS-RSRP measurement. A LS to RAN2 would be needed to inform RAN1’s final decision.

In RAN1 #96, the following agreements have been made

Note: Depending on UE capability discussions, UE may not be required to assume that PDSCH is FDMed with SRS measurement resource

Note: Depending on UE capability discussions, UE may not be required to assume that PDSCH is FDMed with CLI-RSSI measurement resource(s)

If a UE does not support simultaneous reception of PDSCH and CLI measurement, then the FDMed PDSCH and CLI measurement should be regarded as an error case. If a UE does not support this feature but FDMed PDSCH and CLI measurement are configured by network anyway, UE behavior in the error case was not discussed in RAN1. RAN2 and RAN4 have not reach consensus on such UE behavior. RAN2’s agreement prioritizes PDSCH reception over CLI measurement, but RAN4’s scheduling restriction essentially prioritizes CLI measurement. In R1-2001523 [5], RAN4 has asked RAN2 to reconsider UE behavior.

In TS 38.133 version 16.2.0 [6], section 9.7.4, following scheduling restrictions are specified when UE is performing CLI measurements:

For FR1,

- Depending on UE capability, the UE is not expected to receive PDCCH/PDSCH on OFDM symbols on which UE performs CLI measurements, and on 1 data symbol before an OFDM symbol used for CLI measurements for 15 kHz and 30 kHz subcarrier spacing.

- Depending on UE capability, the UE is not expected to receive PDCCH/PDSCH on OFDM symbols on which UE performs CLI measurements, and on 2 data symbols before an OFDM symbol used for CLI measurements for 60 kHz subcarrier spacing.

For FR2,

- Depending on UE capability, the UE is not expected to receive PDCCH/PDSCH on OFDM symbols on which UE performs CLI measurements, and on 1 data symbol before an OFDM symbol used for CLI measurements for 60 kHz subcarrier spacing.

- Depending on UE capability, the UE is not expected to receive PDCCH/PDSCH on OFDM symbols on which UE performs CLI measurements, and on 2 data symbols before an OFDM symbol used for CLI measurements for 120 kHz subcarrier spacing.

RAN1 should discuss UE behavior and the capture the agreement in the “consequence if feature is not supported by the UE” column of UE feature list document. For collision between CLI measurement and PDCCH/PDSCH, we suggest following RAN4’s scheduling restriction.

Proposal 3: For a UE does not support simultaneous reception of PDSCH and CLI measurement feature, capture its behavior in column “consequence if feature is not supported by the UE”, when PDSCH and CLI measurement are FDMed.

- FG 17-3: If not supported, then UE does not expect to receive PDCCH/PDSCH when it performs CLI-RSSI measurement

- FG 17-4: If not supported, then UE does not expect to receive PDCCH/PDSCH when it performs SRS-RSRP measurement

For collision between CLI measurement and DL signals, the prioritizing rule depends on whether the DL signal is cell-specific or UE-specific. The scheduling restrictions can be extended to UE-specific DL signals, e.g., DM-RS and PT-RS, because these signals are used for PDSCH reception. For cell-specific DL signals, the network should avoid conflicting configuration for UEs who do not support simultaneous reception. If an error case is evitable, then UE should prioritize the reception of cell-specific DL signals, such as SSB and periodic CSI-RS.

Proposal 4: If a UE does not support simultaneous reception of DL transmission and CLI measurement and a CLI measurement resource is FDMed with the following cell-specific DL signals, UE should prioritize the reception of DL signals

- SSB

- Periodic CSI-RS

For other DL signals, such as aperiodic CSI-RS, DM-RS, PT-RS, etc., UE does not expect to receive these DL signals when it performs CLI measurement.

# Conclusion

In this contribution, we discussed CLI-related UE features. Based on the discussion, we have the following proposals:

Proposal 1: Rephrase the first component in FG 17-1 and 17-2 as follows:

- FG 17-1: Support CLI-RSSI measurement. The maximum number of CLI-RSSI measurement resource across all CCs shall not exceed 64.

- FG 17-2: Support SRS-RSRP measurement. The maximum number of SRS-RSRP measurement resource across all CCs shall not exceed 32.

Proposal 2: Discuss and decide whether NR supports multi-port SRS-RSRP measurement. A LS to RAN2 would be needed to inform RAN1’s final decision.

Proposal 3: For a UE does not support simultaneous reception of PDSCH and CLI measurement feature, capture its behavior in column “consequence if feature is not supported by the UE”, when PDSCH and CLI measurement are FDMed.

- FG 17-3: If not supported, then UE does not expect to receive PDCCH/PDSCH when it performs CLI-RSSI measurement

- FG 17-4: If not supported, then UE does not expect to receive PDCCH/PDSCH when it performs SRS-RSRP measurement

Proposal 4: If a UE does not support simultaneous reception of DL transmission and CLI measurement and a CLI measurement resource is FDMed with the following cell-specific DL signals, UE should prioritize the reception of DL signals

- SSB

- Periodic CSI-RS

For other DL signals, such as aperiodic CSI-RS, DM-RS, PT-RS, etc., UE does not expect to receive these DL signals when it performs CLI measurement.

# References

| [1] | R1-2001484, “RAN1 UE features list for Rel-16 NR after RAN1#100-E,” 2020. |
| --- | --- |
| [2] | “Chairman's notes RAN1 #96,” 2019. |
| [3] | R1-1903835, “RRC parameters for NR CLI-handling,” 2019. |
| [4] | TS 38.331, “Radio Resource Control (RRC) protocol specification,” 2020. |
| [5] | R1-2001523, “Reply LS on CLI measurement capability,” 2020. |
| [6] | TS 38.133, “Requirements for support of radio resource management,” 2020. |