3GPP TSG-RAN WG1 Meeting #100bis-e	R1-2002913

e-Meeting, 20th – 30th April, 2020

Agenda Item:	7.2.2.1.3

Source:	Moderator (Ericsson)

Title:	FL Summary for [100b-e-NR-unlic-NRU-ULSignalsChannels-02] Email discussion/approval

Document for:	Discussion, Decision

# 1	Introduction

Based on the conclusion of the e-meeting preparation phase [21] and the vice-Chairman’s guidance, the following e-mail discussion has been kicked-off:

[100b-e-NR-unlic-NRU-ULSignalsChannels-02] Email discussion/approval on the following issues

by 4/24; if necessary, followed by endorsing the corresponding TPs by 4/30 – Steve (Ericsson)

- Capture UE procedure related to FDRA field in RAR UL grant

- Editorial correction on SRS

The following topics are included in this email discussion

| Issue | Description | Tdoc References | Class |
| --- | --- | --- | --- |
| 3 | FDRA field in RAR UL grant for operation with shared spectrum channel access - Capture UE interpreation of FDRA field when interlacing configured - Truncation/expansion rule when interlacing not configured - Agree on rule for RB set allocation for PUSCH scheduled by RAR UL grant TP needed to 38.213 §8.3 | R1-2002030: P3,P4 R1-2001875: P4 R1-2001533: P3 R1-2001934: P5 R1-2001758: P1 R1-2002382: P4-P5 R1-2001651: P3 R1-2002383: P2 R1-2001706: P8 | Critical |
| 4 | Align procedure text in 38.213 §6.2.1 with SRS resource configuration in 38.331 capturing that SRS resource can start at any OFDM symbol in a slot. TP needed to 38.214 §6.2.1 | R1-2002075: P1 R1-2002030: P5 R1-2001986: §2.1 R1-2002365: TP1 R1-2002529: P4 R1-2001704: P4 | Editorial |

# 2	Discussion

## 2.1	Issue #3: FDRA Field in RAR UL Grant

Description:

In Section 8.3 in 38.213, either UL resource allocation Type 1 or Type 2 is used for PUSCH scheduled by a RAR UL grant depending on if interlacing is configured. However, the procedure on how the UE interprets the FDRA field of the RAR UL grant is only described for the case when interlacing is not configured. The procedure when interlacing is configured needs to be included. The procedure needs to account for truncation of the FDRA field since only X bits is indicated for the interlace allocation, where X = 6/5 for 15/30 kHz SCS. The procedure also needs to account for what RB set allocation the UE should assume for transmission of PUSCH scheduled by the RAR UL grant, since the FDRA field does not include Y bits as previously agreed.

Another issue is that since the number of bits in the FDRA field is reduced to 12 for operation with shared spectrum channel access, the threshold number of PRBs that the UE uses for processin the FDRA field needs to be reduced from 180 to 90 for the case when interlacing is not configured.

A text proposal (TP#1) to correct these issues is proposed below. There is one FFS point in the TP on the RB set allocation. This is analogous to the RB set allocation rule for DCI 0_0 in a CSS discussed in Email Thread #1. Alt-1 is analogous to Alt-1 in Issues #1-1 in Email Thread #1.

The following is proposed for discussion this week to address the FFS item in TP#1 below, with down selection completed by 4/24.

- For PUSCH scheduled by an UL RAR grant when UL resource allocation Type 2 is configured, down-select to one alternative:

  - Alt-1: The UE assumes that PUSCH is allocated to the RB set of the active UL BWP that intersects the RB set of the active DL BWP in which the DCI 0_1 that schedules the PDSCH with the RAR message is received

  - Other alternatives?

Affected Specification(s):

- 38.213 Section 8.3

- 38.214 Section 6.1.2.2.3

| Company | View/Position |
| --- | --- |
| ZTE | Agree with the TPs |
| Huawei | Support Alt-1. Agree with the TPs. |
| Nokia, NSB | Support Alt-1 and the TP |
| Lenovo, Motorola Mobility | We support RB set 0 is assumed when scheduled by RAR |
| LG Electronics | Agree with the TPs other than RB set allocation part. Regarding to the rule for RB set allocation, we could consider another alternative. Since the FDRA field in RAR grant is 12 bits for operation with shared spectrum channel access, there are 5 or 6 bits left that can be used for RB set indication. For this reason, we suggest the following alternative (i.e., Alt-2). Alt-2 : FDRA field in RAR grant contains X + Y bits - Y is given by size of active UL BWP |
| NTT DOCOMO | Support Alt-1 and agree with TPs #1 and #2 |
| Samsung | Agree with LGE. |
| Qualcomm | Agree with TP. For FFS, can reuse the same design for DCI 0_0 in CSS |
| Intel | Support Alt-1 and the TP |
| OPPO | We don't agree with the RB set determination Alt-1. We propose to follow NR baseline, i.e. Alt-3: UE assumes that PUSCH is in the RB set that intersects with initial UL BWP, if any, or RB set 0 of active UL BWP. |
| vivo | Agree with the TPs, other than Alt 1 to determine the RB set allocation part. We share the same view with Qualcomm, can reuse the same design for DCI 0_0 in CSS. |
| Spreadtrum | Agree with the TP. For FFS part, we prefer RB set 0 is assumed when scheduled by RAR. |
| Sharp | A msg 3 PUSCH transmission can be triggered for a UE in RRC_CONNECTED. Since the gNB cannot know which UE (in RRC_IDLE or RRC_CONEECTED) transmits the msg 3 PUSCH, the FDRA interpretation shouldn’t be dependant on dedicated RRC configuraton as much as possible. Thus, we propose the following revision. - The UE assumes the RB set allocation in a UL BWP for a PUSCH scheduled by the RAR UL grant is given by [FFS: rule for for RB set allocation] - if the active UL BWP and the initial UL BWP have same SCS and same CP length and the active UL BWP includes all RBs of the initial UL BWP, or the active UL BWP is the initial UL BWP, the UL BWP is the initial UL BWP - else, the UL BWP is the active UL BWP |

Reason for changes

- Procedure on how the UE interprets the FDRA field of the RAR UL grant is missing for the case when interlacing is configured

- Clarification on number of bits X in FDRA field of DCI 0_0 that indicates interlace allocation

Summary of changes

Include procedure description for the case when interlacing is configured

Specs/Sections impacted

38.213 Section 8.3

38.214 Section 6.1.2.2.3

Consequences if not approved

Undefined UE behaviour for PUSCH scheduled by a RAR UL grant

---------------------------------------- Text Proposal (TP#1) for 38.213, Section 8.3 ----------------------------------

*** Unchanged text omitted ***

8.3	PUSCH scheduled by RAR UL grant

An active UL BWP, as described in Clause 12 and in [4, TS 38.211], for a PUSCH transmission scheduled by a RAR UL grant is indicated by higher layers.

If useInterlace-PUCCH-PUSCH is neither provided in BWP-UplinkCommon nor BWP-UplinkDedicated, Ffor determining the frequency domain resource allocation for the PUSCH transmission within the active UL BWP

-	if the active UL BWP and the initial UL BWP have same SCS and same CP length and the active UL BWP includes all RBs of the initial UL BWP, or the active UL BWP is the initial UL BWP, the initial UL BWP is used

-	else, the RB numbering starts from the first RB of the active UL BWP and the maximum number of RBs for frequency domain resource allocation equals the number of RBs in the initial UL BWP

The frequency domain resource allocation is by uplink resource allocation type 1 if useInterlacePUSCH-Common is not provided and by uplink resource allocation type 2 if useInterlacePUSCH-Common is provided [6, TS 38.214]. For an initial UL BWP size of  RBs, if useInterlacePUSCH-Common is not provided, a UE processes the frequency domain resource assignment field as follows

-	if , or for operation with shared spectrum channel access if NBWPsize≤90,

-	truncate the frequency domain resource assignment field to its  least significant bits and interpret the truncated frequency resource assignment field as for the frequency resource assignment field in DCI format 0_0 as described in [5, TS 38.212]

-	else

-	insert  most significant bits, or for operation with shared spectrum channel access insert log2NBWPsize∙NBWPsize+12-12 most significant bits, with value set to '0' after the  bits to the frequency domain resource assignment field, where  if the frequency hopping flag is set to '0' and  is provided in Table 8.3-1 if the hopping flag bit is set to '1', and interpret the expanded frequency resource assignment field as for the frequency resource assignment field in DCI format 0_0 as described in [5, TS 38.212]

-	end if

If useInterlace-PUCCH-PUSCH is provided in any of BWP-UplinkCommon and BWP-UplinkDedicated, the frequency domain resource allocation is by uplink resource allocation type 2 [6, TS 38.214]. A UE processes the frequency domain resource assignment field as follows:

- truncate the frequency domain resource assignment field to its X LSBs

- interpret the truncated frequency domain resource assignment field for the active UL BWP as for the X MSBs of the frequency domain resource assignment field in DCI format 0_0 as described in [6, TS 38.214]

- The UE assumes the RB set allocation in the active UL BWP for a PUSCH scheduled by the RAR UL grant is given by [FFS: rule for for RB set allocation]

- *** Unchanged text omitted ***

------------------------------------------------------ End Text Proposal -------------------------------------------------------

------------------------------------ Text Proposal (TP#2) for 38.214, Section 6.1.2.2.3 ------------------------------

*** Unchanged text omitted ***

6.1.2.2.3	Uplink resource allocation type 2

In uplink resource allocation of type 2, the resource block assignment information defined in [5, TS 38.212] indicates to a UE a set of up to M interlace indices, and for DCI 0_1 a set of up to  NRB-setBWP contiguous RB sets, where M and interlace indexing are defined in Clause 4.4.4.6 in [4, TS 38.211]. The UE shall determine the resource allocation in frequency domain as an intersection of the resource blocks of the indicated interlaces and the indicated set of RB sets and intra-cell guard bands defined in Clause 7 between the indicated RB sets, if any.

For µ=0, the X = 6 MSBs of the resource block assignment information indicates to a UE a set of allocated interlace indices m0+l, where the indication consists of a resource indication value (RIV). For 0≤RIV<M(M+1)/2 , l=0,1,⋯L-1 the resource indication value corresponds to the starting interlace index m0 and the number of contiguous interlace indices (). The resource indication value is defined by:

if (L-1)≤M/2 then

RIV=M(L-1)+m0

else

RIV=M(M-L+1)+(M-1-m0)

For ≥M(M+1)/2 , the resource indication value corresponds to the starting interlace index m0 and the set of values  according to Table 6.1.2.2.3-1.

Table 6.1.2.2.3-1: m0  and  for RIV≥M(M+1)/2.

| RIV-M(M+1)/2 | m0 |  |
| --- | --- | --- |
| 0 | 0 | {0, 5} |
| 1 | 0 | {0, 1, 5, 6} |
| 2 | 1 | {0, 5} |
| 3 | 1 | {0, 1, 2, 3, 5, 6, 7, 8} |
| 4 | 2 | {0, 5} |
| 5 | 2 | {0, 1, 2, 5, 6, 7} |
| 6 | 3 | {0, 5} |
| 7 | 4 | {0, 5} |

For µ=1, the X = 5 MSBs of the resource block assignment information comprise a bitmap indicating the interlaces that are allocated to the scheduled UE. The bitmap is of size M bits with one bitmap bit per interlace such that each interlace is addressable, where M and interlace indexing is defined in Clause 4.4.4.6 in [4, TS 38.211]. The order of interlace bitmap is such that interlace 0 to interlace M-1 are mapped from MSB to LSB of the bitmap. An interlace is allocated to the UE if the corresponding bit value in the bitmap is 1; otherwise the interlace is not allocated to the UE.

*** Unchanged text omitted ***

------------------------------------------------------ End Text Proposal -------------------------------------------------------

### 2.1.1	Summary of Discussion

All companies that responded support TPs #1 and #2. TP#1 contains an FFS that needs to be addressed this meeting.

For the FFS on the RB set allocation for PUSCH the following summarizes the situation:

- A majority of companies support the rule in Alt-1

- 3 companies support PUSCH allocated to RB Set 0

- 2 companies support reusing the same rule for RB set allocation as used for PUSCH scheduled by DCI 0_0. This rule is proposed in Email Thread #1 for the UL Signals and Channels AI, and is the same as merging the rules 1. and 2. above

- 2 companies support that the FDRA field of the RAR UL grant contains Y bits to indicate the RB set allocation for PUSCH.

Since it was agreed during the WI that indication of Y bits is not supported for Msg3 PUSCH, the FL recommendation is to not to support rule 4.

Based on this it seems reasonable to support rule 3. which is a merge of 1. and 2.

The FL recommendation is as follows:

Support TP#1 for 38.213 Section 8.3 and TP#2 for 38.214 Section 6.1.2.2.3.

For the FFS in TP#1, FL to update the TP to capture the RB set allocation rule for PUSCH scheduled by the RAR UL grant using the same rule as for PUSCH scheduled by DCI 0_0 in a common search space. Updated TP to be endorsed by 4/30.

### 2.1.2	Further Summary of Discussion

While all companies agree to TP#1, there is continued debaute about the FFS point within TP#1, i.e., the rule for PUSCH allocation which Proposal 3 attempted to resolve. Two companies (OPPO and vivo) have raised concerns on the reflector about Proposal 3 for the deployment scenario of PCell in unlicensed spectrum + SUL (stand alone + SUL).

Furthermore, as I mentioned in Email Thread #1 and #3, it seems we we need further discussion on the case of an UL carrier without guardbands, as it affects the rule for how PUSCH is allocated. In Email Thread #1, several companies previously suggested that we wait for further decisions in the wideband agenda item before making an agreement on DCI 0_0 in a CSS, on which Proposal 3 depends.

As it turns out, in the Wideband Operation (WB-01) email thread that was concluded on 4/23, Seonwook (Feature Lead) will moderate a discussion on two different options for realizing a carrier without intra-cell guard bands during the early part of next week. Companies are encouraged to contribute to the discussion in WB-01 so hopefully that can be concluded. If so, this will help speed progress on PUCCH/PUSCH resource allocation in this agenda item.

Since TP#1 is stable, with the exception of the FFS point, the FL recommends that we adopt TP#1 and TP#2 and leave the PUSCH allocation rule as FFS to be further discussed next week. Furthermore, I would like to narrow down the discussion on the FFS point two alternatives: Alt-1 is the same as in  Proposal 3 above. Alt-2 is the proposal from OPPO and vivo, and the following rationale was provided on the reflector. Companies are encouraged to check this.

OPPO

For NR R15, the RACH procedure can be performed in either normal UL or SUL and it is up to UE to select which one to perform. The Alt-1 certainly precludes this option. Note that standalone SUL is one of the depployment scenarios and it was already captured in the UE feature table. Moreover, Alt-1 involves the association between DL BWP and UL BWP relative positions, which is not the NR design principle, neither has it been tested in the past. We don't know if there is any further postential issues. A safer solution should be either Alt-2 or Alt-3 [Moderator: Alt-3 = Alt-2 shown below] that completely decouple the relation between DL BWP and UL BWP relative locations.

First of all, we support Alt-3, and we have strong concern on Alt-1.

vivo:

For RAR UL grant, we find that it is not suitable to adopt unified solution as DCI 0_0 in CSS after further consideration. Different from DCI 0_0 in CSS, gNB does not know whether the UE is in idle or connected state when sending RAR UL grant. Then there will be some problems if we adopt Alt. 1 even if it is updated as the same solution for current DCI 0_0 in CSS.

Taking the following figure as an example. Assuming UE1 is operated in active BWP (BWP1) of SUL carrier as connected state and UE2 is operated in initial BWP (BWP0) of SUL carrier as idle state, they share the same RACH resource on initial BWP of SUL carrier. Clearly BWP1 includes initial BWP. gNB sends DCI 1_0 in the same DL BWP that schedules the PDSCH with RAR message to both UE1 and UE2. Since there is no intersection between DL and SUL, RB set 0 is used for PUSCH transmission scheduled by RAR UL grant following updated Alt. 1. But for UE1 and UE2, RB set 0 corresponds to different frequency position. In the process of CBRA, gNB has no knowledge on which UE it is (connected or idle) from detected RACH transmission. So if Alt. 1 is adopted, gNB doesn’t know which RB set it should receive the scheduled PUSCH. For Alt. 3 [Moderator: Alt-3 = Alt-2 shown below], the scheduled PUSCH should be in the initial BWP for both UEs in this case and there will be no ambiguity. That’s also why NR Rel-15 adopts similar rule with Alt. 3 for RAR UL grant.

In summary, Alt 3 is preferred by us since it is the most robust solution among the 3 alternatives, which also inherits the design principle in NR Rel15.

Based on this the above, the FL recommendation is as follows:

Support TP#1 for 38.213 Section 8.3 and TP#2 for 38.214 Section 6.1.2.2.3 where the last bullet of TP#1 is modified as follows:

- The UE assumes the RB set allocation in the active UL BWP for a PUSCH scheduled by the RAR UL grant is given by [FFS: rule for for RB set allocation]

- FFS: PUSCH allocation rule within the interlaces indicated by the frequency domain resource allocation field

To resolve the FFS in TP#1, support one of the following two alternatives for the PUSCH allocation rule. Further discuss the alternatives during this week, and if concensus can be reached this meeting, FL to update TP#1.

- When UL resource allocation Type 2 is configured, the UE assumes that PUSCH is allocated as follows:

  - Alt-1: PUSCH is allocated to the RB set of the active UL BWP that intersects the RB set of the active DL BWP in which the DCI 0_1 that schedules the PDSCH containing the RAR UL grant is received. If there is no intersection, PUSCH is allocated to RB Set 0 of the active UL BWP.

  - Alt-2: PUSCH is allocated to the initial UL BWP if the active UL BWP fully overlaps the initial UL BWP, otherwise PUSCH is allocated to RB Set 0 of the active UL BWP.

- FFS: Rule for PUSCH allocation for an UL carrier without intra-cell guard bands. Note: depending on the outcome of the discussion in the Wideband Operation (WB-01) email thread next week, special handling may not be needed if it is decided that RB sets are defined for a carrier without guard bands.

## 2.2	Issue #4: Procedure Text Related to Configuration of SRS

Description:

It was agreed in RAN1#96bis to allow the starting OFDM symbol of an SRS resource to occur at any OFDM symbol of a slot rather than be limited to start only within the last 6 symbols of a slot:

Agreement:

Support RRC configuration of an SRS resource to start at any OFDM symbol within a slot by extending the RRC parameter startPosition of resourceMapping of SRS-Config for Rel-16 to have a value range 0..13.

While 38.331 has been updated to capture this agreement, 38.214 has not.

Affected Specification(s):

- 38.214 Section 6.2.1

| Company | View/Position |
| --- | --- |
| ZTE | Agree with the TP |
| Huawei | Agree with the TP |
| Nokia, NSB | Agree with the TP |
| Lenovo, Motorola Mobility | Agree with the TP |
| LG Electronics | Agree with the TP#3 |
| NTT DOCOMO | Agree with TP#3 |
| Samsung | Agree with the TP |
| Qualcomm | Agree with the TP |
| Intel | Agree with the TP |
| OPPO | OK |
| vivo | Agree with the TP |
| Spreadtrum | Agree with the TP |
| Sharp | Agree with the TP |

Reason for changes

Misalignment between 38.331 on configuration of starting OFDM symbol of an SRS resource and the corresponding procedure text in 38.214.

Summary of changes

Include procedure description for starting OFDM symbol of an SRS resource

Specs/Sections impacted

38.214 Section 6.2.1

Consequences if not approved

Misalignment between procedure text in 38.214 and corresponding configuration in 38.331

----------------------------------- Text Proposal (TP#3) for 38.214, Section 6.2.1 -------------------------------------

*** Unchanged text omitted ***

The UE may be configured by the higher layer parameter resourceMapping in SRS-Resource with an SRS resource occupying  adjacent OFDM symbols at any symbol location within the last 6 symbols of the slot, where all antenna ports of the SRS resources are mapped to each symbol of the resource. When the SRS is configured with the higher layer parameter [SRS-for-positioning] the higher layer parameter resourceMapping in SRS-Resource with an SRS resource occupying NS∈1,2,4,8,12 adjacent symbols anywhere within the slot.

*** Unchanged text omitted ***

------------------------------------------------------ End Text Proposal -------------------------------------------------------

### 2.2.1	Summary of Discussion

There is consensus to support TP#3. Based on this, the FL recommendation is as follows:

Support TP#3 for 38.214 Section 6.2.1

# References

- R1-2001533	Maintainance on uplink signals and channels	Huawei, HiSilicon

- R1-2001651	Remaining issues on physical UL channel design in unlicensed spectrum	vivo

- R1-2001704	Remaining issues on the UL channels for NR-U	ZTE, Sanechips

- R1-2001758	Discussion on the remaining issues of UL signals and channels	OPPO

- R1-2001875	Remaining issues on UL signals and channels for NR-U	Fujitsu

- R1-2001903	Remaining issues on UL signals and channels for NR-U	MediaTek Inc.

- R1-2001934	Remaining issues of UL signals and channels for NR-U	LG Electronics

- R1-2001973	Remaining issues for UL signals and channels for NR-U	Lenovo, Motorola Mobility

- R1-2001986	UL signals and channels for NR-unlicensed	Intel Corporation

- R1-2002030	UL signals and channels	Ericsson

- R1-2002075	TP for SRS configuration	CATT

- R1-2002116	UL signals and channels for NR-U	Samsung

- R1-2002192	Remaining Issues on UL Signals and Channels for NR-U	Nokia, Nokia Shanghai Bell

- R1-2002246	UL signals and channels	ETRI

- R1-2002276	Remaining issues in UL signals and channels	Spreadtrum Communications

- R1-2002321	Remaining issues of UL signals and channels	Apple

- R1-2002365	TPs on uplink signals in NRU	NEC

- R1-2002382	Remaining issues on UL signals/channels for NR-U	Sharp

- R1-2002433	Remaining issues on UL signals and channels for NR-U	NTT DOCOMO, INC.

- R1-2002529	TP for UL signals and channels for NR-U	Qualcomm Incorporated

- R1-2002036, “Feature lead summary for Maintenance of UL Signals and Channels,” Moderator (Ericsson), RAN1#100bis-e, April, 2020.