3GPP TSG RAN WG1 #100bis	R1-2002984

e-Meeting, April 20th – 30th, 2020

Source:	moderator (vivo)

Title:	Editorial corrections on NRU-CG to 38.213

Agenda Item:	7.2.2.2.4

Document for:	Discussion and Decision

Editorial corrections

Reason for change:  some editorial corrections for better reading of spec.

Summary of change:  editorial corrections in sections 10.2 and 10.5.

Consequences if not approved:  readability of is not good

Clauses affected: 38.213 section 10.2, 10.5.

Issue 14: TP on HARQ-ACK information on PUSCH transmission (Editorial/clarification)

2.14.1 TP1

TP for 38.213

----------------------------------------------------------------------------------------------------

10.5	HARQ-ACK information for PUSCH transmissions

A UE can be configured a number of search space sets to monitor PDCCH for detecting a DCI format 0_1 with DFI flag field and CRC scrambled with a CS-RNTI provided by cs-RNTI. The UE determines that the DCI format provides HARQ-ACK information for PUSCH transmissions based on an XYZ a DFI flag field value set to '1', as described in [5, TS 38.212], if a PUSCH transmission is configured by ConfiguredGrantConfig or is an activated PUSCH transmission configured by semiPersistentOnPUSCH.

<unchanged text omitted>

------------------------------------------------------------------------------------------------------

2.14.2 TP2

------------------------------------------------------------------------------------------------------

10.5	HARQ-ACK information for PUSCH transmissions

< Unchanged parts are omitted >

The HARQ-ACK information corresponds to transport blocks in PUSCH transmissions for all UL HARQ processes for a serving cell of a PDCCH reception that provides DCI format 0_1 or, if DCI format 0_1 includes a carrier indicator field, for a serving cell indicated by a value of the carrier indicator field.

< Unchanged parts are omitted >

----------------------------------------------------------------------------------------------------------

Issue 15: Correction related to DFI delay (Editorial/Clarification)

2.15.1 TP1

==============TP for 38.213 10.5==========================

10.5	HARQ-ACK information for PUSCH transmissions

….

For a PUSCH transmission scheduled by a DCI format, HARQ-ACK information for a transport block of a corresponding HARQ process number is valid if a first symbol of the PDCCH reception is after a last symbol of the PUSCH transmission by a number of symbols provided by cg-minDFIDelay-r16 or, if the PUSCH transmission is over multiple slots,

-	after a last symbol of the PUSCH transmission in a first slot from the multiple slots by a number of symbols provided by cg-minDFIDelay-r16, if a value of the HARQ-ACK information is ACK.

-	after a last symbol of the PUSCH transmission in a last slot from the multiple slots by a number of symbols provided by cg-minDFIDelay-r16, if a value of the HARQ-ACK information is NACK.

====================================================

Issue 17: Typo correction in 38.213 (Editorial/clarification)

2.17.1 TP1

10.2	PDCCH validation for DL SPS and UL grant Type 2

< Unchanged parts are omitted >

Table 10.2-1: Special fields for single DL SPS or single UL grant Type 2 scheduling activation PDCCH validation when a UE is provided a single SPS PDSCH or UL grant Type 2 configuration

|  | DCI format 0_0/0_1/0_2 | DCI format 1_0/1_2 | DCI format 1_1 |
| --- | --- | --- | --- |
| HARQ process number | set to all '0's | set to all '0's | set to all '0's |
| Redundancy version | set to all '0's | set to all '0's | For the enabled transport block: set to all '0's |

< Unchanged parts are omitted >

References

[1] R1-2002745, “Summary of prep email discussion on NRU-CG”, RAN1#100b-e