3GPP TSG RAN WG1 Meeting #100bis-e	R1-2003014

E-Meeting, April 20 – 30, 2020

Agenda Item:	6.2.2.4

Source:	Moderator (Huawei)

Title:	Text proposal in 100b-e-LTE-NB_IoTenh3-Coex-NR-01

Document for:	Discussion and Decision

Introduction

This document provides the text proposal as outcomes of the following email discussion [1]:

[100b-e-LTE-NB_IoTenh3-Coex-NR-01] Resource reservation for TDD NB-IoT by 4/24 and corresponding TP (if any) by 4/30 – Yubo (Huawei)

- Issues #1, #4 in R1-2002700

Discussion

TP on resource reservation to special subframes

Reason for changes:

It was agreed in RAN1#100bis-e that symbol-level resource reservation is not applied to special subframes.

Summary of changes:

Specify that symbol-level resource reservation is not applied to special subframes.

Specs/sections impacted:

36.211 sections 10.0.1.2

Consequences if not approved:

Resource reservation may be applied to special subframes incorrectly.

-----------------------------------------------------Start of Text Proposal-----------------------------------

#### 10.0.1.2	Frame structure type 2

Frame structure type 2 is applicable to TDD operation only.

The following restrictions apply:

-	Uplink-downlink configuration 0 and 6 are not supported.

-	UpPTS is not used for NPUSCH or NPRACH.

-	DwPTS and UpPTS in special subframe configuration 10 is not used for transmissions.

-	On an NB-IoT carrier for which higher-layer parameter operationModeInfo indicates inband-SamePCI or inband-DifferentPCI, or higher-layer parameter inbandCarrierInfo is present, or on an NB-IoT carrier for SystemInformationBlockType1-NB for which sib1-carrierInfo indicates non-anchor and the value of the higher layer parameter sib-GuardbandInfo is set to sib-GuardbandInbandSamePCI or sib-GuardbandinbandDiffPCI, DwPTS in special subframe configuration 0 and 5 for normal cyclic prefix is not used for NPDCCH and NPDSCH transmission.

-	Higher-layer parameter symbolBitmap-r16 does not apply to special subframes.

------------------------------------------------------End of Text Proposal------------------------------------

TP on SIB1-NB transmission with resource reservation

Reason for changes:

As the legacy, the subframes containing SIB1-NB transmission should not be considered as NB-IoT downlink subframes.

Summary of changes:

To determine whether a subframe is a NB-IoT downlink subframe, UE shall first determine the subframes containing NPSS/NSSS/NPBCH/SIB1-NB transmission are not assumed as NB-IoT DL subframes.

Specs/sections impacted:

36.213 sections 16.4

Consequences if not approved:

The subframes containing SIB1-NB may be considered as NB-IoT DL subframes incorrectly.

-------------------------------------------------Start of Text Proposal to TS 36.211--------------------------------

<unchanged parts are omitted>

## 16.4	Narrowband physical downlink shared channel related procedures

A NB-IoT UE shall determine whether a downlink subframe or a TDD special subframe configured for NB-IoT DL transmission is a NB-IoT DL subframe as follows

-	If the UE determines that the subframe contains NPSS/NSSS/NPBCH/ SystemInformationBlockType1-NB transmission, then the subframe is not assumed as a NB-IoT subframe

-	Else if higher layer parameter valid-subframe-config-DL or slot-reserved-resource-config-DL is configured

-	for NPDSCH transmission associated with C-RNTI using UE-specific NPDCCH search space

-	if the Resource reservation field in the DCI is set to 0, then the subframe is assumed as a NB-IoT DL subframe

-	else if the Resource reservation field in the DCI is set to 1, then the subframe is assumed as a NB-IoT DL subframe if it is not fully reserved according to the higher layer parameters.

-	for NPDCCH transmission associated with C-RNTI or SPS C-RNTI using UE-specific NPDCCH search space

-	the subframe is assumed as a NB-IoT DL subframe if it is not fully reserved according to the higher layer parameters.

-	In all other cases, a NB-IoT UE shall assume a subframe as a NB-IoT DL subframe if

-	for a NB-IoT carrier that a UE receives higher layer parameter operationModeInfo, the subframe is configured as NB-IoT DL subframe or the subframe is a TDD special subframe configured for NB-IoT DL transmission after the UE has obtained SystemInformationBlockType1-NB.

-	the subframe is configured as NB-IoT DL subframe by the higher layer parameter downlinkBitmapNonAnchor.

-	except when the UE is configured with higher layer parameter additionalTxSIB1-Config set to TRUE, subframe #3 not containing additional SystemInformationBlockType1-NB transmission is assumed as a NB-IoT DL subframe if the UE monitors a NPDCCH UE-specific search space or decodes NPDSCH transmission scheduled by NPDCCH in the UE-specific search space.

For a NB-IoT UE that supports twoHARQ-Processes-r14 or the UE is configured with higher layer parameter multi-TB-Unicast-config, there shall be a maximum of 2 downlink HARQ processes.

<unchanged parts are omitted>

- 16.5	Narrowband physical uplink shared channel related procedures

For a NB-IoT UE that supports twoHARQ-Processes-r14 or the UE is configured with higher layer parameter multi-TB-Unicast-config, there shall be a maximum of 2 uplink HARQ processes.

For a NB-IoT UE and NPUSCH transmission using preconfigured uplink resource, there shall be 1 uplink HARQ process.

A NB-IoT UE shall determine whether a subframe is a NB-IoT UL subframe as follows

-	If higher layer parameter valid-subframe-config-UL or slot-reserved-resource-config-UL is configured

-	for NPUSCH format 1 transmission associated with C-RNTI or SPS C-RNTI using UE-specific NPDCCH search space

-	if the Resource reservation field in the DCI is set to 0, then the subframe is assumed as a NB-IoT UL subframe

-	else if the Resource reservation field in the DCI is set to 1, then the subframe is assumed as a NB-IoT UL subframe if it is not fully reserved according to the higher layer parameters.

-	for NPUSCH format 2 transmission

-	the subframe is assumed as a NB-IoT UL subframe if it is not fully reserved according to the higher layer parameters.

-	In all other cases,

-	for TDD, a NB-IoT UE shall assume a subframe as a NB-IoT UL subframe if, for a NB-IoT carrier, it is configured as NB-IoT UL subframe by higher layers

-	for FDD, a NB-IoT UE shall always assume a subframe as a NB-IoT UL subframe.

<unchanged parts are omitted>

--------------------------------------------------End of Text Proposal to TS 36.211---------------------------------

- References

- R1-2002925	Feature lead summary #1 on 100b-e-LTE-NB_IoTenh3-Coex-NR-01	Moderator(Huawei)