3GPP TSG-RAN WG1 Meeting #100bis-e										R1-2003025

e-meeting, April 20th – 30th, 2020

Agenda Item:	7.2.1

Source:			Moderator (ZTE)

Title:			Text proposal of clarification on the DMRS sequences

Document for:	Discussion

# Introduction

This document is intended to address the following remaining issues by email discussion.

Agreements:

- Continue the discussion in this meeting based on the TP#2 in the Appendix, to clarify the configuration of DMRS sequences for msgA PUSCH.

To check TPs till 4/30

# Text proposal of clarification on the DMRS sequences

12 out of 13 companies supported the TP#2 in [1]. One company thought it is not necessary but seems they also had no problem with the TP. One company had an editorial comment on the ‘preamble(s)’.

[Moderator] The proposal is updated from TP#2 in [1] with the editorial comment captured.

In addition, three companies thought how to determine the nSCID value by the mapping between preambles and PUSCH occasion and associated DMRS resource need be further clarified.

[Moderator] This part can be revisited later once we see some concrete proposals.

## Information for the cover page

Reasons for change

To capture the clarification on the configuration of DMRS sequences.

Summary of changes

Implement the above updates

Specs/Sections impacted

TS 38.211, Section 6.4.1.1.1.1.

## Text proposal

Proposal 1:

- Adopt the following TP to clarify the configuration of DMRS sequences for MsgA PUSCH in TS 38.211.

----------------------------- Start of TP for TS 38.211 ----------------------------

6.4.1.1.1.1	Sequence generation when transform precoding is disabled

<Unchanged Text Omitted>

The quantity nSCID∈0,1 is

-	indicated by the DM-RS initialization field, if present, either in the DCI associated with the PUSCH transmission if DCI format 0_1 or 0_2, in [4, TS 38.212] is used;

-	indicated by the higher layer parameter dmrs-SeqInitialization, if present, for a Type 1 PUSCH transmission with a configured grant;

-	determined by the mapping between preamble(s) and a PUSCH occasion and the associated DMRS resource for a PUSCH transmission of Type-2 random access process in [5, TS 38.213];

-	otherwise nSCID=0.

<Unchanged Text Omitted>

----------------------------- End of TP -----------------------------------------------

Any comments?

| Company | Comment |
| --- | --- |
| Intel | We are fine with the proposal 1 and the TP. |
| CATT | We agree with the proposal 1 and related TP based on FL’s suggestion. |
| Spreadtrum | In general, we are fine with the proposal 1 and the TP, a minor editorial change: determined by the mapping between preamble(s) and a PUSCH occasion and the associated DMRS resource for a PUSCH transmission of Type-2 random access process in [5, TS 38.213]; - otherwise nSCID=0. |
| Apple | We are ok with the Proposal 1. |
| Nokia | We are ok with proposal 1 and the associated TP. The editorial change proposed by Spreadtrum would also be OK with us. |
| Qualcomm | The editorial change suggested by Spreadtrum looks good to us. |
| LG Electronics | We are fine with the Proposal. |

# References

- R1-2002819, Summary of email discussion [100b-e-NR-2step-RACH-04], Moderator (ZTE)

# Appendix

## TP#2 in [1]

----------------------------- Start of TP #2 for TS 38.211 ----------------------------

6.4.1.1.1.1	Sequence generation when transform precoding is disabled

<Unchanged Text Omitted>

The quantity nSCID∈0,1 is

-	indicated by the DM-RS initialization field, if present, either in the DCI associated with the PUSCH transmission if DCI format 0_1 or 0_2, in [4, TS 38.212] is used;

-	indicated by the higher layer parameter dmrs-SeqInitialization, if present, for a Type 1 PUSCH transmission with a configured grant;

-	determined by the mapping between preambles and PUSCH occasion and associated DMRS resource for a PUSCH transmission of Type-2 random access process in [5, TS 38.213];

-	otherwise nSCID=0.

<Unchanged Text Omitted>

----------------------------- End of TP #2 -----------------------------------------------

## Companies’ views collected in the first phase discussion

| Company | Comment |
| --- | --- |
| Samsung | Even though it seems not wrong, such TP is also not necessary to have, since the original text is enough. |
| Huawei, HiSi | Agree as the current spec is actually unclear and very difficult to be based on for figuring out the exact value for scrambling. |
| Ericsson | Agree. |
| CATT | For proposed TP on DMRS scrambling for PUSCH, how to determine nSCID value by the mapping between preambles and PUSCH occasion and associated DMRS resource need be further clarified. |
| NTT DOCOMO | Fine to have the TP. |
| vivo | We agree with the intention of the TP. We also share the similar view as CATT that how to determine nSCID value when nrofDMRS-Sequences is configured as 2 needs to further clarify. |
| LG Electronics | We think TP#2 is needed to clarify the configuration of DMRS sequence for msgA PUSCH. |
| Spreadtrum | We are fine with the TP, it is needed to clarify it. |
| Nokia | Agree to proposal 2 |
| Intel | Agree with the TP. A minor editorial change could be “between preamble(s) and PUSCH occasion and associated DMRS resource”. We also share similar view as other companies that we need to clarify how to determine the nSCID value by the mapping between preambles and PUSCH occasion and associated DMRS resource either in 211 or 213 spec. |
| Qualcomm | Proposal 2 looks fine to us. |
| Apple | We agree with Proposal 2. |