3GPP TSG-RAN WG1 Meeting #100bis-e	Tdoc R1-20xxxxx

E-meeting, April 20th - 30th 2020

Agenda:	5

Source:	Moderator (Ericsson)

Title:	Summary of reply LS on secondary DRX – Apr 22nd

Document for:	Discussion, Decision

# 1	Introduction

RAN2 discussed introduction of a secondary DRX group with Carrier Aggregation in NR during RAN2#108 and made the following conditional agreements:

Conditional on R1 acceptance:

A separate drx-InactivityTimer and drx-onDurationTimer can be configured for the secondary DRX group. R2 understands that this has zero or almost zero impact in R1 and R4

The combination of cross-carrier scheduling and secondary DRX group is not supported

FFS if timers for FR2 DRX configuration are shorter than timers for FR1 DRX configuration.

The intention is to apply secondary DRX configuration to FR2 and existing DRX configuration to FR1

We send an LS to R1, ask whether there is impact, and if so whether the impact is acceptable.

RAN1 received an LS [1] from RAN2 to check if this feature can be introduced under TEI16 with no or limited impact to RAN1 specifications.

As guided by chairman, this summary is to collect companies’ views on the LS and draft the reply based on companies’ input.

[100b-e-LS-08] Email discussion/approval for a potential reply LS to R1-2000165 by 4/24 (Ericsson, Claes)

# 2	Discussion

In the LS [1], RAN2 asked the following question:

RAN2 has the intention to introduce this enhancement under TEI16 and would like to check that this has zero or very little impacts to RAN1 and RAN4.

In the contribution submitted to RAN1#100bis-e, companies have identified various areas where there seem to be impact to RAN1 specifications. Based on this input, it would seem difficult to answer there is zero of very little impact to RAN1 specifications.

Thus, RAN1 should provide an answer describing there is RAN1 impact. It would also be preferable to provide some additional information to RAN2 in which areas there is impact. The company input discusses three areas: reception of DCI format 2_6, i.e. WUS, SCell dormancy and CSI measurements. The company input is summarized below from the first round of discussions is summarized below:

Table 1: Summary of company input

|  | Impact | No or little impact |
| --- | --- | --- |
| WUS | ZTE, vivo, CATT, Ericsson, Huawei/HiSilicon, MTK, Intel, CMCC | Qualcomm, Samsung, Apple |
| SCell dormancy | Vivo, Huawei/HiSilicon | Ericsson, Qualcomm, Samsung, Apple |
| CSI measurements/reporting | ZTE, Huawei/HiSilicon | Ericsson, MTK, Qualcomm, Samsung, Apple |

The moderator interprets “impact” that new definitions, even if they are straightforward, will be needed in RAN1 specifications. There seems to be wide-spread understanding that there is impact on WUS, whereas the views on other areas are diverging.

The moderator notes that the RAN2 LS did not ask RAN1’s opinion about the benefits of secondary DRX.

## 2.1	Proposed response

Based on the initial responses, it seems difficult to directly arrive at a full LS response. Therefore, we will try to agree on parts of the response, to include in the overall description section in the LS response.

### 2.1.1	Minimum response

The minimum response would be the direct answer to RAN2’s question, reflecting the status in RAN1: to the moderator, it would seem clear that RAN1 cannot confirm that secondary DRX has zero of little impact to RAN1, despite the fact that there are three companies that think that there is zero or little impact on RAN1 specifications. However, the LS response should capture the views of RAN1 as a whole. Therefore, we propose to add the following statement to the LS response:

Statement 1:

RAN1 cannot confirm that the introduction of secondary DRX has zero or very little impact to RAN1 specifications.

If necessary, this would be a complete response to the LS. The wording is copied from the RAN2 LS.

Please provide your comments to the above proposal. If you do not support to include the above proposal in the LS response, please provide a motivation why this does not reflect the situation in RAN1.

| Company | Comments |
| --- | --- |

### 2.1.2	An area where RAN1 may agree there is impact.

Based on the first round of inputs, many (but not all) companies expressed the view that there is RAN1 impact if WUS and secondary DRX are configured at the same time. Based on the input, it would seem agreeable to include this as a RAN1 view, since companies are willing to compromise on this. The moderator thus proposes to include a statement in the LS response to explain this, using the proposal from CATT:

Statement 2:

RAN1 has identified that there is RAN1 impact of secondary DRX related to the UE’s behavior of detecting DCI format 2_6 and the respective procedures.

Please provide your comments to the above proposal.

| Company | Comments |
| --- | --- |

Just to reiterate: If we cannot agree to the proposal, we will not include in the LS response.

Regarding SCell dormancy and the CSI measurements, most companies state there is zero or little impact. It is the view of the moderator that RAN1 will not be able to agree to respond that there is impact.

### 2.1.3	Additional input

Several companies have provided additional input on SCell dormancy and CSI measurements. As previously noted, the views are diverging on these two aspects. Arriving at a unified RAN1 view on these topics would be unlikely. However, we may also include information in the LS response that are views of individual companies, where we also include opposite views when applicable. The information should still be beneficial for RAN2, i.e. on a level of technical detail suitable for RAN2. There is thus little point in adding another level of technical detail, since that would not be relevant for RAN2 anyway.

The moderator would like to bring forward a suggestion along these lines for possible inclusion in the LS response:

Statement 3:

Some companies identified that there is RAN1 impact on CSI measurements, whereas some companies stated there is no such impact.

Some companies identified that there is RAN1 impact on SCell dormancy, whereas some companies stated there is no such impact.

Please provide your comments to the above proposal.

| Company | Comments |
| --- | --- |

Again, if we cannot agree to the proposal, we will not include in the LS response.

### 2.1.4	Summary

In summary, the total response will then consist of the parts that RAN1 can agree on. Thus, the full LS response would consist of

- Statement 1, or

- Statement 1 + Statement 2, or

- Statement 1 + Statement 3, or

- Statement 1 + Statement 2 + Statement 3

It is the moderators view that if RAN1 cannot agree on Statement 1, there is little point in sending an LS response.

# 3	References

- R1-2000165, LS on secondary DRX group, RAN2, RAN1#100-e, February 2020

R1-2001581, Discussion on secondary DRX group, ZTE, RAN1#100bis-e, April 2020

R1-2001582, Draft reply LS on secondary DRX group, ZTE, RAN1#100bis-e, April 2020

R1-2001693, Discussion on 2nd DRX group, vivo, RAN1#100bis, April 2020

R1-2001845, Discussion on impact of secondary DRX group, MediaTek Inc., RAN1#100bis-e, April 2020

R1-2002056, Discussion on the RAN1 impacts on Secondary DRX group, CATT, RAN1#100bis-e, April 2020

R1-2002492, Draft LS response on secondary DRX group, Ericsson, RAN1#100bis-e, April 2020

R1-2002493, On secondary DRX group, Ericsson, RAN1#100bis-e, April 2020

R1-2002578, RAN1 impact analysis due to the introduction of secondary DRX cycle, Huawei, HiSilicon, RAN1#100bis-e, April 2020

R1-2002662, Draft reply LS on secondary DRX cycle, Huawei, HiSilicon, RAN1#100bis-e, April 2020