3GPP TSG RAN WG1 Meeting #101	R1-2003312

e-Meeting, May 25th – June 5th, 2020

Agenda item:		7.2.4.4

Source:	Nokia, Nokia Shanghai Bell

Title:	Remaining details of In-device coexistence between LTE and NR sidelinks

Document for:		Discussion and Decision

# Introduction

The V2X WID has the following objectives for LTE – NR coexistence

- Solutions for ‘not co-channel’ in-device coexistence between LTE and NR sidelinks

  - TDM-based solutions as per the study outcome [RAN1, RAN2, RAN4]

  - FDM-based solutions with static power allocation as per the study outcome [RAN4]

    - This will not consider the case where LTE and NR sidelinks are in the same frequency band.

  - No impact to LTE specifications at least from RAN1 and RAN2 perspective.

The FDM solutions are specified in RAN4 but RAN1 specification should enable the use of FDM-based solutions that are based on static power allocation.

# Static power allocation between LTE SL and NR SL

Static power allocation means that higher layers allocate UE Tx power so that maximum Tx power allocated for LTE SL and maximum TX power allocated for NR SL together never exceeds the UE maximum total Tx power. According to the study phase outcome in TR38.885 this is possible based on the following statement:

For inter-band FDM coexistence with static power assignment of Pc, max for each carrier, synchronization between the SLs is not assumed. Under these conditions, inter-band FDM coexistence is feasible when SL transmissions from both RATs overlap. When transmission from one SL overlaps with reception for the other, inter-band FDM coexistence under the above conditions is feasible if the inter-band separation is large enough, but it is not feasible otherwise.

We propose to clarify that FDM method is used in the inter-band case if static power assignment for LTE and NR SL carriers is configured so that UE can transmit both LTE SL and NR SL simultaneously. Otherwise TDM based coexistence is used. The current description of coexistence seems to prevent FDM operation.

Proposal 1: Clarify in 38.213 that FDM-based solutions with static power allocation can be used when frequency separation is large enough and static power allocation between NR and LTE SL is configured.

Regarding Tx/Rx overlap it should be noted that RAN1 agreements do not mandate dropping of lower priority Tx/Rx. The agreement in RAN1#98bis says:

Agreements:

- For Tx/Rx overlap,

  - If packet priorities of both LTE and NR sidelinks are known to both RATs prior to time of transmission/reception (subject to processing time restrictions), then the packet with a higher relative priority is transmitted/received

    - In case the priorities of LTE and NR sidelink packets are the same, then it is up to UE implementation as to which packet is transmitted/received

Currently 38.213 says that only higher priority transmission or reception is performed in Tx/Rx overlap case. We propose to remove the word “only”.

Proposal 2: The word “only” that necessitates dropping of lower priority Tx/Rx is removed from the section 16.2.4.1 of 38.213.

# Conclusion

In this contribution corrections/clarifications to co-existence section of 38.213 are presented. We have the following proposals:

Proposal 1: Clarify in 38.213 that FDM-based solutions with static power allocation can be used when frequency separation is large enough and static power allocation between NR and LTE SL is configured.

Proposal 2: The word “only” that necessitates dropping of lower priority Tx/Rx is removed from the section 16.2.4.1 of 38.213.

We have the following text proposal for 38.213 V16.1.0:

-------------------------- <Unchanged parts are omitted> ----------------------------------------

#### 16.2.4.1	Simultaneous NR and E-UTRA transmission/reception

If the frequency separation of E-UTRA radio access and NR radio access is large enough and static power assignment of Pc, max for E-UTRA carriers and NR radio access carriers allows them to be transmitted simultaneously UE transmits both E-UTRA and NR radio access transmissions. In this case synchronization between the SLs is not assumed.

Otherwise, if a UE

-	would transmit a first channel/signal using E-UTRA radio access and a second channel/signal using NR radio access, and

-	a transmission of the first channel/signal would overlap in time with a transmission of the second channel/signal, and

-	the priorities of the two channels/signals are known to the UE T msec prior to the start of the earlier of the two transmissions

the UE transmits only the channel/signal with the higher priority as determined by the SCI formats scheduling the transmissions or, in case of a S-SS/PSBCH block or a sidelink synchronization signal using E-UTRA radio access, as indicated by higher layers. If a UE

-	would respectively transmit or receive a first channel/signal using E-UTRA radio access and receive or transmit a second channel/signal using NR radio access, and

-	a transmission or reception of the first channel/signal would respectively overlap in time with a reception or transmission of the second channel/signal, and

-	the priorities of the two channels/signals are known to the UE T msec prior to the start of the later transmission or reception

the UE transmits or receives the channel/signal with the higher priority as determined by the SCI formats scheduling the transmissions or, in case of a S-SS/PSBCH block or a sidelink synchronization signal using E-UTRA radio access, as indicated by higher layers.

-------------------------- <Unchanged parts are omitted> ----------------------------------------

# References

- 3GPP TS 38.213 V16.1.0, NR; Physical layer procedures for control

- R1-1913601, Summary of RAN1 Agreements/Working assumptions in WI 5G V2X with NR sidelink, LG Electronics

- 3GPP TR 38.885 V16.0.0, NR; Study on NR Vehicle-to-Everything (V2X)