3GPP TSG RAN WG1 #100	                                             R1-2003357

e-Meeting, May 25th – June 5th, 2020

Source:	vivo

Title:	Discussion on UL codebook subset

Agenda Item:	7.1.2

Document for:	Discussion and Decision

- Discussion

In current version of 38.214v15.9.0 section 6.1.1.1, it is specified that a UE reporting partialAndNonCoherent capability is not expected to be configured with codebook subset with fullyAndPartialAndNonCoherent.

A UE reporting its UE capability of 'partialAndNonCoherent' transmission shall not expect to be configured by codebookSubset with 'fullyAndPartialAndNonCoherent'.

Only an UE with 4Tx will report the capability of partialAndNonCoherent and thus it also specified that an UE is not expected to be configured with codebook subset of partialAndNonCoherent' if the associated SRS resource is two ports.

A UE shall not expect to be configured with the higher layer parameter codebookSubset set to 'partialAndNonCoherent' when higher layer parameter nrofSRS-Ports in an SRS-ResourceSet with usage set to 'codebook' indicates that two SRS antenna ports are configured.

However, it is not clear from the current spec that when a 4Tx UE reporting partialAndNonCoherent capability is configured by gNB with 2 port SRS, which codebook subset can be configured to the UE is not clear. There are two possible codebook subsets associated with 2 port SRS, fullyAndPartialAndNonCoherent or nonCoherent. In such scenario, it should be clarified whether an UE supports 2Tx codebook subset of fullyAndPartialAndNonCoherent or nonCoherent. There are two options to clarify in the spec:

Option1: clarify that a UE reporting capability of partialAndNonCoherent' transmission when configured with 2 port SRS is not expected to be configured by codebookSubset with 'fullyAndPartialAndNonCoherent', that means this UE supports nonCoherent codebook subset for 2Tx.

Option2: clarify that a UE reporting capability of partialAndNonCoherent' transmission when configured with 2 port SRS, the restriction of configurable codebook subset is not applicable.

- References

- 38.214v15.9.0