3GPP TSG RAN WG1 Meeting #101-e	R1-2003512

E-meeting, May 25 - June 5, 2020

Agenda Item:	7.2.2.2.1

Source:	Huawei, HiSilicon

Title:	Maintenance on the channel access procedure

Document for:	Discussion and Decision

# Introduction

In this contribution, we discuss some issues related to the channel access procedures in NR-U Rel-16 as captured by the latest draft specifications v16.1.0 and the endorsed editor CR. We propose some corrections/modifications in the form of TPs targeting respective subclauses to ensure the accuracy and completeness of the final NR-U Rel-16 specifications in terms of capturing the achieved agreements and the common understanding of the group.

# Discussion

## Restrictions to Type 1 DL channel access procedure

In section 4.1.1, it is currently specified that Type 1 channel access procedure is applicable to certain DL transmission(s) initiated by an eNB or a gNB. However, the current subclause implies that Type 1 channel access procedure is not applicable to other PDSCH transmission(s) than unicast with user plane data. Whereas, Type 1 channel access procedure should be also applicable to transmission(s) initiated by a gNB including non-unicast PDSCH such as OSI, paging, and RAR, or unicast PDSCH without user plane data such as RRC signaling.

Furthermore, regarding the two sub-bullets in that section on the CAPC used, the 1st sub-bullet specifies using any CAPC if the transmission(s) includes DRS while the second one specifies using CAPC as applicable to the user plane data when multiplexed with DRS. We observe that these two sub-bullets are in contradiction, and thus, the language of the first sub-bullet should recite “only including discovery burst(s)”.

We therefore propose to adopt TP1 into that section for clarity and to further align the spec with agreement below.

RAN1 #96

Agreement:

For initiation of a gNB transmission:

- LBT other than Cat 4 is not used for DRS multiplexed with unicast data

- LBT other than Cat 4 is not used for PDCCH and/or PDSCH transmission outside of DRS.

Note:

- This does not preclude the use of Cat 2 for transmission on a LBT bandwidth if it is allowed for the case of transmission on multiple LBT bandwidths

## *** <Beginning of Text Proposal 1> *** ### 4.1.1 Type 1 DL channel access procedures This subclause describes channel access procedures to be performed by an eNB/gNB where the time duration spanned by the sensing slots that are sensed to be idle before a downlink transmission(s) is random. The subclause is applicable to the following transmissions: - Transmission(s) initiated by an eNB including PDSCH/PDCCH/EPDCCH, or - Transmission(s) initiated by a gNB including unicast PDSCH with user plane data, or unicast PDSCH with user plane data and unicast PDCCH scheduling user plane data, or - Transmission(s) initiated by a gNB including PDSCH for OSI, paging, RAR and RRC signalling, or - Transmission(s) initiated by a gNB with only discovery burst or with discovery burst multiplexed with non-unicast information, where the transmission(s) duration is larger than 1ms or the transmission causes the discovery burst duty cycle to exceed 1/20. ## *** Unchanged text is omitted *** A gNB may use any channel access priority class for performing the procedures above to transmit transmission(s) including only discovery burst(s) satisfying the conditions described in this subclause. A gNB shall use a channel access priority class applicable to the unicast user plane data multiplexed in PDSCH for performing the procedures above to transmit transmission(s) including unicast PDSCH with user plane data. *** <End of Text Proposal 1> ***

Proposal 1：Adopt TP1 into section 4.1.1 of TS 37.213.

## UL-to-DL COT sharing

### UE choosing between ul-toDL-CO-SharingED-Threshold-r16 and the default one

In the previous meeting, the following agreement was achieved

Agreement:

For at least PUSCH transmissions with configured grants, a UE is allowed to choose between the ED threshold given by ul-toDL-CO-SharingED-Threshold-r16 and the default one. Whether a spec change is required needs further discussion. Discuss and decide the possible TPs in the next meeting.

We note that when the UE is provided with ul-toDL-COT-SharingED-Threshold-r16, the and higher layer parameter cg-COT-SharingList-r16 should be also configured and the COT sharing indication field of length log2C is used in such case. According the curent specifications in section 4.2.3 of TS 37.213 v16.1.0, the UE will have to choose the EDT provided by ul-toDL-COT-SharingED-Threshold-r16 if it intends to share the CO with gNB. If the UE does not intend to share the CO, i.e., does not provision resource for DL transmission and/or chose to use the default EDT, the UE will have to indicate ‘COT sharing not available’ via the COT sharing indication filed bits.

For the case where a UE performs channel access procedures as described in clause 4.2.1.2.1 and shares its corresponding channel occupancy time with the gNB, XThresh_max is set equal to the value provided by the higher layer parameter ul-toDL-COT-SharingED-Threshold-r16, if provided.

Proposal 2：For UE choosing between ul-toDL-CO-SharingED-Threshold-r16 and the default one for a transmission with CG, no further Specification change is needed.

### Restriction of the UL-DL gap to either up to 16us or 25us

It was discussed in the previous meeting whether the constraint on the UL-DL gap duration can be relaxed such that UL-to-DL COT sharing can be used when the gap is > 25us. However, no consensus was reached and it was decided to close the discussion for that meeting. Although the current restriction is not a concern for scheduled UL as it can be avoided by gNB scheduling, it is a concern for sharing a CO initiated by CG-PUSCH as the restriction significantly limits the use of the UL-to-DL sharing mechanism given the slot granularity of the COT sharing indication in the CG-UCI as shown in Fig. 1.

Therefore, it is important to discuss alternative solutions to overcome the inefficiency of the UL-to-DL COT sharing mechanism for CG if allowing a gap duration >25us is not agreeable.

Fig. 1: gNB will often not be able to transmit the DL transmission because a gap of the specified durations cannot be achieved when the CG UL burst ends before the end of the immediately preceding slot

Proposal 3：Alternative solutions should be discussed to overcome the inefficiency of the UL-to-DL COT sharing mechanism for a CO initiated by CG-PUSCH if allowing a gap duration >25us is not agreeable.

## Contention window adjustment procedures for DL transmissions by gNB

In section 4.1.4.2 of the current TS 37.213 v16.1.0 on CWS adjustment for DL transmissions by gNB we observe the following:

1. The current procedure recites a “reference duration corresponding to the earliest DL transmission burst” and also a “reference duration corresponding to the latest DL transmission burst” whereas the same subclause defines the ‘reference duration’ to be “corresponding to a channel occupancy initiated by the gNB”.

2. “HARQ-ACK feedback(s) corresponding to PDSCH(s) in the reference duration for the latest DL transmission burst” is inconsistent with the part highlighted in blue of the agreement quoted below.

As such, we propose to change “transmission burst” to “channel occupancy” in steps 2) and 3).

3. It should be also clarified that for a PDSCH with some CBGs overlapping with the subband (channel) in consideration and some other CBGs overlapping with another subband (channel), the 10% ratio is calculated out of the number of CBGs overlapping with the channel in consideration.

Therefore, to ensure clarity and consistency of this subclause of the spec, we propose to adopt the following TP2 into that section of TS 37.213

RAN1 #99

Agreement:

For CW adjustment for transmissions including PDSCH and PUSCH:

- if new HARQ feedback is available relative to the prior CW update, the feedback for the latest COT for which new feedback is received shall be used:

- if the HARQ feedback is ACK, CW shall be set to CWmin.

- if the HARQ feedback is NACK (or if in absence of feedback within a window as defined below, the gNB or the UE retransmits the TB), CW shall be set to min(CW×2 + 1, CWmax).

  - Window starts at the end of the reference duration and has a duration of max (X ms, the duration of the transmission burst from start of the reference duration + 1 ms)

  - If the absence of other technologies cannot be guaranteed (same condition as in existing specs for other cases), X = 5. Otherwise, X = 10.

- Otherwise (if new HARQ feedback is not available), CW shall remain the same.

- Note: HARQ feedback includes any implicit methods of HARQ feedback determination.

Agreement:

For CWS adjustment for an LBT sub-band when a single contention window is maintained per LBT subband, all CBGs (if any are present) and TBs that partially or fully overlap with that LBT sub-band are taken into account.

- CW is reset if “ACK” is received for at least 10 % of the CBGs or for at least one TB in the reference duration

- Note: Other procedures for contention window adjustment within an LBT subband are also applicable

- A UE can choose to apply feedback only based on TBs for CW adjustment

## *** <Beginning of Text Proposal 2> *** #### 4.1.4.2 Contention window adjustment procedures for DL transmissions by gNB If a gNB transmits transmissions including PDSCH that are associated with channel access priority class p on a channel, the gNB maintains the contention window value CWp and adjusts CWp before step 1 of the procedure described in subclause 4.1.1 for those transmissions using the following steps: - For every priority class p∈1,2,3,4, set CWp=CWmin,p. - If HARQ-ACK feedback is available after the last update of Wp , go to step 3. Otherwise, if the gNB transmission after procedure described in subclause 4.1.1 does not include a retransmission or is transmitted within a duration Tw from the end of the reference duration corresponding to the earliest DL channel occupancy after the last update of CWp , go to step 5; otherwise go to step 4. - The HARQ-ACK feedback(s) corresponding to PDSCH(s) in the reference duration for the latest DL channel occupancy for which HARQ-ACK feedback is available is used as follows: - If at least one HARQ-ACK feedback is ‘ACK’ for PDSCH(s) with transport block based transmissions or at least 10% of HARQ-ACK feedbacks is ‘ACK’ for CBGs overlapping with the channel and in PDSCH(s) with code block group based transmissions go to step 1; otherwise go to step 4. - Increase CWp for every priority class p∈1,2,3,4 to the next higher allowed value. - For every priority class p∈1,2,3,4, maintain CWp as it is; go to step 2. The reference duration and duration Tw in the procedure above are defined as follows: - The reference duration corresponding to a channel occupancy initiated by the gNB including transmission of PDSCH(s) is defined in this subclause as a duration starting from the beginning of the channel occupancy until the end of the first slot where at least one unicast PDSCH is transmitted over all the resources allocated for the PDSCH, or until the end of the first transmission burst by the gNB that contains unicast PDSCH(s) transmitted over all the resources allocated for the PDSCH, whichever occurs earlier. If the channel occupancy includes a unicast PDSCH, but it does not include any unicast PDSCH transmitted over all the resources allocated for that PDSCH, then, the duration of the first transmission burst by the gNB within the channel occupancy that contains unicast PDSCH(s) is the reference duration for CWS adjustment. - Tw=max⁡(TA,TB+1ms) where TB is the duration of the transmission burst from start of the reference duration in ms and TA=5ms if the absence of any other technology sharing the channel can not be guaranteed on a long-term basis (e.g. by level of regulation), and TA=10ms otherwise. ## *** Unchanged text is omitted *** ## *** <End of Text Proposal 2> ***

Proposal 4：Adopt TP2 into section 4.1.4.2 of TS 37.213.

## Contention window adjustment procedures for UL transmissions scheduled/configured by gNB

Similarly, in section 4.2.2.2 of the current TS 37.213 v16.1.0 on CWS adjustment procedures for UL transmissions scheduled/configured by gNB, we observe the following:

1. The current procedure recites a “reference duration corresponding to the earliest UL transmission burst” and also a “reference duration corresponding to the latest UL transmission burst” whereas the same subclause defines the ‘reference duration’ to be “corresponding to a channel occupancy initiated by the UE”.

2. “HARQ-ACK feedback(s) corresponding to PDSCH(s) in the reference duration for the latest DL transmission burst” is inconsistent with the part highlighted in blue of the same agreement quoted in the previous section.

Therefore, we propose to change “transmission burst” to “channel occupancy” in steps 2) and 3).

Proposal 5：Adopt TP3 into section 4.2.2.2 of TS 37.213.

3. It should be also clarified that for a PUSCH with some CBGs overlapping with the subband (channel) in consideration and some other CBGs overlapping with another subband (channel), the 10% ratio is calculated out of the number of CBGs overlapping with the channel in consideration.

4. Furthermore, it should be noted that retransmission of Msg3 PUSCH can be scheduled by DCI format 0_0 scrambled by TC-RNTI. Since Msg 3 PUSCH satisfies the subclause in section 4.2.2.2 of TS 37.213 as a UL channel with implicit HARQ feedback NACK, receiving Msg 4 in response should be considered as an implicit HARQ feedback ACK for the purpose of CWS adjustment.

Therefore, we propose to adopt the following TP4 into section 4.2.2.2 of TS 37.213 to ensure clarity and consistency of this subclause of the specifications.

Proposal 6：Since Msg 3 PUSCH satisfies the subclause in section 4.2.2.2 of TS 37.213 as a UL channel with implicit HARQ feedback NACK, receiving Msg 4 in response should be considered as an implicit HARQ feedback ACK for the purpose of CWS adjustment.

Proposal 7：Adopt TP4 into section 4.2.2.2 of TS 37.213.

## *** <Beginning of Text Proposal 3> *** #### 4.2.2.2 Contention window adjustment procedures for UL transmissions scheduled/configured by gNB If a UE transmits transmissions using Type 1 channel access procedures that are associated with channel access priority class p on a channel, the UE maintains the contention window value CWp and adjusts CWp for those transmissions before step 1 of the procedure described in subclause 4.2.1.1, using the following steps: - For every priority class p∈1,2,3,4, set CWp=CWmin,p; - If HARQ-ACK feedback is available after the last update of CWp, go to step 3. Otherwise, if the UE transmission after procedure described in subclause 4.2.1.1 does not include a retransmission or is transmitted within a duration Tw from the end of the reference duration corresponding to the earliest UL channel occupancy after the last update of CWp, go to step 5; otherwise go to step 4. - The HARQ-ACK feedback(s) corresponding to PUSCH(s) in the reference duration for the latest UL channel occupancy for which HARQ-ACK feedback is available is used as follows: - If at least one HARQ-ACK feedback is ‘ACK’ for PUSCH(s) with transport block (TB) based transmissions or at least 10% of HARQ-ACK feedbacks is ‘ACK’ for CBGs overlapping with the channel and in PUSCH(s) with code block group (CBG) based transmissions go to step 1; otherwise go to step 4. - Increase CWp for every priority class p∈1,2,3,4 to the next higher allowed value; - For every priority class p∈1,2,3,4, maintain CWp as it is; go to step 2. ## *** Unchanged text is omitted *** ## *** <End of Text Proposal 3> ***

## *** <Beginning of Text Proposal 4> *** #### 4.2.2.2 Contention window adjustment procedures for UL transmissions scheduled/configured by gNB ## *** Unchanged text is omitted *** The HARQ-ACK feedback, reference duration and duration Tw in the procedure above are defined as the following: - HARQ-ACK feedback for PUSCH(s) transmissions are expected to be provided to UE(s) explicitly or implicitly where implicit HARQ-ACK feedback for the purpose of contention window adjustment in this subclause, is determined based on either the indication for a new transmission or retransmission in the DCI scheduling PUSCH(s) or the success or failure of receiving of an associated channel/signal in response as follows: - If a new transmission is indicated, ‘ACK’ is assumed for the transport blocks or code block groups in the corresponding PUSCH(s) for the TB-based and CBG-based transmission, respectively. - If a retransmission is indicated for TB-based transmissions, ‘NACK’ is assumed for the transport blocks in the corresponding PUSCH(s). - If a retransmission is indicated for CBG-based transmissions, if a bit value in the code block group transmission information (CBGTI) field is ‘0’ or ‘1’ as described in subclause 5.1.7.2 in [8], ‘ACK’ or ‘NACK’ is assumed for the corresponding CBG in the corresponding PUSCH(s), respectively. - If the UE successfully receives Msg 4 in response to its transmission of Msg 3 PUSCH, ‘ACK’ is assumed for the corresponding TB in the corresponding PUSCH. - The reference duration corresponding to a channel occupancy initiated by the UE including transmission of PUSCH(s) is defined in this subclause as a duration starting from the beginning of the channel occupancy until the end of the first slot where at least one PUSCH is transmitted over all the resources allocated for the PUSCH, or until the end of the first transmission burst by the UE that contains PUSCH(s) transmitted over all the resources allocated for the PUSCH, whichever occurs earlier. If the channel occupancy includes a PUSCH, but it does not include any PUSCH transmitted over all the resources allocated for that PUSCH, then, the duration of the first transmission burst by the UE within the channel occupancy that contains PUSCH(s) is the reference duration for CWS adjustment. ## *** Unchanged text is omitted *** ## *** <End of Text Proposal 4> ***

## Multi-channel access procedures for downlink transmission(s)

In section 4.1.6 of the current TS 37.213 v16.1.0, the highlighted sentence below was added to the procedures of both type B1 and type B2 (when a single CW is maintained for multiple LBT subbands) based on the agreement accounting for any fully or partially overlapping PDSCH with the channel for which the CWS adjustment procedure is conducted. We note that the same behavior is also applicable for both type A1 and type A2 procedures (when a single CW is maintained per LBT subband) and the same sentence should be added to the respective subclauses. We propose as such adopting TP5 into that section of the specifications.

Agreement:

For CWS adjustment for an LBT sub-band when a single contention window is maintained per LBT subband, all CBGs (if any are present) and TBs that partially or fully overlap with that LBT sub-band are taken into account.

- CW is reset if “ACK” is received for at least 10 % of the CBGs or for at least one TB in the reference duration

- Note: Other procedures for contention window adjustment within an LBT subband are also applicable

- A UE can choose to apply feedback only based on TBs for CW adjustment

Agreement:

For CWS adjustment for DL, when a single contention window is maintained for multiple LBT subbands, all CBGs (if any are present) and TBs that partially or fully overlap with those multiple LBT sub-bands are taken into account.

- CW is reset if “ACK” is received for at least 10 % of the CBGs or for at least one TB in the reference duration

- Note: Other procedures for contention window adjustment within an LBT subband are also applicable.

##### 4.1.6.2.1 Type B1 multi-channel access procedure A single CWp value is maintained for the set of channels C. For determining CWp for channel access on channel cj, step 2 of the procedure described in subclause 4.1.4.1 is modified as follows - if at least Z=80% of HARQ-ACK values corresponding to PDSCH transmission(s) in reference subframe k of all channels ci∈C are determined as NACK, increase CWp for each priority class p∈1,2,3,4 to the next higher allowed value; otherwise, go to step 1. For determining CWp for a set of channels C, any PDSCH that fully or partially overlaps with any channel ci∈C, is used in the procedure described in subclause 4.1.4.2. ##### 4.1.6.2.2 Type B2 multi-channel access procedure A CWp value is maintained independently for each channel ci∈C using the procedure described in subclause 4.1.4. For determining CWp for channel ci, any PDSCH that fully or partially overlaps with channel ci, is used in the procedure described in subclause 4.1.4.2. For determining Ninit for channel cj, CWp value of channel cj1∈C is used, where cj1 is the channel with largest CWp among all channels in set C.

Proposal 8：Adopt TP5 into section 4.1.6.1 of TS 37.213.

## *** <Beginning of Text Proposal 5> *** ##### 4.1.6.1.1 Type A1 multi-channel access procedures Counter N as described in subclause 4.1.1 is independently determined for each channel ci and is denoted as Nci. If the absence of any other technology sharing the channel cannot be guaranteed on a long term basis (e.g. by level of regulation), when the eNB/gNB ceases transmission on any one channel cj∈C, for each channel ci≠cj, the eNB/gNB can resume decrementing Nci when idle sensing slots are detected either after waiting for a duration of 4⋅Tsl, or after reinitialising Nci. For determining CWp for channel ci, any PDSCH that fully or partially overlaps with channel ci, is used in the procedure described in subclause 4.1.4.2. ##### 4.1.6.1.2 Type A2 multi-channel access procedures Counter N is determined as described in subclause 4.1.1 for channel cj∈C, and is denoted as Ncj, where cj is the channel that has the largest CWp value. For each channel ci, Nci=Ncj. When the eNB/gNB ceases transmission on any one channel for which Nci is determined, the eNB/gNB shall reinitialise Nci for all channels. For determining CWp for channel ci, any PDSCH that fully or partially overlaps with channel ci, is used in the procedure described in subclause 4.1.4.2. ## *** <End of Text Proposal 5> ***

## Channel access procedures upon detection of a common DCI

In section 4.2.1.0.0 of the current TS 37.213 v16.1.0, the intended behavior is that the UE is allowed to switch from CAT4 to CAT2 within the available RB set and COT duration. However, there is one case in which the gNB indicates all RB set are ‘not available’ while COT duration is a valid value which would typically happen when the GC-PDCCH is at beginning of COT. In such case, and according to the agreement, the UE would not be allowed to switch CAT4 LBT to CAT2 LBT in all RB sets even on the RB set(s) where DCI format 2_0 was detected.

Therefore, in order to improve the efficiency of channel access, we propose that when the gNB transmits Available RB Set Indicator immediately after LBT succeeds in which the gNB indicates all RB sets are not available for reception, i.e., due to insufficient time to update the DCI content, the UE would be allowed to switch CAT4 LBT to CAT2 LBT on the same RB set(s) where DCI format 2_0 carrying the available RB set indicator was detected.

Proposal 9: UE should be allowed to switch CAT4 LBT to CAT2 LBT on the same RB set(s) where DCI format 2_0 carrying available RB set indicator is detected when the available RB set indicators indicate all RB sets are not available for reception.

Proposal 10：Adopt TP6 into section 4.2.1.0.0 of TS 37.213.

## *** <Beginning of Text Proposal 6> *** ##### 4.2.1.0.0 Channel access procedures upon detection of a common DCI *** Unchanged text is omitted *** If a UE determines the duration in time domain and the location in frequency domain of a remaining channel occupancy initiated by the gNB from a DCI format 2_0 as described in subclause 11.1.1 of [7], the following is applicable: - The UE may switch from Type 1 channel access procedures as described in subclause 4.2.1.1 to Type 2A channel access procedures as described in subclause 4.2.1.2.1 for its corresponding UL transmissions within the determined duration in time and location in frequency domain of the remaining channel occupancy. In this case, if the UL transmissions are PUSCH transmissions on configured resources, the UE may assume any priority class for the channel occupancy shared with the gNB. - If the UE determines from a DCI format 2_0 that all RB sets are not available, the UE may switch from Type 1 channel access procedures to Type 2A channel access procedures on the same RB set(s) where the DCI format 2_0 was detected. ## *** Unchanged text is omitted *** ## *** <End of Text Proposal 6> ***

## Multi-channel access procedures for UL transmissions

In section 4.2.1.0.4 of the current TS 37.213 v16.1.0, we observe that the channel access procedures for UL multi-channel transmission(s) are based only on the CA mode of FeLAA and do not account for the cases in which a wideband UL transmission could be scheduled by one UL grant or configured on configured resources on a set of channels. Therefore, we propose to capture these wideband cases for NR-U in the following TP.

Furthermore, it should be noted that the highlighted constraint below ensuing conformance with WiFi channel boding is only necessary for the case wherein the UE uses type 1 channel access procedure on a channel along with type 2 channel access procedures on the remaining channels on which the transmission(s) is scheduled/configured. Therefore, in LTE LAA, it is understood that if such set of channels is not in conformance to the WiFi channel bonding, the UE will perform Type 1 channel access on individual channels.

As such, to avoid further scheduling restrictions in NR-U, we propose that for an NR-U UE scheduled with a PUSCH on multiple RB sets and indicated with type 1 channel access, if the channel frequencies of the corresponding set of channels C is NOT “a subset of one of the sets of channel frequencies defined in subclause 5.7.4 in [2],” the UE performs type 1 channel access procedure on each channel individually.

Proposal 11: For an NR-U UE scheduled with a PUSCH on multiple RB sets and indicated with type 1 channel access, if the channel frequencies of the corresponding set of channels C is NOT a subset of one of the sets of channel frequencies defined in subclause 5.7.4 in [2], the UE performs type 1 channel access procedure on each channel individually.

Proposal 12：Adopt TP7 into section 4.2.1.0.4 of TS 37.213.

## *** <Beginning of Text Proposal 7> *** ##### 4.2.1.0.4 Channel access procedures for UL multi-channel transmission(s) If a UE - is scheduled to transmit on a set of channels C, and if Type 1 channel access procedure is indicated by the UL scheduling grant for the UL transmission on the set of channels C, or - is scheduled to transmit on a set of channels C, and if Type 1 channel access procedure is indicated by the UL scheduling grants for the UL transmissions on the set of channels C, and if the UL transmissions are scheduled to start transmissions at the same time on all channels in the set of channels C , or - intends to perform an uplink transmission on a configured resources on the set of channels C with Type 1 channel access procedure, or - intends to perform an uplink transmission on configured resources on the set of channels C with Type 1 channel access procedure, and if UL transmissions are configured to start transmissions on the same time all channels in the set of channels C, and if the channel frequencies of set of channels C is a subset of one of the sets of channel frequencies defined in subclause 5.7.4 in [2] - the UE may transmit on channel ci∈C using Type 2 channel access procedure as described in subclause 4.2.1.2, - if Type 2 channel access procedure is performed on channel ci immediately before the UE transmission on channel cj∈C, i≠j, and - if the UE has accessed channel cj using Type 1 channel access procedure as described in subclause 4.2.1.1, - where channel cj is selected by the UE uniformly randomly from the set of channels C before performing Type 1 channel access procedure on any channel in the set of channels C. - the UE may not transmit on channel ci∈C within the bandwidth of a carrier, if the UE fails to access any of the channels, of the carrier bandwidth, on which the UE is scheduled or configured by UL resources. if the channel frequencies of set of channels C is not a subset of one of the sets of channel frequencies defined in subclause 5.7.4 in [2], the UE may transmit on channel ci∈C using type 1 channel access procedure as described in subclause 4.2.1.1. ## *** Unchanged text is omitted *** ## *** <End of Text Proposal 7> ***

## Channel access procedures for consecutive UL transmission(s)

### Direct transmission of scheduled UL following configured grant UL transmission(s)

The second subclause under “For UL transmission(s) following autonomous UL transmission(s)” considers the case when a UE is scheduled by an eNB to transmit PUSCH transmission(s) with full BW allocation using Type 1 channel access procedure yet the first scheduled PUSCH starts immediately after AUL transmission(s). The behavior is to simply transmit the scheduled PUSCH(s) without LBT as long as the multiplexing rules of CAPC and MCOT are satisfied, or otherwise the AUL transmission preceding the start of the first scheduled PUSCH is dropped. The condition of full BW allocation on the channel ensures that no FDMed scheduled UL transmissions for other UEs are blocked due to the lack of a gap for LBT.

In the previous meeting, the following agreement was achieved that a similar behavior is supported for back-to-back transmission of GC-PUSCH and dynamically scheduled PUSCH in NR-U with restrictions similar to those in LTE LAA, i.e., the Full BW allocation, MCOT and CAPC restrictions.

Agreement:

Back-to-back transmission of GC-PUSCH and dynamically scheduled PUSCH is supported in NR-U with restrictions similar to those in LTE LAA.

Discuss the exact TP taking TP#9 in R1-2001534 as the starting point (until 4/28)

As such, starting from TP#9 in our contribution R1-2001534, the group has further discussed through email discussion [100b-e-NR-unlic-NRU-ChAcc-01] and provided some edits to improve the flow and accuracy. However, further discussion was needed on how to capture the NR rule for dropping/canceling the transmission on CG-PUSCH.

Therefore, the latest version of the TP from the email discussion has be used as basis to TP8 below.

During the email discussion, some companies pointed out some specific cases to consider for the back-to-back transmission as follows:

- Whether Scheduled UL may be PUCCH or SRS: While TP8 is so far not restricting this back-to-back transmission behavior to only scheduled PUSCH, it worth noting that the full BW allocation condition cannot be met for scheduled PUCCH or aperiodic SRS. For instance, the enhanced PUCCH formats in NR-U may only occupy one or two frequency interlaces. Therefore, when further discussing this TP, the discussion should be focused on scheduled PUSCH only.

- Minimum duration for which transmission on CG-PUSCH is dropped: Since the UE is aware of the CWS it will use for Type 1 channel access before the scheduled UL, we think that the actual duration for which the transmission on CG-PUSCH is dropped should be determined by UE implementation but at least the transmission on the symbols of the last CG-PUSCH before the beginning of the DG-PUSCH should be dropped.

- If the conditions for dropping the transmission on CG-PUSCH are not met: Since the main reason for checking the mechanism for dropping transmission on CG-PUSCH is that the multiplexing rules for the back-to-back transmission were not met, when conditions for dropping the transmission on CG-PUSCH are not met, proceeding with the back-to-back transmission would violate the regulations, and therefore there seems to be no remedy in such a case other than the UE to ignore the scheduling grant.

Proposal 13：For the back-to-back transmission of GC-PUSCH and dynamically scheduled PUSCH in NR-U, the following is applicable:

- The duration for which the transmission on CG-PUSCH is dropped should be determined by UE implementation such that at least the transmission on the symbols of the last CG-PUSCH before the beginning of the DG-PUSCH should be dropped

- If the UE cannot terminate the configured grant UL transmission(s), the UE ignores the scheduling UL grant.

Proposal 14：Adopt TP8 into section 4.2.1.0.1 of TS 37.213.

## *** <Beginning of Text Proposal 8> *** ##### 4.2.1.0.1 Channel access procedures for consecutive UL transmission(s) ## *** Unchanged text is omitted *** For UL transmission(s) following autonomous UL transmission(s), the following are applicable: - If a UE is scheduled by an eNB to transmit on channel ci by a UL grant received on channel cj, i≠j, and if the UE is transmitting using autonomous UL on channel ci, the UE shall terminate the ongoing PUSCH transmissions using the autonomous UL at least one subframe before the UL transmission according to the received UL grant. - If a UE is scheduled by a UL grant received from an eNB on a channel to transmit a PUSCH transmission(s) starting from subframe n on the same channel using Type 1 channel access procedure and if at least for the first scheduled subframe occupies NRBUL resource blocks and the indicated PUSCH starting position is OFDM symbol zero, and if the UE starts autonomous UL transmissions before subframe n using Type 1 channel access procedure on the same channel, the UE may transmit UL transmission(s) according to the received UL grant from subframe n without a gap, if the priority class value of the performed channel access procedure is larger than or equal to priority class value indicated in the UL grant, and the autonomous UL transmission in the subframe preceding subframe n shall end at the last OFDM symbol of the subframe regardless of the higher layer parameter endingSymbolAUL. The sum of the lengths of the autonomous UL transmission(s) and the scheduled UL transmission(s) shall not exceed the maximum channel occupancy time corresponding to the priority class value used to perform the autonomous uplink channel access procedure. Otherwise, the UE shall terminate the ongoing autonomous UL transmission at least one subframe before the start of the UL transmission according to the received UL grant on the same channel. For UL transmission(s) following configured grant UL transmission(s), the following are applicable: - If a UE is scheduled to transmit UL transmission(s) starting from symbol i in slot n using Type 1 channel access procedures without CP extension with a corresponding CAPC, and if the UE starts configured grant UL transmissions before slot n using Type 1 channel access procedures with a corresponding CAPC, and the scheduled UL transmission(s) occupies all the RBs of the same channels occupied by the configured grant UL transmission(s) or all the RBs of a subset thereof, the UE may directly continue to transmit the scheduled UL transmission(s) to the corresponding CAPC from symbol i in slot n without a gap, if the CAPC value of the performed channel access procedure is larger than or equal to the CAPC value corresponding to the scheduled UL transmission(s). The sum of the transmission durations of the configured grant UL transmission(s) and the scheduled UL transmission(s) shall not exceed the MCOT duration corresponding to the CAPC value used to transmit the configured grant UL transmission(s). Otherwise, the UE shall terminate the configured grant UL transmission(s) by dropping the transmission on the symbols of at least the last configured grant UL transmission before symbol i in slot n and attempt to transmit the scheduled UL transmission(s) according to the corresponding CAPC. PUSCH transmission with a configured grant in a slot is dropped according to the mechanism in Clause 11.1 of [7, TS 38.213] relative to a last symbol of a CORESET where the UE detected the scheduling DCI. In this case, if the UE cannot terminate the configured grant UL transmission(s), the UE ignores the scheduling DCI. ## *** Unchanged text is omitted *** ## *** <End of Text Proposal 8> ***

### LBT type after a transmit pause within a set of PUSCH(s) scheduled by one or more UL grants

The first subclause under “For contiguous UL transmissions(s) including a transmission pause” considers the case “If a UE is scheduled to transmit a set of consecutive UL transmissions without gaps using one or more UL grant(s), and if the UE has stopped transmitting during or before one of these UL transmissions”. Given that only Type 1 and Type 2 channel access procedures could be indicated in LAA, and given that this subclause is not specific to UEs scheduled by eNB, the current behavior “the UE may transmit a later UL transmission in the set using Type 2 channel access procedure” should be clarified such that Type 2A is used without applying a CP extension.

Proposal 15：If a UE is scheduled by a gNB to transmit a set of contiguous PUSCHs using one or more UL grants, and if the UE has stopped transmitting during or before one of these UL transmissions, the UE may transmit a later UL transmission in the set using Type 2A UL channel access procedure without applying a CP extension.

Proposal 16：Adopt TP9 into section 4.2.1.0.1 of TS 37.213.

## *** <Beginning of Text Proposal 9> *** ##### 4.2.1.0.1 Channel access procedures for consecutive UL transmission(s) ## *** Unchanged text is omitted *** For contiguous UL transmissions(s) including a transmission pause, the following are applicable: - If a UE is scheduled to transmit a set of consecutive UL transmissions without gaps using one or more UL grant(s), and if the UE has stopped transmitting during or before one of these UL transmissions in the set and prior to the last UL transmission in the set, and if the channel is sensed by the UE to be continuously idle after the UE has stopped transmitting, the UE may transmit a later UL transmission in the set using Type 2 channel access procedure or Type 2A UL channel access procedure without applying a CP extension. - If a channel sensed by a UE is not continuously idle after the UE has stopped transmitting, the UE may transmit a later UL transmission in the set using Type 1 channel access procedure with the UL channel access priority class indicated in the DCI corresponding to the UL transmission. ## *** Unchanged text is omitted *** ## *** <End of Text Proposal 9> ***

## Channel access procedures for Semi-static channel occupancy

In the current subclause in section 4.3 of TS 37.213 v16.1.0, we observe the following:

1. The subclause recites “a periodic channel occupancy can be initiated every Tx within every two consecutive radio frames” which does not restrict that the periodic channel occupancy can only be initiated by the gNB.

2. The subclause also recites “when a gNB or a UE performs sensing for evaluating a channel availability,” which does not restrict the behavior to only the UEs scheduled/configured by a gNB given that the specifications in this document often address UEs scheduled/configured by an eNB as well.

3. For the case “If the channel is sensed to be busy, the gNB shall not perform any transmission during the current channel occupancy time.”, it should be noted that the CO would not be initiated after the channel is sensed busy. Thus, technically, the COT would not exist. Rather, the appropriate description in such a case is that the gNB and its connected UEs are not allowed to transmit for the whole period.

Therefore, we propose adopting TP10 into that section of TS 37.213.

## *** <Beginning of Text Proposal 10> *** ## 4.3 Channel access procedures for Semi-static channel occupancy If the absence of any other technology sharing a channel can be guaranteed on a long-term basis (e.g. by level of regulation) and if a gNB provides UE(s) with higher layer parameters ChannelAccessMode-r16 ='semistatic' by SIB1 or dedicated configuration, a periodic channel occupancy can be initiated by the gNB only every Tx within every two consecutive radio frames, starting from the even indexed radio frame at x∙Tx with a maximum channel occupancy time Ty=0.95Tx, where Tx=Period in ms, is a higher layer parameter provided in semiStaticChannelAccessConfig-r16 and x∈0,1,…,20Tx-1. In the following procedures in this subclause, when a gNB or a UE scheduled/configured by a gNB performs sensing for evaluating a channel availability, the sensing is performed at least during a sensing slot duration Tsl=9us. The corresponding XThresh adjustment for performing sensing by a gNB or a UE is described in subclauses 4.1.5 and 4.2.3, respectively. A channel occupancy initiated by a gNB and shared with UE(s) shall satisfy the following: - The gNB shall transmit a DL transmission burst starting at the beginning of the channel occupancy time immediately after sensing the channel to be idle for at least a sensing slot duration Tsl=9us. If the channel is sensed to be busy, the gNB shall not perform any transmission during the current period. ## *** Unchanged text is omitted *** - The gNB and UEs shall not transmit any transmissions in a set of consecutive symbols for a duration of at least Tz=max0.05Tx , 100us before the start of the next period. ## *** <End of Text Proposal 10> ***

Proposal 17：Adopt TP10 into section 4.3 of TS 37.213.

## LBT failure indication

In response to the reply LS sent from RAN1 in R1-2001397 to RAN2 on the consistent Uplink LBT failure detection mechanism and RAN1’s question about whether the LBT failure indication should be subject to configuration of lbt-FailureRecoveryConfig and/or other higher layer parameters, RAN2 has sent a reply LS in R1-2003254 to RAN1 and stated the following:

“The LBT failure indication is not only used for LBT failure recovery procedure used in MAC but also for the power ramping for RACH, SR counter and configured grant timers. RAN2 has made the agreement that PHY provides LBT failure indications for all uplink transmissions to MAC regardless of the configuration of lbt-FailureRecoveryConfig and/or other higher layer parameters.”

Therefore, the following TPs could be adopted to capture the LBT failure indication for both the dynamic and semi-static channel access modes.

## *** <Beginning of Text Proposal 11> *** ## 4.2 Uplink channel access procedures A UE performing transmission(s) on LAA Scell(s), an eNB scheduling or configuring UL transmission(s) for a UE performing transmission(s) on LAA Scell(s), and a UE performing transmission(s) on channel(s) and a gNB scheduling or configuring UL transmission(s) for a UE performing transmissions on channel(s) shall perform the procedures described in this clause for the UE to access the channel(s) on which the transmission(s) are performed. In this clause, transmissions from a UE are considered as separate UL transmissions, irrespective of having a gap between transmissions or not, and XThresh for sensing is adjusted as described in clause 4.2.3 when applicable. A UE performs channel access procedures in this clause unless the higher layer parameter ChannelAccessMode-r16 is provided and ChannelAccessMode-r16 =' semistatic'. If a UE fails to access the channel(s) prior to a UL transmission intended to a gNB, Layer 1 notifies the higher layers of the channel access failure. ## *** Unchanged text is omitted *** ## *** <End of Text Proposal 11> ***

## *** <Beginning of Text Proposal 12> *** ## 4.3 Channel access procedures for semi-static channel occupancy If the absence of any other technology sharing a channel can be guaranteed on a long-term basis (e.g. by level of regulation) and if a gNB provides UE(s) with higher layer parameters ChannelAccessMode-r16 ='semistatic' by SIB1 or dedicated configuration, a periodic channel occupancy can be initiated every Tx within every two consecutive radio frames, starting from the even indexed radio frame at x∙Tx with a maximum channel occupancy time Ty=0.95Tx, where Tx=Period in ms, is a higher layer parameter provided in semiStaticChannelAccessConfig-r16 and x∈0,1,…,20Tx-1. In the following procedures in this clause, when a gNB or UE performs sensing for evaluating a channel availability, the sensing is performed at least during a sensing slot duration Tsl=9us. The corresponding XThresh adjustment for performing sensing by a gNB or a UE is described in clauses 4.1.5 and 4.2.3, respectively. A channel occupancy initiated by a gNB and shared with UE(s) shall satisfy the following: ## *** Unchanged text is omitted *** If a UE fails to access the channel(s) prior to a UL transmission intended to a gNB, Layer 1 notifies the higher layers of the channel access failure. *** <End of Text Proposal 12> ***

Proposal 18：Adopt TP11 and TP12 into sections 4.2 and 4.3 respectively of TS 37.213.

# Conclusions

Based on the discussion, the following proposals are made:

Proposal 1：Adopt TP1 into section 4.1.1 of TS 37.213.

Proposal 2：For UE choosing between ul-toDL-CO-SharingED-Threshold-r16 and the default one for a transmission with CG, no further Specification change is needed.

Proposal 3：Alternative solutions should be discussed to overcome the inefficiency of the UL-to-DL COT sharing mechanism for a CO initiated by CG-PUSCH if allowing a gap duration >25us is not agreeable.

Proposal 4：Adopt TP2 into section 4.1.4.2 of TS 37.213.

Proposal 5：Adopt TP3 into section 4.2.2.2 of TS 37.213.

Proposal 6：Since Msg 3 PUSCH satisfies the subclause in section 4.2.2.2 of TS 37.213 as a UL channel with implicit HARQ feedback NACK, receiving Msg 4 in response should be considered as an implicit HARQ feedback ACK for the purpose of CWS adjustment.

Proposal 7：Adopt TP4 into section 4.2.2.2 of TS 37.213.

Proposal 8：Adopt TP5 into section 4.1.6.1 of TS 37.213.

Proposal 9: UE should be allowed to switch CAT4 LBT to CAT2 LBT on the same RB set(s) where DCI format 2_0 carrying available RB set indicator is detected when the available RB set indicators indicate all RB sets are not available for reception.

Proposal 10：Adopt TP6 into section 4.2.1.0.0 of TS 37.213.

Proposal 11: For an NR-U UE scheduled with a PUSCH on multiple RB sets and indicated with type 1 channel access, if the channel frequencies of the corresponding set of channels C is NOT a subset of one of the sets of channel frequencies defined in subclause 5.7.4 in [2], the UE performs type 1 channel access procedure on each channel individually.

Proposal 12：Adopt TP7 into section 4.2.1.0.4 of TS 37.213.

Proposal 13：For the back-to-back transmission of GC-PUSCH and dynamically scheduled PUSCH in NR-U, the following is applicable:

- The duration for which the transmission on CG-PUSCH is dropped should be determined by UE implementation such that at least the transmission on the symbols of the last CG-PUSCH before the beginning of the DG-PUSCH should be dropped

- If the UE cannot terminate the configured grant UL transmission(s), the UE ignores the scheduling UL grant.

Proposal 14：Adopt TP8 into section 4.2.1.0.1 of TS 37.213.

Proposal 15：If a UE is scheduled by a gNB to transmit a set of contiguous PUSCHs using one or more UL grants, and if the UE has stopped transmitting during or before one of these UL transmissions, the UE may transmit a later UL transmission in the set using Type 2A UL channel access procedure without applying a CP extension.

Proposal 16：Adopt TP9 into section 4.2.1.0.1 of TS 37.213.

Proposal 17：Adopt TP10 into section 4.3 of TS 37.213.

Proposal 18：Adopt TP11 and TP12 into sections 4.2 and 4.3 respectively of TS 37.213.

# References

- RP-182878, “New WID on NR-based Access to Unlicensed Spectrum”, Qualcomm. Sorrento, Italy, Dec 2019,