3GPP TSG RAN WG1 #101		                                                      R1-2003610

e-Meeting, May 25th – June 5th, 2020

Source:	CATT

Title:	Remaining issues of half-duplex operation in CA

Agenda Item:	7.2.12

Document for:	Discussion and Decision

Introduction

At the RAN1 #100bis e-meeting, three issues were discussed for half-duplex operation in CA including 1) the definition of reference cell, 2) more than 2 cells with different transmission directions and 3) mixed numerology cases. TP for issue 1 was agreed and issue 2 and issue 3 were not discussed.

In this contribution, we discuss issue 2.

Discussion

According to the agreement for half-duplex UE operation, UE does not always follow reference cell transmission direction. For example, in case 3 and case 16 as shown in Table 1, UE will drop the transmission on reference cell for inter-band CA, the issue is when transmission directions of multiple other cells are not aligned, UE cannot determine to follow the transmission direction of which cell.

Table 1 Half-duplex UE behavior in different cases

| No | Ref cell | Other cell | UE behavior | Note |
| --- | --- | --- | --- | --- |
| 3 | Semi SFI D | Dynamic U | Alt 1: Allowed to drop D for inter-band Error case in intra-band | Overriding semi SFI D to F on reference cell for the UE |
| 16 | RRC D | Dynamic U | Alt 1: Allowed to drop D for inter-band Error case in intra-band |  |

As an example shown in Table 2, if UE is configured with three cells, Pcell is configured with semi SFI D or RRC D, a dynamic U is scheduled on Scell 1 and RRC D is configured on Scell2. Then, according to the agreement, Pcell is the reference cell, UE should drop D on reference cell and transmit dynamic U on Scell1, but there is no conflict between Pcell and Scell2, it is not clear whether a half-duplex UE should drop dynamic U on Scell 1 or drop RRC D on Scell 2. From our perspective, UE should prioritize dynamic U on Scell1 in this case according to the principle of case 3 and case 16.

Table 2 Conflict direction on Scells

| Pcell (Reference cell) | Scell1 | Scell2 | UE behavior |
| --- | --- | --- | --- |
| Semi SFI D | Dynamic U | RRC D | drop D or drop U? |
| RRC D | Dynamic U | RRC D | drop D or drop U? |

Proposal 1: For a half-duplex CA UE, if reference cell is semi SFI D or RRC D, UE should drop high layer configured D on other cells if there is dynamic U on one of the other cells.

In addition, the agreements for inter-band CA case are not correctly captured in the specification. Hence, a text proposal is provided below for half-duplex operation in CA in 38.213 section 11.1.

-------------------------------------------------- Start of text proposal ------------------------------------------------------

If the reference cell and another cell for a UE operate in different frequency bands and if the UE

-	is configured with multiple serving cells and is provided half-duplex-behavior-r16 = 'enable',

-	is not capable of simultaneous transmission and reception on any of the multiple serving cells,

-	indicates support of capability for half-duplex operation in CA with unpaired spectrum, and

-	is not configured to monitor PDCCH for detection of DCI format 2-0,

the UE

-	UE assumes symbol on the other cell as flexible, is not required to receive higher layer configured PDCCH, PDSCH, or CSI-RS and not expected to transmit higher layers configured SRS, PUCCH, PUSCH, or PRACH, when tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated indicates symbol as downlink or uplink on the other cell and as uplink or downlink for the reference cell, respectively,

-	transmits a signal/channel on a symbol of the other cell and is not required to receive a higher layer configured PDCCH, PDSCH, or CSI-RS on the symbol on the reference cell and any of the other cells when the symbol is indicated as downlink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigDedicated for the reference cell and if the UE detects a DCI format scheduling the transmission on the symbol on the other cell,

-	transmits a signal/channel on a symbol of the other cell and is not required to receive a higher layer configured PDCCH, PDSCH, or CSI-RS on the symbol on the reference cell and any of the other cells, if the symbol on the reference cell is flexible and the UE is configured by higher layers to receive PDCCH, PDSCH, or CSI-RS on the symbol on the reference cell and the UE detects a DCI format scheduling the transmission on the symbol on the other cell.

----------------------------------------------------- End of text proposal ------------------------------------------------------

Conclusion

In this contribution, we discuss remaining issue for half-duplex operation in CA and give the following proposal:

Proposal 1: For a half-duplex CA UE, if reference cell is semi SFI D or RRC D, UE should drop high layer configured D on other cells if there is dynamic U on one of the other cells.

A text proposal is provided below for half-duplex operation in CA in 38.213 section 11.1.

-------------------------------------------------- Start of text proposal ------------------------------------------------------

If the reference cell and another cell for a UE operate in different frequency bands and if the UE

-	is configured with multiple serving cells and is provided half-duplex-behavior-r16 = 'enable',

-	is not capable of simultaneous transmission and reception on any of the multiple serving cells,

-	indicates support of capability for half-duplex operation in CA with unpaired spectrum, and

-	is not configured to monitor PDCCH for detection of DCI format 2-0,

the UE

-	UE assumes symbol on the other cell as flexible, is not required to receive higher layer configured PDCCH, PDSCH, or CSI-RS and not expected to transmit higher layers configured SRS, PUCCH, PUSCH, or PRACH, when tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated indicates symbol as downlink or uplink on the other cell and as uplink or downlink for the reference cell, respectively,

-	transmits a signal/channel on a symbol of the other cell and is not required to receive a higher layer configured PDCCH, PDSCH, or CSI-RS on the symbol on the reference cell and any of the other cells when the symbol is indicated as downlink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigDedicated for the reference cell and if the UE detects a DCI format scheduling the transmission on the symbol on the other cell,

-	transmits a signal/channel on a symbol of the other cell and is not required to receive a higher layer configured PDCCH, PDSCH, or CSI-RS on the symbol on the reference cell and any of the other cells, if the symbol on the reference cell is flexible and the UE is configured by higher layers to receive PDCCH, PDSCH, or CSI-RS on the symbol on the reference cell and the UE detects a DCI format scheduling the transmission on the symbol on the other cell.

----------------------------------------------------- End of text proposal ------------------------------------------------------