3GPP TSG RAN WG1 Meeting #101                                          	                    R1-2003617

e-Meeting, May 25th – June 5th, 2020

Source:	CATT

Title:	              Feature lead summary #3 on AI 7.2.4.3 Sidelink synchronization mechanism – Email discussion outcomes

Agenda Item:	7.2.4.3

Document for:	Discussion and Decision

- Introduction

In RAN1#101 e-meeting, 3 email discussion threads were taken place.

[101-e--NR-5G_V2X_NRSL-SYNC-01]

Email discussion/approval regarding PSBCH content/TDD configuration indication

- Issue 1: Indication of TDD configuration

- Issue 2: SL-TDD-Config determination

- Issue 4: PSBCH contents and payload size (WAs confirmation)

By 5/29, with potential TP till 6/4 – Teng (CATT)

[101-e--NR-5G_V2X_NRSL-SYNC-02]

Email discussion regarding

- Issue 8: Start symbol of S-SSB

- Issue 17: In-device coexistence between LTE-V2X and NR-V2X

- Issue 5: Sidelink timing definition

Till 5/28. If consensus can be reached, any follow-up TP by 6/3 – Teng (CATT)

[101-e--NR-5G_V2X_NRSL-SYNC-03]

Email discussion regarding

- Issue 12: Collision between S-SSB and DL slots

- Issue 14: (Pre-)configuration of SL BWP

- Issue 18: S-SSB RSRP measurement

Till 5/28. If consensus can be reached, any follow-up TP by 6/3 – Teng (CATT)

After long time discussion, we achieved some agreements and endorsed TP, which are captured in Section 2 in this document.

- Agreements and endorsed LS/TP

[101-e--NR-5G_V2X_NRSL-SYNC-01]

Agreements:

For indication of TDD configuration, the pattern(s) indication (X) and periodicity indication (Y) follows the two tables below:

Table 1. Periodicity indication Y with single TDD pattern (X=0)

| Periodicity indication Y | P (ms) | Single pattern |
| --- | --- | --- |
| 0 | 0.5 | 0.5 |
| 1 | 0.625 | 0.625 |
| 2 | 1 | 1 |
| 3 | 1.25 | 1.25 |
| 4 | 2 | 2 |
| 5 | 2.5 | 2.5 |
| 6 | 4 | 4 |
| 7 | 5 | 5 |
| 8 | 10 | 10 |
| 9-15 | Reserved |  |

Table 2. Periodicity indication Y with two TDD patterns (X=1)

| Periodicity indication Y | P+P2 (ms) | Two patterns |  |
| --- | --- | --- | --- |
|  |  | P | P2 |
| 0 | 1 | 0.5 | 0.5 |
| 1 | 1.25 | 0.625 | 0.625 |
| 2 | 2 | 1 | 1 |
| 3 | 2.5 | 0.5 | 2 |
| 4 | 2.5 | 1.25 | 1.25 |
| 5 | 2.5 | 2 | 0.5 |
| 6 | 4 | 1 | 3 |
| 7 | 4 | 2 | 2 |
| 8 | 4 | 3 | 1 |
| 9 | 5 | 1 | 4 |
| 10 | 5 | 2 | 3 |
| 11 | 5 | 2.5 | 2.5 |
| 12 | 5 | 3 | 2 |
| 13 | 5 | 4 | 1 |
| 14 | 10 | 5 | 5 |
| 15 | 20 | 10 | 10 |

Agreements:

- Confirm the working assumptions in RAN1#99 for the PSBCH contents for NR SL Rel-16, and reserve bits are 2.

Agreements:

- When transmitting PSBCH, UE derives the values of X/Y/Z; from RRC configuration of TDD-UL-DL-ConfigCommon and maps them to the “indication of TDD configuration” field of PSBCH.

Agreements:

- If at least Y-th, (Y+1)-th, ....., (Y+X-1)-th symbols in a slot semi-statically for UL as indicated in TDD-UL-DL-ConfigCommon, where

    - X is sl-LengthSymbols

    - Y is sl-StartSymbol

this slot can be indicated by PSBCH.

NOTE: X and Y in this proposal are different from the X/Y/Z in PSBCH.

Agreements:

For indication of the UL slots by Z,

- If single pattern is configured, Z bits indicate the UL slot number in the pattern is n.

- If two patterns are configured, Z bits indicate the state index derived by the UL slots,

n=n2w*P*2μ+1w+n1w

Where

- n1 is the number of UL slots in the first pattern,

- n2n2 is the number of UL slots in the second pattern,

- PP is the periodicity in units of ms of the first pattern,

- w is the granularity of resource indication,

- μ is (working assumption)

- Alt 1: 0/1/2/3 corresponds to the 15/30/60/120 kHz SCS for SL respectively.

- Alt 2: 2 corresponds to 60 kHz SCS for FR1, and 3 corresponds to 120kHz for FR2.

Agreeemnts:

For indication of the granularity of UL resources,

- If single pattern is configured, the granularity of the number of UL resources indicated by SL-TDD-Config is 1 slot.

- If two patterns are configured, the granularity of the number of UL resources indicated by SL-TDD-Config follows the table below (as a working assumption)

| Periodicity indication Y | P+P2 (ms) | Two patterns |  | Granularity in slots with different SCS |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | P | P2 | 15kHz | 30 kHz | 60 kHz | 120 kHz |
| 0 | 1 | 0.5 | 0.5 | 1 |  |  |  |
| 1 | 1.25 | 0.625 | 0.625 |  |  |  |  |
| 2 | 2 | 1 | 1 |  |  |  |  |
| 3 | 2.5 | 0.5 | 2 |  |  |  |  |
| 4 | 2.5 | 1.25 | 1.25 |  |  |  |  |
| 5 | 2.5 | 2 | 0.5 |  |  |  |  |
| 6 | 4 | 1 | 3 | 1 |  |  | 2 |
| 7 | 4 | 2 | 2 |  |  |  |  |
| 8 | 4 | 3 | 1 |  |  |  |  |
| 9 | 5 | 1 | 4 |  |  |  |  |
| 10 | 5 | 2 | 3 |  |  |  |  |
| 11 | 5 | 2.5 | 2.5 |  |  |  |  |
| 12 | 5 | 3 | 2 |  |  |  |  |
| 13 | 5 | 4 | 1 |  |  |  |  |
| 14 | 10 | 5 | 5 | 1 |  | 2 | 4 |
| 15 | 20 | 10 | 10 | 1 | 2 | 4 | 8 |

[101-e--NR-5G_V2X_NRSL-SYNC-02]

Agreements:

The first OFDM symbol in an S-SS/PSBCH block is mapped to the first OFDM symbol in the slot.

----------------------------------------------------Start of draft TP for 38.211---------------------------------------------------- 8.4.3.1 Time-frequency structure of an S-SS/PSBCH block In the time domain, an S-SS/PSBCH block consists of NsymbS-SSBNsymbS-SSB OFDM symbols, numbered in increasing order from 0 to NsymbS-SSBNsymbS-SSB - 1 within the S-SS/PSBCH block, where S-PSS, S-SSS, and PSBCH with associated DM-RS are mapped to symbols as given by Table 8.4.3.1-1. The number of OFDM symbols in an S-SS/PSBCH block NsymbS-SSBNsymbS-SSB = 13 for normal cyclic prefix and NsymbS-SSBNsymbS-SSB= 11 for extended cyclic prefix. The first OFDM symbol in an S-SS/PSBCH block is the first OFDM symbol in the slot. --------------------------------------------------< Unchanged parts are omitted >------------------------------------------------- -----------------------------------------------------End of draft TP for 38.211-----------------------------------------------------

Agreements:

- For sidelink transmission, when gNB/eNB is used as the synchronization reference, the timing determination mechanism in LTE V2X is reused in NR V2X, i.e. DL timing is used.

- Send an LS to RAN4 asking for feedback, if any

Agreements:

- Adopt the following 38.213 TP

---

16.2.X   Operation for in-device coexistence

If a first channel/signal using E-UTRA radio access and a second channel/signal using NR radio access occur to a UE, when

- the first channel/signal using E-UTRA radio access and the second channel/signal using NR radio access are time-division multiplexed, and

- the frame indexes of the first channel/signal using E-UTRA radio access and the frame indexes of the second channel/signal using NR radio access are known to the UE,

the UE transmits or receives the channel/signal so that the subframe boundary of the second channel/signal using NR radio access is aligned with the subframe boundary of the first channel/signal using E-UTRA radio access. It is up to UE implementation how to achieve subframe boundary alignment.

--------------------------------------------------< Unchanged parts are omitted >-------------------------------------------------

LS? The draft LS is approved. Fianl LS in R1-2004999. R1-2005098.

[101-e--NR-5G_V2X_NRSL-SYNC-03]

Agreements:

S-SSB transmission/reception slots are in cell-specific UL resources in Uu.

Conclusion:

UE assumes that the same SL BWP is (pre-)configured for both RRC idle (or out of coverage) UEs and RRC connected UEs.

Agreements:

- Adopt the following TP (38.213, Section 16.1)

<Unchanged parts omitted>

For reception of a S-SS/PSBCH block, a UE assumes a frequency location corresponding to the subcarrier with index 66 in the S-SS/PSBCH block [4, TS 38.211], is provided by absoluteFrequencySSB-SL. The UE assumes that a S-PSS symbol, a S-SSS symbol, and a PSBCH symbol have a same transmission power. The UE assumes a same numerology of the S-SS/PSBCH as for a SL BWP of the S-SS/PSBCH block reception, and that a bandwidth of the S-SS/PSBCH is within a bandwidth of the SL BWP. The UE assumes the subcarrier with index 0 in the S-SS/PSBCH block is aligned with a subcarrier with index 0 in a RB of the SL BWP.

<Unchanged parts omitted>

Agreements:

- PSBCH-DMRS is used for S-SSB RSRP measurement.

  - S-SSS may be used for S-SSB RSRP measurement additionally, in which case,  it is up to UE implementation to use PSBCH DMRS only or S-SSS + PSBCH DMRS.

Agreements:

- Adopt the following TP

----------------------------------------------------Start of draft TP for 38.215----------------------------------------------------

5.1.22   PSBCH reference signal received power (PSBCH-RSRP)

| Definition | PSBCH Reference Signal Received Power (PSBCH-RSRP) is defined as the linear average over the power contributions (in [W]) of the resource elements that carry demodulation reference signals associated with physical sidelink broadcast channel (PSBCH). For PSBCH-RSRP sidelink secondary synchronization signals in addition to demodulation reference signals for PSBCH may be used. PSBCH-RSRP using sidelink secondary synchronization signals shall be measured by linear averaging over the power contributions of the resource elements that carry corresponding reference signals. For frequency range 1, the reference point for the PSBCH RSRP shall be the antenna connector of the UE. For frequency range 2, PSBCH-RSRP shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch. For frequency range 1 and 2, if receiver diversity is in use by the UE, the reported PSBCH-RSRP value shall not be lower than the corresponding PSBCH-RSRP of any of the individual receiver branches. |
| --- | --- |
| Applicable for | RRC_IDLE intra-frequency, RRC_IDLE inter-frequency, RRC_CONNECTED inter-frequency |

NOTE 1:   The number of resource elements within the considered measurement frequency bandwidth and within the measurement period that are used by the UE to determine PSBCH-RSRP is left up to the UE implementation with the limitation that corresponding measurement accuracy requirements have to be fulfilled.

NOTE 2:   The power per resource element is determined from the energy received during the useful part of the symbol, excluding the CP.

NOTE 3:   It is up to UE implementation to use PSBCH DMRS only or both S-SSS and PSBCH DMRS for PSBCH-RSRP.

-----------------------------------------------------End of draft TP for 38.215-----------------------------------------------------

LS in [101-e--NR-5G_V2X_NRSL-SYNC-02]

- Final LS in R1-2004999, and it is replaced by R1-2005098.

Endorsed TPs

- R1-2005099

- R1-2005103

- R1-2005104