3GPP TSG RAN WG1 #101-e		R1-2003816

e-Meeting, May 25th – June 5th, 2020

Source:	Panasonic

Title: 	Discussion on evaluation methodologies for baseline coverage performance analysis

Agenda Item:		8.4.1.1

Document for:	Discussion

Introduction

A study item on NR coverage enhancement was approved [1]. One of objectives of this study item is to identify baseline coverage performance for both DL and UL for the following scenarios and services based on link-level simulation.

- The target scenarios and services include

  - Urban (outdoor gNB serving indoor UEs) scenario, and rural scenario (including extreme long distance rural scenario) for FR1

  - Indoor scenario (indoor gNB serving indoor UEs), and urban/suburban scenario (including outdoor gNB serving outdoor UEs and outdoor gNB serving indoor UEs) for FR2

  - TDD and FDD for FR1

  - VoIP and eMBB services for FR1

  - eMBB service as first priority and VoIP as second priority for FR2

  - LPWA services and scenarios are not included.

In this document, we provide several views and proposals regarding to the baseline coverage performance evaluation methodology. In addition, we provide preliminary link level evaluation results and coverage performance in FR1 in the Appendix.

Discussion

Evaluation metrics

To evaluate the baseline coverage performance, what metrics should be used for link-level evaluation need to be agreed. Generally, the identification of coverage issues uses the MCL (Maximum Coupling Loss) methodology. The coupling loss is defined as the total long-term channel loss over the link between the UE antenna ports and the gNB antenna ports, and includes in practical antenna gains, path loss, shadowing, body loss, etc. The MCL is the limit value of the coupling loss at which the service can be delivered, and therefore defines the coverage of the service. The MCL is evaluated via link budget analysis (supported by link-level simulation). The following link budget template in Section 5.2.3.3 of Report ITU-R M.2411-0 [2] can be starting point of MCL calculation template, e.g., MCL is defined as (29a) and (29b) in Table 1. According to the previous experiences such as LTE Coverage Enhancements [3], more simplified link budge analysis template can also be considered.

Table 1: Link budget template

| Transmitter |  |
| --- | --- |
| (1) Number of transmit antennas |  |
| (2) Maximal transmit power per antenna (dBm) |  |
| (3) Total transmit power = function of (1) and (2) (dBm) |  |
| (4) Transmitter antenna gain (dBi) |  |
| (5) Transmitter array gain (depends on transmitter array configurations and technologies such as adaptive beamforming, CDD, etc.) (dB) |  |
| (6) Control channel power boosting gain (dB) |  |
| (7) Data channel power loss due to pilot/control boosting (dB) |  |
| (8) Cable, connector, combiner, body losses, etc. (enumerate sources) (dB) (feeder loss must be included for and only for downlink) |  |
| (9a) Control channel e.i.r.p. = (3) + (4) + (5) + (6) – (8) dBm |  |
| (9b) Data channel e.i.r.p = (3) + (4) + (5) – (7) – (8) dBm |  |
| Receiver |  |
| (10) Number of receive antennas |  |
| (11) Receiver antenna gain (dBi) |  |
| (12) Cable, connector, combiner, body losses, etc. (enumerate sources) (dB) (feeder loss must be included for and only for uplink) |  |
| (13) Receiver noise figure (dB) |  |
| (14) Thermal noise density (dBm/Hz) |  |
| (15) Receiver interference density (dBm/Hz) |  |
| (16) Total noise plus interference density = 10log (10(((13)+(14))/10) + 10(15)/10) dBm/Hz |  |
| (17) Occupied channel bandwidth (for meeting the requirements of the traffic type) (Hz) |  |
| (18) Effective noise power = (16) + 10log ((17)) dBm |  |
| (19a) Required SNR for the control channel (dB) |  |
| (19b) Required SNR for the data channel (dB) |  |
| (20) Receiver implementation margin (dB) |  |
| (21a) HARQ gain for control channel (dB) |  |
| (21b) HARQ gain for data channel (dB) |  |
| (22a) Receiver sensitivity for control channel = (18) + (19a) + (20) – (21a) dBm |  |
| (22b) Receiver sensitivity for data channel = (18) + (19b) + (20) – (21b) dBm |  |
| (23a) Hardware link budget for control channel = (9a) + (11) – (22a) dB |  |
| (23b) Hardware link budget for data channel = (9b) + (11) – (22b) dB |  |
| Calculation of available pathloss |  |
| (24) Lognormal shadow fading std deviation (dB) |  |
| (25) Shadow fading margin (function of the cell area reliability and (24)) (dB) |  |
| (26) BS selection/macro-diversity gain (dB) |  |
| (27) Penetration margin (dB) |  |
| (28) Other gains (dB) (if any please specify) |  |
| (29a) Available path loss for control channel = (23a) – (25) + (26) – (27) + (28) – (12) dB |  |
| (29b) Available path loss for data channel = (23b) – (25) + (26) – (27) + (28) – (12) dB |  |

Proposal 1: Table 1 of link budget template in Section 5.2.3.3 of Report ITU-R M.2411-0 can be starting point of MCL calculation.

General parameters for link level evaluation assumption

For the baseline coverage performance evaluation, following general parameters can be considered as a starting point based on according to the previous study in NR [4-6].

Table 2: General assumptions

|  | FR1 |  | FR2 |  |
| --- | --- | --- | --- | --- |
| Scenario | Urban | Rural | Indoor | Urban/Suburban |
| Carrier frequency | Around 4 GHz | Around 4 GHz (or Around 700 MHz) | Around 30 GHz |  |
| Frequency bandwidth | 100 MHz |  | 400 MHz |  |
| Subcarrier spacing | 30 kHz |  | 120 kHz |  |
| Channel model | TDL-A / TDL-C |  |  |  |
| Delay spread | 240 ns | 363 ns | 20 ns | 66 ns |
| Mobile speed | 3 km/h |  |  |  |
| gNB antenna configuration | 4 Tx, 4 Rx |  | 2 Tx, 2 Rx |  |
| UE antenna configuration | 2 Tx, 4 Rx |  | 2 Tx, 2 Rx |  |

Proposal 2: Table 2 can be starting point of general parameters for evaluation assumption.

Channel-specific parameters

For the link-evaluation of each channel (PDCCH, PDSCH, PUCCH, and PUSCH), the performance target should be defined. Based on the previous evaluation of LTE Coverage Enhancements [3], the following performance target should be considered for baseline coverage performance evaluation.

PDCCH: 1% BLER

PDSCH: 10% iBLER for VoIP, and 10% iBLER for eMBB

PUCCH: 1% Pmiss/Pfa for PUCCH format 0/1, and 1% BLER for PUCCH format 2/3/4

PUSCH: 2% rBLER for VoIP, and 10% iBLER for eMBB

The acronyms used in the above are iBLER for initial BLER, rBLER for residual BLER after retransmission, Pmiss for Probability of missed detection, and Pfa for Probability of false alarm, respectively.

The following channel specific parameters in Table 3 should be defined for baseline coverage performance evaluation. The table is basically based on the previous evaluation of LTE Coverage Enhancements [3]. The parameters with TBI (to be indicated) may not be specified, so that each company has to indicate its values when presenting the results.

Table 3: Channel-specific parameters

|  |  | PDCCH | PDSCH | PUCCH | PUSCH |
| --- | --- | --- | --- | --- | --- |
| Performance target | VoIP | 1% BLER | 10 % iBLER | 1% Pmiss/Pfa / 1% BLER | 2% rBLER |
|  | eMBB |  | 10 % iBLER |  | 10% iBLER |
| Max number of HARQ retransmissions |  | N/A | TBI | N/A | TBI |
| Number of RBs |  | TBI | TBI | TBI | TBI |
| MCS number |  | TBI | TBI | TBI | TBI |
| Aggregation level |  | TBI | N/A | N/A | N/A |
| Frequency hopping |  | N/A | N/A | TBI | TBI |
| Number of repetitions |  | N/A | TBI | TBI | TBI |
| PUCCH format |  | N/A | N/A | TBI | N/A |
| Payload size |  | 24 | N/A | TBI | N/A |

Proposal 3: Table 3 can be considered as channel-specific parameters for baseline coverage performance evaluation.

Even if the simulations are based on the same values for the parameters shown in Table 3, the variance of the simulation results provided by different companies may be large. Therefore, in order to compare these results or calibrate the simulators, following parameters can also be discussed/defined or each company should indicate values when presenting the results.

- PDCCH:

  - Symbol duration, CCE-to-REG mapping type (interleaved or non-interleaved), interleaver size, precoder granularity, channel estimation

- PDSCH/PUSCH

  - Symbol allocation (S, L), number of layers, Tx precoding, mapping type, DMRS assumptions (DMRS length, additional DMRS symbol position, DMRS configuration type), channel estimation

- PUCCH

  - PUCCH duration, frequency hopping offset, detection scheme/channel estimation

Conclusion

In this contribution, we discussed the baseline coverage performance evaluation methodology. We made following proposals.

Proposal 1: Table 1 of link budget template in Section 5.2.3.3 of Report ITU-R M.2411-0 can be starting point of MCL calculation.

Proposal 2: Table 2 can be starting point of general parameters for evaluation assumption.

Proposal 3: Table 3 can be considered as channel-specific parameters for baseline coverage performance evaluation.

We also provided preliminary link level evaluation results and coverage performance in FR1 in the Appendix.

Reference

[1] RP-193240, “New SID on NR coverage enhancement,” China Telecom, RAN#86, December 2019.

[2] Report ITU-R M.2411-0, “Requirements, evaluation criteria and submission templates for the development of IMT-2020,” November 2017.

[3] TR36.824, “LTE coverage enhancements (Release 11),” February 2012.

[4] TR38.913, “Study on scenarios and requirements for next generation access technologies (Release 14),” June 2017.

[5] TR37.910, “Study on self evaluation toward IMT-2020 submission (Release 16),” September 2019.

[6] TR38.901, “Study on channel model for frequencies from 0.5 to 100 GHz (Release 16),” December 2019.

[7] RP-191531, “Link budge template – RIT,”

- Appendix A: Preliminary link level evaluation results and coverage performance in FR1

Simulation assumptions

General evaluation assumptions for link-level evaluation are summarized in Table A-1.

Table A-1: General assumption

| Frequency range | FR1 |  |
| --- | --- | --- |
| Scenario | Urban | Rural |
| Carrier frequency | 4.1 GHz |  |
| Frequency bandwidth | 100 MHz |  |
| Subcarrier spacing | 30 kHz |  |
| Channel model | TDL-C |  |
| Delay spread | 240 ns | 363 ns |
| Mobile speed | 3 km/h |  |
| gNB antenna configuration | 4 Tx, 4 Rx |  |
| UE antenna configuration | 2 Tx, 4 Rx |  |

Channel-specific parameters for link-level evaluation are summarized in Table A-2 ~ A-4. In this contribution, PDCCH, PDSCH, and PUSCH are evaluated.

Table A-2: Simulation assumptions for PDCCH evaluation

| Performance target | 1% BLER |
| --- | --- |
| Number of RBs | 48 |
| Symbol duration | 2 |
| Data modulation | QPSK |
| Channel coding | Polar code |
| Aggregation level | 16 |
| CCE-to-REG mapping | Interleaved |
| Interleaver size | 2 |
| REG-bundle size | 6 |
| Precoder granularity | REG-bundle |
| Payload size | 24 |

Table A-3: Simulation assumptions for PDSCH evaluation

|  | VoIP | eMBB |
| --- | --- | --- |
| Performance target | 10% iBLER | 10% iBLER |
| Max number of HARQ retransmissions | No HARQ | No HARQ |
| Number of RBs | 1 | 40 (Urban) 4 (Rural) |
| Number of layers | 1 | 2 |
| Code rate | 30/1024 | 370/1024 |
| Data modulation | QPSK |  |
| MCS number | MCS 0 (Table 3) | MCS 9 (Table 3) |
| Channel coding | LDPC |  |
| PDSCH mapping type | Type A |  |
| DMRS Type A position | pos2 |  |
| DMRS length | 1 |  |
| Additional DMRS symbol positions | pos3 |  |
| DMRS configuration type | Type 1 |  |
| Channel estimation | 2D MMSE |  |

Table A-4: Simulation assumptions for PUSCH evaluation

|  | VoIP | eMBB |
| --- | --- | --- |
| Performance target | 2% rBLER | 10% iBLER |
| Max number of HARQ retransmissions | 4 | No HARQ |
| Number of RBs | 4 | 15 (Urban) 2 (Rural) |
| Number of layers | 1 | 2 |
| Code rate | 30/1024 | 370/1024 |
| Data modulation | QPSK |  |
| MCS number | MCS 0 (Table 3) | MCS 9 (Table 3) |
| Channel coding | LDPC |  |
| PDSCH mapping type | Type A |  |
| DMRS Type A position | pos2 |  |
| DMRS length | 1 |  |
| Additional DMRS symbol positions | pos3 |  |
| DMRS configuration type | Type 1 |  |
| Channel estimation | 2D MMSE |  |

Link level evaluation results

Figure A-1, A-2, and A-3 shows the link level evaluation results of PDCCH, PDSCH, and PUSCH in FR1, respectively. The required SNR for achieving the performance target is summarized in Table A-5.

Figure A-1: PDCCH

(a) VoIP                                                                    (b) eMBB

Figure A-2: PDSCH

(a) VoIP                                                                    (b) eMBB

Figure A-2: PUSCH

Table A-5: Required SNR for achieving performance target

|  | VoIP |  | eMBB |  |
| --- | --- | --- | --- | --- |
|  | Urban | Rural | Urban | Rural |
| PDCCH | -12.0 dB | -12.0 dB | -12.0 dB | -12.0 dB |
| PDSCH | -4.5 dB | -4.7 dB | 0.1 dB | 1.8 dB |
| PUSCH | -12.7 dB | -12.8 dB | -0.1 dB | 1.6 dB |

Coverage performance

MCL for each channel is calculated using link budget template of Table 1. The required SNR obtained from above link-level evaluation is used for “(19a) Required SNR for the control channel” and “(19b) Required SNR for the data channel”. For other items, the values in [7] (Channel Model A, NLOS O-to-I DU-eMBB (4GHz, NR DDDSU)) are used. For rural scenario, the difference of penetration margin between urban and rural area is reflected for the calculation (i.e., 26.25 dB for urban (O-to-I) and 12.5 dB for rural).

Table A-6: MCL evaluation results for FR1

| MCL (dB) | VoIP |  | eMBB |  |
| --- | --- | --- | --- | --- |
|  | Urban | Rural | Urban | Rural |
| PDCCH | 135.0 | 148.7 | 135.0 | 148.7 |
| PDSCH | 147.3 | 161.1 | 126.7 | 148.8 |
| PUSCH | 128.0 | 141.7 | 109.7 | 130.5 |