3GPP TSG RAN WG1 #101-e		  R1- 2003985

e-Meeting, May 25th – June 5th, 2020

Agenda Item:     7.2.6.3

Source:	Spreadtrum Communications

Title:	Discussion on remaining issues on multi-beam operation

Document for:	Discussion and decision

Introduction

In this contribution, we share our views on the remaining issues for multi-beam operation in Rel-16.

Discussion

Clarification on default spatial relation and path-loss RS for multi-slot PUCCH

In RAN-1 #99, we have the following agreement for default spatial relation and path-loss RS for PUCCH.

Agreement The default spatial relation for dedicated-PUCCH/SRS for a CC in FR2, at least when no pathloss RSs are configured by RRC is determined by - in case when CORESET(s) are configured on the CC, the TCI state / QCL assumption of the CORESET with the lowest ID, or - The PL RS to be used is the QCL-TypeD RS of the same TCI state / QCL assumption of the CORESET with the lowest ID - Note: The PL RS should be periodic RS - in case when any CORESETs are not configured on the CC, the activated TCI state with the lowest ID applicable to PDSCH in the active DL-BWP of the CC - Above applies at least for UEs supporting beam correspondence - Above applies at least for the single TRP case

However, it did not consider the multi-slot PUCCH case. As illustrated in Figure-1, multi-slot PUCCH transmission may partially overlap with the 3ms region right after the slot of receiving ACK for MAC-CE of PDCCH/CORESET TCI state indication. Thus, some clarifications are necessary to determine the spatial relation and path-loss RS of the four PUCCH transmissions.

A similar discussion is the default beam of PDSCH aggregation in Rel-15, where a single default beam is applied to all PDSCHs. Therefore, for ease of implementation, we prefer a single default spatial relation and path-loss RS for all PUCCH transmissions.

Figure-1: Ambiguous cases for multi-slot PUCCH transmission

Proposal 1: Adopt text proposals (TP1/TP2) in 38.213.

Text Proposal-1(TP1): for Section 7.2.1 of 38.213:

-------------------------------------------------------------------------------------------------------------------------- < Unchanged parts are omitted > - If the UE - is not provided pathlossReferenceRSs, and - is not provided PUCCH-SpatialRelationInfo, and - is provided enableDefaultBeamPlForPUCCH the UE determines a RS resource index providing a periodic RS resource with 'QCL-TypeD' in the TCI state or the QCL assumption of a CORESET with the lowest index in the active DL BWP of the primary cell. For multi-slot PUCCH, the qd derived from 1st PUCCH applies to all PUCCHs. < Unchanged parts are omitted > ----------------------------------------------------------------------------------------------------------------------------

Text Proposal-2(TP2): for Section 9.2.2 of 38.213:

-------------------------------------------------------------------------------------------------------------------------- < Unchanged parts are omitted > If a UE - reports beamCorrespondenceWithoutUL-BeamSweeping, - is not provided pathlossReferenceRSs in PUCCH-PowerControl, - is provided enableDefaultBeamPlForPUCCH, and - is not provided PUCCH-SpatialRelationInfo, a spatial setting for a PUCCH transmission from the UE is same as a spatial setting for PDCCH receptions by the UE in the CORESET with the lowest ID on the active DL BWP of the PCell. For multi-slot PUCCH, the spatial setting for 1st PUCCH transmission also applies to all PUCCHs. < Unchanged parts are omitted > ----------------------------------------------------------------------------------------------------------------------------

Conclusions

Proposal 1: Adopt text proposals (TP1/TP2) in 38.213..

References

3GPP TS 38.213: "NR; Physical layer procedures for data", g10