3GPP TSG RAN WG1 Meeting #101-e 		 		R1-2004131

e-Meeting, May 25th – June 5th, 2020

Agenda Item:	7.2.1.2

Source: 	LG Electronics

Title: 	Remaining details of Procedure for 2-step RACH

Document for:	Discussion and Decision

- Introduction

In RAN1#100b-e meeting, we discussed how to capture the text proposal for preventing mapping of PRACH occasions not associated with SS/PBCH blocks. In this document, we continue the discussion and provide the corresponding text proposal.

- PRACH occasions not associated with SS/PBCH blocks

If ‘The PRACH occasions not associated with SS/PBCH block after an integer number of association periods’ are mapped to PUSCH occasion associated with a DMRS resource, the PUSCH resources are useless. So, additional specification is required to prevent this unnecessity mapping.

Since the PRACH occasion are regarded as valid PRACH occasions, it is not necessity to make extra validation procedure. It is better to simply add a sentence for explaining that the ROs are not used for preamble to PUSCH resource mapping at the end of the description for the PRACH occasions not associated with SS/PBCH block.

Conclusion

Proposal 1: Adopt the following TP to capture in TS 38.213 Section 8.1.

---------- Text Proposal starts for TS38.213 [1] ----------

## 8.1	Random access preamble

<Unchanged Text Omitted>

An association period, starting from frame 0, for mapping SS/PBCH blocks to PRACH occasions is the smallest value in the set determined by the PRACH configuration period according Table 8.1-1 such that  SS/PBCH blocks are mapped at least once to the PRACH occasions within the association period, where a UE obtains  from the value of ssb-PositionsInBurst in SIB1 or in ServingCellConfigCommon. If after an integer number of SS/PBCH blocks to PRACH occasions mapping cycles within the association period there is a set of PRACH occasions or PRACH preambles that are not mapped to  SS/PBCH blocks, no SS/PBCH blocks are mapped to the set of PRACH occasions or PRACH preambles. An association pattern period includes one or more association periods and is determined so that a pattern between PRACH occasions and SS/PBCH blocks repeats at most every 160 msec. PRACH occasions not associated with SS/PBCH blocks after an integer number of association periods, if any, are not used for PRACH transmissions, and the PRACH occasions are not used for mapping preamble of a PRACH slot to a PUSCH occasion associated with a DMRS resource for Type-2 random access procedure.

<Unchanged Text Omitted>

---------- Text Proposal ends for TS38.213 ----------

Reference

- [1] 3GPP TS 38.213 (v16.1.0): "NR; Physical layer procedures for control"