3GPP TSG-RAN WG1 Meeting #101-e	R1- 2004551

e-Meeting, May 25th - June 5th, 2020	(Revision of R1-2002241)

Agenda Item:	7.2.4.8

Source:	Ericsson

Title:	Remaining details on resource pool determination

Document for:	Discussion, Decision

# 1	Introduction

This contribution discusses the remaining details of resource pool determination.

# 2	NR V2X resource pool determination

RAN1#100e made the following agreement:

Agreement - For resource pool configuration, slots for a resource pool is (pre-)configured with bitmap, which is applied with periodicity. Agreement For derivation of the set of slots to be included in the resource pool, the baseline is the derivation with bitmap and periodicity based on Subclause 14.1.5 of TS36.213 with the following modifications. - FFS: Periodicity and L_bitmap value - The slot index is relative to slot#0 of the radio frame corresponding to SFN 0 of the serving cell if serving cell timing reference is in use, or DFN 0 otherwise - The following procedure is used. - The set includes all the slots except the following slots: - Slots in which SLSS resource is configured, - (Working assumption) slots not having at least Y-th, (Y+1)-th, ....., (Y+X-1)-th symbols in a slot semi-statically for UL as indicated in TDD-UL-DL-ConfigCommon, where - X is sl-LengthSymbols - Y is sl-StartSymbol - (Working assumption) reserved slots which are determined by the similar steps in Subclause 14.1.5 of TS36.213 Working assumption - For the number of PRBs for resource pool, allow configuration of all number of PRBs in a SL BWP. - FFS until RAN1#100bis-e whether/how to deal with remaining PRBs if the configured PRBs for resource pool is not a multiple of subchannel size.

For resource pool determination, we propose to reuse the LTE procedure as defined in Subclause 14.1.5 of TS36.213. This means that the working assumptions made in RAN1#100-e holds true and should be confirmed.

Confirm the working assumption that slots not having at least Y-th, (Y+1)-th, ....., (Y+X-1)-th symbols in a slot semi-statically for UL as indicated in TDD-UL-DL-ConfigCommon, where  X is sl-LengthSymbols and Y is sl-StartSymbol.

Confirm the working assumption that reserved slots which are determined by the similar steps in Subclause 14.1.5 of TS36.213.

- Additionally, during last meeting an issue regarding how to identify UL slots as valid sidelink slots was also discussed in the synchronization agenda item. We have given more details about this issue in our companion paper regarding the TDD configuration signalled in PSBCH [1].

When it comes to the periodicity value, in LTE SFN cycle is used which contains 10240 subframes. Also, in NR, we propose to use the fixed periodicity value of SFN period i.e.10240 subframes or 10240ms.

Periodicity value of 10240 ms is fixed.

Furthermore, there has been extensive discussion on whether/how to use the remaining PRBs if the configured PRBs for resource pool is not a multiple of subchannel size. In our view, such optimization is not necessary at this point in time as it may lead to different subchannel sizes within a resource pool. Given that all the procedures are defined considering same subchannel size, such optimization may lead to large standardization efforts which is absolutely undesirable at this phase. Furthermore, we believe that a proper configuration of resource pool and subchannel size can mitigate the resource wastage.

No further enhancements are needed for PRB configuration of resource pool.

# 3	Conclusion

Based on the discussion in the previous section we propose the following:

Proposal 1	Confirm the working assumption that slots not having at least Y-th, (Y+1)-th, ....., (Y+X-1)-th symbols in a slot semi-statically for UL as indicated in TDD-UL-DL-ConfigCommon, where  X is sl-LengthSymbols and Y is sl-StartSymbol.

Proposal 2	Confirm the working assumption that reserved slots which are determined by the similar steps in Subclause 14.1.5 of TS36.213.

Proposal 3	Periodicity value of 10240 slot is considered.

Proposal 4	No further enhancements are needed for PRB configuration of resource pool.

# References

R1-2004546, “Synchronization mechanism for NR SL”, Ericcson, RAN1#101-e, May 2020.