3GPP TSG RAN WG1 Meeting #101                                          	                    R1-2005099

e-Meeting, May 25th – June 5th, 2020

Source:	CATT

Title:	              Test proposal from email discussion thread #01 for AI 7.2.4.3 sidelink synchronization mechanism

Agenda Item:	7.2.4.3

Document for:	Discussion and Decision

- Introduction

[101-e--NR-5G_V2X_NRSL-SYNC-01]

Email discussion/approval regarding PSBCH content/TDD configuration indication

- Issue 1: Indication of TDD configuration

- Issue 2: SL-TDD-Config determination

- Issue 4: PSBCH contents and payload size (WAs confirmation)

By 5/29, with potential TP till 6/4 – Teng (CATT)

- Endorsed TPs

## 2.1 Endorsed TP 1 to TS 38.213

Coversheet information

- Reason for change: In current specification, TDD-UL-DL-ConfigCommon indicates the DL/UL/FL resources in time domain to in-coverage UEs. When UE uses PSBCH to indicate TDD configuration to other (Out of coverage) UEs, the indication field in PSBCH is only 12 bits. A conversion mechanism is needed to transfer the NR Uu UL slot number to a potential SL slot number that can be contained in PSBCH. RAN1 agreed that this conversion is processed at UE side with different granularity.

- Summary of changes: After a UE received TDD-UL-DL-ConfigCommon, the UE converts the TDD configuration of NR Uu to sl-TDD-Config which has 12 bits payload size and is contained in PSBCH. When one pattern is configured, the UL slots indication is 1 slot granularity. When two patterns are configured, different granularities according to different periodicity/SCS are used to scale down the indication states in PSBCH. Beside, partial UL slots also can be included in the TDD indication of PSBCH.

- Specs/sections impacted: TS 38.213, section 16.1

- Consequences if not approved: Incomplete specification; NR Uu TDD configuration indication cannot be indicated and contained by PSBCH.

------------------------------------------------------ Start of Draft TP of 38.213--------------------------------------------------

16.1	Synchronization procedures

For transmission of an S-SS/PSBCH block, a UE assumes that PSBCH payload containing a bit sequence a0, a1, a2, a3, …, a11 indicated by sl-TDD-Config provides the slot format over a number of slots:

- A number of patterns indicated by TDD-UL-DL-ConfigCommon as described in Subclause 11.1 by a0

-	If a pattern1 is provided, a0=0

-	If a pattern1 and a pattern2 are provided, a0=1

-	A period or two periods indicated by dl-UL-TransmissionPeriodicity by a1, a2, a3,a4

-	j=i=14ai∙24-i denotes an index in Table 16.1-1 if a0=0 or Table 16.1-2 if a0=1

-	uslotsSL, the indicator of number of slots including uplink symbols by a5, a6, a7, a8, a9, a10, a11

-	if a0=0, uslotsSL=uslots*2μ-μref+usym*2μ-μrefL+I1,

-	if a0=1, uslotsSL=uslots,2*2μ-μref+usym,2*2μ-μrefL+I2w*P*2μ+1w+uslots*2μ-μref+usym*2μ-μrefL+I1w

where

- L is the number of symbols in a slot: L = 12 if cyclicPrefix-SL is enabled to “ECP”, otherwise L = 14;

- I1 is 1 if usym*2μ-μref mod L≥ L-Y, otherwise I1 is 0; I2 is 1 if usym,2*2μ-μref mod L≥L-Y, otherwise I2 is 0. Y is the sidelink starting symbol index represented by sl-StartSymbol;

- w is the granularity of slots indication as described in Table 16.1-2,

- μref, uslots, usym, uslots,2, usym,2, P msec and P2 msec are described in Subclause 11.1,

- μ=0, 1, 2, 3 corresponds to SL SCS as defined in [4, TS 38.211].

-	a5, a6, a7, a8, a9, a10, a11 equal the binary representation of uslotsSL according to i=511ai∙211-i.

Table 16.1-1: Slot configuration period when one pattern is indicated

| Index j | Slot configuration period of pattern1 P (msec) |
| --- | --- |
| 0 | 0.5 |
| 1 | 0.625 |
| 2 | 1 |
| 3 | 1.25 |
| 4 | 2 |
| 5 | 2.5 |
| 6 | 4 |
| 7 | 5 |
| 8 | 10 |
| 9 – 15 | Reserved |

Table 16.1-2: Slot configuration period and granularity when two patterns are indicated

| Index j | Slot configuration period of pattern1 P (msec) | Slot configuration period of pattern2 P2 (msec) | Granularity w in slots with different SCS |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 15kHz | 30 kHz | 60 kHz | 120 kHz |
| 0 | 0.5 | 0.5 | 1 |  |  |  |
| 1 | 0.625 | 0.625 |  |  |  |  |
| 2 | 1 | 1 |  |  |  |  |
| 3 | 0.5 | 2 |  |  |  |  |
| 4 | 1.25 | 1.25 |  |  |  |  |
| 5 | 2 | 0.5 |  |  |  |  |
| 6 | 1 | 3 | 1 |  |  | 2 |
| 7 | 2 | 2 |  |  |  |  |
| 8 | 3 | 1 |  |  |  |  |
| 9 | 1 | 4 |  |  |  |  |
| 10 | 2 | 3 |  |  |  |  |
| 11 | 2.5 | 2.5 |  |  |  |  |
| 12 | 3 | 2 |  |  |  |  |
| 13 | 4 | 1 |  |  |  |  |
| 14 | 5 | 5 | 1 |  | 2 | 4 |
| 15 | 10 | 10 | 1 | 2 | 4 | 8 |

------------------------------------------------ < Unchanged parts are omitted > --------------------------------------------

----------------------------------------------------- End of Draft TP of 38.213 ------------------------------------------------

- Appendix: Agreements of Email Thread#01

Agreements:

For indication of TDD configuration, the pattern(s) indication (X) and periodicity indication (Y) follows the two tables below:

Table 1. Periodicity indication Y with single TDD pattern (X=0)

| Periodicity indication Y | P (ms) | Single pattern |
| --- | --- | --- |
| 0 | 0.5 | 0.5 |
| 1 | 0.625 | 0.625 |
| 2 | 1 | 1 |
| 3 | 1.25 | 1.25 |
| 4 | 2 | 2 |
| 5 | 2.5 | 2.5 |
| 6 | 4 | 4 |
| 7 | 5 | 5 |
| 8 | 10 | 10 |
| 9-15 | Reserved |  |

Table 2. Periodicity indication Y with two TDD patterns (X=1)

| Periodicity indication Y | P+P2 (ms) | Two patterns |  |
| --- | --- | --- | --- |
|  |  | P | P2 |
| 0 | 1 | 0.5 | 0.5 |
| 1 | 1.25 | 0.625 | 0.625 |
| 2 | 2 | 1 | 1 |
| 3 | 2.5 | 0.5 | 2 |
| 4 | 2.5 | 1.25 | 1.25 |
| 5 | 2.5 | 2 | 0.5 |
| 6 | 4 | 1 | 3 |
| 7 | 4 | 2 | 2 |
| 8 | 4 | 3 | 1 |
| 9 | 5 | 1 | 4 |
| 10 | 5 | 2 | 3 |
| 11 | 5 | 2.5 | 2.5 |
| 12 | 5 | 3 | 2 |
| 13 | 5 | 4 | 1 |
| 14 | 10 | 5 | 5 |
| 15 | 20 | 10 | 10 |

Agreements:

- Confirm the working assumptions in RAN1#99 for the PSBCH contents for NR SL Rel-16, and reserve bits are 2.

Agreements:

- When transmitting PSBCH, UE derives the values of X/Y/Z; from RRC configuration of TDD-UL-DL-ConfigCommon and maps them to the “indication of TDD configuration” field of PSBCH.

Agreements:

- If at least Y-th, (Y+1)-th, ....., (Y+X-1)-th symbols in a slot semi-statically for UL as indicated in TDD-UL-DL-ConfigCommon, where

    - X is sl-LengthSymbols

    - Y is sl-StartSymbol

this slot can be indicated by PSBCH.

NOTE: X and Y in this proposal are different from the X/Y/Z in PSBCH.

Agreements:

For indication of the UL slots by Z,

- If single pattern is configured, Z bits indicate the UL slot number in the pattern is n.

- If two patterns are configured, Z bits indicate the state index derived by the UL slots,

n=n2w*P*2μ+1w+n1w

Where

- n1 is the number of UL slots in the first pattern,

- n2n2 is the number of UL slots in the second pattern,

- PP is the periodicity in units of ms of the first pattern,

- w is the granularity of resource indication,

- μ is (working assumption)

- Alt 1: 0/1/2/3 corresponds to the 15/30/60/120 kHz SCS for SL respectively.

- Alt 2: 2 corresponds to 60 kHz SCS for FR1, and 3 corresponds to 120kHz for FR2.

Agreeemnts:

For indication of the granularity of UL resources,

- If single pattern is configured, the granularity of the number of UL resources indicated by SL-TDD-Config is 1 slot.

- If two patterns are configured, the granularity of the number of UL resources indicated by SL-TDD-Config follows the table below (as a working assumption)

| Periodicity indication Y | P+P2 (ms) | Two patterns |  | Granularity in slots with different SCS |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | P | P2 | 15kHz | 30 kHz | 60 kHz | 120 kHz |
| 0 | 1 | 0.5 | 0.5 | 1 |  |  |  |
| 1 | 1.25 | 0.625 | 0.625 |  |  |  |  |
| 2 | 2 | 1 | 1 |  |  |  |  |
| 3 | 2.5 | 0.5 | 2 |  |  |  |  |
| 4 | 2.5 | 1.25 | 1.25 |  |  |  |  |
| 5 | 2.5 | 2 | 0.5 |  |  |  |  |
| 6 | 4 | 1 | 3 | 1 |  |  | 2 |
| 7 | 4 | 2 | 2 |  |  |  |  |
| 8 | 4 | 3 | 1 |  |  |  |  |
| 9 | 5 | 1 | 4 |  |  |  |  |
| 10 | 5 | 2 | 3 |  |  |  |  |
| 11 | 5 | 2.5 | 2.5 |  |  |  |  |
| 12 | 5 | 3 | 2 |  |  |  |  |
| 13 | 5 | 4 | 1 |  |  |  |  |
| 14 | 10 | 5 | 5 | 1 |  | 2 | 4 |
| 15 | 20 | 10 | 10 | 1 | 2 | 4 | 8 |