### Maintenance of NR Mobility Enhancements

R1-2003747	Issue Summary for NR Mobility Enhancements	Intel Corporation

#### Physical Layer Aspects for Mobility Enhancements during HO and SCG Change

R1-2003330	Remaining issues on NR mobility enhancements in physical layer	ZTE

R1-2003506	Remaining issues on DAPS-HO	Huawei, HiSilicon

R1-2003676	Remaining issues on Physical Layer Aspects for  DAPS-HO	MediaTek Inc.

R1-2003748	Corrections to Physical layer aspects of NR mobility enhancement	Intel Corporation

R1-2003890	Remaining issues for NR Mobility Enhancement	Samsung

R1-2004202	Remaining issues on mobility enhancements	Ericsson

R1-2004235	On remaining issues on NR mobility enhancements	Apple

R1-2004580	Remaining physical layer aspects of dual active protocol stack based HO	Nokia, Nokia Shanghai Bell

[101-e-NR-Mob-Enh-01]	Email discussion/approval of Issue #1 (UL cancellation for DAPS) and #3 (UL overlapping transmission) in R1-2003747 by 5/29; if necessary, endorse the associated TPs by 6/4 – Daewon (Intel)

R1-2004957	Summary of email discussions for [101-e-NR-Mob-Enh-01]	Moderator (Intel Corporation)

Agreement:

- Introduce the following new FG 21-2d for inter-frequency DAPS-HO and adopt UL transmission based cancellation.

- Update the intra-frequency DAPS HO to include UL transmission cancellation feature.

| 21-1a | Intra-frequency DAPS HO | Support of intra-frequency DAPS-HO 1) Support of simultaneous DL reception of PDCCH and PDSCH from source and target cell in DAPS-HO 2) Support of PDCCH blind decoding capability in the first MCG and second MCG. 3) Support of cancelling UL transmission to the source cell for intra-frequency DAPS-HO | DAPS (Note: RAN2 feature) | Yes | N/A | The network cannot configure UE with DAPS HO | Per Band | No | N/A | N/A |  | [Optional with capability signalling] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 21-2d | UL transmission cancellation | Indicates support of cancelling UL transmission to the source cell for inter-frequency DAPS-HO | 21-1b | Yes | N/A | UE does not support scheduling of overlapping PUSCH/PUCCH/SRS transmissions to source and target cells for inter-frequency DAPS-HO | per band combination | No | N/A | N/A |  | Optional with capability signalling |

Agreement:

Adopt TP#1-16 of R1- R1-2004957 for Clause 15 of TS38.213

Agreement:

Adopt TP#2-1 of R1-2004957 for Clause 15 of TS38.213

Agreement:

Adopt TP#2-6 of R1-2004957 for Clause 15 of TS38.213

[101-e-NR-Mob-Enh-02]	Email discussion/approval of Issue #5 (Power sharing mode for UL DAPS-HO) in R1-2003747 by 5/28; if necessary, endorse the associated TPs by 6/3– Daewon (Intel)

Agreement:

Adopt TP#8 in R1-2004748 for Clause 15 of TS 38.213.

[101-e-NR-Mob-Enh-03]	Email discussion/approval of Issue #6 (PDCCH monitoring in DL DAPS-HO) in R1-2003747 by 5/28; if necessary, endorse the associated TPs by 6/2– Daewon (Intel)

Agreement:

Adopt TP#2 and TP#4 in R1-2004749 for Clause 15 of TS 38.213

#### Others

R1-2003331	Discussion on FR2 mobility interruption enhancements	ZTE

R1-2004148	Remaining PHY aspects for CHO	Huawei, HiSilicon