3GPP TSG RAN WG1 #101                                                                               									R1-20xxxxx

e-Meeting, May 25th – June 5th, 2020

Source:	vivo

Title:	Summary of email discussion [101-e-NR-7.1CRs-06]

Agenda Item:	7.1.3

Document for:	Discussion and Decision

- Introduction

The document provides a summary for email discussion thread [101-e-NR-7.1CRs-06]. Note that the deadline for the email thread is set to be 5/29. Please provide the first round of comments by 5/27 UTC 12:00.

[101-e-NR-7.1CRs-06] Correction on PUSCH skipping with overlapping UCI on PUCCH (R1-2003364, R1-2003363, R1-2003705, R1-2004616, R1-2004617) by 5/29 – Xiaohang (vivo)

Email discussion summary

Discussions

In RAN1 #100-e meeting, there were discussions on the issue by RAN2 LS R1-2000163 regarding dynamic UL skipping. Following two cases related to dynamic PUSCH skipping and the handling of overlapping CSI/HARQ-ACK on PUCCH were discussed in the RAN1#100-e email discussion thread [100e-5LS-02].

- Case 1: dynamic PUSCH skipping without overlapping CSI/HARQ-ACK on PUCCH

- Case 2: dynamic PUSCH skipping with overlapping CSI/HARQ-ACK on PUCCH

For case 1, RAN1 reached the following conclusion

- When a UL grant without UL-SCH field or UL-SCH =1 (if present) is detected by a UE configured with skipUplinkTxDynamic, the corresponding PUSCH transmission is skipped by the UE if no transport block for the PUSCH transmission is generated by MAC and there is no CSI/HARQ-ACK on PUCCH overlapping with the PUSCH.

- Current RAN1 spec should be corrected, the CR is to be discussed/decided after the conclusion of case 2 is reached.

For case 2, there were a majority of companies in RAN1 that agree with the RAN2 common understanding on the desired UE behavior for UCI handling, as described in the LS R1-2000163. However, no consensus could be made in RAN1#100-e, due to implementation concerns raised from both gNB (increased gNB blind detection) and UE side (change of UCI multiplexing behavior).

- Issue 1: What is the UE behaviour in case of dynamic PUSCH skipping with overlapping CSI/HARQ-ACK on PUCCH, i.e. Case 2?

RAN1 sent an LS (R1-2001376) to RAN2 in RAN1#100-e requesting RAN2 input to resolve this issue. In the RAN2#109bis-e meeting, RAN2 made the following conclusion in response to this LS. According to RAN2’s conclusion, RAN1 needs to discuss what is the behavior for case 2.

For Case 2 in the LS R2-2002515 (i.e. dynamic PUSCH skipping with overlapping CSI/HARQ-ACK on PUCCH), RAN2 assumes MAC does not generate a MAC PDU as in the current MAC specification: no changes to MAC are needed. RAN2 waits for further input from RAN1.

In RAN1#100-e, for case 2, two options were proposed for the UE behaviors for UCI handling. Based on the inputs from the companies in RAN1#100-e and RAN1#101-e, companies’ positions are summarized as below:

- Option 1: for case 2, the PUSCH is skipped and CSI/HARQ-ACK are transmitted in the corresponding PUCCH

  - Option 1 is supported by: CATT, DOCOMO, vivo, Ericsson, Nokia, ASUSTeK, Intel, MediaTek, Samsung, LG

  - Arguments:

    - This is the same as LTE behaviour, and aligned with RAN2 common understanding for NR as in LS R1-2000163

    - Concern on option 1

      - Change of UCI multiplexing behaviour

  - If option 1 is adopted, spec change as provided by draft 38.214 CR on Case 2 in R1-2003364 [2] can be considered.

A UE shall upon detection of a PDCCH with a configured DCI format 0_0 or 0_1 transmit the corresponding PUSCH as indicated by that DCI, if a transport block for the corresponding PUSCH transmission is generated as described in [10, TS38.321]. Upon detection of a DCI format 0_1 with "UL-SCH indicator" set to "0" and with a non-zero "CSI request" where the associated "reportQuantity" in CSI-ReportConfig set to "none" for all CSI report(s) triggered by "CSI request" in this DCI format 0_1, the UE ignores all fields in this DCI except the "CSI request" and the UE shall not transmit the corresponding PUSCH as indicated by this DCI format 0_1. For any HARQ process ID(s) in a given scheduled cell, the UE is not expected to transmit a PUSCH that overlaps in time with another PUSCH. For any two HARQ process IDs in a given scheduled cell, if the UE is scheduled to start a first PUSCH transmission starting in symbol j by a PDCCH ending in symbol i, the UE is not expected to be scheduled to transmit a PUSCH starting earlier than the end of the first PUSCH by a PDCCH that ends later than symbol i. The UE is not expected to be scheduled to transmit another PUSCH by DCI format 0_0 or 0_1 scrambled by C-RNTI or MCS-C-RNTI for a given HARQ process until after the end of the expected transmission of the last PUSCH for that HARQ process.

- Option 2: for case 2, the PUSCH shall not be skipped and CSI/HARQ-ACK are multiplexed with the PUSCH

  - Option 2 is supported by: ZTE, Huawei, Qualcomm

  - Arguments:

    - No change for UCI multiplexing behaivor

    - Concern on option 2

      - This is different from LTE behaviour, and not aligned with RAN2 common understanding for NR as in LS R1-2000163

      - Since RAN2 has concluded no changes to MAC in Rel.15 are needed for case 2, behaviors for transmitting CSI/HARQ-ACK multiplexed with PUSCH without UL-SCH need to specify in RAN1. This is not supported in current RAN1 spec and would require significant change for spec or implementation.

  - If option 2 is adopted, spec change as provided Draft 38.213 CR on Case 2 in R1-2004616 [4] can be considered.

If a UE - would multiplex UCI in a PUCCH transmission that overlaps with a PUSCH transmission, and - the PUSCH and PUCCH transmissions fulfill the conditions in Subclause 9.2.5 for UCI multiplexing, and Regardless of whether a transport block corresponding to the HARQ process of the corresponding PUSCH transmission is generated or not, the UE - multiplexes only HARQ-ACK information, if any, from the UCI in the PUSCH transmission and does not transmit the PUCCH if the UE multiplexes aperiodic or semi-persistent CSI reports in the PUSCH; - multiplexes only HARQ-ACK information and CSI reports, if any, from the UCI in the PUSCH transmission and does not transmit the PUCCH if the UE does not multiplex aperiodic or semi-persistent CSI reports in the PUSCH.

Please share your views on the following aspects for Case 2 using the table format

Q1-1: Please share your view regarding option 1 and the corresponding draft 38.214 CR for Case 2.

- Whether to support this option and the key justification/reason for that

- Any other comments?

| Company | view/comments |
| --- | --- |

Q1-2: Please share your view regarding option 2 and the corresponding draft 38.213 CR for Case 2.

- Whether to support this option and the key justification/reason for that

- Any other comments?

| Company | view/comments |
| --- | --- |

List of contributions

- R1-2003363		Disucssion on PUSCH skipping with overlapping UCI on PUCCH		vivo

- R1-2003364		Correction on PUSCH skipping with overlapping UCI on PUCCH		vivo

- R1-2003705		Clarification on dynamic PUSCH skipping with overlapping CSI HARQ-ACK on PUCCH			ZTE

- R1-2004616		Corrections on dynamic PUSCH skipping with overlapping with PUCCH		Huawei, HiSilicon

- R1-2004617		Corrections on dynamic PUSCH skipping without overlapping with PUCCH		Huawei, HiSilicon