3GPP TSG RAN WG1 #102	R1- 2005646

e-Meeting, August 17th – 28th, 2020

Agenda item:	7.2.4.1

Source: 	MediaTek Inc.

Title: 	Discussion on sidelink physical layer structure

Document for:	Discussion/Decision

# Introduction

In this contribution, we provide our views on the remaining issues of the physical channel structure based on the agreements and the feature lead summary in the previous meetings.

# Discussions on Remaining Issues

## Issue 1. Resource Pool/BWP

- BWP switching between SL BWP and Uu BWP

In RAN1 #1901 meeting, some agreements for SL BWP design are approved as follow.

- Configuration for SL BWP is separated from Uu BWP configuration signalling. UE is not expected to use different numerology in the configured SL BWP and active UL BWP in the same carrier at a given time. - FFS the time scale - FFS relation to DL BWP including initial Uu BWP - FFS relation in terms of frequency location and bandwidth

It means there is no SL BWP switching when V2X-UE is out-of-coverage on the V2X sidelink carrier. It can be FFS for the case of in-coverage operation.

Observation 1: No SL BWP switching when Sidelink UE is out of coverage.

In RAN4 #88 meeting agreement [1] for BWP switching delay, there are several components as follow.

- DCI parsing/RRC reconfiguration

- RF parameter calculating and loading

- BB parameter calculating and loading

- Apply the new parameters

Figure 1. BWP switching delay time partition

DCI parsing/RRC reconfiguration

From legacy LTE agreement [2], the Uu-SL switching do not need to happen immediately after UE receiving DCI command, but at least 4 slots later than DCI command. It means DCI parsing time should not be considered in V2X BWP switching.

TS 36.213 If the UE receives in subframe n DCI format 5A with the CRC scrambled by the SL-V-RNTI, one transmission of PSCCH is in the PSCCH resource (described in Subclause 14.2.4) in the first subframe that is included in and that starts not earlier than TDL-NTA2×TS+4+m×10-3. is the value indicated by "Lowest index of the sub-channel allocation to the initial transmission" associated with the configured sidelink grant (described in [8]), is determined by Subclause 14.1.5, the value m is indicated by 'SL index' field in the corresponding DCI format 5A according to Table 14.2.1-1 if this field is present and m=0 otherwise, TDL is the start of the downlink subframe carrying the DCI, and NTA and TS are described in [3].

Observation 2: UE doesn’t need to switch to SL immediately after receiving SL DCI command in LTE.

Proposal 1: SL BWP switching time should not include DCI parsing time.

RF parameter calculating and loading

In NR BWP switching, another major factor is the RF/BB parameter calculating and loading. In NR, the BWPs could have different SCS, center frequency, and bandwidth. All these factor’s change will impact the RF calculating, loading and switching time. Based on RAN4’s agreement in NR, the RF switching time would be about 200us if the center frequency of BWPs is different while the RF switching time would be about 20us if the center frequency of BWPs is the same. Thus, there is no benefit to permit SL BWP switching to Uu BWP with different center frequency in V2X. It suggests SL BWP should use the same center frequency and SCS with at least one of DL/UL BWP to speed up the BWP switching between SL and Uu.

Proposal 2: SL BWP should use the same center frequency and SCS as the Uu BWP to fasten SL BWP switching.

BB parameter calculating and loading

On the other side, the reason why BB parameter calculating and loading will also take much time in NR is that UE may have to prepare for changing/setting hundreds of parameters when switching between DL BWP1 and DL BWP2. However, BWP switching between DL BWP and UL BWP could be faster because DL BWP and UL BWP parameters without any change can be stored in one Rx-Tx pair and switching between them will not impact each other. Similarly, SL BWP could be stored independently with Uu BWP parameters and switching between Uu BWP and SL BWP has no impact with Uu DL-UL parameters supposing there is no change on uu BWP parameters and SL BWP parameters themselves during switching.

Proposal 3: SL BWP switching time should not include BB parameter calculating and loading time because SL BWP parameters are independently with Uu BWP parameters.

Apply the new parameters

SL BWP RF switching time could follow agreed RAN4 NR RF switching time[3] as the following Table.

Table 1. SL BWP RF switching time

| Scenarios | RF switching time(us) |
| --- | --- |
| FR1 | 13 |
| Co-exist with TDD LTE in FR1 | 20 |
| FR2 | 7 |

Proposal 4: SL BWP RF switching time should be 20us when SL BWP has the same center frequency with Uplink BWP.

Uu BWP- SL BWP – Uu BWP BWP switching pair

There are two basic scenarios for SL BWP switching to Uu BWP after Uu BWP1 switching to SL BWP:

- SL BWP switching to a new Uu BWP2

- SL BWP switching back to Uu BWP1

BWP switching from BWP1 to BWP2 will spend at least 3 slots(refer on RAN4’s spec TS38.133) in NR because of parameters changing between BWP1 and BWP2. Thus, the BWP switching time from SL BWP to Uu BWP2 is much longer than Uu BWP1 to SL BWP because UE needs to prepare lots of baseband parameters changing between Uu BWP2 and Uu BWP1 in figure 1(a). It’s not reasonable to insert SL BWP in Uu BWP switching procedure, and also it’s not reasonable to borrow SL BWP to finish Uu BWP switching. The reasonable V2X BWP switching procedure is that switching Uu BWP to SL BWP when UE needs to execute SL service and switching back to the same Uu BWP after finishing SL service shown in figure 1(b).

Figure 2. Uu BWP – SL BWP switching pair

Proposal 5: Once Uu BWP switching to SL BWP happens, only permit SL BWP switch back to the same Uu BWP.

The SL BWP switching could be divided in the following categories based on the discussion above.

- Case 1: DL BWP -> SL BWP

- Case 2: UL BWP -> SL BWP

- Case 3: SL BWP -> DL BWP

- Case 4: SL BWP -> UL BWP

In the Figure 3 below, SL has TA/2 advance to align with BS timing following the LTE design[2].

Observation 3: UE SL timing has TA/2 advance in LTE to guarantee the last symbol in SL can cover the UL timing advance.

Typical GP will be 2 symbols to cover 2 times BS-UE distance (TA/2) plus RF switching time (TAoffset), and 1 symbol time equals TA/2 plus TAoffset. It means DL slot with TA/2 delay and RF switching (TAoffset) will only overlap with SL within the 1st symbol in SL. Owing to UE SL 1st symbol is used to AGC retuning, it’s no impact to SL in the Figure 3 below. At the same time, UE UL signal will have TA advance to UE DL signal so that it’s also no impact when UE switch UL BWP to SL BWP.

Observation 4: There is no impact for Uu DL/UL BWP switching to SL BWP because DL slot TA/2 delay plus RF switching time can cover in SL first symbol.

It’s also no impact when SL BWP switch to DL BWP because UE DL slot has TA/2 delay than BS DL slot and the last symbol plus TA/2 of SL can enough cover the RF switching from SL to DL.

Observation 5: There is no impact for SL BWP switching to Uu DL BWP because RF switching time can cover in SL last symbol.

Figure 3. (DL, SL, UL) BWP switching timing

Now coming to SL BWP switching to Uu UL BWP, this case could be different with legacy LTE V2X because numerology has been introduced in NR V2X. From the Figure 3 below, the last symbol for SL should cover RF switching time from SL to UL and Uu UL TA advance which equals TAoffset plus TA/2. Owing to the last symbol time changing for different numerology, the permitted BS coverage will be limited by SL symbol length. In legacy LTE, the permitted coverage will be about 9.4km, but in NR the 1symbol GP in SL for coverage would be a problem because of smaller symbol length in SCS=30KHz ,60KHz and 120KHz(in the Table 2 below). If we want to the similar coverage of LTE V2X in NR FR1 and feasibility network deployment in FR2, the GP symbols in sidelink channel structure should be larger than 1 symbol when SL BWP switching to UL BWP. It should be also noted that if the transient period 10us in FR1 and 5us in FR2 is considered, the coverage will further reduced and the required GP symbol number may be also increased.

Observation 6: 1 symbol GP in SL is not enough for SL->UL BWP switching for different numerologies.

Proposal 6: RAN1 sends LS to RAN4 to further check the SL-Uu BWP switching time in detail.

## Issue 2. Slot Structure

- GP symbols in a slot

As discussed in the last meeting, whether/how to have more SL GP symbols with the larger SCS (30/60khz) for accommodating the sufficient timing advance in uu transmission has been raised for discussions.

Agreements: - One symbol is used for gap right after PSSCH transmission. - One symbol is used for gap right after PSFCH transmission. - FFS more gap symbols are defined in order to handle timing advance for shared carrier for Uu operation. - FFS whether to additionally support: - For 15/30 kHz SCS, - Half symbol is used for gap right after PSSCH transmission. - Half symbol is used for gap right after PSFCH transmission.

In FR1, the number of SL GP symbols needs to be set according to the numerology for achieving the same BS coverage as legacy LTE V2X. The permitted GP symbols in the last of SL slot are shown in the Table 2 below. The GP length will be at most 2 symbols for SCS=30kHz and up to 4 symbols for SCS=60kHz. In FR2, we think 2km is enough for FR2’s BS coverage because the coverage could be much smaller than FR1. The GP for SCS=60, 120kHz will be 2,4 symbols.

Multiple GP symbols will only be used when SL switches to UL in uu. The 1-symbol GP between each SL slots won’t be a problem when UE handle consecutive SL slots’ service. It will be a huge overhead for channel structure design if we always set the GP as 1, 2, 4 symbols for SCS=15, 30, 60kHz in FR1 and 2,4 symbols for SCS=60,120kHz in FR2. A feasible solution is to use 2 bits signalling in DCI/SCI to dynamically indicate sidelink GP symbol number.

Table 2. BS Coverage with SL 1 symbol GP

|  | SCS(KHz) | 1symb (us) | BWP switching (us) | TAoffset(us) | Permit TA/2(us) | Coverage(km) | GP Symb Num |
| --- | --- | --- | --- | --- | --- | --- | --- |
| LTE | 15 | 71.3 | 20 | 20 | 31.3 | 9.39 | 1 |
| NR FR1 | 15 | 71.3 | 20 | 13 | 38.3 | 11.49 | 1 |
|  | 30 | 35.7 | 20 | 13 | 2.7 | 0.81 | 2 |
|  | 60 | 17.8 | 20 | 13 | -15.2 | -4.56 | 3-4 |
| Coexist with TDD LTE FR1 | 15 | 71.3 | 20 | 20 | 31.3 | 9.39 | 1 |
|  | 30 | 35.7 | 20 | 20 | -4.3 | -1.29 | 2 |
|  | 60 | 17.8 | 20 | 20 | -22.2 | -6.66 | 3-4 |
| NR FR2 | 60 | 17.8 | 20 | 7 | -9.2 | -2.76 | 2 |
|  | 120 | 8.9 | 20 | 7 | -18.1 | -5.43 | 4 |

Proposal 7: In case of 30/60khz SL SCS, More GP symbols in SL are needed for switching from SL to uu UL due to the insufficient timing advance for uu within one 30/60khz GP symbol.

On the other hand, more GP symbols may degrade the overall system performance. Considering the trade-off between the uu coverage and the SL overall performance, one solution is to configure one more GP symbols at the end of the slot for every N slots for accommodating the sufficient timing advance in uu. Then gNB can schedule SL capable UE with the larger TA to switch on those SL slot for uplink transmission. Besides, in case of mode 2 SL operation which is transparent to gNB, gNB can’t schedule the SL UE (in the cell edge) for uplink transmission without any impact on the SL operation.

Observation 7: It is not feasible to address TA issue by fully relying on BS scheduling since Mode 2 operation is unknown to gNB.

Proposal 8: At least 2 GP symbols at the end of the slot are (pre-)configured for every N slots to support the sufficient timing advance in uu.

## Issue 3. 2nd SCI design w/wo CRC scrambled by Dest L1-ID

- CRC scrambled by Dest L1-ID in 2nd SCI

To reduce the payload of the 2nd SCI, L1-ID can be scrambled in the 2nd SCI, at least for the unicast and/or groupcast communication with known Dest ID at the Rx UE. For the broadcast communication, the dest ID if applied can be carried in 2nd SCI explicitly with a different 2nd SCI size or format than unicast/groupcast. Then at least 16 bits payload can be saved for the unicast/groupcast.

To be noticed, the false alarm is not an issue due to 24 bits of CRC and the limited number of Dest IDs for the unicast/groupcast from the Rx UE perspective.

Proposal 9: The 2nd SCI CRC can be scrambled by 16 bits of Dest ID at least for unicast/groupcast communication to reduce the payload overhead.

## Issue 4. PSCCH design

- QCL between PSCCH-DMRS band PSSCH-DMRS

For SCI contents carried in 1st SCI, the following fields should be considered in addition to the list summarized in the feature lead summary.

QCL indicator in 1st SCI (1 bit): to indicate whether the 1st SCI DMRS are QCL’ed with the data DMRS. If they are QCL’ed, 1st SCI DMRSs can be used jointly with data DMRS for channel estimation to improve the performance of 2nd SCI and data. For broadcast transmission, UE can assumes that PSCCH and PSSCH are QCL’ed to improve the channel estimation with the minimized DMRS overhead.

Alternatively, UE assumes that PSCCH is QCL’ed with PSSCH at least for broadcast transmission.

Proposal 10: UE assumes that PSCCH-DMRS is QCL’ed with PSSCH-DMRS at least for broadcast transmission.

## Issue 5. PSSCH design

- Number of TB per PSSCH

In RAN1#96bis, the working assumption was made only to use 1 TB per PSSCH. It was also considered that only up to 2 layers can be used for PSSCH. So it is preferred to confirm the working assumption and consider more TBs per PSSCH in the future release.

Proposal 11: Confirm the working assumption to support 1 TB per PSSCH.

- PSCCH location in PSSCH subchannels

In LTE V2X, no matter a PSCCH and the associated PSSCH are adjacent or non-adjacent, there is a one-to-one mapping between the starting RB of the PSCCH and the starting RB of the PSSCH. As an example, for the adjacent case, if a PSSCH uses Subchannels k, k+1, …, k+LsubCH-1, where LsubCH is the number of used subchannels, then the corresponding PSCCH is located in Subchannel k. An advantage of zero offset is that no additional signaling is required to indicate the starting subchannel of PSSCH.

For the advanced use cases supported in NR V2X, it is likely that a UE uses a large number of subchannels in a slot. Then, the LTE V2X design has the following two drawbacks: First, PSCCH collisions occur frequently because most PSCCHs are sent in subchannels with small indices. Second, when two PSSCHs use the same set of subchannels, the corresponding PSCCHs must collide.

Unless equipped with an advanced receiver with interference cancellation capability, it is challenging to decode two PSSCHs at the same location. Lower PSCCH collision probability is beneficial since 1) PSSCH using low MCS may survive a collision with other PSSCHs; and 2) combining of multiple HARQ transmissions is impossible without successful SCI decoding. To improve the SCI decoding performance without using high-complexity receivers, it is desirable that there is some flexibility between the located subchannel of a PSCCH and the starting subchannel of the associated PSSCH.

As shown in Figure 4 and Figure 5, there is a clear performance gain for the random offset to avoid control channel collision with the same data channel locations. It can be up to 2.0% gain for highway scenario and 2.5% gain for urban scenario

| Figure 4. performance with random offset vs. fixed offset for highway scenario | Figure 5. performance with random offset vs. fixed offset for urban scenario |
| --- | --- |

Proposal 12:  Support PSCCH in the highest PSSCH subchannel based on a rule to avoid control channel collision.

## Issue 6. CSI-RS issues

- Rate-matching or puncturing for CSI-RS

For SL CSI-RS transmission for SL CSI measurement, it can be rated matched according to the presence derived from SCI field (e.g., 2nd SCI) for CSI request and the configuration of CSI-RS resources. Additionally, CSI-RS resources can be mapped on the PSSCH resources transmitting TBs. In other words, it can’t be mapped to PSSCH transmitting 2nd SCI and/or PSSCH carrying 1st SCI. Because UE can know how to do de-ratemaching only after decoding 1st SCI and 2nd SCI carrying CSI request field. Due to the resource size for 2nd SCI may vary, the exact CSI-RS resource location may vary as well to avoid collision with the 2nd SCI and 1st SCI resources. More specifically, CSI-RS resources can be only mapped on the symbols with PSSCH for TB transmission (i.e., without any 1st SCI and 2nd SCI transmission). Another approach is to do puncturing for CSI-RS so that it is transparent to UE receiver with reduced complexity.

Proposal 13: CSI-RS can be punctured due to at most 2 REs per RB in SL.

# Conclusion

The following summarizes the observations in this contribution:

Observation 1: No SL BWP switching when Sidelink UE is out of coverage.

Observation 2: UE doesn’t need to switch to SL immediately after receiving SL DCI command in LTE.

Observation 3: UE SL timing has TA/2 advance in LTE to guarantee the last symbol in SL can cover the UL timing advance.

Observation 4: There is no impact for Uu DL/UL BWP switching to SL BWP because DL slot TA/2 delay plus RF switching time can cover in SL first symbol.

Observation 5: There is no impact for SL BWP switching to Uu DL BWP because RF switching time can cover in SL last symbol.

Observation 6: 1 symbol GP in SL is not enough for SL->UL BWP switching for different numerologies.

Observation 7: It is not feasible to address TA issue by fully relying on BS scheduling since Mode 2 operation is unknown to gNB.

The following proposals are provided for discussion/decision:

Proposal 1: SL BWP switching time should not include DCI parsing time.

Proposal 2: SL BWP should use the same center frequency and SCS as the Uu BWP to fasten SL BWP switching.

Proposal 3: SL BWP switching time should not include BB parameter calculating and loading time because SL BWP parameters are independently with Uu BWP parameters.

Proposal 4: SL BWP RF switching time should be 20us when SL BWP has the same center frequency with Uplink BWP.

Proposal 5: Once Uu BWP switching to SL BWP happens, only permit SL BWP switch back to the same Uu BWP.

Proposal 6: RAN1 sends LS to RAN4 to further check the SL-Uu BWP switching time in detail.

Proposal 7: In case of 30/60khz SL SCS, More GP symbols in SL are needed for switching from SL to uu UL due to the insufficient timing advance for uu within one 30/60khz GP symbol.

Proposal 8: At least 2 GP symbols at the end of the slot are (pre-)configured for every N slots to support the sufficient timing advance in uu.

Proposal 9: The 2nd SCI CRC can be scrambled by 16 bits of Dest ID at least for unicast/groupcast communication to reduce the payload overhead.

Proposal 10: UE assumes that PSCCH-DMRS is QCL’ed with PSSCH-DMRS at least for broadcast transmission.

Proposal 11: Confirm the working assumption to support 1 TB per PSSCH.

Proposal 12:  Support PSCCH in the highest PSSCH subchannel based on a rule to avoid control channel collision.

Proposal 13: CSI-RS can be punctured due to at most 2 REs per RB in SL.

# References

- 3GPP RAN4 #88 Chairman’s Notes

- 3GPP TS36.213 v15.1.0

- R4-1702029, [Draft] Reply LS on UE RF Bandwidth Adaptation in NR, Mediatek Inc.

- 3GPP RAN1 #96 Chairman’s Notes

- 3GPP RAN1 #96bis Chairman’s Notes

- 3GPP RAN1 #97 Chairman’s Notes

- 3GPP RAN1 #98 Chairman’s Notes

- 3GPP RAN1 #98bis Chairman’s Notes

- 3GPP RAN1 #99 Chairman’s Notes

- 3GPP RAN1 #100_e Chairman’s Notes

- 3GPP RAN1 #100bis_e Chairman’s Notes

- 3GPP RAN1 #101_e Chairman’s Notes