3GPP TSG-RAN WG1 Meeting #102-e	R1-2006435

e-Meeting, August 17th – 28th, 2020

Agenda Item:	7.2.4.2.2

Source:	Ericsson

Title:	Open issues and corrections on Mode 2 resource allocation

Document for:	Discussion, Decision

# 1	Introduction

In this contribution some corrections to the NR SL specifications regarding resource allocation mode 2 are indicated.

# 2	Corrections to 38.212

## 2.1	Fields of SCI format 1-A

TS 38.212 describes the information transmitted by means of SCI format 1-A. For some of the fields a reference is added pointing to the appropriate specification. We note that this correction affects Mode 1 in some cases too. We believe that the references for some fields should be changed as indicated in the following TP.

Correct the references in the description of the fields for SCI format 1-A.

-------------------------- Start of Text Proposal for TS 38.212 -------------------------- <Unchanged parts omitted> #### 8.3.1.1 SCI format 1-A SCI format 1-A is used for the scheduling of PSSCH and 2nd-stage-SCI on PSSCH The following information is transmitted by means of the SCI format 1-A: - Priority – 3 bits as defined in clause 5.4.3.3 of [12, TS 23.287]. - Frequency resource assignment – log2(N subChannel SLN subChannel SL + 12) bits when the value of the higher layer parameter sl-MaxNumPerReserve is configured to 2; otherwise log2(N subChannel SLN subChannel SL + 12N subChannel SL + 16) bits when the value of the higher layer parameter sl-MaxNumPerReserve is configured to 3, as defined in clause 16.4 of [5, TS 38.213]. - Time resource assignment – 5 bits when the value of the higher layer parameter sl-MaxNumPerReserve is configured to 2; otherwise 9 bits when the value of the higher layer parameter sl-MaxNumPerReserve is configured to 3, as defined in clause 16.4 of [5, TS 38.213]. - Resource reservation period –log2Nrsv_period bits as defined in clause 16.4 of [5, TS 38.213], where Nrsv_period is the number of entries in the higher layer parameter sl-ResourceReservePeriodList, if higher layer parameter sl-MultiReserveResource is configured; 0 bit otherwise. <Unchanged parts omitted>

In TS 38.212 and TS 38.214, different terminology is used to refer to the field used for signaling reservations corresponding to periodic allocations: ‘resource reservation period’ (TS 38.212) and ‘resource reservation interval’ (TS 38.214).

Align the terminology ‘resource reservation period’ (TS 38.212) and ‘resource reservation interval’ (TS 38.214)

-------------------------- Start of Text Proposal for TS 38.212 -------------------------- <Unchanged parts omitted> #### 8.3.1.1 SCI format 1-A SCI format 1-A is used for the scheduling of PSSCH and 2nd-stage-SCI on PSSCH The following information is transmitted by means of the SCI format 1-A: <Unchanged parts omitted> - Resource reservation interval –log2Nrsv_period bits as defined in clause 8.1.4 of [6, TS 38.214], where Nrsv_period is the number of entries in the higher layer parameter sl-ResourceReservePeriodList, if higher layer parameter sl-MultiReserveResource is configured; 0 bit otherwise. <Unchanged parts omitted> ------------------------------------ End of Text Proposal ------------------------------------

## 2.2	Fields of SCI format 2-A

TS 38.212 describes the information transmitted by means of SCI format 2-A. For most fields a reference is adding to the appropriate specification. However, several of the references point to incorrect sections. We note that this correction affects Mode 1 in some cases too.

Correct the references in the description of the fields for SCI format 2-A.

-------------------------- Start of Text Proposal for TS 38.212 -------------------------- <Unchanged parts omitted> #### 8.4.1.1 SCI format 2-A SCI format 2-A is used for the decoding of PSSCH, with HARQ operation when HARQ-ACK information includes ACK or NACK, or when there is no feedback of HARQ-ACK information. The following information is transmitted by means of the SCI format 2-A: - HARQ process number – log2Nprocess bits as defined in clause 8.1 of [6, TS 38.214]. - New data indicator – 1 bit as defined in clause 8.1 of [6, TS 38.214].. - Redundancy version – 2 bits as defined in clause 8.1 of [6, TS 38.214].. - Source ID – 8 bits as defined in clause 8.1 of [6, TS 38.214]. - Destination ID – 16 bits as defined in clause 8.1 of [6, TS 38.214]. - HARQ feedback enabled/disabled indicator – 1 bit as defined in clause 8.1 of [6, TS 38.214].. - Cast type indicator – 2 bits as defined in Table 8.4.1.1-1 and in clause 8.1 of [6, TS 38.214]. - CSI request – 1 bit as defined in clause 8.2.1 of [6, TS 38.214] and in clause 8.1 of [6, TS 38.214]. <Unchanged parts omitted> ------------------------------------ End of Text Proposal ------------------------------------

# 3	Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	Correct the references in the description of the fields for SCI format 1-A.

Proposal 2	Align the terminology ‘resource reservation period’ (TS 38.212) and ‘resource reservation interval’ (TS 38.214)

Proposal 3	Correct the references in the description of the fields for SCI format 2-A.