3GPP TSG-RAN WG1 Meeting #102-e	R1- 2006437

e-Meeting, August 17th – 28th, 2020

Agenda Item:	7.2.4.3

Source:	Ericsson

Title:	Corrections on sidelink synchronization mechanism

Document for:	Discussion, Decision

# 1	Introduction

In this contribution some corrections to the NR SL specifications regarding the SL synchronization procedure are discussed.

# 2	Corrections

## 2.1 	Correction to TS 38.212 Clause 8.1.1

----------------------------------------------------begin text proposal for 38.212-----------------------------------------------

## 8.1	Sidelink broadcast channel

The processing for SL-BCH transport channel follows the BCH according to clause 7.1, with the following changes:

-	Clause 7.1.1 for PBCH payload generation is not performed.

-	Clause 7.1.2 for scrambling is not performed.

-	In clause 7.1.5, the rate matching output sequence length E = 1386 when higher layer parameter cyclicPrefix is configured, otherwise, E = 1782.

### 8.1.1	PSBCH payload generation

## 8.2	Sidelink shared channel

----------------------------------------------------end text proposal for 38.212-----------------------------------------------

In the current version of the TS 38.212 specification, we propose to have the following TP regarding the PSBCH payload generation.

Since the process performed in clause 7.1.1 for PBCH does not apply to the payload generation agreed for PSBCH, in our view the clause [8.1.1, TS 38.212] should be remove from the specification.

Remove the clause 8.1.1 in TS 38.212 since the process does not apply to PSBCH and make some modifications to the text in 8.1 to address this issue.

In [16.1, TS 38.213], the determination of the payload for a portion of the PSBCH – the indication of the TDD Configuration – is defined as agreed in RAN1. Therefore, in order to address this issue, the specification in [5.8.9.4.3, TS 38.331] needs to be modified since the SL-TDD configuration cannot be directly copied from TDD-UL-DL-ConfigCommon. In addition, we note that for pre-configuration, the RRC specification should use a similar procedure as for configuration, given that the pre-configuration does not directly contain the values to put in PSBCH but a IE like the one used for conveying the UL DL TDD configuration.

The RRC specification should include a reference to [16.1, TS 38.213] in [5.8.9.4.3, TS 38.331] in order to generate the PSBCH SL-TDD configuration values from the TDD-UL-DL-ConfigCommon for both RRC configuration and pre-configuration as agreed in RAN1.

Although the relevant agreements have been made by RAN1, the corresponding text is part of the RAN2 specification. We have submitted the corresponding CR in [1].

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	The RRC specification should include a reference to [16.1, TS 38.213] in [5.8.9.4.3, TS 38.331] in order to generate the PSBCH SL-TDD configuration values from the TDD-UL-DL-ConfigCommon for RRC configuration and pre-configuration as agreed in RAN1.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Remove the clause 8.1.1 in TS 38.212 since the process does not apply to PSBCH and make some modifications to the text in 8.1 to address this issue.

# 4	References

- R2-2007395, “Correction to transmission of MasterInformationBlockSidelink”, Ericsson, 3GPP TSG-RAN WG2 Meeting #111-e, Electronic Meeting, 17th – 28th August 2020