3GPP TSG-RAN WG1 Meeting #102-e											R1-200xxxx

e-Meeting, August 17th – 28th, 2020

Agenda Item:	7.2.1

Source:			Moderator (ZTE)

Title:			FL summary on the maintenance of 2-step RACH

Document for:	Discussion

# Introduction

This document contains the feature lead summary of issues related to maintenance of the Rel-16 2-step RACH WI.

The issues mentioned in the submitted TDocs are collected and summarized in Section 2, and the feature lead recommendation for the first round email discussion can be found in Section 3.

# Maintenance issues

| # | Issue | Description | Related TDoc # |
| --- | --- | --- | --- |
| 1 | Alignment on RRC parameters | Align the RRC parameter names with 38.331 in RAN1 specs | R1-2005664 R1-2006284 R1-2006407 Proposal 5 in R1-2006609 |
| 2 | Modulation order of MsgB PDSCH | The UE is not expected to decode a PDSCH scheduled with msgB-RNTI and Qm > 2 | R1-2005605 |
| 3 | Subset RO sharing | Adding the condition “within an SSB-RO mapping cycle” according to the agreement | Proposal 1 in R1-2006091 |
| 4 | PUSCH occasion across slot boundary | POs derived for one slot could be across the slot boundary and/or extend to the next slot. | Proposal 2 in R1-2006091 |
| 5 | Gap between MsgA PRACH and PUSCH | Clarify that MsgA PUSCH is not transmitted if the gap between PRACH and PUSCH is not satisfied | Proposal 1 in R1-2006609 |
| 6 | Resource determination for CFRA | Capture MsgA PUSCH resource determination for CFRA in 38.213 | Proposal 2 in R1-2006609 |
| 7 | TDRA for MsgA PUSCH | 7.1 Editorial issues related to the time domain resource allocation of MsgA PUSCH | Proposal 4 in R1-2006407 and Proposal 3 in R1-2006609 |
|  |  | 7.2 Capture the default TDRA table 6.1.2.1.1-3 for extended CP in 38.213 | Proposal 3 in R1-2006609 |
| 8 | Resource overhead of MsgA PUSCH | Align the resource overhead determination with Msg3 for MsgA PUSCH in 38.214 | Proposal 4 in R1-2006609 |

# Summary and recommendation

The budget for email discussion of 2-step RACH in this meeting is only 1. Please indicate your views on the priority of the listed issues into the following table by Wednesday 8/12.

| Company | Priority |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Issue#1 | Issue#2 | Issue#3 | Issue#4 | Issue#5 | Issue#6 | Issue#7.1 | Issue#7.2 | Issue#8 |
| ZTE | High | Medium | Medium | Low | Low | Medium | High | Medium | High |
| Ericsson | High | Medium | High | Low | Low | Medium | High | High | High |

Email thread #1:

To be updated based on companies’ feedback…

Any comments?

| Company | Comment |
| --- | --- |
| ZTE | For issue#4, we think PO should not across the slot boundary. For issue#5, it has been discussed in the last meeting, but no conclusion was achieved. |
| Ericsson | Since there’re not that may controversial issues this time, we assume all the issues listed will be in the scope for discussion and the priority just means how much effort needed to conclude the discussions. Based on this understanding we provide our views as below. Low: Issue #4, issue #5 may need more discussions in this meeting to at least get some conclusions in the coming 2 weeks. Note that issue #5 that we proposed in last meeting was not discussed due to NR-U was still discussing whether same gap requirement used in licensed band should be applied to NR-U as well. And now NR-U has agreed to apply the N gap requirement between preamble and PUSCH for both licensed and unlicensed operation, which is why we proposed to discuss this in this meeting. Medium (some effort but easy to conclude): Issue #2 was discussed in email earlier when editor tried to capture 2-step RACH in a first version, it looks companies had concerns on this at that time. We’re fine to discuss more on this now although we’re a bit preferring to align MsgB with Msg2 on this as ZTE proposed. Issue 6, in our view, it is necessary to capture CFRA resource determination in RAN1 spec. as well. Although I remember in last meeting, we agreed that no RAN1 changes for CFRA, we also said some clarification TPs might be needed in 38.213 among the options to be down-selected by RAN2. We would like to see other companies views on this on whether and how to capture this. High (little effort needed): For all other issues, we think it is obvious that they need to be addressed with related TPs in some way, but maybe some polishing is needed on the original proposed TPs which we can discuss in the coming meetings, e.g. the TPs related to restricted set configuration may need to be merged among companies. Issue 7.1 and 7.2 can be merged since the other default table for extended CP was just missed by editors in the beginning, as there’s no intention to support only normal CP for 2-step RACH in RAN1. Issue 3 is also just to capture the agreement we made, which was missed by editor. |

# References

- R1-2005329	Remaining issues on 2-step RACH	vivo

- R1-2005605	Text proposal on the modulation order of MsgB PDSCH	ZTE, Sanechips

- R1-2005664	Remaining issues on 2-step RACH for NR	CATT

- R1-2006091	Remaining issues for two step RACH for NR	Samsung

- R1-2006284	Remaining issues on channel structure for 2-step RACH	Spreadtrum Communications

- R1-2006407	Remaining issues for 2-step RACH	Huawei, HiSilicon

- R1-2006609	Maintenance of Two step RACH	Ericsson

- R1-2006692	Maintenance for Two-step RACH	NTT DOCOMO, INC.

# Appendix

List of proposals in the submitted contributions.

| TDoc | Proposals |
| --- | --- |
| R1-2005329, vivo | Proposal 1: Keep current specification unchanged for the determination of scrambling for msgA PUSCH of both CBRA and CFRA 2-step RACH. Proposal 2: Keep current specification unchanged for the determination of time domain resource allocation for msgA PUSCH of both CBRA and CFRA 2-step RACH. Proposal 3: RAN1 make conclusion on msgA-RSRP-ThresholdSSB in RACH-ConfigCommonTwoStepRA is used for the selection of the SSB for 2-step RACH triggered by BFR. - No spec change is needed. |
| R1-2005605, ZTE | The following change to TS 38.214 is proposed ----------------------------------------- Start of TP ------------------------------------------------ 5.1.3.1 Modulation order and target code rate determination <---------------------------Other parts are omitted-------------------------------> The UE is not expected to decode a PDSCH scheduled with P-RNTI, RA-RNTI, msgB-RNTI, SI-RNTI and Qm > 2 <---------------------------Other parts are omitted-------------------------------> ---------------------------------------- End of TP ---------------------------------------------------- |
| R1-2005664, CATT | Proposal 1: We suggest applying RRC parameter names for 2-step RACH in TS38.331 to section 8 in 38.213 in order to make RAN1 spec clearer and make parameter names consistent on 2 step RACH between RAN1 spec and RRC spec. Below TPs for TS 38.213 are adopted. --------------------------Start of TP for TS 38.213 ------------------------------------------- ## 8.1 Random access preamble <Unchanged Text Omitted> For Type-2 random access procedure with common configuration of PRACH occasions with Type-1 random access procedure, a UE is provided a number N of SS/PBCH block indexes associated with one PRACH occasion by ssb-perRACH-OccasionAndCB-PreamblesPerSSB and a number Q of contention based preambles per SS/PBCH block index per valid PRACH occasion by msgA-CB-PreamblesPerSSB-PerSharedRO. The PRACH transmission can be on a subset of PRACH occasions associated with a same SS/PBCH block index for a UE provided with a PRACH mask index by msgA-ssb-sharedRO-MaskIndex according to [11, TS 38.321]. For Type-2 random access procedure with separate configuration of PRACH occasions with Type-1 random access procedure, a UE is provided a number N of SS/PBCH block indexes associated with one PRACH occasion and a number R of contention based preambles per SS/PBCH block index per valid PRACH occasion bymsgA-SSB-PerRACH-OccasionAndCB-PreamblesPerSSB when provided; otherwise, by ssb-perRACH-OccasionAndCB-PreamblesPerSSB. <Unchanged Text Omitted> A UE determines a first interlace or first RB for a first PUSCH occasion in an active UL BWP respectively from interlaceIndexFirstPO-MsgA-PUSCH or from frequencyStartMsgA-PUSCH that provides an offset, in number of RBs in the active UL BWP, from a first RB of the active UL BWP. A PUSCH occasion includes a number of interlaces or a number of RBs provided by nrofInterlacesPerMsgA-PO or by nrofPRBs-perMsgA-PO, respectively. Consecutive PUSCH occasions in the frequency domain of an UL BWP are separated by a number of RBs provided by guardBandMsgA-PUSCH. A number Nf of PUSCH occasions in the frequency domain of an UL BWP is provided by nrofMsgA-PO-FDM. <Unchanged Text Omitted> Consecutive PUSCH occasions within each slot are separated by guardPeriodMsgA-PUSCH symbols and have same duration. A number Nt of time domain PUSCH occasions in each slot is provided by nrofMsgA-PO-perSlot and a number Ns of consecutive slots that include PUSCH occasions is provided by nrofSlotsMsgA-PUSCH. A UE is provided a DMRS configuration for a PUSCH transmission in a PUSCH occasion in an active UL BWP by msgA-DMRS-Config. <Unchanged Text Omitted> A PUSCH occasion for PUSCH transmission is defined by a frequency resource and a time resource, and is associated with a DMRS resource. The DMRS resources are provided by msgA-DMRS-Config. <Unchanged Text Omitted> where Npreamble=ceilTpreambleTPUSCH, Tpreamble is a total number of valid PRACH occasions per association pattern period multiplied by the number of preambles per valid PRACH occasion provided by rach-ConfigCommonTwoStepRA, and TPUSCH is a total number of valid PUSCH occasions per PUSCH configuration per association pattern period multiplied by the number of DMRS resource indexes per valid PUSCH occasion provided by msgA-DMRS-Config. <Unchanged Text Omitted> -----------------------End of TP for TS 38.213 --------------------------------------------- |
| R1-2006091, Samsung | Observation 1: whether the POs derived for one slot could be extended to the next slot is not clear; Observation 2: allowing the POs derived for one slot could be extended to the next slot can make efficient use of the UL resource and reduce the delay. Proposal 1: Adopt following TP in section 8.1 in TS38.213: ========== section 8.1 in TS38.213 unchanged part omitted ============ For Type-2 random access procedure with common configuration of PRACH occasions with Type-1 random access procedure, a UE is provided a number N of SS/PBCH block indexes associated with one PRACH occasion by ssb-perRACH-OccasionAndCB-PreamblesPerSSB and a number Q of contention based preambles per SS/PBCH block index per valid PRACH occasion by msgA-CB-PreamblesPerSSB. The PRACH transmission can be on a subset of PRACH occasions associated with a same SS/PBCH block index within a SSB-RO mapping cycle for a UE provided with a PRACH mask index by msgA-ssb-sharedRO-MaskIndex according to [11, TS 38.321]. ========== End ========================== Proposal 2: the POs derived for one slot could be across the slot boundary and/or extend to the next slot. Proposal 3: adopt following TP in section 8.2 of TS38.213 =========== section 8.2 in TS38.213 unchanged part omitted =========== For mapping one or multiple preambles of a PRACH slot to a PUSCH occasion associated with a DMRS resource, a UE determines a first slot for a first PUSCH occasion in an active UL BWP from msgA-PUSCH-TimeDomainOffset that provides an offset, in number of slots in the active UL BWP, relative to the start of a PUSCH slot including the start of each PRACH slot. The UE does not expect to have a PRACH preamble transmission and a PUSCH transmission with a msgA in a PRACH slot or in a PUSCH slot, or to have overlapping msgA PUSCH occasions for a MsgA PUSCH configuration. The UE expects that a first PUSCH occasion in each slot has a same SLIV [6, TS 38.214] for a PUSCH transmission that is provided by startSymbolAndLengthMsgA-PO. Consecutive PUSCH occasions within each slot are separated by guardPeriodMsgA-PUSCH symbols and have same duration. A number Nt of time domain PUSCH occasions in each slot is provided by nrofMsgA-PO-perSlot and a number Ns of consecutive slots that include PUSCH occasions is provided by nrofSlotsMsgA-PUSCH. The POs derived for one slot might be extended the next slot. ============= End =========================== |
| R1-2006284, Spreadtrum | Proposal 1: To complete the restricted set application for 2-step RACH, adopt TP#1 in the Appendix. Proposal 2: Align RO related configuration parameters in TS38.213 with TS 38.331, adopt TP#2 in the Appendix. Proposal 3: To correct the new PRACH configuration index for 2-step RACH, adopt TP#1 in the Appendix. ## Text Proposal1 for TS38.211 --------------------------------- Start of Text Proposal ---------------------------------------- -------------------------------- Unchanged parts omitted --------------------------------------- #### 6.3.3.1 Sequence generation The set of random-access preambles shall be generated according to from which the frequency-domain representation shall be generated according to where , , LRA=1151, or LRA=571 depending on the PRACH preamble format as given by Tables 6.3.3.1-1 and 6.3.3.1-2. There are 64 preambles defined in each time-frequency PRACH occasion, enumerated in increasing order of first increasing cyclic shift of a logical root sequence, and then in increasing order of the logical root sequence index, starting with the index obtained from the higher-layer parameter prach-RootSequenceIndex or rootSequenceIndex-BFR or by msgA-PRACH-RootSequenceIndex-r16if configured and a type-2 random-access procedure is initiated as described in clause 8.1 of [5, TS 38.213]. Additional preamble sequences, in case 64 preambles cannot be generated from a single root Zadoff-Chu sequence, are obtained from the root sequences with the consecutive logical indexes until all the 64 sequences are found. The logical root sequence order is cyclic; the logical index 0 is consecutive to LRA-2. The sequence number is obtained from the logical root sequence index according to Tables 6.3.3.1-3 to 6.3.3.1-4B. The cyclic shift is given by where is given by Tables 6.3.3.1-5 to 6.3.3.1-7, the higher-layer parameter restrictedSetConfig for Type-1 random access procedure and the higher-layer parameter msgA-RestrictedSetConfig for Type-2 random access procedure determine the type of restricted sets (unrestricted, restricted type A, restricted type B), and Tables 6.3.3.1-1 and 6.3.3.1-2 indicate the type of restricted sets supported for the different preamble formats. #### 6.3.3.2 Mapping to physical resources The preamble sequence shall be mapped to physical resources according to where is an amplitude scaling factor in order to conform to the transmit power specified in [5, TS38.213], and is the antenna port. Baseband signal generation shall be done according to clause 5.3 using the parameters in Table 6.3.3.1-1 or Table 6.3.3.1-2 with given by Table 6.3.3.2-1. Random access preambles can only be transmitted in the time resources obtained from Tables 6.3.3.2-2 to 6.3.3.2-4 and depends on FR1 or FR2 and the spectrum type as defined in [8, TS38.104]. The PRACH configuration index in Tables 6.3.3.2-2 to 6.3.3.2-4 is - for Table 6.3.3.2-3 given by the higher-layer parameter prach-ConfigurationIndex-v1610if configured, otherwise by the higher-layer parameter prach-ConfigurationIndex, or by msgA-PRACH-ConfigurationIndex-r16 if configured; and - for Tables 6.3.3.2-2 and 6.3.3.2-4 given by the higher-layer parameter prach-ConfigurationIndex, or by msgA-PRACH-ConfigurationIndex-r16 if configured.------------------------------ Unchanged parts omitted -------------------------------------- ------------------------------End of Text Proposal ------------------------------------------ ## Text Proposal2 for TS38.213 --------------------------- Start of Text Proposal ------------------------------------------- --------------------------Unchanged parts omitted ----------------------------------------- ### 8.1 Random access preamble Physical random access procedure is triggered upon request of a PRACH transmission by higher layers or by a PDCCH order. A configuration by higher layers for a PRACH transmission includes the following: - A configuration for PRACH transmission [4, TS 38.211]. - A preamble index, a preamble SCS, , a corresponding RA-RNTI, and a PRACH resource. A PRACH is transmitted using the selected PRACH format with transmission power , as described in Clause 7.4, on the indicated PRACH resource. For Type-1 random access procedure, a UE is provided a number N of SS/PBCH block indexes associated with one PRACH occasion and a number R of contention based preambles per SS/PBCH block index per valid PRACH occasion by ssb-perRACH-OccasionAndCB-PreamblesPerSSB. For Type-2 random access procedure with common configuration of PRACH occasions with Type-1 random access procedure, a UE is provided a number N of SS/PBCH block indexes associated with one PRACH occasion by ssb-perRACH-OccasionAndCB-PreamblesPerSSB and a number Q of contention based preambles per SS/PBCH block index per valid PRACH occasion by msgA-CB-PreamblesPerSSB-PerSharedRO-r16. The PRACH transmission can be on a subset of PRACH occasions associated with a same SS/PBCH block index for a UE provided with a PRACH mask index by msgA-SSB-SharedRO-MaskIndex-r16 according to [11, TS 38.321]. For Type-2 random access procedure with separate configuration of PRACH occasions with Type-1 random access procedure, a UE is provided a number N of SS/PBCH block indexes associated with one PRACH occasion and a number R of contention based preambles per SS/PBCH block index per valid PRACH occasion by msgA-SSB-PerRACH-OccasionAndCB-PreamblesPerSSB-r16 when provided; otherwise, by ssb-perRACH-OccasionAndCB-PreamblesPerSSB. For Type-1 random access procedure, or for Type-2 random access procedure with separate configuration of PRACH occasions from Type 1 random access procedure, if N<1, one SS/PBCH block index is mapped to 1N consecutive valid PRACH occasions and R contention based preambles with consecutive indexes associated with the SS/PBCH block index per valid PRACH occasion start from preamble index 0. If N≥1, R contention based preambles with consecutive indexes associated with SS/PBCH block index n, 0≤n≤N-1, per valid PRACH occasion start from preamble index n⋅NpreambletotalN where Npreambletotal is provided by totalNumberOfRA-Preambles for Type-1 random access procedure, or by msgA-TotalNumberOfRA-Preambles-r16for Type-2 random access procedure with separate configuration of PRACH occasions from a Type 1 random access procedure, and is an integer multiple of N. For Type-2 random access procedure with common configuration of PRACH occasions with Type-1 random access procedure, if N<1, one SS/PBCH block index is mapped to 1N consecutive valid PRACH occasions and Q contention based preambles with consecutive indexes associated with the SS/PBCH block index per valid PRACH occasion start from preamble index R. If N≥1, Q contention based preambles with consecutive indexes associated with SS/PBCH block index n, 0≤n≤N-1, per valid PRACH occasion start from preamble index n⋅NpreambletotalN+R, where Npreambletotal is provided by totalNumberOfRA-Preambles for Type-1 random access procedure. ----------------------------- Unchanged parts omitted -------------------------------------- ----------------------------- End of Text Proposal ---------------------------------------- |
| R1-2006407, Huawei | ## Text proposal #1 for TS 38.211 Clause 5.3.2: ==================== Unchanged part omitted ==================== 5.3.2 OFDM baseband signal generation for PRACH The time-continuous signal on antenna port p for PRACH is defined by sl(p,μ)t=k=0LRA-1ak(p,RA)ej2πk+Kk1+kΔfRAt-NCP,lRATc-tstartRA K=ΔfΔfRA k1=k0μ+NBWP,istart-Ngridstart,μNscRB+nRAstartNscRB+nRANRBRANscRB-Ngridsize,μNscRB2 k0μ=Ngridstart,μ+Ngridsize,μ2NscRB-Ngridstart,μ0+Ngridsize,μ02NscRB2μ0-μ where and - is given by clause 6.3.3; - is the subcarrier spacing of the initial uplink bandwidth part during initial access. Otherwise, is the subcarrier spacing of the active uplink bandwidth part; - μ0 is the largest μ value among the subcarrier spacing configurations by the higher-layer parameter scs-SpecificCarrierList; - is the lowest numbered resource block of the initial uplink bandwidth part and is derived by the higher-layer parameter initialUplinkBWP during initial access. Otherwise, is the lowest numbered resource block of the active uplink bandwidth part and is derived by the higher-layer parameter BWP-Uplink; - nRAstart is the frequency offset of the lowest PRACH transmission occasion in frequency domain with respect to physical resource block 0 of the active uplink bandwidth part. The quantity nRAstart is given by the higher-layer parameter msgA-RO-FrequencyStart-r16 if configured and a type-2 random-access procedure is initiated as described in clause 8.1 of [5, TS 38.213], otherwise by msg1-FrequencyStart as described in clause 8.1 of [5 TS 38.213]; ==================== Unchanged part omitted ==================== ## Text proposal #2 for TS 38.211 Clause 6.3.3.2: ==================== Unchanged part omitted ==================== Random access preambles can only be transmitted in the frequency resources given by either the higher-layer parameter msg1-FrequencyStart or msgA-RO-FrequencyStart-r16 if configured as described in clause 8.1 of [5 TS 38.213]. The PRACH frequency resources nRA∈0,1,…,M-1, where M equals the higher-layer parameter msg1-FDM or msgA-RO-FDM if configured, are numbered in increasing order within the initial uplink bandwidth part during initial access, starting from the lowest frequency. Otherwise, nRA are numbered in increasing order within the active uplink bandwidth part, starting from the lowest frequency. ==================== Unchanged part omitted ==================== ## Text proposal #3 for TS 38.211 Clause 6.3.3.1: ==================== Unchanged part omitted ==================== The cyclic shift is given by where is given by Tables 6.3.3.1-5 to 6.3.3.1-7, the higher-layer parameter restrictedSetConfig determines the type of restricted sets (unrestricted, restricted type A, restricted type B), or the higher-layer parameter msgA-RestrictedSetConfig-r16, if provided, determines the type of restricted sets (unrestricted, restricted type A, restricted type B) if a type-2 random-access procedure is initiated as described in clause 8.1 of [TS 38.213]. and Tables 6.3.3.1-1 and 6.3.3.1-2 indicate the type of restricted sets supported for the different preamble formats. ==================== Unchanged part omitted ==================== ## Text proposal #4 for TS 38.213 Clause 8.1A: ==================== Unchanged part omitted ==================== If a UE does not have dedicated RRC configuration, or has an initial UL BWP as an active UL BWP, or is not provided startSymbolAndLengthMsgA-PO, msgA-PUSCH-timeDomainAllocation provides a SLIV and a PUSCH mapping type for a PUSCH transmission by indicating - one of the first maxNrofUL-Allocations values from PUSCH-TimeDomainResourceAllocationList, if PUSCH-TimeDomainResourceAllocationList is provided in PUSCH-ConfigCommon - one of the entries from table 6.1.2.1.1-2 in [6, TS 38.214], if PUSCH-TimeDomainResourceAllocationList is not provided in PUSCH-ConfigCommon else, the UE is provided a SLIV by startSymbolAndLengthMsgA-PO, and a PUSCH mapping type by mappingTypeMsgA-PUSCH for a PUSCH transmission. For mapping one or multiple preambles of a PRACH slot to a PUSCH occasion associated with a DMRS resource, a UE determines a first slot for a first PUSCH occasion in an active UL BWP from msgA-PUSCH-TimeDomainOffset that provides an offset, in number of slots in the active UL BWP, relative to the start of a PUSCH slot including the start of each PRACH slot. The UE does not expect to have a PRACH preamble transmission and a PUSCH transmission with a msgA in a PRACH slot or in a PUSCH slot, or to have overlapping msgA PUSCH occasions for a MsgA PUSCH configuration. The UE expects that a first PUSCH occasion in each slot has a same SLIV [6, TS 38.214] for a PUSCH transmission that is provided by startSymbolAndLengthMsgA-PO or by msgA-PUSCH-timeDomainAllocation. =================== Unchanged part omitted ================== |
| R1-2006609, Ericsson | Proposal 1 If the MsgA resource the UE determines does not meet the gap requirement between MsgA preamble and PUSCH, the PUSCH is not transmitted, according to text proposal TP1. --------------------------start of TP1 for 38.213 section 8.1A-------------------------------- ## 8.1A PUSCH for Type-2 random access procedure For a Type-2 random access procedure, a UE transmits a PUSCH, when applicable, after transmitting a PRACH. The UE encodes a transport block provided for the PUSCH transmission using redundancy version number 0. The PUSCH transmission is after the PRACH transmission by at least N symbols where N=2 for μ=0 or μ=1, N=4 for μ=2 or μ=3, and μ is the SCS configuration for the active UL BWP. A UE does not transmit a PUSCH in a PUSCH occasion if the PUSCH occasion associated with a DMRS resource is not mapped to a preamble of valid PRACH occasions or if the associated PRACH preamble is not transmitted as described in Clause 7.5 or Clause 11.1 or if the time gap between the PUSCH occasion and corresponding PRACH occasion is less than N symbols. A UE can transmit a PRACH preamble in a valid PRACH occasion if the PRACH preamble is not mapped to a valid PUSCH occasion. A mapping between one or multiple PRACH preambles and a PUSCH occasion associated with a DMRS resource is per PUSCH configuration. ----------------------------unchanged text omitted--------------------------------- -------------------------------end of TP1------------------------------------------ Proposal 2 Capture the MsgA PUSCH resource determination in CFRA in 38.213, according to text proposal TP2. ----------------------start of TP2 for 38.213 section 8.1A--------------------------------- ------------------------unchanged text omitted--------------------------------- A PUSCH occasion for PUSCH transmission is defined by a frequency resource and a time resource, and is associated with a DMRS resource. The DMRS resources are provided by msgA-DMRS-Configuration. For contention-based Type-2 random access procedure, Eeach consecutive number of Npreamble preamble indexes from valid PRACH occasions in a PRACH slot - first, in increasing order of preamble indexes within a single PRACH occasion - second, in increasing order of frequency resource indexes for frequency multiplexed PRACH occasions - third, in increasing order of time resource indexes for time multiplexed PRACH occasions within a PRACH slot are mapped to a valid PUSCH occasion and the associated DMRS resource - first, in increasing order of frequency resource indexes fid for frequency multiplexed PUSCH occasions - second, in increasing order of DMRS resource indexes within a PUSCH occasion, where a DMRS resource index DMRSid is determined first in an ascending order of a DMRS port index and second in an ascending order of a DMRS sequence index [4, TS 38.211] - third, in increasing order of time resource indexes tid for time multiplexed PUSCH occasions within a PUSCH slot - fourth, in increasing order of indexes for Ns PUSCH slots where Npreamble=ceilTpreambleTPUSCH, Tpreamble is a total number of valid PRACH occasions per association pattern period multiplied by the number of preambles per valid PRACH occasion provided by msgA-PUSCH-PreambleGroup, and TPUSCH is a total number of valid PUSCH occasions per PUSCH configuration per association pattern period multiplied by the number of DMRS resource indexes per valid PUSCH occasion provided by msgA-DMRS-Configuration. For contention-free Type-2 random access procedure, UE determines the PUSCH occasion and the associated DMRS resource provided by dedicated higher layer parameter msgA-PUSCH-resource-Index [12, TS 38.331] if present, otherwise UE determines a PUSCH occasion with fid = 0, tid = 0, Ns=0, a DMRS sequence provided by msgA-ScramblingID0 and a DMRS port 0 for MsgA PUSCH transmission. -------------------------unchanged text omitted------------------------------ -------------------------------end of TP2--------------------------------------------------- Proposal 3 Capture the default TDRA table 6.1.2.1.1-3 for extended CP for MsgA PUSCH and correct the typo according to text proposal TP3. ------------------start of TP3 for 38.213 section 8.1A----------------------------------- -----------------------unchanged text omitted------------------------------- If a UE does not have dedicated RRC configuration, or has an initial UL BWP as an active UL BWP, or is not provided startSymbolAndLengthMsgA-PO, msgA-PUSCH-timeDomainAllocation provides a SLIV and a PUSCH mapping type for a PUSCH transmission by indicating - first maxNrofUL-Allocations values from PUSCH-TimeDomainResourceAllocationList, if PUSCH-TimeDomainResourceAllocationList is provided in PUSCH-ConfigCommon - entries from table 6.1.2.1.1-2 for normal CP or table 6.1.2.1.1-3 for extended CP in [6, TS 38.214] according to the higher layer parameter cyclicPrefix, if PUSCH-TimeDomainResourceAllocationList is not provided in PUSCH-ConfigCommon else, the UE is provided with a SLIV by startSymbolAndLengthMsgA-PO, and a PUSCH mapping type by mappingTypeMsgA-PUSCH for a PUSCH transmission. -------------------------unchanged text omitted-------------------------------- -------------------------------------end of TP3----------------------------------------- Proposal 4 Align the resource overhead determination with Msg3 for MsgA PUSCH, according to text proposal TP4. ------------------------start of TP4 for 38.214 section 6.1.4.2------------------------------- ---------------------------unchanged text omitted------------------------------- The UE shall first determine the number of REs (NRE) within the slot: - A UE first determines the number of REs allocated for PUSCH within a PRB by - , where is the number of subcarriers in the frequency domain in a physical resource block, NsymbslotNsymbslotis the number of symbols L of the PUSCH allocation according to Clause 6.1.2.1 for scheduled PUSCH or Clause 6.1.2.3 for configured PUSCH, is the number of REs for DM-RS per PRB in the allocated duration including the overhead of the DM-RS CDM groups without data, as described for PUSCH with a configured grant in Clause 6.1.2.3 or as indicated by DCI format 0_1 or DCI format 0_2 or as described for DCI format 0_0 in Clause 6.2.2, and is the overhead configured by higher layer parameter xOverhead in PUSCH-ServingCellConfig. If the is not configured (a value from 6, 12, or 18), the is assumed to be 0. For Msg3 or MsgA PUSCH transmission the is always set to 0. In case of PUSCH repetition Type B, is determined assuming a nominal repetition with the duration of L symbols without segmentation. -----------------------unchanged text omitted----------------------------- --------------------------------end of TP4----------------------------------------------- Proposal 5 msgA-PreambleReceivedTargetPower, if provided, should be used for the power control of MsgA PUSCH, according to text proposal TP5. --------------------------start of TP5 for 38.213 section 7.1.1----------------------------- -------------------------unchanged text omitted----------------------------- ### 7.1.1 UE behaviour If a UE transmits a PUSCH on active UL BWP of carrier of serving cell using parameter set configuration with index and PUSCH power control adjustment state with index , the UE determines the PUSCH transmission power in PUSCH transmission occasion as [dBm] where, - is the UE configured maximum output power defined in [8-1, TS 38.101-1], [8-2, TS38.101-2] and [8-3, TS38.101-3] for carrier of serving cell in PUSCH transmission occasion . - is a parameter composed of the sum of a component and a component where . - If a UE established dedicated RRC connection using a Type-1 random access procedure, as described in Clause 8, and is not provided P0-PUSCH-AlphaSet or for a PUSCH transmission scheduled by a RAR UL grant as described in Clause 8.3, , , and , where PO_PRE is provided by preambleReceivedTargetPower [11, TS 38.321] and ΔPREAMBLE_Msg3 is provided by msg3-DeltaPreamble, or dB if msg3-DeltaPreamble is not provided, for carrier of serving cell - If a UE established dedicated RRC connection using a Type-2 random access procedure, as described in Clause 8, and is not provided P0-PUSCH-AlphaSet, or for a PUSCH transmission for Type-2 random access procedure as described in Clause 8.1A, j=0, PO_UE_PUSCH,b,f,c(0)=0, and PO_NOMINAL_PUSCH,f,c(0)=PO_PRE+ΔMsgA_PUSCH, where PO_PRE is provided by msgA-preambleReceivedTargetPower, or by preambleReceivedTargetPower if msgA-preambleReceivedTargetPoweris not provided and ΔMsgA_PUSCH is provided by msgADeltaPreamble, or ΔMsgA_PUSCH=ΔPREAMBLE_Msg3 dB if msgADeltaPreamble is not provided, for carrier f of serving cell c -----------------------unchanged text omitted---------------------------- -------------------------------end of TP5------------------------------------------ |
| R1-2006692, Docomo | Proposal 1: There is no need to add the parameter “msgA-RSRP-ThresholdSSB” to section 6 in TS 38.213. |