3GPP TSG RAN WG1 #103-e	R1-2007547

e-Meeting, October 26 – November 13, 2020

Agenda Item:	8.1.5

Source:	Futurewei

Title:	Sounding enhancement for interference probing in TDD massive MIMO

Document for:	Discussion/Decision

# Introduction

In 3GPP RAN Meeting #86, a new work item (WI) on Further enhancements on MIMO for NR (NR_FeMIMO, see RP-193133) was approved. Among the multiple objectives in the WI, the following is concerned with SRS enhancement:

- Enhancement on SRS, targeting both FR1 and FR2:

  - Identify and specify enhancements on aperiodic SRS triggering to facilitate more flexible triggering and/or DCI overhead/usage reduction

  - Specify SRS switching for up to 8 antennas (e.g., xTyR, x = {1, 2, 4} and y = {6, 8})

  - Evaluate and, if needed, specify the following mechanism(s) to enhance SRS capacity and/or coverage: SRS time bundling, increased SRS repetition, partial sounding across frequency

In this contribution, the SRS enhancements are discussed in detail. Specifically, this contribution is focused on sounding enhancement for interference probing in TDD massive MIMO, “SRS probing” for short. TDD Massive MIMO is capable of providing high spectrum efficiency (SE) and is widely considered as a key technology for 5G NR. However, the current 5G NR TDD system provides only moderate performance benefit over 4G LTE TDD system with the same antenna setting. Further enhancement of TDD Massive MIMO for even higher SE is desirable for future releases of NR. In this contribution, we propose a SRS probing mechanism that enhances UL SRS to probe and mitigate DL interference (intra-cell and inter-cell) in a TDD Massive MIMO system, resulting in significant SE gains on top of current Rel-16 compatible mechanisms. It is suggested that RAN1 support this mechanism in Rel-17.

This contribution is mainly an updated version based on R1-2005291 submitted to RAN1#102-e, with system-level evaluation results generated according to the latest RAN1 agreements.

# Cooperative Massive MIMO: potential and issues

Massive MIMO is considered as the major technique to achieve very high SE performance. One of the main issues in Massive MIMO is how to obtain accurate knowledge of the channels to enable spatial multiplexing of transmissions to multiple users without strong inter-user interference. For FDD systems, a relatively smaller number of antennas in the system limits the potential of performance, and in addition, without channel reciprocity, CSI acquisition needs to rely on CSI feedback which can lead to higher overhead, longer latency, lower accuracy, and hence limited SE performance. For TDD systems, Massive MIMO works relatively well thanks to accurate CSI acquisition through channel UL-DL reciprocity and generally provides higher SE than FDD.

However, channel UL-DL reciprocity is so far only utilized for single-cell (non-cooperative) MU-MIMO for TDD system and the gain is still far from what is promised with the massive number of antennas.

On the other hand, cooperative MIMO (e.g., CoMP in LTE) has not been found successful in practice. Here are some insights on the causes:

- Reciprocity in TDD is utilized (only) for channel information estimation for desired signals.

- Intra-cell interference (from MU-pairing) is handled relatively well, but for most scenarios with Massive MIMO inter-cell interference becomes the dominant limiting factor.

- Handling inter-cell interference using channel information of neighboring cells is not successful even for TDD system:

  - Each individual interfering link may not be strong enough to obtain reliable channel information, by CSI feedback or by SRS.

  - The number of interfering links to consider is generally large, resulting in overhead, complexity, and robustness issues.

  - Each cell (or cell group) uses a centralized approach, which leads to significant complexity, robustness, backhaul, and edge-effect issues.

To resolve these issues and obtain the promised gain from the massive number of antennas, an approach for TDD Cooperative MIMO is presented here. Instead of trying to obtain channel state information of interfering links and then determine precodings jointly at a centralized entity, SRS is enhanced to directly reflect DL interference spatial information (utilizing UL-DL reciprocity). Each gNB measures the corresponding SRS resources to obtain such information and adjusts its precoding to achieve interference coordination/avoidance. More details are provided in the next section.

# Flexible A-SRS Triggering Enhancements for TDD Cooperative MIMO

Here we introduce a mechanism to obtain spatial information of DL inter/intra-cell interference through cooperative UL sounding based on UL-DL reciprocity.

Traditionally, DL interference and UL transmissions are not directly related even in a TDD system, and hence UL-DL reciprocity is not applicable for interference acquisition purposes. To utilize TDD UL-DL reciprocity for interference measurement, the key is to tie some sounding activities to scheduled DL transmissions, which may be referred to as DL Interference Probing or SRS Probing. In other words, after gNBs make DL scheduling decisions including resource allocation and MU pairing, the system first goes through a step in which the gNBs trigger the to-be-scheduled UEs to sound on resources corresponding to the DL scheduling. That is, only (and all) the UEs/layers prescheduled for PDSCH will send SRS, and the SRS reflects the PDSCH resource/layer allocation. Then by TDD reciprocity, a gNB seeing strong UL interference from a certain spatial direction on the SRS resources (for example, via estimating the UL spatial covariance of interference signal) implies that in DL transmission the gNB will cause strong interference in that direction. The gNB can then adjust the precoding for DL interference avoidance. Finally, the gNBs can perform the actual DL transmissions with resource allocations and MU pairing the same as the prescheduling. The gNBs in the network can coordinate the probing SRS resources and the time delay between the prescheduling and actual DL transmission in a semi-static fashion a priori, but essentially no inter-gNB information exchange is required on the fly. To summarize, DL Interference Probing from UL is possible if the network controls the UEs to transmit SRS in a way that best reflects prospective DL interference.

The above approach is illustrated in Figure 1, in which the sounding from UEs is based on prescheduling and reflects UE receive beamforming capabilities. As a result, the gNB can acquire DL interference spatial information. The gNB adjusts the precoders and thus DL SINR and SE can be improved. Furthermore, depending on the network implementation, more users/layers may be paired for MU-MIMO (such as via less conservative scheduling), yielding even higher SE gains. This approach of interference probing and mitigation, namely SRS probing, can be used to enable a variety of communication schemes to be implemented, generally in the category of Cooperative Massive MIMO.

Figure 1: TDD DL SE performance enhancement via interference probing and mitigation. To convey spatial information about the interference (both intra-cell and inter-cell) to the network, flexible A-SRS triggering based on prescheduling may be used

The above approach is effective to suppress both inter-cell and intra-cell interference. As discussed above, in Massive MIMO systems, the MU interference or intra-cell interference can be effectively controlled by existing solutions such as zero-forcing or WMMSE, leaving inter-cell interference the main bottleneck for further SE improvement. Thus, precoding against both intra-cell and inter-cell interference is highly effective in achieving optimized system level performance. This approach also has the following advantages:

- Distributed across gNBs (or TRPs) in the network, with low computational complexity for each gNB

- No or little channel information exchange among gNBs

- Can adapt to inter-cell interference, including unknown interference from non-cooperative gNBs or outside of the network (e.g., from other service providers, small cells, etc.)

- No need to estimate element-wise channel, reducing the complexity and overhead.

One specific way to implement is called bi-directional training (BiT); see, e.g., [1][2][10] (and references therein) for details for algorithms for narrowband systems, wideband systems, and dynamic TDD systems. Iterative interference probing and mitigation may be carried out starting from random initial gNB precoding, i.e., the UL sounding and DL transmission with adjusted precoders shown in Figure 1 will be executed multiple times until convergence to optimal/sub-optimal precoders, and then the PDSCH is transmitted with the optimal/sub-optimal precoders. Alternatively, BiT without multiple iterations (i.e., only one shot of interference probing before the PDSCH transmission, as shown in Figure 1) can be performed, in which beamformed sounding based on conventional DL channel measurement and interference measurement is used. The latter is more practical for typical wireless communications. Nevertheless, these implementations rely on coordinated SRS transmissions to probe DL interference and then mitigate the DL interference via precoder adjustment, and hence the term “SRS probing” (which we may use interchangeably with BiT).

We remark that the proposed scheme with prescheduling and interference probing bears certain resemblance to the NZP CSI-RS based interference probing scheme described in [4] (Sec. 3.1) and [5]. Both Cooperative Massive MIMO schemes have a step of DL interference probing following prescheduled outcome. However, in the NZP CSI-RS based scheme, the DL interference probing is via DL CSI-RS, and the gNB adjusts MCS (but not precoding), whereas in the currently proposed scheme, the DL interference probing is via UL SRS, and the gNB adjusts precoding for interference avoidance (but not MCS).

The key standard component needed to support BiT is flexible A-SRS triggering, that is, to trigger A-SRS with flexibility resembling PDSCH. This fits into Rel-17 FeMIMO WI flexible A-SRS trigging topic very well.

Observation 1: Flexible A-SRS triggering is motivated at least by TDD cooperative SRS probing, which utilizes coordinated A-SRS for DL interference probing and mitigation.

## Special case for single-cell massive MIMO (ZF) with DL interference probing

The above scheme can also be applied to the special case of ZF without any inter-cell cooperation such as semi-static coordination. A-SRS can be used to probe DL channel/interference for the paired UEs within a cell. That is, a UE transmits A-SRS on the PRBs indicated by the network which are to be used for PDSCH, and the network adjust PDSCH precoding based on these partial bandwidth A-SRS, instead of based on P/SP SRS sent by UE regularly to cover the entire wideband and with longer periodicity in typical ZF schemes. In fact, when ignoring the inter-cell interference (or the covariance matrix for inter-cell interference), the above scheme reduces to single-cell massive MIMO, i.e., ZF, except that the sounding used for ZF is based on A-SRS whose frequency-domain resources, ports, and beamforming are tied to the PDSCH. As we will see, the A-SRS based ZF with DL interference probing outperforms P/SP-SRS based ZF. The key standard component needed to support this enhancement is still flexible A-SRS triggering, similar to BiT.

# Performance Evaluation of Cooperative SRS Probing

A large variety of simulations have been done to evaluate the performance of BiT with one-shot interference probing in a multi-cell wideband TDD system, and in most of the simulations, substantial SE performance improvements have been observed. To best understand the scheme and the reasons for the performance improvements, it is helpful to start with simple system simulations to conceptually grasp the essence of the scheme and then move to more sophisticated, full-blown system simulations.

## Performance evaluation with “conceptual” simulator

On https://github.com/futureweiwirelesslab/bi-directionalTraining/ [6], we published a minimal demo of one type of BiT implementation in Matlab with a number of ideal and simplifying assumptions. The demo shows an SINR improvement of about tens of dB and SE gain of about 100% over baseline ZF scheme, due to the superb capability of interference mitigation by BiT. The code and channel data samples can be downloaded, and the resulting gains can be verified.

## Performance evaluation with full-blown simulator and agreed EVM assumptions

More realistic evaluations are provided. The deployment scenarios are based on 3GPP models (see Table 1 in Appendix for more details). One baseline is ZF based Massive MIMO. With one-shot BiT, significant UPT gains (8% ~ 46% gain for mean UPT, and 44% to more than 2x gains for 5%ile UPT) are achieved over ZF for different RUs (~20%, 50%, and 70%) as shown in Figure 2. The gains primarily come from increased DL SINR and also the increased total number of paired layers due to effective inter-cell interference avoidance. In Figure 3, we also see that BiT can substantially reduce the RU under the same traffic load as the ZF, which results from the increased spectrum efficiency with BiT, and the reduced RU in turn contributes to reduce interference in the network, further improving the SINR and UPT in the system.

Figure 2: Performance comparison of BiT (with one-shot interference probing) and ZF-based Massive MIMO under different traffic arrival rates. The performance metrics are: mean UPT, 5%ile UPT, and 50%ile UPT. In the bars, the value “1” is for the baseline and the other value (such as “1.46”) is relative to the baseline.

Figure 3: RU comparison of BiT (with one-shot interference probing) and ZF-based Massive MIMO under different traffic arrival rates. In the bars, the value “1” is for the baseline and the other value (such as “0.91”) is relative to the baseline.

We point out the proposed scheme may have increased latency (depending on the time gap between prescheduling and DL transmission) for some transmissions, and potentially increased SRS resource overhead. However, the drastically improved SINR/SE offsets the probing latency and overhead, for example, the overall packet-level latency is much reduced (also observed in NZP CSI-RS based interference probing schemes). Additionally, the latency and overhead can be limited by various techniques via implementation and standardization, and the scheme is well justified given its substantial gains.

## Other simulations

A wide variety of simulations have been conducted, such as with full-buffer traffic and with different numbers of UEs in the network, UEs with 1T4R, SRS with UL power control, with more realistic impairments and scheduling methods, enhanced rank adaptation, jointly with NZP CSI-RS based interference probing, etc. These simulations have demonstrated generally similarly high gains of BiT over baseline ZF. These results may be submitted later or elsewhere.

Overall, we conclude that BiT can provide substantial SE performance gains over baseline ZF.

Observation 2: BiT can provide substantial SE performance gains over baseline ZF in a TDD system.

## Special case for single-cell massive MIMO (ZF) with DL interference probing

Now we compare the performance of the A-SRS based ZF with DL interference probing versus P/SP-SRS based ZF. Some assumptions are:

- P/SP SRS based ZF

Traditional ZF is based on P/SP SRS in general. The SRS has to cover the wideband (with or without hopping), and all active UEs have to sound. Therefore, the periodicity of P/SP SRS cannot be small for all UEs, otherwise the SRS would lead to significant overhead. In the simulation, the P/SP SRS periodicity is set as 10 TTIs or 20 TTIs (but not 5 TTIs).

- A-SRS based ZF with DL interference probing

Only UEs to be scheduled with PDSCH in the next 5 TTIs will transmit A-SRS. The A-SRS PRBs, ports, and beamforming are the same as the prospective PDSCH. The A-SRS is used by the network to adjust ZF precoders for the PDSCH. Therefore, the aging of A-SRS will not exceed 5 TTIs, which ensures high accuracy of the precoding/beamforming leading to high SE.

SRS used for other purposes, such as channel measurement, scheduling, etc., can be based on P/SP SRS with long periodicity, such as 20 TTIs or longer. This is because the properties needed for these purposes generally evolve much slower than interference dynamics.

Figure 4 shows the UPT performance and RU values. Clearly, with A-SRS, there is significant performance gains over P-SRS, especially for cell edge UPT and/or medium/high load traffic.

Figure 4: Performance comparison of ZF based on P-SRS and ZF based on A-SRS under different traffic arrival rates. In the bars, the value “1” is for the baseline and the other value (such as “1.71”) is relative to the baseline.

Observation 3: TDD ZF performance can be significantly improved by flexible A-SRS based DL interference probing.

# Objectives for Rel-17 standard support

## A-SRS triggering enhancements for TDD cooperative SRS probing

In order to effectively convey information about dynamic interference conditions to the network, a gNB can indicate UEs how the SRS should be transmitted, including the time/frequency resource allocation and port selection for the SRS corresponding to the prospective PDSCH. This means that the network needs to dynamically adjust more SRS transmission parameters (PRB allocations, port selection) than with conventional SRS transmission.

The proposed Cooperative Massive MIMO mechanism mostly relies on network implementation, but some steps also require standard support:

- Operations requiring standard impact

  - SRS transmission with parameters tied to DL transmission, including PRB allocation and port allocation

    - A-SRS PRB/port allocation

The SRS PRB/port allocation should be the same as the prospective PDSCH and should be dynamically indicated.

    - A-SRS beamforming

The SRS beamforming can be based on DL CMR, and to better reflect potential DL interference, it is more desirable to be based on DL CMR and IMR, one or both of which may be dynamically indicated.

    - A-SRS triggering offset

To utilize A-SRS to estimate interference for interference mitigation via precoding adjustment, the gNBs do not need to detect each UE’s SRS sequences. Received SRS power accumulated on each gNB receiving antenna port should be sufficient. Thus, the A-SRS can be transmitted on overlapping resources to reduce overhead. However, the A-SRS triggers may be sent to different UEs at different times. To enable A-SRS overlap, A-SRS triggering offsets may be dynamically indicated to different UEs.

The triggering offset may be similar to the k0 and SLIV design for PDSCH. To further reduce overhead, k0 and SLIV may not refer to the absolute slot/symbol offsets but slot/symbol offsets based on configured SRS slots/symbols.

    - Higher priority for A-SRS with newly introduced flexibility

The A-SRS with newly introduced parameters in a SRS request field to support BiT and other enhancements may be assigned with higher priority, so that when it collides with other SRS/UL transmissions, the other transmissions are dropped.

- Operations requiring no standard impact

  - gNB scheduling/prescheduling, MU pairing, interference measurement on SRS resources, generating interference covariance matrix, precoding adjustment, time gap between prescheduling and actual DL transmission, how to tie the signalled SRS transmission parameters with parameters for DL transmission, etc.

The following proposal captures the above discussions, and see companion contribution [9] for detailed design.

Proposal: Support at least the following flexible A-SRS triggering in R17 FeMIMO:

- SRS enhancements with dynamically indicated parameters associated with corresponding DL transmissions

  - Flexible A-SRS triggering with dynamically indicated PRB allocation and port allocation

  - Flexible A-SRS triggering with flexible triggering offset

  - Flexible A-SRS triggering with dynamically indicated DL CMR and/or IMR

  - Flexible A-SRS triggering with higher priority

# Conclusion

In this contribution, we proposed a mechanism that enhances UL SRS to probe and mitigate DL interference (intra-cell and inter-cell) in a TDD Massive MIMO system, resulting in significant SE gains on top of current Rel-16 compatible mechanisms. The following are observed and proposed:

Observation 1: Flexible A-SRS triggering is motivated at least by TDD cooperative SRS probing, which utilizes coordinated A-SRS for DL interference probing and mitigation.

Observation 2: BiT can provide substantial SE performance gains over baseline ZF in a TDD system.

Observation 3: TDD ZF performance can be significantly improved by flexible A-SRS based DL interference probing.

Proposal: Support at least the following flexible A-SRS triggering in R17 FeMIMO:

- SRS enhancements with dynamically indicated parameters associated with corresponding DL transmissions

  - Flexible A-SRS triggering with dynamically indicated PRB allocation and port allocation

  - Flexible A-SRS triggering with flexible triggering offset

  - Flexible A-SRS triggering with dynamically indicated DL CMR and/or IMR

  - Flexible A-SRS triggering with higher priority

It is suggested that RAN1 consider the above proposal in FeMIMO.

See also past contributions [7,8] for previous proposals to RAN1 and RAN plenary regarding this scheme, as well as companion contribution [9].

# References

H. Zhou, J. Liu, Q. Cheng, D. Maamari, W. Xiao, and A. C. K. Soong, "Bi-Directional Training with Rank Optimization and Fairness Control," in 2018 IEEE 88th Vehicular Technology Conference (VTC-Fall), 2018.

J. Liu, Q. Cheng, W. Xiao, D. Maamari, and A. C. K. Soong, "Bi-directional Training for Wideband Systems," in 2019 IEEE 89th Vehicular Technology Conference (VTC-Fall), 2019.

3GPP, TR 38.901, Study on channel model for frequencies from 0.5 to 100 GHz.

R1-1709932, "Channel and interference measurement for CSI acquisition," Huawei, HiSilicon, 3GPP TSG RAN WG1 NR Ad Hoc Meeting, Qingdao, China, June 27 – 30, 2017.

R1-1719424, "Channel and interference measurement for CSI acquisition," Huawei, HiSilicon, 3GPP TSG RAN WG1 Meeting#91, Reno, USA, November 27 – December 1, 2017.

https://github.com/futureweiwirelesslab/bi-directionalTraining/

R1-1909018, "TDD cooperative MIMO enhancement via sounding based interference probing," FUTUREWEI, 3GPP TSG RAN WG1 Meeting #98, Prague, Czech Republic, August 26-30, 2019.

RP-191869, "Rel-17 work scope on NR MIMO," FUTUREWEI, 3GPP TSG RAN Meeting #85, Newport Beach, USA, September 16-20, 2019.

R1-2007544, "Enhancements on SRS flexibility, coverage and capacity," FUTUREWEI, 3GPP TSG RAN WG1 #103-e, October 26 – November 13, 2020.

A. Tölli, H. G. Ghauch, J. Kaleva, P. Komulainen, M. Bengtsson, M. Skoglund, M. L. Honig, E. Lähetkangas, E. Tiirola, and K. Pajukoski, Distributed Coordinated Transmission with Forward-Backward Training for 5G Radio Access,” IEEE Communications Magazine 57(1): 58-64 (2019).

# Appendix

Table 1: Simulation settings used in numerical evaluation.

| Items | Parameters |
| --- | --- |
| Carrier | 3.5 GHz with 20 MHz bandwidth and 30 kHz SCS |
| BS Deployment | UMi, ISD = 200 m (with wraparound), 25 m antenna height |
| UE distribution | 10 per cell/sector, 80% indoor (3 km/h), 20% outdoor (30 km/h). 4 drops |
| Channel | 3D UMi channel [3] |
| Traffic statistics | Non-full buffer, FTP 3 with 0.5 MB files |
| Antenna configurations | BS: (M, N, P, Mg,Ng; Mp, Np) = (8,8,2,1,1,4,8). (dH,dV) = (0.5, 0.8)λ (8 columns, 4 rows, cross polarization, totally 64 ports) UE: 4T4R 2x1x2 (2 columns, 1 row, cross polarization, 4 ports), 0.5 λ |
| UE power | 23 dBm |
| BS power | 44 dBm |
| Precoding granularity | Subband (6 resource blocks, 72 tones) |
| Channel/covariance estimation | For scheduler: ideal serving cell channel information, ideal interference covariance estimation at the gNB For UE demodulation: realistic channel/covariance estimation |
| SRS periodicity | Aperiodic SRS with SRS resource configured per 5 TTIs |
| UE receiver noise figure | 9 dB |
| Modulation | Up to 256QAM |
| UE receiver | MMSE-IRC |