3GPP TSG RAN WG1 Meeting #103-e	R1-2007555

e-Meeting, October 26th – November 13th, 2020

Agenda Item:	8.14.1

Source:	Futurewei

Title:	XR applications and scenarios

Document for:	Discussion and decision

# Introduction

In RAN#88-e meeting, the revised Rel. 17 FS_NR_XR_eval SID [1] was approved. The objectives specified in the SID [1] are:

- Confirm XR and Cloud Gaming applications of interest - Identify the traffic model for each application of interest taking outcome of SA WG4 work as input, including considering different upper layer assumptions, e.g. rendering latency, codec compression capability etc. - Identify evaluation methodology to assess XR and CG performance along with identification of KPIs of interest for relevant deployment scenarios - Once traffic model and evaluation methodologies are agreed, carry out performance evaluations towards characterization of identified KPIs

In this contribution, we present our views on XR applications and scenarios, key traffic model parameters and QoS requirements relevant to RAN1, and proposals for moving forward.

# XR Applications and Scenarios

Extended Reality (XR) extends the reality we experience by blending the “virtual” and “real” worlds to create a fully immersive experience.  Many XR use cases are discussed in [2] including a collection of 23 use cases.  In the revised Rel. 17 FS_NR_XR_eval SID [1], it is stated that “The following applications are to be considered as starting points for this study”:

- VR1: “Viewport dependent streaming”

- VR2: “Split Rendering: Viewport rendering with Time Warp in device”

- AR1: “XR Distributed Computing”

- AR2: “XR Conversational”

- CG: Cloud Gaming

In our opinion, the above five XR applications may be categorized into two groups according to their architecture and traffic characteristics.  The first group comprises four XR applications: VR1, VR2, AR1, and CG, as they could share a similar architecture and traffic characteristic, while the second group comprises AR2, which may have its own architecture and traffic characteristic.

The four XR applications in the first group may have similar architecture such as the split rendering architecture [2]. Split-rendering refers to the architecture where the real-time viewport-rendering of the XR scene is off-loaded to a powerful XR server (back-end), while the XR User Equipment (UE) performs only the less intensive tasks of decoding and pose correction such as Asynchronous Time Warping (ATW).  Figure 1 illustrates examples of deployment of multiple split rendered XR services over 5G network.

Figure 2 shows a more detailed diagram [2] for split rendering with ATW Correction. With this architecture, the following key steps are required to support the XR service [3]:

1)	An XR Device connects to the network and joins XR application

a)	Sends static device information and capabilities (supported decoders, viewport)

2)	Based on this information, the XR server sets up encoders and formats

3)	Loop

a)	XR Device collects XR pose (or a predicted XR pose)

b)	XR Pose is sent to XR Server

c)	The XR Server uses the pose to pre-render the XR viewport

d)	XR Viewport is encoded with 2D media encoders

e)	The compressed media is sent to XR device along with XR pose that it was rendered for

f)	The XR device decompresses video

g)	The XR device uses the XR pose provided with the video frame and the actual XR pose for an improved prediction using and to correct the local pose, e.g. using ATW.

Out of the steps above, Step 3-b and 3-e are more RAN related, e.g., the UL traffic mainly contains the XR pose sent in Step 3-b while the DL traffic mainly contains the compressed media sent to the XR device in Step 3-e.

Figure 1: Deployment of multiple split rendered XR services over 5G network

Figure 2: Split Rendering with Asynchronous Time Warping (ATW) Correction

According to [2], the four applications in the first group also share some similar traffic characteristics, e.g., their DL traffics are heavier than the UL traffics, whereas the “AR2: XR Conversational” application in the second group has similar DL and UL traffic load.  Also the four applications in the first group have more stringent latency requirements (e.g., 10-50 ms) than the AR2 application (e.g., ~150 ms).

Since the four applications in the first group share some similar traffic characteristics, it will be more efficient for RAN1 to select one application (e.g., VR2: “Split Rendering: Viewport rendering with Time Warp in device”, or CG: Cloud Gaming) as the focus of study, at least in the initial stage of the study item.

Based on the above analysis, we have the following proposal:

Proposal 1: FS_NR_XR_eval considers selecting one application (e.g., VR2: “Split Rendering: Viewport rendering with Time Warp in device”, or CG: Cloud Gaming) as the focus of study, at least in the initial stage of the study item.

# Key Traffic Model Parameters and QoS Requirements

The SA WG4 is currently working on a study item on “Feasibility Study on Typical Traffic Characteristics for XR Services and other Media” [4].  It is expected that SA WG4 will provide XR traffic model as an outcome of this study item.  However, it is unclear in what format the traffic model provided by SA4 will be.  For example, in [3], V-Traces is provided as one type of traffic model, where the V-Traces includes the exact size and time stamp of each packet for a sequence of packets.  However, from RAN1’s point of view, it will be more convenient if a statistical XR traffic model can be provided for system performance evaluation purpose.

The two key parameters for the statistical XR traffic model are distribution of the inter packet arrival time and distribution of the packet size.  These two parameters are highly dependent on the format of the video signal in the XR traffic.

- Regarding the inter packet arrival time, it can be modeled as a constant (i.e., the packet arrival at the gNB is periodic).  For example, if the frame rate is 60 fps, an inter packet arrival time of 16.67 ms can be used.

- Regarding the packet size distribution, it can be modeled as Gaussian distributed [5].  The mean packet size is a function of the average data rate and the frame rate.  Table 1 shows a list of data rates and mean packet sizes for different video formats.  In the calculation in Table 1, it is assumed that YUV: 4:2:0 pixel encoding and the video codec H.264/AVC are used with a compression ratio of 120, and the factor of 2 is multiplied to account for both left and right eye frames.  As indicated in Table 1, for video format of 2Kx2K with 60 fps, the mean packet size is about 0.8 Mbits.  The variance of the packet size can be assumed to be a fraction of the mean packet size (e.g., 0.1 - 0.2 of the mean packet size) or it can be calculated from the V-Traces file provided by SA4.

Table 1: Data Rates and Mean Packet Sizes for Different Video Formats

|  | 1080P FHD | 2K x 2K | 4K UHD | 8K UHD |
| --- | --- | --- | --- | --- |
| Horizontal Resolution (pixel) | 1920 | 2000 | 3840 | 7680 |
| Vertical Resolution (pixel) | 1080 | 2000 | 2160 | 4320 |
| Frame Rate (fps) | 60 | 60 | 60 | 60 |
| Number of bits/pixel | 12 | 12 | 12 | 12 |
| Compression ratio | 120 | 120 | 120 | 120 |
| Data rate after compression (Mbps) | 12.4 | 24.0 | 49.8 | 199.1 |
| Total number of streams for both eyes | 2 | 2 | 2 | 2 |
| Total data rate for both eyes (Mbps) | 24.9 | 48.0 | 99.5 | 398.1 |
| Packet size (Mbits) | 0.4 | 0.8 | 1.7 | 6.6 |

Based on the above analysis, we have the following proposal:

Proposal 2: RAN1 notifies SA4 that RAN1 prefers a statistical XR traffic model in FS_NR_XR_eval studies and kindly asks SA4 to confirm that such a statistical XR traffic model can be provided as an outcome of SA4’s related study item on XR.

Regarding the DL data rate, from Table 1 we observe that for 2K x 2K at 60 fps and 4K UHD at 60 fps, the average data rate for DL XR traffic is around 50 Mbps and 100 Mbps, respectively.  In [2] and [3], a DL data rate of 50 to 100 Mbps are listed for XR traffic.  It seems that a DL data rate of 50 Mbps is a good starting point to be considered for this study, at least in the initial stage of the study item.

Regarding the UL data rate, it was mentioned in [2] that the UL is predominantly the pose information, and the UL data rates are several 100 kbps, assuming a typical pose sampling rate of 1Khz in the XR environment.  Compared to the DL data rate (e.g., 50 Mbps), the UL data rate is much lower.  Given the much lower required data rate on the UL, the DL is more likely to become the bottleneck to support XR.  Therefore, it will be more efficient for RAN1 to focus on DL for the study, at least in the initial stage of the study item.  The UL aspect in the modeling/evaluation can be simplified, e.g., by assuming a fixed UL latency (e.g., 10 ms) as part of the overall end-to-end latency.

Based on the above analysis, we have the following proposal:

Proposal 3: FS_NR_XR_eval sets higher priority on evaluation of DL for XR, at least in the initial stage of the study item.  The UL aspect in the modeling/evaluation can be simplified by assuming a fixed UL latency as part of the overall end-to-end latency.

The key QoS requirement in XR is the roundtrip interaction delay [2].  This delay could be understood as the total delay incurred from Step 3-a to Step 3-g described in Section 2.  In [2], four categories are considered with respect to roundtrip interaction delay:

- Ultra-Low-Latency applications: roundtrip interaction delay threshold of at most 50ms latency.

- Low-Latency applications: roundtrip interaction delay threshold of at most 100ms latency.

- Moderate latency applications: roundtrip interaction delay threshold of at most 200ms latency.

- Non- critical latency applications: roundtrip interaction delay threshold higher than 200ms latency.

From RAN1’s perspective, the DL and UL latency are more relevant in the evaluation of XR performance. The DL latency is measured as the time interval between the packet arrival at the gNB and the successful packet delivery at the UE, while the UL latency is measured as the time interval between the packet generated at the UE and the successful packet delivery at the gNB.  According to [2] and [3], typical DL latency requirements are in the range of 20 to 60 ms.  DL packets that are not successfully delivered to the UE within this latency requirement should be counted as dropped packet in the evaluation. Regarding the UL latency, ss discussed previously in this section, the UL aspect in the modeling/evaluation can be simplified by assuming a fixed UL latency (e.g., 10 ms).

Based on the above analysis, we have the following proposal:

Proposal 4: FS_NR_XR_eval considers DL latency requirements of 20 ms for XR traffic, at least in the initial stage of the study item.

# Conclusions

In this contribution, we present our views on XR applications and scenarios, key traffic model parameters and QoS requirements relevant to RAN1.  Based on the discussions in the previous sections we propose the following:

Proposal 1: FS_NR_XR_eval considers selecting one application (e.g., VR2: “Split Rendering: Viewport rendering with Time Warp in device”, or CG: Cloud Gaming) as the focus of study, at least in the initial stage of the study item.

Proposal 2: RAN1 notifies SA4 that RAN1 prefers a statistical XR traffic model in FS_NR_XR_eval studies and kindly asks SA4 to confirm that such a statistical XR traffic model can be provided as an outcome of SA4’s related study item on XR.

Proposal 3: FS_NR_XR_eval sets higher priority on evaluation of DL for XR, at least in the initial stage of the study item.  The UL aspect in the modeling/evaluation can be simplified by assuming a fixed UL latency as part of the overall end-to-end latency.

Proposal 4: FS_NR_XR_eval considers DL latency requirements of 20 ms for XR traffic, at least in the initial stage of the study item.

# References

RP-201145, “Revised SID on XR Evaluations for NR”, Qualcomm, RAN#88e.

TR 26.928, “Extended Reality (XR) in 5G (Release 16)”.

S4-201245, “FS_XRTraffic: Permanent document, v0.3.0”, Qualcomm, SA4#110e.

SP-200054, “Feasibility Study on Extensions to Typical Traffic Characteristics”, Qualcomm et al., SA#87e.

R1-2005412, “Views on XR and Cloud Gaming evaluation”, Vivo, RAN1#102-e.