3GPP TSG RAN WG1 #103-e		R1-2007560

e-meeting, October 26th – November 13th, 2020

Source:	Lenovo, Motorola Mobility

Title: 	Additional evaluations for NR beyond 52.6GHz

Agenda Item:   8.2.3

Document for:	Discussion

# 1	Introduction

In RAN#86, a new SI has been approved for supporting NR from 52.6 GHz to 71 GHz in NR Rel. 17 with following objectives captured in [1]:

- Study of required changes to NR using existing DL/UL NR waveform to support operation between 52.6 GHz and 71 GHz

  - Study of applicable numerology including subcarrier spacing, channel BW (including maximum BW), and their impact to FR2 physical layer design to support system functionality considering practical RF impairments [RAN1, RAN4].

  - Identify potential critical problems to physical signal/channels, if any [RAN1].

- Study of channel access mechanism, considering potential interference to/from other nodes, assuming beam-based operation, in order to comply with the regulatory requirements applicable to unlicensed spectrum for frequencies between 52.6 GHz and 71 GHz [RAN1].

  - Note: It is clarified that potential interference impact, if identified, may require interference mitigation solutions as part of channel access mechanism.

In this contribution, we provide additional link level simulations results and observations for different SCS. Also currently, there is an on-going email discussion for collecting results for both link level and system level simulations. In the appendix, we also include the results in the tables form that we provide for the email discussion and some additional results with TDL-A channel. Furthermore, other results and analysis are also provided in our contribution R1-2007560 [2].

# 2	Discussion

## 2.1 BLER vs SNR for PDSCH with CDL-B channel

Figure 1: BLER vs SNR for PDSCH with 400MHz BW for 60GHz carrier frequency, delay spread 20ns, NCP

Figure 2: BLER vs SNR for PDSCH with 400MHz BW for 60GHz carrier frequency, delay spread 50ns, NCP

Figure 3: BLER vs SNR for PDSCH with 400MHz BW for 60GHz carrier frequency, delay spread 20ns, ECP

Figure 4: BLER vs SNR for PDSCH with 400MHz BW for 60GHz carrier frequency, delay spread 50ns, ECP

Based on the results for BLER vs SNR, following observations can be made:

Observation 1: For lower delay spread, low-mid range MCS and normal cyclic prefix, there is no significant performance difference for target BLER between different subcarrier spacing values

Observation 2: For lower delay spread, higher MCS and normal cyclic prefix, 960kHz subcarrier spacing performs the best and with significant performance difference for target BLER of 1%, that might be needed for URLLC

Observation 3: For higher delay spread and normal cyclic prefix, 960kHz subcarrier spacing performs the worst and target BLER cannot be reached for high MCS

Observation 4: For higher delay spread and extended cyclic prefix, 960kHz subcarrier spacing performance is significantly improved compared to normal cyclic prefix

## 2.2 Spectral efficiency vs SNR for PDSCH with CDL-B channel

Figure 5: Spec. Eff. vs SNR for PDSCH with 400MHz BW for 60GHz carrier frequency, delay spread 20ns, NCP

Figure 6: Spec. Eff. vs SNR for PDSCH with 400MHz BW for 60GHz carrier frequency, delay spread 50ns, NCP

Figure 7: Spec. Eff. vs SNR for PDSCH with 400MHz BW for 60GHz carrier frequency, delay spread 20ns, ECP

Figure 8: Spec. Eff. vs SNR for PDSCH with 400MHz BW for 60GHz carrier frequency, delay spread 50ns, ECP

Based on the results for spectral efficiency, following observations can be made:

Observation 5: For lower delay spread, low-mid range MCS and normal cyclic prefix, there is no significant performance difference in terms of spectral efficiency between different subcarrier spacing values

Observation 6: For lower delay spread, higher MCS and normal cyclic prefix, 960kHz subcarrier spacing performs the best and reaches the peak spectral efficiency at much lower SNR

Observation 7: For higher delay spread and normal cyclic prefix, 960kHz subcarrier spacing performs the worst

Observation 8: For higher delay spread and extended cyclic prefix, 960kHz subcarrier spacing performance is significantly improved compared to normal cyclic prefix and it performs slightly better than 480kHz for high MCS

# 3	Conclusion

Here we summarize the observations from the above section:

Observation 1: For lower delay spread, low-mid range MCS and normal cyclic prefix, there is no significant performance difference for target BLER between different subcarrier spacing values

Observation 2: For lower delay spread, higher MCS and normal cyclic prefix, 960kHz subcarrier spacing performs the best and with significant performance difference for target BLER of 1%, that might be needed for URLLC

Observation 3: For higher delay spread and normal cyclic prefix, 960kHz subcarrier spacing performs the worst and target BLER cannot be reached for high MCS

Observation 4: For higher delay spread and extended cyclic prefix, 960kHz subcarrier spacing performance is significantly improved compared to normal cyclic prefix

Observation 5: For lower delay spread, low-mid range MCS and normal cyclic prefix, there is no significant performance difference in terms of spectral efficiency between different subcarrier spacing values

Observation 6: For lower delay spread, higher MCS and normal cyclic prefix, 960kHz subcarrier spacing performs the best and reaches the peak spectral efficiency at much lower SNR

Observation 7: For higher delay spread and normal cyclic prefix, 960kHz subcarrier spacing performs the worst

Observation 8: For higher delay spread and extended cyclic prefix, 960kHz subcarrier spacing performance is significantly improved compared to normal cyclic prefix and it performs slightly better than 480kHz for high MCS

# 4	References

[1]	3GPP RP-193259, “New SID: Study on supporting NR from 52.6GHz to 71”, Intel

[2]	R1-2007558, “Discussion on physical layer impacts for NR beyond 52.6 GHz”, Lenovo, Motorola Mobility

# 5	Appendix

## 5.1 Link-level simulation parameters

| Parameters | Values |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Waveform | CP-OFDM |  |  |  |  |
| Carrier Freqeuncy | 60 GHz |  |  |  |  |
| Channel | PDSCH |  |  |  |  |
| Phase Noise Model | TR 38.803, 6.1.11, Example 2 (Tx:BS, Rx: UE for PDSCH) |  |  |  |  |
| Start smbol, Length | 1,12 |  |  |  |  |
| PTRS Config | K=4, L=1 |  |  |  |  |
| Channel | CDL-B (config. 2) (Mg,Ng,M,N,P) = (1,1,4,8,2) BS with (0.5 dv, 0.5 dH) (Mg,Ng,M,N,P) = (1,1,2,2,2) UE with (0.5 dv, 0.5 dH) |  |  |  |  |
| Mobility | 3 kmph |  |  |  |  |
| DMRS | 1-symbol front-loaded |  |  |  |  |
| MCS | 7,16,22 |  |  |  |  |
| Transmission | Rank 1 |  |  |  |  |
| Channel Estimation | Real |  |  |  |  |
| Noise Estimation | Ideal |  |  |  |  |
| PN Compensation | With CPE |  |  |  |  |
| Performance Metric | BLER vs SNR, Spectral Efficiency vs SNR |  |  |  |  |
| Delay Spread (ns) | 20,50 (CDL-B) |  |  |  |  |
| BW (MHz) | 400 MHz |  |  |  | 2000 MHz |
| Sampling Rate | 0.491520 GHz |  |  |  | 3.93216 GHz |
| SCS, # of RBs | 120 kHz,256 | 240 kHz, 128 | 480 kHz, 64 | 960 kHz, 32 | 960 kHz, 160 |
| FFT Size | 4096 | 2048 | 1024 | 512 | 4096 |
| Symbol length (w/o CP) | 8.3333 us | 4.1666 us | 2.0833 us | 1.0416 us | 1.0416 us |
| Normal Cyclic Prefix (l=0,7) / (l≠0,7) | 0.651 µs / 0.586 µs | 0.325 µs / 0.293 µs | 0.163 µs / 0.146 µs | 0.081 µs / 0.073 µs | 0.081 µs / 0.073 µs |
| Extended Cyclic Prefix | 2.08 µs | 1.04 µs | 0.52 µs | 0.26 µs | 0.26 µs |

## 5.2 Results in table template submitted for email discussion

Table B.1.1-1: LLS template: SINR in dB achieving PDSCH BLER of 10% /1%

| Tdoc / Source | MCS | Channel | 120KHz /400MHz | 240KHz /400MHz | 480KHz /400MHz | 960KHz /400MHz | 960KHz /2GHz |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Lenovo, Motorola Mobility | 7 | TDL-A, 5ns | 3/ 5.2 | 3.2/ 5.17 | 3.2/5.22 | 3.7/5.9 | 2.4/ 3.9 |
|  |  | TDL-A, 10ns | 2.6/ 4.5 | 2.7/ 4.7 | 2.7/ 4.7 | 3.5/5.5 | 2.4/ 3.9 |
|  |  | TDL-A, 20ns | 2.4/ 3.9 | 2.6/ 4.1 | 3.1/ 4.5 | 4/ 5.9 | 2.4/ 3.8 |
|  |  | CDL-B, 20ns | 4.9/5.9 | 5.2/6.1 | 6/6.8 | 5.2/6.2 | 5/6.2 |
|  |  | CDL-B, 50ns | 5.1/7 | 5.3/6.8 | 7/8.3 | 7.8/8.9 | 7.5/8.7 |
|  | 16 | TDL-A, 5ns | 11.2/ 14.4 | 11.2/ 14.2 | 10.9/ 13.5 | 11 /13.8 | 10.9/ 12.8 |
|  |  | TDL-A, 10ns | 10.8/ 13 | 10.8/ 13 | 10.5/ 12.5 | 11/13.7 | 10.9/ 12.9 |
|  |  | TDL-A, 20ns | 10.7/ 12.6 | 10.7/ 12.6 | 10.7/ 12.6 | 11.5/ 13.8 | 10.5/ 12.3 |
|  |  | CDL-B, 20ns | 13/14.5 | 12.3/12.9 | 13.1/13.8 | 12/12.7 | 12.3/13.2 |
|  |  | CDL-B, 50ns | 13.2/14.5 | 13.7/15.5 | 14.7/15.6 | 18/19.7 | 18.1/20.9 |
|  | 22 | TDL-A, 5ns | 27/ - | 20/ - | 18/33 | 17.8 / 26.8 | 24/ - |
|  |  | TDL-A, 10ns | -/ - | 25.2/ - | 19.1/ - | 18.5/ 33 | - /- |
|  |  | TDL-A, 20ns | -/ - | -/ - | 23.5/ - | 20/ - | -/- |
|  |  | CDL-B, 20ns | -/ - | -/ - | 21.2/25.8 | 17.4/18.4 | 19.3/21.8 |
|  |  | CDL-B, 50ns | -/ - | -/ - | -/ - | -/ - | -/ - |
|  |  | CDL-B, 50ns (ECP) |  |  | 21.4/26.1 | 19.1/21.4 | 21.4/25.3 |

## 5.2 Additional results with TDL-A

| 400 MHz bandwidth |  |
| --- | --- |
| BW; 400MHz. Channel; TDL-A. Ds; 20ns. PTRS; L=1, K=4. Ch. Est; Real. PN comp.; CPE. Speed; 3Km/h. CP; NCP. Ant; 2X2. | BW; 400MHz. Channel; TDL-A. Ds; 5ns. PTRS; L=1, K=4. Ch. Est; Ideal. PN comp.; CPE. Speed; 3Km/h. CP; NCP. Ant; 2X2. |
| BW; 400MHz. Channel; TDL-A. Ds; 10ns. PTRS; L=1, K=4. Ch. Est; Ideal. PN comp.; CPE. Speed; 3Km/h. CP; NCP. Ant; 2X2. | BW; 400MHz. Channel; TDL-A. Ds; 20ns. PTRS; L=1, K=4. Ch. Est; Ideal. PN comp.; CPE. Speed; 3Km/h. CP; NCP. Ant; 2X2. |
| BW; 400MHz. Channel; TDL-A. Ds; 5ns. PTRS; L=1, K=4. Ch. Est; Real. PN comp.; CPE. Speed; 3Km/h. CP; NCP. Ant; 2X2. | BW; 400MHz. Channel; TDL-A. Ds; 5ns. PTRS; L=1, K=4. Ch. Est; Ideal. PN comp.; CPE. Speed; 3Km/h. CP; NCP. Ant; 2X2. |
| BW; 400MHz. Channel; TDL-A. Ds; 10ns. PTRS; L=1, K=4. Ch. Est; Ideal. PN comp.; CPE. Speed; 3Km/h. CP; NCP. Ant; 2X2. | BW; 400MHz. Channel; TDL-A. Ds; 20ns. PTRS; L=1, K=4. Ch. Est; Ideal. PN comp.; CPE. Speed; 3Km/h. CP; NCP. Ant; 2X2. |
| BW; 400MHz. Channel; TDL-A. Ds; 5ns. PTRS; L=1, K=4. Ch. Est; Ideal. PN comp.; CPE. Speed; 3Km/h. CP; NCP. Ant; 2X2. | BW; 400MHz. Channel; TDL-A. Ds; 10ns. PTRS; L=1, K=4. Ch. Est; Ideal. PN comp.; CPE. Speed; 3Km/h. CP; NCP. Ant; 2X2. |
| BW; 400MHz. Channel; TDL-A. Ds; 10ns. PTRS; L=1, K=2. Ch. Est; Real. PN comp.; CPE. Speed; 3Km/h. CP; NCP. Ant; 2X2. | BW; 400MHz. Channel; TDL-A. Ds; 20ns. PTRS; L=1, K=2. Ch. Est; Real. PN comp.; CPE. Speed; 3Km/h. CP; NCP. Ant; 2X2. |
| BW; 400MHz. Channel; TDL-A. Ds; 10ns. PTRS; L=1, K=1. Ch. Est; Real. PN comp.; CPE. Speed; 3Km/h. CP; NCP. Ant; 2X2. | BW; 400MHz. Channel; TDL-A. Ds; 20ns. PTRS; L=1, K=4. Ch. Est; Real. PN comp.; CPE. Speed; 3Km/h. CP; ECP. Ant; 2X2. |
| BW; 400MHz. Channel; TDL-A. Ds; 40ns. PTRS; L=1, K=4. Ch. Est; Real. PN comp.; CPE. Speed; 3Km/h. CP; NCP. Ant; 2X2. | BW; 400MHz. Channel; TDL-A. Ds; 40ns. PTRS; L=1, K=4. Ch. Est; Real. PN comp.; CPE. Speed; 3Km/h. CP; ECP. Ant; 2X2. |
| 2GHz bandwidth |  |
| BW; 2GHz. Channel; TDL-A. Ds; 5ns. PTRS; L=1, K=2. Ch. Est; Real. PN comp.; CPE. Speed; 3Km/h. CP; NCP. Ant; 2X2. | BW; 2GHz. Channel; TDL-A. Ds; 10ns. PTRS; L=1, K=2. Ch. Est; Real. PN comp.; CPE. Speed; 3Km/h. CP; NCP. Ant; 2X2. |
| BW; 2GHz. Channel; TDL-A. Ds; 20ns. PTRS; L=1, K=2. Ch. Est; Real. PN comp.; CPE. Speed; 3Km/h. CP; NCP. Ant; 2X2. | BW; 2GHz. Channel; TDL-A. Ds; 5ns. PTRS; L=1, K=1. Ch. Est; Real. PN comp.; CPE. Speed; 3Km/h. CP; NCP. Ant; 2X2. |
| BW; 2GHz. Channel; TDL-A. Ds; 10ns. PTRS; L=1, K=1. Ch. Est; Real. PN comp.; CPE. Speed; 3Km/h. CP; NCP. Ant; 2X2. | BW; 2GHz. Channel; TDL-A. Ds; 20ns. PTRS; L=1, K=1. Ch. Est; Real. PN comp.; CPE. Speed; 3Km/h. CP; NCP. Ant; 2X2. |