3GPP TSG RAN WG1 Meeting #103-e	R1-2007573

E-meeting, October 26 – November 13, 2020

Agenda Item:	8.15.2

Source:	Huawei, HiSilicon

Title:	Solutions to support IoT in NTN

Document for:	Discussion and Decision

# Introduction

The study on NB-IoT/eMTC support for NTN was approved with the following RAN1 objectives [1]:

RAN1

The second objective is, for the above identified scenarios, to study and recommend necessary changes to support NB-IoT and eMTC over satellite, reusing as much as possible the conclusions of the studies performed for NR NTN in TR38.821. This objective will address the following items:

-	Aspects related to random access procedure/signals [RAN1, RAN2]

-	Mechanisms for time/frequency adjustment including Timing Advance, and UL frequency compensation indication [RAN1, RAN2]

-	Timing offset related to scheduling and HARQ-ACK feedback [RAN1, RAN2]

- Aspects related to HARQ operation [RAN2, RAN1]

In this contribution, we discuss the timing relationship enhancements, time/frequency synchronization, and HARQ operation for IoT over NTN.

# Discussion

## Timing relationship enhancements

The propagation delay in NTN can be from several milliseconds to hundreds of milliseconds, which is much larger than that in terrestrial networks [2]. Similar to NR NTN UE, an IoT device in case of NTN deployment also needs to apply a large TA value, which leads to a large offset in its DL and UL frame timing. Therefore, some timing relationship cases need to be enhanced. For example, transmission timing for NPUSCH scheduled by UL grant, transmission timing for HARQ-ACK on NPUSCH and so on.

Fortunately, the Koffset introduced in NR NTN can be reused in IoT NTN to enhance the timing relationships involving DL-UL timing interactions. As shown in [2] and [3], the signalling details of Koffset are discussed. When signalling details are determined in NR NTN, IoT NTN can also reuse or revise them if needed.

Observation 1: The Koffset introduced in NR NTN can be reused in IoT NTN to enhance the timing relationships.

Proposal 1: Reuse the Koffset introduced in NR NTN to enhance the timing relationships in IoT NTN.

An NB-IoT UE works in half duplex mode, which means that the UE cannot receive DL signals when transmitting UL signals. In NTN scenarios, the differential RTD, i.e., differential TA, in one cell is large, the maximum differential delay within a cell could be up to 10.3 ms [2]. The above-mentioned large TA and differential TA may introduce collisions of simultaneous UL transmission and DL reception at the UE side, which go against the half duplex mode.

Observation 2: The large TA and large differential TA in a cell may introduce collisions of simultaneous UL transmission and DL reception for a half-duplex UE.

Proposal 2: Study solutions for the possible collision between UL transmission and DL reception for half- duplex UE caused by the large TA and large differential TA.

## UL time and frequency synchronization enhancement

In IoT NTN, UE with GNSS capability is assumed [1]. Through detecting DL reference signals, UE position and satellite position, UE can acquire the UL frequency/timing offset of service link. Therefore, both the estimation and pre-compensation of UE-specific UL frequency/timing offset of service link can be conducted at the UE side. While, the frequency offset of feeder link can be compensated by gNB. As a result, UL frequency correction indication is not needed. The timing offset of feeder link can be compensated fully or partly by gNB, and the indicated common TA is needed for UE [3]. Overall, the UL time and frequency synchronization enhancement in IoT NTN can reuse relevant design and conclusions of NR NTN.

Observation 3: The UL time and frequency synchronization enhancement of NR NTN can be applied to IoT NTN.

Proposal 3: Reuse the UL time and frequency synchronization enhancement design and conclusions of NR NTN in IoT NTN.

As mentioned above, an NB-IoT UE works in half duplex mode. It is noted that NB-IoT can keep transmitting UL signals more than 2 s, because there are repetitions. Even though there is 40 ms UL gap duration after 256 ms UL transmission for receiving DL reference signals. This UL gap may not be sufficient to update UL timing adjustment.

Proposal 4: Study the effects of long UL transmission duration on UL time synchronization.

## HARQ enhancement

In NB-IoT, the UE supports only two HARQ processes at most. Extension of HARQ process number may not be needed given that the delay and data rate are not that critical for IoT applications. Meanwhile, in NR NTN, the discussion on indication of HARQ process, enabling/disabling of HARQ feedback and HARQ-ACK codebook enhancement are ongoing. IoT NTN can discuss aspects related to HARQ until NR NTN has made sufficient progress.

Observation 4: The HARQ aspects for IoT NTN can wait until NR NTN has made sufficient progress if needed.

# Conclusion

In this contribution, we discuss the timing relationship enhancements, the time/frequency adjustment, and HARQ operation in IoT NTN. The following observations and proposals are presented:

Observation 1: The Koffset introduced in NR NTN can be reused in IoT NTN to enhance the timing relationships.

Observation 2: The large TA and large differential TA in a cell may introduce collisions of simultaneous UL transmission and DL reception for a half-duplex UE.

Observation 3: The UL time and frequency synchronization enhancement of NR NTN can be applied to IoT NTN.

Observation 4: The HARQ aspects for IoT NTN can wait until NR NTN has made sufficient progress if needed.

Proposal 1: Reuse the Koffset introduced in NR NTN to enhance the timing relationships in IoT NTN.

Proposal 2: Study solutions for the possible collision between UL transmission and DL reception for half- duplex UE caused by the large TA and large differential TA.

Proposal 3: Reuse the UL time and frequency synchronization enhancement design and conclusions of NR NTN in IoT NTN.

Proposal 4: Study the effects of long UL transmission duration on UL time synchronization.

# References

RP-193235, “Study on NB-Io/eMTC support for Non-Terrestrial Network”, MediaTek Inc.

TR 38.821, “Solutions on NR to support non-terrestrial networks”.

RAN1#102-e Chairman's Notes