3GPP TSG RAN WG1 Meeting #103-e                                                                                  R1-2007606

E-meeting, October 26th – November 13th, 2020

Agenda Item:	7.2.2

Source:	Huawei, HiSilicon

Title:	Maintenance on initial access signals and channels

Document for:	Discussion and Decision

# Introduction

In this paper, we discuss the remaining issues after RAN1#102-e [1] on the MsgA PUSCH mapping when multiple FDMed ROs are across multiple RB sets.

# On the FDMed RO

Agreement:

For connected mode UE, at least for PRACH sequence length 1151 and 571, support configuring multiple FDMed ROs in active UL BWP over multiple RB sets, where each RO will be confined within one RB set.

- Legacy msg1-FrequencyStart or msgA-RO-FrequencyStart-r16 indicates the position of the first RO as in Rel-15, and thus implicitly indicates the first lowest-indexed RB set configured with a RO in the UL BWP. The RB offset between the lowest CRB of the lowest-indexed RB set configured with an RO and the lowest RB of first RO is applied to higher indexed RB sets in the UL BWP for configuring the positions of the remaining ROs

  - ROs are mapped to contiguous set of RB-sets, one RO per RB-set

Note: the number of FDM’d ROs is provided by msg1-FDM or msgA-RO-FDM-r16.

In RAN1#102-e, RAN1 agreed to adjust the resource mapping of FDMed RO for PRACH sequence length of 1151 and 571 ensuring each RO confined with RB set. The corresponding TP was captured in TS38.211.

During the discussion, there is no consensus on whether to support multiple FDMed ROs in active UL BWP over multiple RB set for short PRACH sequence of 139.

In NR Rel-15, At most 8 FDMed ROs can be configured in frequency domain. It corresponds to 34.5MHz BW assuming 30kHz SCS, which is larger than a RB set. If FDMed ROs across RB set is not allowed in NR-U, there are at most 4 FDMed ROs available. Considering the size of default intra cell guard is 5 or 6 PRB which might impact at most two consecutive ROs in frequency domain, same RO mapping in frequency domain in Rel-15 can be reused and the RO(s) overlap with intra cell guard band should not be used.

Proposal 1: For connected mode UE, for PRACH sequence length of 139, support configuring multiple FDMed ROs in active UL BWP over multiple RB sets. The RO(s) overlapping with the UL intra cell guard should not be used. The corresponding TP#1 is provided in Appendix.

# On MsgA PUSCH

There was discussion in RAN1#102-e [1] on the frequency domain allocation for MsgA PUSCH when there are multiple RB set in an active UL BWP. According to the current specification TS38.213 as copied below, the FDRA of the first PO in the UL BWP (interlaceIndexFirstPO-MsgA-PUSCH/ nrofInterlacesPerMsgA-PO or frequencyStartMsgA-PUSCH/nrofPRBs-perMsgA-PO) and a number of consecutive FDMed PO (nrMsgA-PO-FDM) are configured to UE. At most 1 RB can be configured

## 8.1A	PUSCH for Type-2 random access procedure

….

A UE determines a first interlace or first RB for a first PUSCH occasion in an active UL BWP respectively from interlaceIndexFirstPO-MsgA-PUSCH or from frequencyStartMsgA-PUSCH that provides an offset, in number of RBs in the active UL BWP, from a first RB of the active UL BWP. A PUSCH occasion includes a number of interlaces or a number of RBs provided by nrofInterlacesPerMsgA-PO or by nrofPRBs-perMsgA-PO, respectively. Consecutive PUSCH occasions in the frequency domain of an UL BWP are separated by a number of RBs provided by guardBandMsgA-PUSCH. A number Nf of PUSCH occasions in the frequency domain of an UL BWP is provided by nrMsgA-PO-FDM.

….

Each consecutive number of Npreamble preamble indexes from valid PRACH occasions in a PRACH slot

-	first, in increasing order of preamble indexes within a single PRACH occasion

-	second, in increasing order of frequency resource indexes for frequency multiplexed PRACH occasions

-	third, in increasing order of time resource indexes for time multiplexed PRACH occasions within a PRACH slot

are mapped to a valid PUSCH occasion and the associated DMRS resource

-	first, in increasing order of frequency resource indexes fid for frequency multiplexed PUSCH occasions

-	second, in increasing order of DMRS resource indexes within a PUSCH occasion, where a DMRS resource index DMRSid is determined first in an ascending order of a DMRS port index and second in an ascending order of a DMRS sequence index [4, TS 38.211]

-	third, in increasing order of time resource indexes tid for time multiplexed PUSCH occasions within a PUSCH slot

-	fourth, in increasing order of indexes for Ns PUSCH slots

…

between 2 consecutive PO in frequency domain by guardBandMsgA-PUSCH.

For interlaced PUSCH, it is not clear whether an MsgA PUSCH is mapped to all PRB of an interlace across all RB sets of an UL BWP or confined within an RB set. We prefer the latter because it provides higher channel access probability and larger coverage considering the PSD limitation. The UE can assume the PO locating in the same RB set as its associated RO because there is no need to introduce additional RRC parameter in such as late stage of maintenance phase. It also follows the same design for Msg1 and Msg3 in 4 step RACH. The RO to PO mapping sequence defined in 2 step RACH WI can be reused for UE to determine the index of interlace on which MsgA PUSCH should map.

For non-interlaced PUSCH, the PO might be overlapped with the UL intra cell guard bands when the total bandwidth of all FDMed POs are larger than a RB set. Moreover, the PO might be in a different RB set from its associated RO according to current RO to PO mapping sequence. Similar as interlaced MsgA PUSCH, it is also beneficial to confine the PO in the same RB set where the associated RO locates. Thus, the RO to PO mapping sequence should be restricted with the same RB set.

Proposal 2: For both interlaced and non-interlaced PUSCH, the MsgA PUSCH should be confined within the same RB set as its associated RO.

Proposal 3: For non-interlaced PUSCH, the RO to PO mapping sequence should be restricted within the RB set.

The corresponding TP#2 is provided in Appendix.

# Conclusions

In the paper, we discussed FDMed RO and PO mapping for MsgA PUSCH across multiple RB sets. We have the following proposals.

Proposal 1: For connected mode UE, for PRACH sequence length of 139, support configuring multiple FDMed ROs in active UL BWP over multiple RB sets. The RO(s) overlapping with the UL intra cell guard should not be used. The corresponding TP#1 is provided in Appendix.

Proposal 2: For both interlaced and non-interlaced PUSCH, the MsgA PUSCH should be confined within the same RB set as its associated RO.

Proposal 3: For non-interlaced PUSCH, the RO to PO mapping sequence should be restricted within the RB set.

# References

R1-2007192, “FL summary for initial access signals and channels for NR-U”, Qualcomm

# Appendix

## TP#1: TS38.213

8.1	Random access preamble

***Unchanged text omitted***

For unpaired spectrum,

-	if a UE is not provided tdd-UL-DL-ConfigurationCommon, a PRACH occasion in a PRACH slot is valid if it does not precede a SS/PBCH block in the PRACH slot and starts at least Ngap symbols after a last SS/PBCH block reception symbol, where Ngap is provided in Table 8.1-2 and, if ChannelAccessMode-r16 = semistatic is provided, does not overlap with a set of consecutive symbols before the start of a next channel occupancy time where the UE does not transmit [15, TS 37.213].

-	the candidate SS/PBCH block index of the SS/PBCH block corresponds to the SS/PBCH block index provided by ssb-PositionsInBurst in SIB1 or in ServingCellConfigCommon , as described in Clause 4.1

-	If a UE is provided tdd-UL-DL-ConfigurationCommon, a PRACH occasion in a PRACH slot is valid if

-	it is within UL symbols, or

-	it does not precede a SS/PBCH block in the PRACH slot and starts at least Ngap symbols after a last downlink symbol and at least Ngap symbols after a last SS/PBCH block symbol, where Ngap is provided in Table 8.1-2, and if ChannelAccessMode-r16 = semistatic is provided, does not overlap with a set of consecutive symbols before the start of a next channel occupancy time where there shall not be any transmissions, as described in [15, TS 37.213]

-	the candidate SS/PBCH block index of the SS/PBCH block corresponds to the SS/PBCH block index provided by ssb-PositionsInBurst in SIB1 or in ServingCellConfigCommon, as described in Clause 4.1.

-	For operation with shared spectrum channel access, if a UE is configured with intraCellGuardBandUL-r16 or determines the nominal intra-cell guard band and RB set pattern as specified in [8, TS 38.101-1], a PRACH occasion with PRACH sequence length of 139 in a PRACH slot is valid if the PRACH occasion is not overlapped with the intra-cell guard band.

***Unchanged text omitted***

## TP#2: TS38.213

8.1A	PUSCH for Type-2 random access procedure

***Unchanged text omitted***

A UE determines a first interlace or first RB for a first PUSCH occasion in an active UL BWP respectively from interlaceIndexFirstPO-MsgA-PUSCH or from frequencyStartMsgA-PUSCH that provides an offset, in number of RBs in the active UL BWP, from a first RB of the active UL BWP. A PUSCH occasion includes a number of interlaces or a number of RBs provided by nrofInterlacesPerMsgA-PO or by nrofPRBs-perMsgA-PO, respectively. Consecutive PUSCH occasions in the frequency domain of an UL BWP are separated by a number of RBs provided by guardBandMsgA-PUSCH. A number Nf of PUSCH occasions in the frequency domain of an UL BWP is provided by nrMsgA-PO-FDM. If a UE is configured with intraCellGuardBandUL-r16 or determines the nominal intra-cell guard band and RB set pattern as specified in [8, TS 38.101-1], the PUSCH occasion in an active UL BWP is confined in the RB set same as its associated PRACH occasion.

***Unchanged text omitted***

Each consecutive number of Npreamble preamble indexes from valid PRACH occasions in a PRACH slot and in a RB set if configured

-	first, in increasing order of preamble indexes within a single PRACH occasion

-	second, in increasing order of frequency resource indexes for frequency multiplexed PRACH occasions

-	third, in increasing order of time resource indexes for time multiplexed PRACH occasions within a PRACH slot

are mapped to a valid PUSCH occasion and the associated DMRS resource in the same RB set if configured

-	first, in increasing order of frequency resource indexes fid for frequency multiplexed PUSCH occasions

-	second, in increasing order of DMRS resource indexes within a PUSCH occasion, where a DMRS resource index DMRSid is determined first in an ascending order of a DMRS port index and second in an ascending order of a DMRS sequence index [4, TS 38.211]

-	third, in increasing order of time resource indexes tid for time multiplexed PUSCH occasions within a PUSCH slot

-	fourth, in increasing order of indexes for Ns PUSCH slots

where Npreamble=ceilTpreambleTPUSCH, Tpreamble is a total number of valid PRACH occasions in a RB set if configured per association pattern period multiplied by the number of preambles per valid PRACH occasion provided by msgA-PUSCH-PreambleGroup, and TPUSCH is a total number of valid PUSCH occasions in the same RB set if configured per PUSCH configuration per association pattern period multiplied by the number of DMRS resource indexes per valid PUSCH occasion provided by msgA-DMRS-Configuration.

A PUSCH occasion is valid if it does not overlap in time and frequency with any PRACH occasion associated with either a Type-1 random access procedure or a Type-2 random access procedure. Additionally, for unpaired spectrum and for SS/PBCH blocks with indexes provided by ssb-PositionsInBurst in SIB1 or by ServingCellConfigCommon

-	if a UE is not provided tdd-UL-DL-ConfigurationCommon, a PUSCH occasion is valid if the PUSCH occasion

-	does not precede a SS/PBCH block in the PUSCH slot, and

-	starts at least Ngap symbols after a last SS/PBCH block symbol, where Ngap is provided in Table 8.1-2

-	if a UE is provided tdd-UL-DL-ConfigurationCommon, a PUSCH occasion is valid if the PUSCH occasion

-	is within UL symbols, or

-	does not precede a SS/PBCH block in the PUSCH slot, and

-	starts at least Ngap symbols after a last downlink symbol and at least Ngap symbols after a last SS/PBCH block symbol, where Ngap is provided in Table 8.1-2 and, if ChannelAccessMode-r16 = semistatic is provided, does not overlap with a set of consecutive symbols before the start of a next channel occupancy time where the UE does not transmit [15, TS 37.213].

-	For operation with shared spectrum channel access, if a UE is configured with intraCellGuardBandUL-r16 or determines the nominal intra-cell guard band and RB set pattern as specified in [8, TS 38.101-1], a PUSCH occasion in a PUSCH slot is valid if the PUSCH occasion is not overlapped with the intra-cell guard band.