3GPP TSG RAN WG1 #103-e		R1-2007665

e-Meeting, October 26th –November 13th, 2020

Source:	vivo

Title:	Evaluation of NR positioning performance

Agenda Item:	8.5.2

Document for:	Discussion & Decision

Introduction

In RAN1 #102-e, some conclusions and agreements have been reached in [1]. In this contribution, based on the output of last meeting, some evaluation results of achievable positioning accuracy, latency, network and UE efficiency are presented and discussed.

Accuracy evaluation

2.1 Performance analysis of Rel-16 positioning solutions

### 2.1.1 Horizontal accuracy evaluation

In this section, we provide our horizontal accuracy evaluation results for downlink, uplink, downlink and uplink evaluations for Rel-16 positioning solutions. The simulation results are listed in the following tables. The detailed parameter configuration and CDF figures are provided in Appendix A and Appendix B. It is noted the horizontal accuracy evaluation results in this section are only for R16 techniques without outlier determination/rejection techniques (i.e., without RAIM).

Downlink only evaluation

The evaluation results with Rel.16 downlink only techniques are summarized in Table 1.

In our companion contribution [2], the LOS probability of different DH clutter parameters has been analyzed. The clutter parameter {60%, 6m, 2m} is an important part in evaluation to assess the positioning performance when the environment is not so good due to high NLOS probability. It is observed that when the NLOS probability is high, there are almost no LOS links for most UEs, thus the positioning accuracy can hardly be met. It may be beneficial to identify the performance gap in this case.

Table 1 Rel.16 NR positioning - horizontal accuracy results (m) with downlink only techniques

|  |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| [Case 1], [SH, perfect sync], [FR1], [DL-TDOA, MUSIC, select based on RSRP] | Convex UEs | 0.041 | 0.055 | 0.099 | 4.15 |
|  | (Optional) All UEs | 0.059 | 0.17 | 1.40 | 6.73 |
| [Case 2], [SH, sync error 50ns], [FR1], [DL-TDOA, MUSIC, select based on RSRP] | Convex UEs | 11.49 | 14.48 | 19.44 | 24.06 |
|  | (Optional) All UEs | 13.41 | 18.91 | 25.07 | 35.48 |
| [Case 3], [SH, perfect sync], [FR2], [DL-TDOA, MUSIC, select based on RSRP] | Convex UEs | 0.010 | 0.013 | 0.021 | 2.97 |
|  | (Optional) All UEs | 0.014 | 0.029 | 0.35 | 4.82 |
| [Case 4], [SH, sync error 50ns], [FR2], [DL-TDOA, MUSIC, select based on RSRP] | Convex UEs | 11.67 | 13.44 | 17.49 | 23.21 |
|  | (Optional) All UEs | 13.45 | 17.71 | 23.46 | 33.34 |
| [Case 5], [DH, perfect sync], [FR1], [DL-TDOA, MUSIC, select based on RSRP] | Convex UEs | 0.056 | 0.33 | 2.97 | 5.92 |
|  | (Optional) All UEs | 0.10 | 1.23 | 4.60 | 9.80 |
| [Case 6], [DH, sync error 50ns], [FR1], [DL-TDOA, MUSIC, select based on RSRP] | Convex UEs | 9.95 | 13.09 | 16.43 | 23.79 |
|  | (Optional) All UEs | 12.65 | 16.29 | 20.59 | 30.46 |
| [Case 7], [DH, perfect sync], [FR2], [DL-TDOA, MUSIC, select based on RSRP] | Convex UEs | 0.012 | 0.23 | 2.72 | 5.77 |
|  | (Optional) All UEs | 0.023 | 1.06 | 4.08 | 9.67 |
| [Case 8], [DH, sync error 50ns], [FR2], [DL-TDOA, MUSIC, select based on RSRP] | Convex UEs | 9.39 | 12.89 | 15.05 | 22.90 |
|  | (Optional) All UEs | 11.79 | 14.79 | 17.49 | 22.93 |
| [Case 9], [DH {0.6,6,2}, perfect sync], [FR1], [DL-TDOA, MUSIC, select based on RSRP] | Convex UEs | 6.13 | 7.76 | 13.38 | 18.71 |
|  | (Optional) All UEs | 6.49 | 10.35 | 15.33 | 24.59 |
| [Case 10], [DH {0.6,6,2}, perfect sync], [FR2], [DL-TDOA, MUSIC, select based on RSRP] | Convex UEs | 6.63 | 9.01 | 13.6 | 15.09 |
|  | (Optional) All UEs | 8.04 | 12.84 | 15.23 | 23.12 |
| [Case 11], [SH, perfect sync], [FR1], [DL-TDOA, MUSIC, select based on first/median peak] | Convex UEs | 0.038 | 0.049 | 0.069 | 0.094 |
|  | (Optional) All UEs | 0.052 | 0.080 | 0.15 | 0.30 |
| [Case 12], [SH, sync error 50ns], [FR1], [DL-TDOA, MUSIC, select based on first/median peak] | Convex UEs | 11.20 | 14.93 | 18.32 | 26.09 |
|  | (Optional) All UEs | 13.82 | 18.12 | 23.77 | 36.35 |
| [Case 13], [SH, perfect sync], [FR2], [DL-TDOA, MUSIC, select based on first/median peak] | Convex UEs | 0.011 | 0.013 | 0.017 | 0.031 |
|  | (Optional) All UEs | 0.013 | 0.020 | 0.046 | 0.11 |
| [Case 14], [SH, sync error 50ns], [FR2], [DL-TDOA, MUSIC, select based on first/median peak] | Convex UEs | 10.12 | 13.01 | 17.49 | 25.67 |
|  | (Optional) All UEs | 12.27 | 17.54 | 23.28 | 32.26 |
| [Case 15], [DH, perfect sync], [FR1], [DL-TDOA, MUSIC, select based on first/median peak] | Convex UEs | 0.062 | 0.079 | 0.11 | 0.60 |
|  | (Optional) All UEs | 0.093 | 0.15 | 0.25 | 3.23 |
| [Case 16], [DH, sync error 50ns], [FR1], [DL-TDOA, MUSIC, select based on first/median peak] | Convex UEs | 10.14 | 13.00 | 14.93 | 20.30 |
|  | (Optional) All UEs | 12.24 | 15.42 | 20.30 | 31.59 |
| [Case 17], [DH, perfect sync], [FR2], [DL-TDOA, MUSIC, select based on first/median peak] | Convex UEs | 0.011 | 0.015 | 0.018 | 0.049 |
|  | (Optional) All UEs | 0.014 | 0.023 | 0.041 | 0.60 |
| [Case 18], [DH, sync error 50ns], [FR2], [DL-TDOA, MUSIC, select based on first/median peak] | Convex UEs | 9.71 | 12.14 | 15.19 | 20.16 |
|  | (Optional) All UEs | 11.36 | 14.65 | 17.97 | 25.55 |

Observation 1

- For Rel.16 DL-TDOA positioning, the performance target [0.2m 90%] can be achieved in InF-SH with perfect synchronization for convex UEs, and in InF-SH with perfect synchronization for all UEs for FR2, but cannot be achieved in other cases in InF-SH.

Observation 2

- For Rel.16 DL-TDOA positioning, the performance target [0.2m 90%] can be achieved in InF-DH with perfect synchronization for convex UEs for FR2, but cannot be achieved in other cases in InF-DH.

Observation 3

- The positioning performance of DL-TDOA degraded significantly with 50ns synchronization error.

Observation 4

- For DL-TDOA positioning with the DH clutter parameter {0.6,6,2}, the performance target [0.2m 90%] cannot be achieved and the performance gap is nearly 18 m.

Uplink only evaluation

The evaluation results with Rel.16 uplink only techniques are summarized in Table 2. The evaluated techniques are UL-TDOA, UL-AOAand UL-TDOA+AOA.

The evaluation results show that the performance degradation caused by synchronization error for UL-AOA and UL-TDOA+AOA is not significant. In other words, UL-AOA and UL-TDOA+AOA can mitigate the positioning error caused by synchronization error effectively.

Besides this, compared to UL-AOA technique, UL-TDOA+AOA can improve the accuracy to some extent. The reason is that when the first path is detected by UL-TDOA+AOA, UL-TDOA+AOA can distinguish the angle information of the first path, while UL-AOA itself can only detect the angle information of multipaths.

Table 2 Rel.16 NR positioning - horizontal accuracy results (m) with uplink only techniques

|  |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| [Case 19], [SH, perfect sync], [FR1], [UL-TDOA, MUSIC, select based on RSRP] | Convex UEs | 0.041 | 0.051 | 0.087 | 4.22 |
|  | (Optional) All UEs | 0.057 | 0.13 | 1.40 | 6.68 |
| [Case 20], [SH, sync error 50ns], [FR1], [UL-TDOA, MUSIC, select based on RSRP] | Convex UEs | 11.50 | 14.77 | 19.49 | 24.51 |
|  | (Optional) All UEs | 13.81 | 19.44 | 26.32 | 35.51 |
| [Case 21], [SH, perfect sync], [FR2], [UL-TDOA, MUSIC, select based on RSRP] | Convex UEs | 0.010 | 0.013 | 0.033 | 4.07 |
|  | (Optional) All UEs | 0.015 | 0.036 | 0.54 | 5.61 |
| [Case 22], [SH, sync error 50ns], [FR2], [UL-TDOA, MUSIC, select based on RSRP] | Convex UEs | 11.70 | 13.82 | 18.11 | 23.21 |
|  | (Optional) All UEs | 13.65 | 18.95 | 23.67 | 33.34 |
| [Case 23], [DH, perfect sync], [FR1], [UL-TDOA, MUSIC, select based on RSRP] | Convex UEs | 0.060 | 0.19 | 2.77 | 5.85 |
|  | (Optional) All UEs | 0.10 | 1.04 | 4.48 | 9.14 |
| [Case 24], [DH, sync error 50ns], [FR1], [UL-TDOA, MUSIC, select based on RSRP] | Convex UEs | 9.92 | 13.47 | 16.02 | 22.90 |
|  | (Optional) All UEs | 12.31 | 15.40 | 19.04 | 23.85 |
| [Case 25], [DH, perfect sync], [FR2], [UL-TDOA, MUSIC, select based on RSRP] | Convex UEs | 0.013 | 0.35 | 2.85 | 5.76 |
|  | (Optional) All UEs | 0.025 | 1.09 | 4.60 | 9.95 |
| [Case 26], [DH, sync error 50ns], [FR2], [UL-TDOA, MUSIC, select based on RSRP] | Convex UEs | 10.52 | 13.46 | 16.09 | 18.92 |
|  | (Optional) All UEs | 12.22 | 15.26 | 18.72 | 23.64 |
| [Case 27], [SH, perfect sync], [FR1], [UL-TDOA, MUSIC, select based on first/median peak] | Convex UEs | 0.038 | 0.055 | 0.070 | 0.087 |
|  | (Optional) All UEs | 0.055 | 0.078 | 0.13 | 0.29 |
| [Case 28], [SH, sync error 50ns], [FR1], [UL-TDOA, MUSIC, select based on first/median peak] | Convex UEs | 11.23 | 15.14 | 20.06 | 27.70 |
|  | (Optional) All UEs | 13.95 | 18.53 | 24.10 | 36.35 |
| [Case 29], [SH, perfect sync], [FR2], [UL-TDOA, MUSIC, select based on first/median peak] | Convex UEs | 0.011 | 0.013 | 0.017 | 0.032 |
|  | (Optional) All UEs | 0.013 | 0.020 | 0.038 | 0.11 |
| [Case 30], [SH, sync error 50ns], [FR2], [UL-TDOA, MUSIC, select based on first/median peak] | Convex UEs | 11.11 | 14.30 | 18.27 | 25.67 |
|  | (Optional) All UEs | 13.38 | 18.28 | 23.63 | 32.51 |
| [Case 31], [DH, perfect sync], [FR1], [UL-TDOA, MUSIC, select based on first/median peak] | Convex UEs | 0.058 | 0.075 | 0.11 | 0.60 |
|  | (Optional) All UEs | 0.088 | 0.16 | 0.26 | 2.83 |
| [Case 32], [DH, sync error 50ns], [FR1], [UL-TDOA, MUSIC, select based on first/median peak] | Convex UEs | 10.14 | 13.35 | 16.32 | 22.01 |
|  | (Optional) All UEs | 12.14 | 15.29 | 18.76 | 27.05 |
| [Case 33], [DH, perfect sync], [FR2], [UL-TDOA, MUSIC, select based on first/median peak] | Convex UEs | 0.010 | 0.014 | 0.019 | 0.051 |
|  | (Optional) All UEs | 0.015 | 0.023 | 0.052 | 0.89 |
| [Case 34], [DH, sync error 50ns], [FR2], [UL-TDOA, MUSIC, select based on first/median peak] | Convex UEs | 9.93 | 13.20 | 16.64 | 19.74 |
|  | (Optional) All UEs | 12.15 | 15.34 | 18.35 | 24.03 |
| [Case 35], [SH, perfect sync], [FR1], [UL-AOA] | Convex UEs | 1.25 | 2.15 | 3.28 | 5.93 |
|  | (Optional) All UEs | 1.35 | 2.21 | 3.38 | 6.08 |
| [Case 36], [SH, sync error 50ns], [FR1], [UL-AOA] | Convex UEs | 1.31 | 2.24 | 3.75 | 6.20 |
|  | (Optional) All UEs | 1.43 | 2.41 | 4.02 | 6.47 |
| [Case 37], [DH, perfect sync], [FR1], [UL-AOA] | Convex UEs | 2.28 | 3.39 | 4.29 | 5.48 |
|  | (Optional) All UEs | 2.45 | 3.63 | 4.70 | 6.86 |
| [Case 38], [DH, sync error 50ns], [FR1], [UL-AOA] | Convex UEs | 2.45 | 3.56 | 4.45 | 5.76 |
|  | (Optional) All UEs | 2.63 | 3.68 | 4.75 | 7.06 |
| [Case 39], [SH, perfect sync], [FR1], [UL-TDOA+UL-AOA] | Convex UEs | 0.067 | 0.11 | 0.21 | 0.41 |
|  | (Optional) All UEs | 0.078 | 0.14 | 0.25 | 0.41 |
| [Case 40], [SH, sync error 50ns], [FR1], [UL-TDOA+UL-AOA] | Convex UEs | 0.098 | 0.17 | 0.26 | 0.43 |
|  | (Optional) All UEs | 0.11 | 0.21 | 0.33 | 0.63 |
| [Case 41], [DH, perfect sync], [FR1], [UL-TDOA+UL-AOA] | Convex UEs | 0.047 | 0.094 | 0.18 | 0.68 |
|  | (Optional) All UEs | 0.055 | 0.11 | 0.41 | 4.12 |
| [Case 42], [DH, sync error 50ns], [FR1], [UL-TDOA+UL-AOA] | Convex UEs | 0.059 | 0.10 | 0.26 | 0.77 |
|  | (Optional) All UEs | 0.083 | 0.17 | 0.44 | 4.23 |

Observation 5

- For Rel.16 UL-TDOA positioning, the performance target [0.2m 90%] can be achieved in InF-SH with perfect synchronization for convex UEs, and in InF-SH with perfect synchronization for all UEs for FR2, but cannot be achieved in other cases in InF-SH.

Observation 6

- For Rel.16 UL-TDOA positioning, the performance target [0.2m 90%] can be achieved in InF-DH with perfect synchronization for convex UEs for FR2, but cannot be achieved in other cases in InF-DH.

Observation 7

- For Rel.16 UL-AOA positioning, the performance target [0.2m 90%] cannot be achieved in InF-SH and InF-DH.

Observation 8

- Compared to DL-TDOA or UL-TDOA, sync error has little effect on positioning performance on UL-AOA.

Observation 9

- For Rel.16 UL-TDOA+AOA positioning, the performance target [0.2m 90%] cannot be achieved in InF-SH and InF-DH.

Observation 10

- Compared to UL-AOA, UL-TDOA+AOA can improve the positioning accuracy to some extent.

Downlink and uplink evaluation

The evaluation results with Rel.16 downlink and uplink techniques are summarized in Table 3. The evaluated techniques are Multi-RTT. The evaluation results show that the performance degradation caused by synchronization error for Multi-RTT is not significant. In other words, Multi-RTT can mitigate the positioning error caused by synchronization error effectively.

Table 3 Rel.16 NR positioning - horizontal accuracy results (m) with downlink & uplink techniques

|  |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| [Case 43], [SH, perfect sync], [FR1], [Multi-RTT, MUSIC, select based on RSRP] | Convex UEs | 0.039 | 0.050 | 0.095 | 4.25 |
|  | (Optional) All UEs | 0.054 | 0.12 | 1.34 | 6.70 |
| [Case 44], [SH, sync error 50ns], [FR1], [Multi-RTT, MUSIC, select based on RSRP] | Convex UEs | 0.042 | 0.055 | 0.11 | 4.71 |
|  | (Optional) All UEs | 0.060 | 0.14 | 1.49 | 6.72 |
| [Case 45], [SH, perfect sync], [FR2], [Multi-RTT, MUSIC, select based on RSRP] | Convex UEs | 0.010 | 0.011 | 0.033 | 3.96 |
|  | (Optional) All UEs | 0.014 | 0.027 | 0.43 | 5.19 |
| [Case 46], [SH, sync error 50ns], [FR2], [Multi-RTT, MUSIC, select based on RSRP] | Convex UEs | 0.010 | 0.013 | 0.034 | 4.13 |
|  | (Optional) All UEs | 0.014 | 0.037 | 0.50 | 5.87 |
| [Case 47], [DH, perfect sync], [FR1], [Multi-RTT, MUSIC, select based on RSRP] | Convex UEs | 0.068 | 0.13 | 2.89 | 5.88 |
|  | (Optional) All UEs | 0.092 | 1.09 | 4.55 | 9.82 |
| [Case 48], [DH, sync error 50ns], [FR1], [Multi-RTT, MUSIC, select based on RSRP] | Convex UEs | 0.068 | 0.23 | 3.18 | 6.20 |
|  | (Optional) All UEs | 0.091 | 1.34 | 5.49 | 10.71 |
| [Case 49], [DH, perfect sync], [FR2], [Multi-RTT, MUSIC, select based on RSRP] | Convex UEs | 0.013 | 0.28 | 2.61 | 5.74 |
|  | (Optional) All UEs | 0.024 | 1.04 | 3.31 | 9.24 |
| [Case 50], [DH, sync error 50ns], [FR2], [Multi-RTT, MUSIC, select based on RSRP] | Convex UEs | 0.010 | 0.30 | 2.80 | 6.23 |
|  | (Optional) All UEs | 0.025 | 1.51 | 4.53 | 10.10 |
| [Case 51], [SH, perfect sync], [FR1], [Multi-RTT, MUSIC, select based on first/median peak] | Convex UEs | 0.040 | 0.050 | 0.069 | 0.10 |
|  | (Optional) All UEs | 0.052 | 0.074 | 0.12 | 0.25 |
| [Case 52], [SH, sync error 50ns], [FR1], [Multi-RTT, MUSIC, select based on first/median peak] | Convex UEs | 0.042 | 0.058 | 0.069 | 0.10 |
|  | (Optional) All UEs | 0.048 | 0.080 | 0.15 | 0.32 |
| [Case 53], [SH, perfect sync], [FR2], [Multi-RTT, MUSIC, select based on first/median peak] | Convex UEs | 0.010 | 0.014 | 0.017 | 0.031 |
|  | (Optional) All UEs | 0.013 | 0.020 | 0.037 | 0.11 |
| [Case 54], [SH, sync error 50ns], [FR2], [Multi-RTT, MUSIC, select based on first/median peak] | Convex UEs | 0.010 | 0.012 | 0.017 | 0.030 |
|  | (Optional) All UEs | 0.013 | 0.021 | 0.039 | 0.10 |
| [Case 55], [DH, perfect sync], [FR1], [Multi-RTT, MUSIC, select based on first/median peak] | Convex UEs | 0.061 | 0.082 | 0.11 | 0.60 |
|  | (Optional) All UEs | 0.076 | 0.12 | 0.20 | 3.13 |
| [Case 56], [DH, sync error 50ns], [FR1], [Multi-RTT, MUSIC, select based on first/median peak] | Convex UEs | 0.063 | 0.092 | 0.12 | 0.78 |
|  | (Optional) All UEs | 0.087 | 0.13 | 0.28 | 4.72 |
| [Case 57], [DH, perfect sync], [FR2], [Multi-RTT, MUSIC, select based on first/median peak] | Convex UEs | 0.010 | 0.013 | 0.018 | 0.051 |
|  | (Optional) All UEs | 0.014 | 0.022 | 0.039 | 0.63 |
| [Case 58], [DH, sync error 50ns], [FR2], [Multi-RTT, MUSIC, select based on first/median peak] | Convex UEs | 0.011 | 0.015 | 0.021 | 0.055 |
|  | (Optional) All UEs | 0.016 | 0.023 | 0.041 | 0.69 |

Observation 11

- For Rel.16 Multi-RTT positioning, the performance target [0.2m 90%] can be achieved in InF-SH for convex UEs, and in InF-SH for all UEs for FR2, but cannot be achieved in InF-SH in other cases.

Observation 12

- For Rel.16 Multi-RTT positioning, the performance target [0.2m 90%] can be achieved in InF-DH for convex UEs for FR2, but cannot be achieved in other cases in InF-DH.

Observation 13

- Compared to DL-TDOA and UL-TDOA, Multi-RTT can mitigate the positioning error caused by synchronization error between network nodes.

### 2.1.2 Vertical accuracy evaluation

In this section, we provide our vertical accuracy evaluation results for Rel-16 positioning solutions. The simulation results are listed in the following tables. The detailed parameter configuration and CDF figures are provided in Appendix A and Appendix B. It is noted the vertical accuracy evaluation results for DL-TDOA are evaluated with outlier determination/rejection techniques (i.e., with RAIM) in this section.

In RAN1 101-e meeting, the baseline for vertical positioning has been agreed that the fixed BS height is 8m and the fixed UE height is 1.5m. And in last RAN1 meeting, the conclusion is that RAN1 will not define additional optional values for UE and gNB antenna heights for evaluations. Therefore, only baseline vertical evaluation results are presented in Table 4.

Table 4 Rel.16 NR positioning - vertical accuracy results (m)

|  |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| [Case 59], [SH, perfect sync], [FR1], [DL-TDOA, MUSIC, select based on first/median peak] | Convex UEs | 0.19 | 0.30 | 0.46 | 0.58 |
|  | (Optional) All UEs | 0.19 | 0.33 | 0.54 | 0.84 |
| [Case 60], [DH, perfect sync], [FR1], [DL-TDOA, MUSIC, select based on first/median peak] | Convex UEs | 0.12 | 0.20 | 0.41 | 0.64 |
|  | (Optional) All UEs | 0.13 | 0.22 | 0.41 | 1.39 |
| [Case 61], [SH, perfect sync], [FR1], [AOA+ZOA, MUSIC] | Convex UEs | 0.28 | 0.43 | 0.52 | 0.66 |
|  | (Optional) All UEs | 0.48 | 0.54 | 0.58 | 0.90 |
| [Case 62], [DH, perfect sync], [FR1], [AOA+ZOA, MUSIC] | Convex UEs | 0.48 | 0.63 | 0.83 | 1.12 |
|  | (Optional) All UEs | 0.44 | 0.63 | 0.83 | 1.27 |

Observation 14

- For vertical evaluation with DL-TDOA, the performance target [1m 90%] can be achieved in InF-SH and InF-DH scenarios for FR1 with baseline assumptions.

Observation 15

- For vertical evaluation with AOA+ZOA technique, the performance target [1m 90%] can be achieved in InF-SH scenario but not in InF-DH scenario for FR1 with baseline assumptions.

### 2.1.3 Horizontal accuracy evaluation for IOO scenario

In RAN1#101-e meeting, it was agreed that for the evaluation of commercial use cases, no baseline scenario is defined. As far as we are concerned, the evaluation for general commercial use cases should focus on the Indoor scenario. In this section, the horizontal accuracy evaluation results for IOO scenario with perfect synchronization for FR1 and FR2 are presented in Table 5. It is noted the horizontal accuracy evaluation for IOO in this section are performed on Rel-16 techniques with outlier determination/rejection techniques (i.e., with RAIM).

Table 5 Rel.16 NR positioning - horizontal accuracy results (m) for IOO scenario

|  |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| [Case 63], [IOO scenario, perfect sync], [FR1], [DL-TDOA, MUSIC, select based on first/median peak] | all UEs | 0.11 | 0.19 | 0.35 | 0.80 |
| [Case 64], [IOO scenario, perfect sync], [FR2], [DL-TDOA, MUSIC, select based on first/median peak] | all UEs | 0.03 | 0.06 | 0.13 | 0.54 |
| [Case 65], [IOO scenario, perfect sync], [FR1], [UL-TDOA, MUSIC, select based on first/median peak] | all UEs | 0.11 | 0.19 | 0.38 | 0.84 |
| [Case 66], [IOO scenario, perfect sync], [FR2], [UL-TDOA, MUSIC, select based on first/median peak] | all UEs | 0.03 | 0.06 | 0.13 | 0.56 |
| [Case 67], [IOO scenario, perfect sync], [FR1], [Multi-RTT, MUSIC, select based on first/median peak] | all UEs | 0.11 | 0.17 | 0.37 | 0.68 |
| [Case 68], [IOO scenario, perfect sync], [FR2], [Multi-RTT MUSIC, select based on first/median peak] | all UEs | 0.03 | 0.05 | 0.11 | 0.50 |

Observation 16

- For IOO scenario, the performance target [1m 90%] can be achieved with perfect synchronization.

2.2 Performance of NR positioning enhancements

2.2.1 Performance of Outlier determination/rejection techniques

In this section, we provide horizontal accuracy evaluation results for NR positioning enhancements with outlier determination/rejection techniques. The simulation results are listed in the following tables. The detailed parameter configuration and CDF figures are provided in Appendix A and Appendix C.

In the following enhancement simulations, the outlier determination/rejection technique is performed by the method that the TRP which introduces relatively large error is deleted by the intermediate variable through the taylor series algorithm (RAIM). RAIM is an implement algorithm in the receiver to estimate whether a set of measurements (such as RSTD) includes element(s) with a larger error, and exclude this element(s) if there is. For example, if the position calculation equation is

y=GX+ε

Where y is the vector of measured TOA or RSTD, G is the coefficient matrix, X is the vector of position coordinates to be solved, GX is  the vector of calculated TOA or RSTD by calculating positioning X, and ε is initial residual vector. According to the principle of least squares, the least square solution of vector X is

X=(GTG)-1GTy

Then the residual vector ε can be

ε=y-G(GTG)-1GTy=I-GGTG-1GTy=Sy

where S=I-GGTG-1GT .

Then the WSSE of the residual vector can be

εWSSE=WεT(Wε)

where W is the weight vector.

If εWSSE is larger than the threshold, the max element of residual vector  ε  will be excluded and the RAIM will be processed again until εWSSE is not larger than the threshold.

- 2.2.1.1 Downlink only evaluation

The evaluation results for NR positioning enhancements with downlink techniques are summarized in Table 6

Table 6  Rel.16 NR positioning enhancements - horizontal accuracy results (m) with downlink only techniques

|  |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| [Case E1], [SH, perfect sync], [FR1], [DL-TDOA, MUSIC, select based on RSRP, RAIM] | Convex UEs | 0.039 | 0.049 | 0.072 | 0.099 |
|  | (Optional) All UEs | 0.049 | 0.080 | 0.20 | 0.89 |
| [Case E2], [SH, sync error 50ns], [FR1], [DL-TDOA, MUSIC, select based on RSRP, RAIM] | Convex UEs | 10.25 | 13.23 | 17.16 | 26.57 |
|  | (Optional) All UEs | 13.09 | 17.89 | 25.44 | 34.70 |
| [Case E3], [SH, perfect sync], [FR2], [DL-TDOA, MUSIC, select based on RSRP, RAIM] | Convex UEs | 0.009 | 0.012 | 0.016 | 0.024 |
|  | (Optional) All UEs | 0.013 | 0.020 | 0.042 | 0.34 |
| [Case E4], [SH, sync error 50ns], [FR2], [DL-TDOA, MUSIC, select based on RSRP, RAIM] | Convex UEs | 9.78 | 12.77 | 15.84 | 26.39 |
|  | (Optional) All UEs | 13.52 | 17.85 | 23.58 | 33.52 |
| [Case E5], [DH, perfect sync], [FR1], [DL-TDOA, MUSIC, select based on RSRP, RAIM] | Convex UEs | 0.058 | 0.10 | 0.18 | 4.43 |
|  | (Optional) All UEs | 0.11 | 0.26 | 0.85 | 5.72 |
| [Case E6], [DH, sync error 50ns], [FR1], [DL-TDOA, MUSIC, select based on RSRP, RAIM] | Convex UEs | 9.32 | 11.97 | 14.63 | 21.94 |
|  | (Optional) All UEs | 10.80 | 14.46 | 17.66 | 23.84 |
| [Case E7], [DH, perfect sync], [FR2], [DL-TDOA, MUSIC, select based on RSRP, RAIM] | Convex UEs | 0.010 | 0.013 | 0.026 | 4.35 |
|  | (Optional) All UEs | 0.015 | 0.027 | 0.21 | 4.98 |
| [Case E8], [DH, sync error 50ns], [FR2], [DL-TDOA, MUSIC, select based on RSRP, RAIM] | Convex UEs | 10.52 | 13.61 | 16.58 | 18.89 |
|  | (Optional) All UEs | 11.74 | 15.26 | 18.18 | 22.95 |
| [Case E9], [SH, perfect sync], [FR1], [DL-TDOA, MUSIC, select based on first/median peak, RAIM] | Convex UEs | 0.037 | 0.048 | 0.069 | 0.094 |
|  | (Optional) All UEs | 0.051 | 0.074 | 0.13 | 0.30 |
| [Case E10], [SH, sync error 50ns], [FR1], [DL-TDOA, MUSIC, select based on first/median peak, RAIM] | Convex UEs | 11.42 | 13.97 | 17.67 | 25.67 |
|  | (Optional) All UEs | 13.67 | 17.73 | 23.65 | 37.47 |
| [Case E11], [SH, perfect sync], [FR2], [DL-TDOA, MUSIC, select based on first/median peak, RAIM] | Convex UEs | 0.009 | 0.012 | 0.017 | 0.024 |
|  | (Optional) All UEs | 0.012 | 0.019 | 0.033 | 0.10 |
| [Case E12], [SH, sync error 50ns], [FR2], [DL-TDOA, MUSIC, select based on first/median peak, RAIM] | Convex UEs | 10.84 | 13.91 | 17.53 | 23.80 |
|  | (Optional) All UEs | 12.57 | 16.84 | 24.54 | 34.93 |
| [Case E13], [DH, perfect sync], [FR1], [DL-TDOA, MUSIC, select based on first/median peak, RAIM] | Convex UEs | 0.056 | 0.075 | 0.11 | 0.17 |
|  | (Optional) All UEs | 0.087 | 0.14 | 0.25 | 0.62 |
| [Case E14], [DH, sync error 50ns], [FR1], [DL-TDOA, MUSIC, select based on first/median peak, RAIM] | Convex UEs | 8.91 | 12.07 | 14.26 | 19.69 |
|  | (Optional) All UEs | 11.42 | 15.01 | 17.54 | 24.96 |
| [Case E15], [DH, perfect sync], [FR2], [DL-TDOA, MUSIC, select based on first/median peak, RAIM] | Convex UEs | 0.011 | 0.015 | 0.018 | 0.034 |
|  | (Optional) All UEs | 0.015 | 0.022 | 0.040 | 0.32 |
| [Case E16], [DH, sync error 50ns], [FR2], [DL-TDOA, MUSIC, select based on first/median peak, RAIM] | Convex UEs | 9.94 | 13.35 | 15.10 | 18.17 |
|  | (Optional) All UEs | 11.78 | 14.66 | 18.03 | 21.48 |

Observation 17

- For DL-TDOA positioning, the performance target [0.2m 90%] can be achieved in InF-SH and InF-DH with perfect synchronization for convex UEs with enhancement.

Observation 18

- For DL-TDOA positioning, the performance target [0.2m 90%] can be achieved in InF-SH for FR2 with perfect synchronization for all UEs with enhancement, but cannot be achieved in other cases for all UEs.

- 2.2.1.2 Uplink only evaluation

The evaluation results for NR positioning enhancements with uplink only techniques are summarized in Table 7.

Table 7 Rel.16 NR positioning enhancements - horizontal accuracy results (m) with uplink only techniques

|  |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| [Case E17], [SH, perfect sync], [FR1], [UL-TDOA, MUSIC, select based on RSRP, RAIM] | Convex UEs | 0.037 | 0.049 | 0.061 | 0.10 |
|  | (Optional) All UEs | 0.051 | 0.086 | 0.17 | 0.98 |
| [Case E18], [SH, sync error 50ns], [FR1], [UL-TDOA, MUSIC, select based on RSRP, RAIM] | Convex UEs | 9.80 | 13.23 | 17.04 | 26.38 |
|  | (Optional) All UEs | 13.09 | 17.30 | 25.36 | 35.25 |
| [Case E19], [SH, perfect sync], [FR2], [UL-TDOA, MUSIC, select based on RSRP, RAIM] | Convex UEs | 0.009 | 0.012 | 0.016 | 0.034 |
|  | (Optional) All UEs | 0.012 | 0.019 | 0.036 | 0.31 |
| [Case E20], [SH, sync error 50ns], [FR2], [UL-TDOA, MUSIC, select based on RSRP, RAIM] | Convex UEs | 10.40 | 13.17 | 19.07 | 26.42 |
|  | (Optional) All UEs | 13.17 | 17.91 | 24.74 | 35.56 |
| [Case E21], [DH, perfect sync], [FR1], [UL-TDOA, MUSIC, select based on RSRP, RAIM] | Convex UEs | 0.058 | 0.098 | 0.18 | 5.48 |
|  | (Optional) All UEs | 0.10 | 0.21 | 1.14 | 5.98 |
| [Case E22], [DH, sync error 50ns], [FR1], [UL-TDOA, MUSIC, select based on RSRP, RAIM] | Convex UEs | 9.91 | 13.28 | 16.02 | 22.90 |
|  | (Optional) All UEs | 12.26 | 15.28 | 18.57 | 23.85 |
| [Case E23], [DH, perfect sync], [FR2], [UL-TDOA, MUSIC, select based on RSRP, RAIM] | Convex UEs | 0.010 | 0.020 | 0.089 | 4.55 |
|  | (Optional) All UEs | 0.016 | 0.029 | 0.50 | 5.96 |
| [Case E24], [DH, sync error 50ns], [FR2], [UL-TDOA, MUSIC, select based on RSRP, RAIM] | Convex UEs | 10.52 | 13.61 | 16.21 | 18.75 |
|  | (Optional) All UEs | 12.23 | 15.25 | 18.43 | 23.60 |
| [Case E25], [SH, perfect sync], [FR1], [UL-TDOA, MUSIC, select based on first/median peak, RAIM] | Convex UEs | 0.037 | 0.048 | 0.060 | 0.083 |
|  | (Optional) All UEs | 0.050 | 0.072 | 0.13 | 0.25 |
| [Case E26], [SH, sync error 50ns], [FR1], [UL-TDOA, MUSIC, select based on first/median peak, RAIM] | Convex UEs | 11.50 | 14.13 | 17.81 | 26.09 |
|  | (Optional) All UEs | 13.61 | 17.81 | 23.77 | 37.64 |
| [Case E27], [SH, perfect sync], [FR2], [UL-TDOA, MUSIC, select based on first/median peak, RAIM] | Convex UEs | 0.009 | 0.010 | 0.016 | 0.032 |
|  | (Optional) All UEs | 0.011 | 0.018 | 0.033 | 0.11 |
| [Case E28], [SH, sync error 50ns], [FR2], [UL-TDOA, MUSIC, select based on first/median peak, RAIM] | Convex UEs | 11.50 | 14.38 | 18.06 | 26.01 |
|  | (Optional) All UEs | 14.13 | 16.70 | 24.49 | 36.40 |
| [Case E29], [DH, perfect sync], [FR1], [UL-TDOA, MUSIC, select based on first/median peak, RAIM] | Convex UEs | 0.053 | 0.073 | 0.11 | 0.19 |
|  | (Optional) All UEs | 0.086 | 0.16 | 0.27 | 0.65 |
| [Case E30], [DH, sync error 50ns], [FR1], [UL-TDOA, MUSIC, select based on first/median peak, RAIM] | Convex UEs | 10.0 | 13.35 | 16.53 | 22.54 |
|  | (Optional) All UEs | 12.77 | 15.12 | 18.62 | 26.28 |
| [Case E31], [DH, perfect sync], [FR2], [UL-TDOA, MUSIC, select based on first/median peak, RAIM] | Convex UEs | 0.011 | 0.014 | 0.019 | 0.043 |
|  | (Optional) All UEs | 0.015 | 0.022 | 0.051 | 0.34 |
| [Case E32], [DH, sync error 50ns], [FR2], [UL-TDOA, MUSIC, select based on first/median peak, RAIM] | Convex UEs | 9.93 | 13.21 | 16.55 | 20.23 |
|  | (Optional) All UEs | 12.11 | 15.60 | 19.18 | 24.03 |

Observation 19

- For UL-TDOA positioning, the performance target [0.2m 90%] can be achieved in InF-SH and InF-DH with perfect synchronization for convex UEs with enhancement.

Observation 20

- For UL-TDOA positioning, the performance target [0.2m 90%] can be achieved in InF-SH for FR2 with perfect synchronization for all UEs with enhancement, but cannot be achieved in other cases for all UEs.

- 2.2.1.3 Downlink and uplink evaluation

The evaluation results for NR positioning enhancements with downlink & uplink techniques are summarized in Table 8.

Table 8 Rel.16 NR positioning enhancements - horizontal accuracy results (m) with downlink & uplink techniques

|  |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| [Case E33], [SH, perfect sync], [FR1], [Multi-RTT, MUSIC, select based on RSRP, RAIM] | Convex UEs | 0.040 | 0.052 | 0.071 | 0.11 |
|  | (Optional) All UEs | 0.054 | 0.076 | 0.12 | 0.51 |
| [Case E34], [SH, sync error 50ns], [FR1], [Multi-RTT, MUSIC, select based on RSRP, RAIM] | Convex UEs | 0.043 | 0.055 | 0.069 | 0.14 |
|  | (Optional) All UEs | 0.051 | 0.074 | 0.13 | 0.62 |
| [Case E35], [SH, perfect sync], [FR2], [Multi-RTT, MUSIC, select based on RSRP, RAIM] | Convex UEs | 0.010 | 0.014 | 0.019 | 0.049 |
|  | (Optional) All UEs | 0.013 | 0.021 | 0.036 | 0.14 |
| [Case E36], [SH, sync error 50ns], [FR2], [Multi-RTT, MUSIC, select based on RSRP, RAIM] | Convex UEs | 0.011 | 0.015 | 0.019 | 0.12 |
|  | (Optional) All UEs | 0.014 | 0.021 | 0.034 | 0.19 |
| [Case E37], [DH, perfect sync], [FR1], [Multi-RTT, MUSIC, select based on RSRP, RAIM] | Convex UEs | 0.068 | 0.12 | 0.23 | 4.89 |
|  | (Optional) All UEs | 0.091 | 0.14 | 0.78 | 6.39 |
| [Case E38], [DH, sync error 50ns], [FR1], [Multi-RTT, MUSIC, select based on RSRP, RAIM] | Convex UEs | 0.063 | 0.092 | 0.26 | 5.12 |
|  | (Optional) All UEs | 0.092 | 0.16 | 0.88 | 6.84 |
| [Case E39], [DH, perfect sync], [FR2], [Multi-RTT, MUSIC, select based on RSRP, RAIM] | Convex UEs | 0.013 | 0.019 | 0.11 | 4.12 |
|  | (Optional) All UEs | 0.018 | 0.065 | 0.59 | 4.82 |
| [Case E40], [DH, sync error 50ns], [FR2], [Multi-RTT, MUSIC, select based on RSRP, RAIM] | Convex UEs | 0.013 | 0.019 | 0.039 | 4.26 |
|  | (Optional) All UEs | 0.033 | 0.081 | 0.78 | 5.49 |
| [Case E41], [SH, perfect sync], [FR1], [Multi-RTT, MUSIC, select based on first/median peak, RAIM] | Convex UEs | 0.040 | 0.050 | 0.068 | 0.092 |
|  | (Optional) All UEs | 0.044 | 0.066 | 0.097 | 0.24 |
| [Case E42], [SH, sync error 50ns], [FR1], [Multi-RTT, MUSIC, select based on first/median peak, RAIM] | Convex UEs | 0.048 | 0.058 | 0.067 | 0.092 |
|  | (Optional) All UEs | 0.049 | 0.069 | 0.12 | 0.23 |
| [Case E43], [SH, perfect sync], [FR2], [Multi-RTT, MUSIC, select based on first/median peak, RAIM] | Convex UEs | 0.010 | 0.014 | 0.018 | 0.030 |
|  | (Optional) All UEs | 0.013 | 0.019 | 0.030 | 0.055 |
| [Case E44], [SH, sync error 50ns], [FR2], [Multi-RTT, MUSIC, select based on first/median peak, RAIM] | Convex UEs | 0.012 | 0.016 | 0.021 | 0.037 |
|  | (Optional) All UEs | 0.016 | 0.021 | 0.033 | 0.072 |
| [Case E45], [DH, perfect sync], [FR1], [Multi-RTT, MUSIC, select based on first/median peak, RAIM] | Convex UEs | 0.059 | 0.083 | 0.11 | 0.19 |
|  | (Optional) All UEs | 0.078 | 0.12 | 0.18 | 0.31 |
| [Case E46], [DH, sync error 50ns], [FR1], [Multi-RTT, MUSIC, select based on first/median peak, RAIM] | Convex UEs | 0.060 | 0.091 | 0.13 | 0.20 |
|  | (Optional) All UEs | 0.086 | 0.13 | 0.19 | 0.35 |
| [Case E47], [DH, perfect sync], [FR2], [Multi-RTT, MUSIC, select based on first/median peak, RAIM] | Convex UEs | 0.012 | 0.016 | 0.020 | 0.048 |
|  | (Optional) All UEs | 0.013 | 0.018 | 0.029 | 0.11 |
| [Case E48], [DH, sync error 50ns], [FR2], [Multi-RTT, MUSIC, select based on first/median peak, RAIM] | Convex UEs | 0.013 | 0.016 | 0.021 | 0.051 |
|  | (Optional) All UEs | 0.014 | 0.020 | 0.026 | 0.11 |

Observation 21

- For Multi-RTT positioning, the performance target [0.2m 90%] can be achieved in InF-SH and InF-DH for convex UEs with enhancement.

Observation 22

- For Multi-RTT positioning, the performance target [0.2m 90%] can be achieved in InF-SH and InF-DH for FR2 for all UEs with enhancement, but cannot be achieved in other cases for all UEs.

2.2.2 Outlier determination/rejection techniques and LOS detection

In this section, we evaluate the performance of outlier determination/rejection techniques(RAIM) and different LOS classification/detection condition. We compare the performance of LOS/NLOS classification/detection with RAIM. The case of known LOS without RAIM represents selecting TRPs with known LOS information, that is, if the UE has more than 4 LOS TRPs, it chooses TRPs for positioning form the LOS TRPs, if the UE has less than 4 LOS TRPs, it chooses all LOS TRPs and additional NLOS TRP to reach 4 TRPs.  The case of known LOS + RAIM means selecting TRPs with known LOS information, while the RAIM algorithm is also applied.

The results are in Table 9, it can be seen that the RAIM algorithm without any LOS detection can reach a high accuracy better than LOS detection without RAIM. In other words, if the RAIM algorithm is applied, with sufficient LOS links, better TRPs can be chosen and LOS detection is not effective.

Table 9 Rel.16 NR positioning enhancements - horizontal accuracy results (m)

|  |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| [Case E49], [SH, perfect sync], [FR1], [DL-TDOA, RAIM] | Convex UEs | 0.037 | 0.048 | 0.069 | 0.094 |
| [Case E50], [SH, perfect sync], [FR1], [DL-TDOA, known LOS without RAIM] | Convex UEs | 0.037 | 0.049 | 0.073 | 0.096 |
| [Case E51], [SH, perfect sync], [FR1], [DL-TDOA, known LOS+ RAIM] | Convex UEs | 0.037 | 0.047 | 0.063 | 0.083 |
| [Case E52], [SH, perfect sync], [FR1], [DL-TDOA, 95% LOS without RAIM] | Convex UEs | 0.039 | 0.052 | 0.080 | 2.86 |
| [Case E53], [SH, perfect sync], [FR1], [DL-TDOA, 90% LOS without RAIM] | Convex UEs | 0.041 | 0.052 | 0.083 | 4.54 |
| [Case E54], [SH, perfect sync], [FR1], [DL-TDOA, unknown LOS without RAIM] | Convex UEs | 0.042 | 0.055 | 0.11 | 4.62 |
| [Case E55], [DH, perfect sync], [FR1], [DL-TDOA, RAIM] | Convex UEs | 0.056 | 0.075 | 0.11 | 0.17 |
| [Case E56], [DH, perfect sync], [FR1], [DL-TDOA, known LOS without RAIM] | Convex UEs | 0.060 | 0.097 | 0.15 | 0.33 |
| [Case E57], [DH, perfect sync], [FR1], [DL-TDOA, known LOS+ RAIM] | Convex UEs | 0.055 | 0.071 | 0.11 | 0.17 |
| [Case E58], [DH, perfect sync], [FR1], [DL-TDOA, 95% LOS without RAIM] | Convex UEs | 0.042 | 0.064 | 0.30 | 3.40 |
| [Case E59], [DH, perfect sync], [FR1], [DL-TDOA, 90% LOS without RAIM] | Convex UEs | 0.043 | 0.065 | 1.82 | 3.43 |
| [Case E60], [DH, perfect sync], [FR1], [DL-TDOA, unknown LOS without RAIM] | Convex UEs | 0.048 | 1.05 | 3.49 | 8.64 |

Based on the above evaluation results, the positioning performance of LOS detection/identification degrades as LOS detection error probability increases. Even in the ideal case with 100% LOS path detection assumed, its’ positioning performance is still worse than that of implementation-based solutions (RAIM algorithm with the first/median) for 90% UE in SH and DH cases. The reason is that the implementation-based solution (RAIM) can eliminate the TRP who introduces relatively large error.

Observation 23

- Outlier determination/rejection techniques have better positioning accuracy performance than LOS detection.

- The positioning performance of LOS/NLOS detection method degrades as LOS detection error probability increases.

It’s obvious that benefits of  LOS/NLOS detection method will diminish if it cannot determine the LOS path correctly. Based on the evaluation results in Table 9, 5% LOS/NLOS identification error will lead to nearly 3m positioning error.

Proposal 1

- LOS/NLOS detection/identification should not be considered in Rel-17.

2.2.3 Timing measurement reporting granularity

In the last RAN1 meeting, the Agreement 1 has been reached. The evaluation results of different granularity of timing measurement reports are shown in Table 10.

Agreement 1:

- FFS whether Rel.16 granularity of timing measurement reports is enough to avoid degradation in I-IoT scenarios and meet positioning requirements

In Rel.16, the reporting granularity for the UE/gNB timing measurements (DL RSTD, the UE Rx-Tx time difference, UL RTOA, gNB Rx-Tx time difference) is defined as T=Tc2k, where k is a configuration parameter with a minimum value of at most 0. The minimum granularity is 0.5ns. As shown in the results, when the granularity of timing measurement reports is 0.5ns, the accuracy is 0.16m for SH and 0.17m for DH for 90% UE, which can still meet the performance requirements.

Table 10 Rel.16 NR positioning enhancements - horizontal accuracy results (m)

|  |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| [Case E61], [SH, perfect sync], [FR1], [DL-TDOA] [granularity 0.5ns] | Convex UEs | 0.070 | 0.092 | 0.12 | 0.16 |
| [Case E62], [SH, perfect sync], [FR1], [DL-TDOA] [granularity 1ns] | Convex UEs | 0.10 | 0.13 | 0.16 | 0.21 |
| [Case E63], [SH, perfect sync], [FR1], [DL-TDOA] [granularity 2ns] | Convex UEs | 0.21 | 0.28 | 0.32 | 0.47 |
| [Case E64], [DH, perfect sync], [FR1], [DL-TDOA] [granularity 0.5ns] | Convex UEs | 0.063 | 0.086 | 0.11 | 0.17 |
| [Case E65], [DH, perfect sync], [FR1], [DL-TDOA] [granularity 1ns] | Convex UEs | 0.12 | 0.15 | 0.21 | 0.35 |
| [Case E66], [DH, perfect sync], [FR1], [DL-TDOA] [granularity 2ns] | Convex UEs | 0.22 | 0.27 | 0.33 | 0.59 |

Observation 24

- The minimum Rel.16 granularity of timing measurement reports is enough to avoid degradation in IIoT scenarios and meet positioning requirements.

2.2.4 Tx/Rx timing error

In the last RAN1 meeting, the following conclusion about Tx/Rx timing error was reached.

Conclusion:

- It is observed that calibration errors of UE/gNB Tx/Rx timing may negatively impact accuracy of timing-based methods of Rel.16 positioning solutions when precise UE positioning is targeted.

- Note: Additional evaluations need to be performed before deciding whether and how to capture the above in the TR

To further evaluate how Tx/Rx timing error affect positioning accuracy, we evaluate the performance of different value of Tx/Rx timing error with DL-TDOA and Multi-RTT. The results are provided in Table 11. For DL-TDOA, the UE timing error seems have little impact on accuracy, which is because RSTD calculation can eliminate most timing error at UE side, as shown in case E47 - E51; but the BS timing error causes a great degradation on positioning performance, as shown in case E52 - E55. For Multi-RTT, the UE timing error and BS timing error both cause a degradation in accuracy. Therefore, the Tx/Rx timing error may be more urgent to solve in Multi-RTT technique. Some methods such as differential positioning may be a good way to solve the timing error problem, which is discussed in detail in our contribution [3].

Table 11 Rel.16 NR positioning enhancements - horizontal accuracy results (m)

|  |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| [Case E67], [SH, perfect sync], [FR1], [DL-TDOA] [BS timing error 0.5ns, UE timing error 0.5ns] | Convex UEs | 0.14 | 0.19 | 0.23 | 0.30 |
| [Case E68], [SH, perfect sync], [FR1], [DL-TDOA] [BS timing error 0.5ns, UE timing error 1ns] | Convex UEs | 0.14 | 0.18 | 0.23 | 0.34 |
| [Case E69], [SH, perfect sync], [FR1], [DL-TDOA] [BS timing error 0.5ns, UE timing error 2ns] | Convex UEs | 0.14 | 0.18 | 0.23 | 0.36 |
| [Case E70], [SH, perfect sync], [FR1], [DL-TDOA] [BS timing error 0.5ns, UE timing error 3ns] | Convex UEs | 0.14 | 0.18 | 0.22 | 0.35 |
| [Case E71], [SH, perfect sync], [FR1], [DL-TDOA] [BS timing error 0.5ns, UE timing error 5ns] | Convex UEs | 0.14 | 0.17 | 0.23 | 0.37 |
| [Case E72], [SH, perfect sync], [FR1], [DL-TDOA] [BS timing error 1ns, UE timing error 0.5ns] | Convex UEs | 0.18 | 0.23 | 0.29 | 0.42 |
| [Case E73], [SH, perfect sync], [FR1], [DL-TDOA] [BS timing error 2ns, UE timing error 0.5ns] | Convex UEs | 0.37 | 0.45 | 0.60 | 0.83 |
| [[Case E74], [SH, perfect sync], [FR1], [DL-TDOA] [BS timing error 3ns, UE timing error 0.5ns] | Convex UEs | 0.47 | 0.61 | 0.76 | 1.07 |
| [Case E75], [SH, perfect sync], [FR1], [DL-TDOA] [BS timing error 5ns, UE timing error 0.5ns] | Convex UEs | 0.80 | 1.00 | 1.24 | 1.87 |
| [Case E76], [DH, perfect sync], [FR1], [DL-TDOA] [BS timing error 0.5ns, UE timing error 0.5ns] | Convex UEs | 0.14 | 0.18 | 0.23 | 0.31 |
| [Case E77], [DH, perfect sync], [FR1], [DL-TDOA] [BS timing error 0.5ns, UE timing error 1ns] | Convex UEs | 0.13 | 0.17 | 0.21 | 0.32 |
| [Case E78], [DH, perfect sync], [FR1], [DL-TDOA] [BS timing error 0.5ns, UE timing error 2ns] | Convex UEs | 0.13 | 0.18 | 0.21 | 0.32 |
| [Case E79], [DH, perfect sync], [FR1], [DL-TDOA] [BS timing error 0.5ns, UE timing error 3ns] | Convex UEs | 0.13 | 0.17 | 0.20 | 0.28 |
| [Case E80], [DH, perfect sync], [FR1], [DL-TDOA] [BS timing error 0.5ns, UE timing error 5ns] | Convex UEs | 0.12 | 0.18 | 0.22 | 0.31 |
| [Case E81], [DH, perfect sync], [FR1], [DL-TDOA] [BS timing error 1ns, UE timing error 0.5ns] | Convex UEs | 0.18 | 0.24 | 0.33 | 0.40 |
| [Case E82 [DH, perfect sync], [FR1], [DL-TDOA] [BS timing error 2ns, UE timing error 0.5ns] | Convex UEs | 0.40 | 0.48 | 0.62 | 0.76 |
| [[Case E83], [DH, perfect sync], [FR1], [DL-TDOA] [BS timing error 3ns, UE timing error 0.5ns] | Convex UEs | 0.54 | 0.62 | 0.75 | 0.88 |
| [Case E84], [DH, perfect sync], [FR1], [DL-TDOA] [BS timing error 5ns, UE timing error 0.5ns] | Convex UEs | 0.78 | 1.08 | 1.19 | 1.94 |
| [Case E85], [SH, perfect sync], [FR1], [Multi-RTT] [BS timing error 0.5ns, UE timing error 0.5ns] | Convex UEs | 0.089 | 0.13 | 0.18 | 0.24 |
| [Case E86], [SH, perfect sync], [FR1], [Multi-RTT] [BS timing error 0.5ns, UE timing error 1ns] | Convex UEs | 0.094 | 0.13 | 0.17 | 0.23 |
| [Case E87], [SH, perfect sync], [FR1], [Multi-RTT] [BS timing error 0.5ns, UE timing error 2ns] | Convex UEs | 0.12 | 0.15 | 0.19 | 0.27 |
| [Case E88], [SH, perfect sync], [FR1], [Multi-RTT] [BS timing error 0.5ns, UE timing error 3ns] | Convex UEs | 0.13 | 0.17 | 0.23 | 0.33 |
| [Case E89], [SH, perfect sync], [FR1], [Multi-RTT] [BS timing error 0.5ns, UE timing error 5ns] | Convex UEs | 0.16 | 0.25 | 0.34 | 0.44 |
| [Case E90], [SH, perfect sync], [FR1], [Multi-RTT] [BS timing error 1ns, UE timing error 0.5ns] | Convex UEs | 0.12 | 0.17 | 0.22 | 0.28 |
| [Case E91], [SH, perfect sync], [FR1], [Multi-RTT] [BS timing error 2ns, UE timing error 0.5ns] | Convex UEs | 0.17 | 0.21 | 0.28 | 0.34 |
| [[Case E92], [SH, perfect sync], [FR1], [Multi-RTT] [BS timing error 3ns, UE timing error 0.5ns] | Convex UEs | 0.39 | 0.49 | 0.61 | 0.76 |
| [Case E93], [SH, perfect sync], [FR1], [Multi-RTT] [BS timing error 5ns, UE timing error 0.5ns] | Convex UEs | 0.56 | 0.69 | 0.88 | 1.26 |
| [Case E94], [DH, perfect sync], [FR1], [Multi-RTT] [BS timing error 0.5ns, UE timing error 0.5ns] | Convex UEs | 0.10 | 0.12 | 0.17 | 0.24 |
| [Case E95], [DH, perfect sync], [FR1], [Multi-RTT] [BS timing error 0.5ns, UE timing error 1ns] | Convex UEs | 0.10 | 0.13 | 0.17 | 0.24 |
| [Case E96], [DH, perfect sync], [FR1], [Multi-RTT] [BS timing error 0.5ns, UE timing error 2ns] | Convex UEs | 0.11 | 0.15 | 0.20 | 0.28 |
| [Case E97], [DH, perfect sync], [FR1], [Multi-RTT] [BS timing error 0.5ns, UE timing error 3ns] | Convex UEs | 0.13 | 0.17 | 0.23 | 0.36 |
| [Case E98], [DH, perfect sync], [FR1], [Multi-RTT] [BS timing error 0.5ns, UE timing error 5ns] | Convex UEs | 0.18 | 0.25 | 0.33 | 0.46 |
| [Case E99], [DH, perfect sync], [FR1], [Multi-RTT] [BS timing error 1ns, UE timing error 0.5ns] | Convex UEs | 0.14 | 0.20 | 0.26 | 0.34 |
| [Case E100], [DH, perfect sync], [FR1], [Multi-RTT] [BS timing error 2ns, UE timing error 0.5ns] | Convex UEs | 0.21 | 0.26 | 0.35 | 0.48 |
| [[Case E101], [DH, perfect sync], [FR1], [Multi-RTT] [BS timing error 3ns, UE timing error 0.5ns] | Convex UEs | 0.37 | 0.51 | 0.62 | 0.87 |
| [Case E102], [DH, perfect sync], [FR1], [Multi-RTT] [BS timing error 5ns, UE timing error 0.5ns] | Convex UEs | 0.57 | 0.71 | 0.98 | 1.28 |

Observation 25

- With the TX/RX timing error 0.5ns, the performance target [0.2m 90%] cannot be achieved in both InF-SH and InF-DH.

Observation 26

- For DL-TDOA, the UE timing error has little impact on positioning accuracy, while the BS timing error causes some degradation on positioning performance.

Observation 27

- For Multi-RTT, the UE timing error and BS timing error both cause degradation in accuracy.

2.2.5 Aggregation of DL positioning frequency layers

In the last RAN1 meeting, Agreement 2 has been agreed that Aggregating multiple DL positioning frequency layers will be investigated in Rel-17. However, how much gain of aggregation is not known. In this section, the evaluation of aggregation of multiple DL positioning frequency is presented in Table 12, where the performance benefits of aggregating are evaluated in the ideal scenarios (without considering the impact of timing offset, phase offset, frequency error, and power imbalance among CCs). It is observed that the performance of aggregating which is 50+50M is better than 50M and worse than 100M.

Agreement 2:

- Aggregating multiple DL positioning frequency layers of the same or different bands for improving positioning performance for both intra-band and inter-band scenarios will be investigated in Rel-17, which may take into account at least the following

- The scenarios and performance benefits of aggregating multiple DL positioning frequency layers

- The impact of channel spacing, timing offset, phase offset, frequency error, and power imbalance among CCs to the positioning performance for intra-band contiguous/ non-contiguous and inter-band scenarios

- UE complexity considerations

- Note: What is captured in the TR will be discussed separately.

Table 12 Rel.16 NR positioning enhancements - horizontal accuracy results (m)

|  |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| [Case E103], [SH, perfect sync], [FR1], [50M] | Convex UEs | 0.12 | 0.16 | 0.20 | 0.31 |
|  | (Optional) All UEs | 0.16 | 0.25 | 0.38 | 0.95 |
| [Case E104], [SH, perfect sync], [FR1], [100M] | Convex UEs | 0.037 | 0.048 | 0.069 | 0.094 |
|  | (Optional) All UEs | 0.051 | 0.074 | 0.13 | 0.25 |
| [Case E105], [SH, perfect sync], [FR1], [50M+50M] | Convex UEs | 0.072 | 0.12 | 0.16 | 0.21 |
|  | (Optional) All UEs | 0.11 | 0.17 | 0.28 | 0.58 |
| [Case E106], [DH, perfect sync], [FR1], [50M] | Convex UEs | 0.11 | 0.16 | 0.21 | 0.44 |
|  | (Optional) All UEs | 0.17 | 0.26 | 0.50 | 0.98 |
| [Case E107], [DH, perfect sync], [FR1], [100M] | Convex UEs | 0.056 | 0.075 | 0.11 | 0.17 |
|  | (Optional) All UEs | 0.087 | 0.14 | 0.25 | 0.62 |
| [Case E108], [DH, perfect sync], [FR1], [50M+50M] | Convex UEs | 0.069 | 0.095 | 0.15 | 0.23 |
|  | (Optional) All UEs | 0.11 | 0.20 | 0.39 | 0.91 |

Observation 28

- For ideal aggregation of multiple DL positioning frequency 50M+50M, performance target [0.2m 90%] cannot be achieved in both InF-SH and InF-DH.

Observation 29

- For ideal aggregation of multiple DL positioning frequency 50M+50M, the performance is worse than 100M but better than 50M.

Proposal 2

- The performance impact of non-ideal aggregation of multiple DL positioning frequency should be further studied.

2.3 Observations on NR positioning enhancements

In this section, we provide our observations on NR positioning enhancements for horizontal accuracy evaluation in Table 13. The main cases are DL-TDOA, UL-TDOA and Multi RTT where selecting TRPs based on first/median peak with and without RAIM for convex UEs with perfect sync.

Table 13 NR positioning enhancements – horizontal accuracy performance summary

| Simulation case (Horizontal Error) | Gain vs Rel.16 solution, @[90%], [m] | Commercial horizontal accuracy requirements [1]m @[90]% are met - Yes/No. If no, provide performance gaps | IIoT horizontal accuracy requirements of [0.2]m @[90]%are met - Yes/No. If no, provide performance gaps | IIoT horizontal accuracy requirements of [0.5]m @[90]%are met -Yes/No. If no, provide performance gaps |
| --- | --- | --- | --- | --- |
| [[Case E9], [SH, perfect sync], [FR1], [DL-TDOA, MUSIC, select based on first/median peak, RAIM] | 0 | \ | Yes | Yes |
| [Case E11], [SH, perfect sync], [FR2], [DL-TDOA, MUSIC, select based on first/median peak, RAIM] | 0.007 | \ | Yes | Yes |
| [Case E13], [DH, perfect sync], [FR1], [DL-TDOA, MUSIC, select based on first/median peak, RAIM] | 0.43 | \ | Yes | Yes |
| [Case E15], [DH, perfect sync], [FR2], [DL-TDOA, MUSIC, select based on first/median peak, RAIM] | 0.015 | \ | Yes | Yes |
| [Case E25], [SH, perfect sync], [FR1], [UL-TDOA, MUSIC, select based on first/median peak, RAIM] | 0.004 | \ | Yes | Yes |
| [Case E27], [SH, perfect sync], [FR2], [UL-TDOA, MUSIC, select based on first/median peak, RAIM] | 0 | \ | Yes | Yes |
| [Case E29], [DH, perfect sync], [FR1], [UL-TDOA, MUSIC, select based on first/median peak, RAIM] | 0.41 | \ | Yes | Yes |
| [Case E31], [DH, perfect sync], [FR2], [UL-TDOA, MUSIC, select based on first/median peak, RAIM] | 0.00/8 | \ | Yes | Yes |
| [Case E41], [SH, perfect sync], [FR1], [Multi-RTT, MUSIC, select based on first/median peak, RAIM] | 0.008 | \ | Yes | Yes |
| [Case E43], [SH, perfect sync], [FR2], [Multi-RTT, MUSIC, select based on first/median peak, RAIM] | 0.001 | \ | Yes | Yes |
| [Case E45], [DH, perfect sync], [FR1], [Multi-RTT, MUSIC, select based on first/median peak, RAIM] | 0.41 | \ | Yes | Yes |
| [Case E47], [DH, perfect sync], [FR2], [Multi-RTT, MUSIC, select based on first/median peak, RAIM] | 0.003 | \ | Yes | Yes |

- Latency evaluation

## 3.1	The evaluation of physical layer latency for Rel.16 solutions

Agreement 3:

Physical layer latency for DL only, UL only, DL+UL positioning solutions for UE-based and UE-assisted approaches are separately studied

Agreement 4

Physical Layer Latency Start and End times are defined as follows:

| Method | Start | End |
| --- | --- | --- |
| UE assisted DL-only & DL-ECID & Multi-RTT | Transmission of the PDSCH from the gNB carrying the LPP Request Location Information message | Successful decoding of the PUSCH carrying the LPP Provide Location Information message |
| UL-only method & UL ECID & Multi-RTT | Reception by the gNB of the NRPPa measurement request message | The transmission by the gNB of the NRPPa measurement response message |
| UE-based | Transmission of the PDSCH from the gNB carrying the LPP Request Location Information if applicable, otherwise, · Alt. 1: transmission of the PUSCH carrying the MG Request from the UE. · Alt. 2: Transmission of the PDSCH from the gNB carrying the LPP message containing the assistance data · Alt. 3: Start of the Reception of DL PRS Note: Suggest to downselect this at the next meeting. Note: The high layers latency components may be subject to adjustment for different alternatives. | Successful decoding of the PUSCH at gNB carrying the LPP Provide Location Information message if applicable, otherwise Calculation of Location Estimate at the UE |

Agreement 5

- At least the following information is provided for positioning physical layer latency analysis:

  - Source initiating request for positioning measurements/location for a given UE (UE, Network)

  - Destination awaiting for positioning measurements/location for a given UE (UE, Network)

  - Start and end triggers/events for physical layer latency evaluation

    - For Rel.16 solutions, it is based on specification for each solution

  - Initial and final RRC State of positioned UE (RRC IDLE, INACTIVE, CONNECTED) at the start and end time for the physical layer latency evaluation

  - Positioning

    - technique (enumeration): (1) DL-TDOA, (2) DL AoD, (3) UL-TDoA, (4) UL-AoA, (5) Multi-RTT, (6) E-CID

    - type: DL, UL, DL+UL

    - mode: UE-based, UE-assisted

  - Latency component w/ value range and description, including information on any parallel (simultaneous) components

  - Total latency value

- Latency components are recommended to be captured in table and ordered consequently in time starting from the earliest one:

| Source [UE, NW]/Destination [UE, NW] Positioning technique [DL-TDOA, E-CID, …], type [DL, UL, DL+UL], mode [UE-A, UE-B], Initial and Final RRC States [IDLE, INACTIVE, CONNECTED] |  |  |
| --- | --- | --- |
| Latency Component | Value Range | Description of Latency Component |
| Start trigger |  |  |
| Name of component 1 |  |  |
| Name of component 2 |  |  |
| Name of the last component |  |  |
| End trigger |  |  |
| Total values |  |  |

According to Agreement 3 in RAN1 102-e, physical layer latency needs be studied separately for DL only, UL only, DL+UL positioning solutions for UE-based and UE-assisted approaches. In the following subsections, we present our views on latency components and values for physical layer latency for different approaches separately.

### 3.1.1 The physical layer latency for UE assisted DL only

The Physical Layer Latency Start and End times of UE assisted DL only have been defined in Agreement 4.

In this subsection, we present our views on latency components with corresponding range of values between Start and End times for UE assisted DL only. Firstly, latency components are listed below and the PHY procedure as Figure 1.

Figure 1 The PHY procedure for UE assisted DL only Positioning

Start time: Transmission of the PDSCH from the gNB carrying the LPP Request Location Information message.

1: Successful decoding of the PDSCH carrying the LPP Request Location Information message at the UE side.

2: Transmission of the PUSCH from the UE carrying the Measurement gap request message.

3: Successful decoding of the PUSCH carrying the Measurement gap request message at the gNB side.

4: Transmission of the PDSCH from the gNB carrying the Measurement gap configuration message.

5: Successful decoding of the PDSCH carrying the Measurement gap configuration at the UE side.

6: DL measurement &process delay.

7: Transmission of the PUSCH from the UE carrying the LPP Provide Location Information message.

End Time: Successful decoding of the PUSCH carrying the LPP Provide Location Information message at the gNB side

It is observed that the above latency components can be classified into four types and its value can be assumed as below:

- Start time and flow 4 are DL data transmission time. The value can be assumed as 0.5ms-1ms for URLLC case based on [LS: R1-1901470 and 6.4.1, TS 38.824]. From the LS, the 0.5ms one-way latency target can be achieved for both DL and UL for FDD for 30kHz (and higher) SCS and TDD for 60kHz (and higher) SCS.

And the value can be assumed as 7.5ms for eMBB based on [section 5.2.1, TS 36.881].

R1-1901470 RAN1 has performed some latency and reliability analysis for the use case I provided in the LS. The 0.5 ms latency target analysis lead to the following RAN1 conclusions: - For 15 kHz SCS, the 0.5ms one-way latency target cannot be achieved with Rel-15 NR. - For FDD, the 0.5ms one-way latency target can be achieved for both DL and UL for 30kHz (and higher) SCS with Rel-15 NR for single shot transmission. - For TDD: - For 30kHz SCS, some analysis shows the 0.5ms one-way latency target can be achieved with the respectively assumed UL/DL configuration, whereas other analysis indicates that it cannot be achieved with Rel-15 NR for the respectively assumed UL/DL configuration. - For 60kHz (and higher) SCS, the 0.5ms one-way latency target can be achieved with the respectively assumed UL/DL configuration for both DL and UL with Rel-15 NR for single-shot transmission.

Meanwhile, DL data transmission time needs to consider below components based on [ A.4, TS 38.824]

Figure 2 The components for DL data transmission time For evaluating the impact of processing times on DL data transmission time: - the gNB processing time (Tprocess time_pdsch )and the alignment delay(Talign time ) - The alignment delay includes the gap between the two consecutive PDCCH monitoring occasions for FDD, the UL/DL configuration for TDD, and the scheduling constraint due to the slot boundaries. - The alignment delay should also be considered for scheduling the later PDSCHs. - PDCCH duration (TPDCCH ) - Overlap time between PDCCH and PDSCH - PDSCH duration (TPDSCH ) - PUCCH duration (TPUCCH ) - gNB's processing time for transmission of the initial PDSCH and gNB's PUCCH-to-PDCCH processing time for re-transmission of the PDSCH: - Case1: UE's N2/2 + X for scheduling the initial PDSCH and UE's N2 + X for re-transmission. - UE decoding time for the PDSCH(TDecoding time_PDSCH ) and feedback HARQ

Flow 2 and flow 7 are UL data transmission time. Similar with DL data transmission time,  the value also can be assumed as 0.5ms-1ms for URLLC case based on [LS: R1-1901470], and 12.5ms for eMBB based on [section 5.2.1, TS 36.881].

And UL data transmission time needs to consider below components based on [ A.4, TS 38.824].

Figure 3 The components for UL data transmission time For evaluating the impact of processing times on uplink latency: -The alignment delay(Talign time ). - For the case of SR-based PUSCH, the alignment delay includes the gap between the two consecutive SR occasions for FDD, the SR transmission latency due to the UL/DL configuration for TDD, and the scheduling constraint due to the slot boundaries. - For the case of grant-free PUSCH, the alignment delay includes the transmission constraint due to the grant-free UL occasions for the initial transmission, and the scheduling constraint due to the slot boundaries for the grant-based re-transmission. - For both SR-based PUSCH and grant-free PUSCH, the alignment delay should also be considered for PUSCH re-transmission triggered by a dynamic grant. - UE's processing time of the initial transmission for the case of grant-free PUSCH (Tprocess time_pusch ), - gNB's PUSCH-to-PDCCH processing time (note that PDCCH alignment has to be included separately) is UE's N1 + X - X = 2/4/8 symbols for SCS = 30/60/120KHz, respectively. - gNB's decoding time for the last PUSCH (TDecoding time_PUSCH ) - PUSCH duration (TPUSCH ) - PDCCH duration(TPDCCH ) - For SR-based PUSCH: - gNB's processing time for SR - Duration of the PUCCH for SR - Number of SR occasions per slot

- End time, flow 1, flow 3, and flow 5 can be seen as equivalent RRC processing time, which value is 10ms [12, TS 38.331]

- Flow 6 is DL measurement &process delay, for one occasion and without considering beam sweeping case, which equals LCMTPRS,  T measGap∪ TProcess time,where LCMTPRS,  T measGap is the least common multiple of PRS and MG periodicity, TProcess time is the processing time up to the UE ability and the signal duration N1 that needs to measure. Whose value is variate and depends on the periodicity and the configuration of PRS and the value range is 22ms~11514ms. For multiple positioning occasion and multiple beam sweeping case, whose value equals LCMTPRS,  T measGap×NRxBeam×NPosOccasion∪ TProcess time, and the value range is 646ms~328960ms

where - LCMTPRS, T measGap is the least common multiple of PRS and MG periodicity, which value is {20,40,80,160,320,640,1280,2560,5120,10240} ms - TPRS is the periodicity of PRS, which includes 2μ4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 160, 320, 640, 1280, 2560, 5120, 10240 slots. - T measurement gap is the periodicity of the measurement gap, which includes 20, 40, 80, 160ms, and MGL is {1.5, 3, 3.5, 4, 5, 5.5, 6,10,20}ms. It is noted that 10ms only used for 80ms periodicity and 20ms is for 160ms. - TProcess time, which value is T1ms= N1/N× T ms, that is multiples of {8, 16, 20, 30, 40, 80, 160, 320, 640, 1280}ms - TProcess time is T1ms up to UE ability and the signal duration N1 that needs to measure, where N(N = {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50}ms) is a duration of DL PRS symbols in ms processed every Tms (T = {8, 16, 20, 30, 40, 80, 160, 320, 640, 1280}ms)for a given maximum bandwidth (B) in MHz supported by UE. - LCMTPRS, T measGap∪ TProcess time is the union of the least common multiple of PRS and MG periodicity and PRS processing time. It equals the least common multiple of PRS and MG periodicity if UE can figure out all the PRS that need be processed, otherwise, it is the least common multiple of PRS and MG periodicity plus the extra processing latency. Whose values equals {20,40,80,160,320,640,1280,2560,5120,10240} ms+ N1/N×{8, 16, 20, 30, 40, 80, 160, 320, 640, 1280}ms- N ms, the minimum value is 22ms if N1 is 2ms and N is 6ms, the maximum value is near 10240+1280-6=11514ms - LCMTPRS, T measGap×NRxBeam×NPosOccasion∪ TProcess time is the union of the least common multiple of PRS and MG periodicity multiple the number of Rx beam sweeping and Positioning occasion number and PRS processing time. So, the minimum value is (20×8×4+2)ms if NRxBeam=8 , NPosOccasion=4, and the maximum value is near (10240×8×4+1280-6)=328954ms

In short, the latency component and values ranges are summarized in Table 14.

Table 14 The latency component and values range for UE-assisted DL only

| [Case 1], [IIoT/ Commercial], [Frequency Band], [DL-TDOA/AoD] Source [UE, Network]/Destination [UE,Network] Positioning technique [DL-TDOA/AoD], type [DL], mode [UE-A], Initial and Final RRC States [IDLE, INACTIVE, CONNECTED] |  |  |
| --- | --- | --- |
| Latency Component | Value Range (ms) | Description of Latency Component |
| Start trigger | [0.5-1,7.5] | Transmission of the PDSCH from the gNB carrying the LPP Request Location Information message. Which is DL data transmission time - The value can be assumed as 0.5ms-1ms for URLLC case based on [LS: R1-1901470 and 6.4.1, TS 38.824]. - The value can be assumed as 7.5ms for non-URLLC based on [section 5.2.1, TS 36.881]. |
| 1: Successful decoding of the PDSCH carrying the LPP Request Location Information message at the UE side. | 10 | Successful decoding of the PDSCH carrying the LPP Request Location Information message can be equivalent to RRC processing time, which value is 10ms [12, TS 38.331] |
| 2: Transmission of the PUSCH from the UE carrying the measurement gap request message. | [0.5-1, 12.5] | It is equivalent to UL data transmission time - The value also can be assumed as 0.5ms-1ms for URLLC case based on [LS: R1-1901470 and 6.4.1, TS 38.824] - The value can be assumed as 12.5ms for non-URLLC based on [section 5.2.1, TS 36.881]. |
| 3: Successful decoding of the PUSCH carrying the Measurement gap request message at the gNB side. | 10 | Equivalent to RRC processing time, which value is 10ms [12, TS 38.331] |
| 4: Transmission of the PDSCH from the gNB carrying the Measurement gap configuration message. | [0.5-1, 7.5] | It is equivalent to DL data transmission time - The value also can be assumed as 0.5ms-1ms for URLLC case based on [LS: R1-1901470 and 6.4.1, TS 38.824] - The value can be assumed as 7.5ms for non-URLLC based on [section 5.2.1, TS 36.881]. |
| 5: Successful decoding of the PDSCH carrying the Measurement gap configuration at the UE side. | 10 | Equivalent to RRC processing time, which value is 10ms [12, TS 38.331] |
| 6: DL measurement &process delay. | [22~11514] Or [646~328960] | -LCMTPRS, T measGap∪ TProcess time for one occasion and without considering beam sweeping case. - LCMTPRS, T measGap×NRxBeam×NPosOccasion∪ TProcess time for multiple occasion and beam sweeping case It is noted the extra process time is 2ms. |
| 7: Transmission of the PUSCH from the UE carrying the LPP Provide Location Information message. | [0.5-1, 12.5] | It is equivalent to UL data transmission time - The value also can be assumed as 0.5ms-1ms for URLLC case based on [LS: R1-1901470 and 6.4.1, TS 38.824] - The value can be assumed as 12.5ms for non-URLLC based on [section 5.2.1, TS 36.881]. |
| End trigger | 10 | Successful decoding of the PUSCH carrying the LPP Provide Location Information message |
| Total values | 64~ | The minimum total value is 64ms if only consider the minimum value for every step. |

### 3.1.2 The physical layer latency for UE based DL only

For case 2(Network-to-Network)

The latency components with corresponding range of values between Start and End times for Network-to-Network UE based DL only are listed in Table 15 and the PHY procedure as  Figure 4.

Figure 4 The PHY procedure for Network-to-Network UE based DL only Positioning

Table 15 The latency component and values ranges for Network-to-Network UE based DL only

| [Case 2], [IIoT/ Commercial], [Frequency Band], [DL-TDOA/AoD] Source [Network]/Destination [Network] Positioning technique [DL-TDOA], type [DL], mode [UE-B], Initial and Final RRC States [IDLE, INACTIVE, CONNECTED] |  |  |
| --- | --- | --- |
| Latency Component | Value Range (ms) | Description of Latency Component |
| Start trigger | [0.5-1,7.5] | For Physical Layer Latency Start time of UE based method, Alt.2 is selected. Transmission of the PDSCH from the gNB carrying LPP message containing the assistance data. Which is DL data transmission time - The value can be assumed as 0.5ms-1ms for URLLC case based on [LS: R1-1901470 and 6.4.1, TS 38.824]. - The value can be assumed as 7.5ms for non-URLLC based on [section 5.2.1, TS 36.881]. |
| 1: Successful decoding of the PDSCH carrying the LPP Request Location Information message at the UE side. | 10 | Successful decoding of the PDSCH carrying the LPP Request Location Information message can be equivalent to RRC processing time, which value is 10ms [12, TS 38.331] |
| 2: Transmission of the PUSCH from the UE carrying the Measurement gap request message. | [0.5-1, 12.5] | It is equivalent to UL data transmission time - The value also can be assumed as 0.5ms-1ms for URLLC case based on [LS: R1-1901470 and 6.4.1, TS 38.824] - The value can be assumed as 12.5ms for non-URLLC based on [section 5.2.1, TS 36.881]. |
| 3: Successful decoding of the PUSCH carrying the Measurement gap request message at the gNB side. | 10 | Equivalent to RRC processing time, which value is 10ms [12, TS 38.331] |
| 4: Transmission of the PDSCH from the gNB carrying the Measurement gap configuration message. | [0.5-1, 7.5] | It is equivalent to DL data transmission time - The value also can be assumed as 0.5ms-1ms for URLLC case based on [LS: R1-1901470 and 6.4.1, TS 38.824] - The value can be assumed as 7.5ms for non-URLLC based on [section 5.2.1, TS 36.881]. |
| 5: Successful decoding of the PDSCH carrying the Measurement gap configuration at the UE side. | 10 | Equivalent to RRC processing time, which value is 10ms [12, TS 38.331] |
| 6: DL measurement &process delay. | [22~11514] Or [646~328960] | -LCMTPRS, T measGap∪ TProcess time for one occasion and without considering beam sweeping case. - LCMTPRS, T measGap×NRxBeam×NPosOccasion∪ TProcess time for multiple occasion and beam sweeping case It is noted the extra process time is 2ms. |
| 7: Calculation of Location Estimate at the UE | 2 | Calculation of Location Estimate at the UE |
| 8: Transmission of the PUSCH from the UE carrying the LPP Provide Location Information message. | [0.5-1, 12.5] | It is equivalent to UL data transmission time - The value also can be assumed as 0.5ms-1ms for URLLC case based on [LS: R1-1901470 and 6.4.1, TS 38.824] The value can be assumed as 12.5ms for non-URLLC based on [section 5.2.1, TS 36.881]. |
| End trigger | 10 | Successful decoding of the PUSCH carrying the LPP Provide Location Information message |
| Total values | 66~ | The minimum total value is 66ms if only consider the minimum value for every step. |

For case 3 (UE-to-UE)

The latency components with corresponding range of values between Start and End times for UE-to- UE UE based DL only are listed as Table 16 and the PHY procedure as Figure 5.

Figure 5 The PHY procedure for UE-to- UE UE based DL only Positioning

Table 16 the latency component and values ranges for UE-to- UE UE based DL only

| [Case 3], [IIoT/ Commercial], [Frequency Band], [DL-TDOA/AoD] Source [UE]/Destination [UE] Positioning technique [DL-TDOA], type [DL], mode [UE-B], Initial and Final RRC States [IDLE, INACTIVE, CONNECTED] |  |  |
| --- | --- | --- |
| Latency Component | Value Range (ms) | Description of Latency Component |
| Start trigger | [0.5-1,7.5] | Transmission of the PDSCH from the gNB carrying LPP message containing the assistance data. Which is DL data transmission time - The value can be assumed as 0.5ms-1ms for URLLC case based on [LS: R1-1901470 and 6.4.1, TS 38.824]. - The value can be assumed as 7.5ms for non-URLLC based on [section 5.2.1, TS 36.881]. |
| 1: Successful decoding of the PDSCH carrying the LPP Request Location Information message at the UE side. | 10 | Successful decoding of the PDSCH carrying the LPP Request Location Information message can be equivalent to RRC processing time, which value is 10ms [12, TS 38.331] |
| 2: Transmission of the PUSCH from the UE carrying the Measurement gap request message. | [0.5-1, 12.5] | It is equivalent to UL data transmission time - The value also can be assumed as 0.5ms-1ms for URLLC case based on [LS: R1-1901470 and 6.4.1, TS 38.824] - The value can be assumed as 12.5ms for non-URLLC based on [section 5.2.1, TS 36.881]. |
| 3: Successful decoding of the PUSCH carrying the Measurement gap request message at the gNB side. | 10 | Equivalent to RRC processing time, which value is 10ms [12, TS 38.331] |
| 4: Transmission of the PDSCH from the gNB carrying the Measurement gap configuration message. | [0.5-1, 7.5] | It is equivalent to DL data transmission time - The value also can be assumed as 0.5ms-1ms for URLLC case based on [LS: R1-1901470 and 6.4.1, TS 38.824] - The value can be assumed as 7.5ms for non-URLLC based on [section 5.2.1, TS 36.881]. |
| 5: Successful decoding of the PDSCH carrying the Measurement gap configuration at the UE side. | 10 | Equivalent to RRC processing time, which value is 10ms [12, TS 38.331] |
| 6: DL measurement &process delay. | [22~11514] Or [646~328960] | -LCMTPRS, T measGap∪ TProcess time for one occasion and without considering beam sweeping case. - LCMTPRS, T measGap×NRxBeam×NPosOccasion∪ TProcess time for multiple occasion and beam sweeping case It is noted the extra process time is 2ms. |
| 7: Calculation of Location Estimate at the UE | 2 | Calculation of Location Estimate at the UE |
| Total values | 55.5~ | The minimum total value is 55.5ms if only consider the minimum value for every step. |

### 3.1.3 The physical layer latency for UE assisted UL only

For periodic SRS

The latency components with corresponding range of values between Start and End times for UL only positioning with periodic SRS are listed as Table 17 and the PHY procedure as Figure 6.

Figure 6 The PHY procedure for UL only Positioning with periodic SRS

Table 17 The latency component and values range for UL only Positioning with periodic SRS

| [Case 4], [IIoT/ Commercial], [Frequency Band], [UL-TDOA/UL-AoA] Source [UE,Network]/Destination [UE,Network] Positioning technique [UL-TDOA/UL-AoA], type [DL], mode [UE-A], Initial and Final RRC States [IDLE, INACTIVE, CONNECTED] |  |  |
| --- | --- | --- |
| Latency Component | Value Range (ms) | Description of Latency Component |
| Start trigger | 10 | Reception and Successful decode the NRPPa measurement request message from the LMF at gNB, the value is equivalent to RRC processing time, which value is 10ms [12, TS 38.331] |
| 1: UL measurement &process delay. | FR1: 0.5-2560 FR2: 0.125~640 | For case 1: one occasion and without considering beam sweeping - UL measurement equals to the periodicity of SRS - SRS periodicity is {1, 2, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 160, 320, 640, 1280, 2560}slots - 15kHz 1ms-2560ms - 30kHz 0.5ms-1280ms - 60kHz 0.25ms-640ms - 120kHz 0.125ms-320ms For case 2: Multiple positioning occasion and beam sweeping - UL measurement equals to the periodicity of SRS×NRxBeam×NPosOccasion gNB processing delay is assumed as zero |
| End trigger | 0.5 | The transmission by the gNB of the NRPPa measurement response message. Which value less than the DL/UL data transmission time. It can be seen as the gNB processing time (Tprocess time_pdsch) and smaller than 0.5ms. |
| Total values | 30.5~ | The minimum total value is 30.5 ms if the periodicity of SRS is 20ms and the same as the DL minimum periodicity. |

For Aperiodic SRS

The latency components with corresponding range of values between Start and End times for UL only positioning with Aperiodic SRS are listed as Table 18 and the PHY procedure as Figure 7.

Figure 7 The PHY procedure for UL only Positioning with Aperiodic SRS

Table 18 the latency component and values ranges for UL only Positioning with Aperiodic SRS

| [Case 5], [IIoT/ Commercial], [Frequency Band], [UL-TDOA/UL-AoA] Source [UE,Network]/Destination [UE,Network] Positioning technique [UL-TDOA/UL-AoA], type [DL], mode [UE-A], Initial and Final RRC States [IDLE, INACTIVE, CONNECTED] |  |  |
| --- | --- | --- |
| Latency Component | Value Range (ms) | Description of Latency Component |
| Start trigger | 10 | Reception and Successful decode the NRPPa SRS activation and measurement request message from the LMF at gNB, the value is equivalent to RRC processing time, which value is 10ms [12, TS 38.331] |
| 1. Transmission of the PDCCH from the gNB carrying activate SRS message | 0.5 | Transmission of the PDCCH from the gNB carrying activate SRS message. Which value less than the DL/UL data transmission time. It needs to consider Tprocess time_pdcch, Talign time andTPDCCH as below and smaller than 0.5ms. - the gNB processing time (Tprocess time_pdcch )and the alignment delay(Talign time ) - The alignment delay includes the gap between the two consecutive PDCCH monitoring occasions for FDD, the UL/DL configuration for TDD, and the scheduling constraint due to the slot boundaries. - PDCCH duration (TPDCCH ) |
| 2: UL measurement &process delay. | 0-32 | The slot offset of aperiodic is 0-32 slots - 15kHz 0ms-32ms - 30kHz 0ms-16ms - 60kHz 0ms-8ms - 120kHz 0ms-4ms |
| End trigger | 0.5-1 | The transmission by the gNB of the NRPPa measurement response message |
| Total values | 11~ | The minimum total value is 11ms if only considers the minimum value for every step. |

### 3.1.4 The physical layer latency for UE assisted multi-RTT

Considering multi-RTT is the combination of UE assisted DL only and UE assisted UL only, there are many cases need be considered. Such as the combination of periodic PRS with periodic/semi-persistence/aperiodic SRS. In the subsection, let’s take the periodic positioning as an example, to analyze the physical layer latency for  UE assisted multi-RTT.

For the ideal case, the physical layer latency for multi-RTT equals the max(TUE assisted DL only, TUE assisted UL only), in this case, the Start and End trigger for UL only positioning is between DL positioning procedure, and vice versa. For the general case, the physical layer latency for multi-RTT equals the sum(TUE assisted DL only, TUE assisted UL only, TAlign_DL_UL ).TAlign_DL_UL is the gap between End trigger of DL positioning and Start trigger of UL positioning. If TAlign_DL_UL>0, Start trigger timing of UL positioning is later than End trigger timing of DL positioning and the multi-RTT latency will bigger than the sum(TUE assisted DL only, TUE assisted UL only). If - TUE assisted DL only < TAlign_DL_UL<0 Start trigger timing of UL positioning is earlier than End trigger timing of DL positioning and the multi-RTT latency will less than the sum(TUE assisted DL only, TUE assisted UL only). And the latency components with corresponding range of values are listed in Table 19 and the PHY procedure as Figure 8.

Figure 8 The PHY layer latency for Multi-RTT

Table 19 The latency component and values ranges for UE assisted multi-RTT

| [Case 6], [IIoT/ Commercial], [Frequency Band], [Multi-RTT] Source [UE,Network]/Destination [UE,Network] Positioning technique [Multi-RTT], type [DL], mode [UE-A], Initial and Final RRC States [IDLE, INACTIVE, CONNECTED] |  |  |
| --- | --- | --- |
| Latency Component | Value Range (ms) | Description of Latency Component |
| Start trigger | [0.5-1,7.5] | Transmission of the PDSCH from the gNB carrying the LPP Request Location Information message. Which is DL data transmission time - The value can be assumed as 0.5ms-1ms for URLLC case based on [LS: R1-1901470 and 6.4.1, TS 38.824]. - The value can be assumed as 7.5ms for non-URLLC based on [section 5.2.1, TS 36.881]. |
| 1: Successful decoding of the PDSCH carrying the LPP Request Location Information message at the UE side. | 10 | Successful decoding of the PDSCH carrying the LPP Request Location Information message can be equivalent to RRC processing time, which value is 10ms [12, TS 38.331] |
| 2: Transmission of the PUSCH from the UE carrying the Measurement gap request message. | [0.5-1, 12.5] | It is equivalent to UL data transmission time - The value also can be assumed as 0.5ms-1ms for URLLC case based on [LS: R1-1901470 and 6.4.1, TS 38.824] The value can be assumed as 12.5ms for non-URLLC based on [section 5.2.1, TS 36.881]. |
| 3: Successful decoding of the PUSCH carrying the Measurement gap request message at the gNB side. | 10 | Equivalent to RRC processing time, which value is 10ms [12, TS 38.331] |
| 4: Transmission of the PDSCH from the gNB carrying the Measurement gap configuration message. | [0.5-1, 7.5] | It is equivalent to DL data transmission time - The value also can be assumed as 0.5ms-1ms for URLLC case based on [LS: R1-1901470 and 6.4.1, TS 38.824] The value can be assumed as 7.5ms for non-URLLC based on [section 5.2.1, TS 36.881]. |
| 5: Successful decoding of the PDSCH carrying the Measurement gap configuration at the UE side. | 10 | Equivalent to RRC processing time, which value is 10ms [12, TS 38.331] |
| 6: DL measurement &process delay. | [22~11514] Or [646~328960] | -LCMTPRS, T measGap∪ TProcess time for one occasion and without considering beam sweeping case. - LCMTPRS, T measGap×NRxBeam×NPosOccasion∪ TProcess time for multiple occasion and beam sweeping case It is noted the extra process time is 2ms. |
| 7: Transmission of the PUSCH from the UE carrying the LPP Provide Location Information message. | [0.5-1, 12.5] | It is equivalent to UL data transmission time - The value also can be assumed as 0.5ms-1ms for URLLC case based on [LS: R1-1901470 and 6.4.1, TS 38.824] The value can be assumed as 12.5ms for non-URLLC based on [section 5.2.1, TS 36.881]. |
| 8. Successful decoding of the PUSCH carrying the LPP Provide Location Information message | 10 | Successful decoding of the PUSCH carrying the LPP Provide Location Information message |
| 9. The alignment delay is the gap between End trigger of DL positioning and Start trigger of UL positioning | [TBD] | TAlign_DL_UL: The alignment delay is the gap between End trigger of DL positioning and Start trigger of UL positioning. If TAlign_DL_UL>0 , Start trigger timing of UL positioning is later than End trigger timing of DL positioning and the multi-RTT latency will bigger than the sum(TUE assisted DL only, TUE assisted UL only). If - TUE assisted DL only < TAlign_DL_UL<0, Start trigger timing of UL positioning is earlier than End trigger timing of DL positioning and the multi-RTT latency will less than the sum(TUE assisted DL only, TUE assisted UL only). |
| 10. Reception and Successful decode the NRPPa measurement request message from the LMF at gNB | 10 | Reception and Successful decode the NRPPa measurement request message from the LMF at gNB, the value is equivalent to RRC processing time, which value is 10ms [12, TS 38.331] |
| 11: UL measurement &process delay. | 0.125-2560 | For case 1: one occasion and without considering beam sweeping - UL measurement equals to the periodicity of SRS - SRS periodicity is {1, 2, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 160, 320, 640, 1280, 2560} slots - 15kHz 1ms-2560ms - 30kHz 0.5ms-1280ms - 60kHz 0.25ms-640ms - 120kHz 0.125ms-320ms For case 2: Multiple positioning occasion and beam sweeping - UL measurement equals to the periodicity of SRS×NRxBeam×NPosOccasion gNB processing delay is assumed as zero |
| End trigger | 0.5 | The transmission by the gNB of the NRPPa measurement response message. Which value less than the DL/UL data transmission time. It can be seen as the gNB processing time (Tprocess time_pdsch) and smaller than 0.5ms. |
| Total values | 94.5+TAlign_DL_UL |  |

### 3.1.5 Latency analysis for Rel.16 solutions

In conclusion, the performance gap and evaluation results of all the above scenarios and assumptions are summarized in Table 20. And the numerical value in below table is the minimum of the latency for every case and it will be increased with the periodicity.

Table 20 Rel-16 solutions- Physical layer latency summary [X]

| Description Evaluation Case | L1 Latency | Commercial requirements [100]ms are met -Yes/No - If no, provide performance gaps | IIoT requirements of [10ms] are met - Yes/No. If no, provide performance gaps | IIoT requirements of [100]ms are met - Yes/No. If no, provide performance gaps |
| --- | --- | --- | --- | --- |
| [Case 1], [IIoT/ Commercial], [Frequency Band], [DL-TDOA/AoD],[UE-A] | 64ms~ |  | 54ms~ |  |
| [Case 2], [IIoT/ Commercial], [Frequency Band], [DL-TDOA/AoD],[UE-B] Source [Network]/Destination [Network] | 66 ms ~ |  | 56ms~ |  |
| [Case 3], [IIoT/ Commercial], [Frequency Band], [DL-TDOA/AoD],[UE-B] Source [UE]/Destination [UE] | 55.5ms~ |  | 45.5ms~ |  |
| [Case 4], [IIoT/ Commercial], [Frequency Band], [UL-TDOA/UL-AoA], [periodic SRS] | 30.5ms~ |  | 20.5ms~ |  |
| [Case 5], [IIoT/ Commercial], [Frequency Band], [UL-TDOA/UL-AoA], [A- SRS] | 11ms~ |  | 1ms~ |  |
| [Case 6], [IIoT/ Commercial], [Frequency Band], [Multi-RTT] | 94.5+ TAlign_DL_UL ~ |  | 84.5+TAlign_DL_UL~ |  |

Observation 30

- The longer period of positioning reference signal and measurement gap, the greater the latency.

Observation 31

- 10ms physical layer latency cannot be reached with Rel-16  solutions.

Proposal 3

- Physical layer latency needs to be reduced in Rel-17.

## 3.2 The evaluation of physical layer latency for NR enhancements

### 3.2.1 Aperiodic PRS for physical layer latency

For case 7(Aperiodic PRS)

Aperiodic PRS is for reducing the latency of waiting for a period to complete the measurement, which is reducing the latency of step 6 (DL measurement &process delay) in DL positioning. Based on the analyze in 3.1.1, it is observed the latency of DL measurement by Rel-16 solution is the least common multiple of PRS and MG periodicity LCMTPRS,  T measGap and the value range is  {20,40,80,160,320,640,1280,2560,5120,10240} ms. While, for Aperiodic PRS, the latency only includes the PRS scheduling latency (Tprocess time_pdcch, Talign time and TPDCCH) and scheduling offset(TSlot offset), the DL measurement latency will be reduced. And the network efficiency may be reduced considering the network can stop transmit and only transmit the needed PRS.

It is noted that the on-demand PRS has the same characteristics as the aperiodic PRS if the resource type of on-demand PRS is aperiodic. And the details of on-demand are in our contribution [3].

The comparison of step 6 (DL measurement &process delay) between Rel-16 positioning technique and NR enhancement with Aperiodic PRS are listed in Table 21 and Figure 9.

Figure 9 The procedure of DL measurement &process delay (step 6) of Rel-16 positioning technique and NR enhancement with Aperiodic PRS

Table 21 the latency component and values range for step 6 of Rel-16 positioning technique and NR enhancement with Aperiodic PRS

| [Case 7], [IIoT/ Commercial], [Frequency Band], [DL-TDOA/AoD VS Aperiodic PRS] Source [UE]/Destination [UE] Positioning technique [DL-TDOA/AoD], type [DL], mode [UE-B], Initial and Final RRC States [IDLE, INACTIVE, CONNECTED] |  |  |  |
| --- | --- | --- | --- |
| Latency Component | Rel-16 positioning technique Value Range (ms) | NR enhancement with on-demand PRS/Aperiodic PRS | Description of Latency Component of NR enhancement with Aperiodic PRS |
| 6: DL measurement &process delay. | [22~11514] Or [646~328960] | 2.5-34.5 | - Dynamic Scheduling latency - Transmission of the PDCCH from the gNB carrying activate aperiodic/on-demand PRS message. Which value less than the DL/UL data transmission time, including Tprocess time_pdcch, Talign time andTPDCCH and smaller than 0.5ms. - Scheduling offset(TSlot offset) - The slot offset can be 0-32 slots if the scheduling offset of SRS and PRS are consistent - Process delay is assumed as 2ms |

Observation 32

- DL measurement &process delay will be reduced by Aperiodic PRS, and the physical layer latency is reduced by nearly 20ms.

### 3.2.2 Positioning BWP/on-demand MG for physical layer latency

For case 8 (on-demand /aperiodic MG)

On-demand /aperiodic MG is for reducing the latency of waiting for a period of measurement gap to complete the measurement and the latency of measurement requesting and measurement configuration (step 2-5). Based on the analyze in 3.1.1, it is observed the latency of DL measurement (step 6) by Rel-16 solution is the least common multiple of PRS and MG periodicityLCMTPRS,  T measGap and the value range is  {20,40,80,160,320,640,1280,2560,5120,10240} ms. While the latency of  DL measurement  by on-demand /aperiodic MG doesn’t need to consider the periodicity of MG and equals the periodicity of TPRS , which value is less than LCMTPRS,  T measGap, and is  4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 160, 320, 640, 1280, 2560, 5120, 10240 ms. Besides, the latency of measurement requesting and measurement configuration(that is step 2-5 in 3.1.1) is 21ms. While, for on-demand /aperiodic MG, the latency only includes the MG scheduling latency (Tprocess time_pdcch,Talign time and TPDCCH) will reduce the latency by 20.5ms.

The comparison of step 2-6 between Rel-16 positioning technique and NR enhancement with on-demand MG/Aperiodic MG are listed in Table 22 and Figure 10.

Figure 10 The procedure of step 2-6 of Rel-16 positioning technique and NR enhancement with on-demand /Aperiodic MG

Table 22 the latency component and values range for step 2-6 of Rel-16 positioning technique and NR enhancement with on-demand/Aperiodic MG

| [Case 8], [IIoT/ Commercial], [Frequency Band], [DL-TDOA/AoD VS on-demand/aperiodic MG] Source [UE]/Destination [UE] Positioning technique [DL-TDOA/AoD], type [DL], mode [UE-B], Initial and Final RRC States [IDLE, INACTIVE, CONNECTED] |  |  |  |
| --- | --- | --- | --- |
| Latency Component | Rel-16 positioning technique Value Range (ms) | NR enhancement with on-demand/Aperiodic MG | Description of Latency Component of NR enhancement with on-demand MG/Aperiodic MG |
| 2: Transmission of the PUSCH from the UE carrying the Measurement gap request message. | [0.5-1, 12.5] | 0.5 | - Dynamic Scheduling latency - Transmission of the PDCCH from the gNB carrying activate aperiodic/on-demand MG message. Which value less than the DL/UL data transmission time, including Tprocess time_pdcch, Talign time andTPDCCH and smaller than 0.5ms. |
| 3: Successful decoding of the PUSCH carrying the Measurement gap request message at the gNB side. | 10 |  |  |
| 4: Transmission of the PDSCH from the gNB carrying the Measurement gap configuration message. | [0.5-1, 7.5] |  |  |
| 5: Successful decoding of the PDSCH carrying the Measurement gap configuration at the UE side. | 10 |  |  |
| 6: DL measurement &process delay. | [22~11514] Or [646~328960] | [6~10240] Or [130~32770] | - TPRS, which value is {4,5,8,10,16,20,32,40,64,80,160,320,640,1280,2560,5120, 10240}ms - Process delay is assumed as 2ms |

Observation 33

- DL measurement delay is impacted by PRS periodicity instead of the least common multiple of PRS and MG periodicity if on-demand /aperiodic MG is introduced, and the physical layer latency is reduced by nearly 16 ms for DL measurement.

- The latency of measurement requesting and measurement configuration can also be reduced by on-demand /aperiodic MG. The physical layer latency is reduced by nearly 20ms for measurement requesting and measurement configuration if on-demand /aperiodic MG is introduced.

For case 9 (Positioning BWP)

Positioning BWP  is similar to on-demand /aperiodic MG for reducing the latency of waiting for a period of measurement gap to complete the measurement and the latency of measurement requesting and measurement configuration(step 2-5).

The comparison of step 2- 6 between Rel-16 positioning technique and NR enhancement with Positioning BWP are listed in Table 23 and in Figure 11.

Figure 11 The procedure of step 2-6 of Rel-16 positioning technique and NR enhancement with Positioning BWP

Table 23 the latency component and values range for step 2-6 of Rel-16 positioning technique and NR enhancement with Positioning BWP

| [Case 9], [IIoT/ Commercial], [Frequency Band], [DL-TDOA/AoD VS Positioning BWP] Source [UE]/Destination [UE] Positioning technique [DL-TDOA/AoD], type [DL], mode [UE-B], Initial and Final RRC States [IDLE, INACTIVE, CONNECTED] |  |  |  |
| --- | --- | --- | --- |
| Latency Component | Rel-16 positioning technique Value Range (ms) | NR enhancement with positioning BWP | Description of Latency Component of NR enhancement with Positioning BWP |
| 2: Transmission of the PUSCH from the UE carrying the Measurement gap request message. | [0.5-1, 12.5] | 1.5 | - Dynamic Scheduling latency - Dynamic Scheduling latency. Transmission of the PDCCH from the gNB carrying activate Positioning BWP message. Whose value is less than the DL/UL data transmission time, including Tprocess time_pdcch, Talign time andTPDCCH and smaller than 0.5ms. - BWP switching latency. The value of BWP switching time can refer to Table 8.6.2-1 in TS38.133. Here we take a typical value of 1ms as the BWP switching latency. |
| 3: Successful decoding of the PUSCH carrying the Measurement gap request message at the gNB side. | 10 |  |  |
| 4: Transmission of the PDSCH from the gNB carrying the Measurement gap configuration message. | [0.5-1, 7.5] |  |  |
| 5: Successful decoding of the PDSCH carrying the Measurement gap configuration at the UE side. | 10 |  |  |
| 6: DL measurement &process delay. | [22~11514] Or [646~328960] | [6~10240] Or [130~32770] | - TPRS, which value is {4,5,8,10,16,20,32,40,64,80,160,320,640,1280,2560,5120, 10240}ms - Process delay is assumed as 2ms |

Observation 34

- DL measurement delay is impacted by PRS periodicity instead of the least common multiple of PRS and MG periodicity if Positioning BWP is introduced, and the physical layer latency is reduced by nearly 16 ms for DL measurement.

- The latency of measurement requesting and measurement configuration can also be reduced by Positioning BWP. The physical layer latency is reduced by nearly 20ms for measurement requesting and measurement configuration if Positioning BWP is introduced.

### 3.2.3 Low layer triggered positioning requesting/ positioning reporting for physical layer latency

For case 10 (low layer triggered positioning requesting/ positioning reporting)

Low layer triggered positioning requesting/ positioning reporting is for reducing the latency of decoding of the PDSCH carrying the LPP Request Location Information message at the UE side (step 1 in 3.1.1) and the PUSCH carrying the LPP Provide Location Information message (End trigger in 3.1.1). Based on the analysis in 3.1.1, it is observed the latency of step 1 and End trigger step by Rel-16 solution is 20ms. While the latency of Low layer triggered positioning requesting/ positioning reporting doesn’t need to consider the RRC process time, which value of step 1 and End trigger step is 0 if the physical layer triggered, and is 3ms if the MAC layer triggered. The enhancement will reduce the latency by 13~20ms.

The comparison of step (Start trigger, 1, 7, End trigger) between Rel-16 positioning technique and NR enhancement with Low layer triggered positioning requesting/ positioning reporting are listed in Table 24 and Figure 12.

Figure 12 The procedure of step (Start trigger, 1, 7, End trigger) of Rel-16 positioning technique and NR enhancement with physical layer triggered requesting/reporting

Table 24 The latency component and values range for step 2-6 of Rel-16 positioning technique and NR enhancement with physical layer triggered requesting/reporting

| [Case 10], [IIoT/ Commercial], [Frequency Band], [DL-TDOA/AoD VS physical layer triggered] Source [UE]/Destination [UE] Positioning technique [DL-TDOA/AoD], type [DL], mode [UE-B], Initial and Final RRC States [IDLE, INACTIVE, CONNECTED] |  |  |  |
| --- | --- | --- | --- |
| Latency Component | Rel-16 positioning technique Value Range (ms) | NR enhancement with Low layer triggered positioning requesting/ positioning reporting | Description of Latency Component of NR enhancement with Low layer triggered positioning requesting/ positioning reporting |
| Start trigger | [0.5-1,7.5] | 0.5 ms | - Dynamic Scheduling latency for positioning request - Transmission of the PDCCH from the gNB carrying Positioning requesting message. Which value less than the DL/UL data transmission time, including Tprocess time_pdcch, Talign time andTPDCCH and smaller than 0.5ms. |
| 1: Successful decoding of the PDSCH carrying the LPP Request Location Information message at the UE side. | 10 |  |  |
| 7: Transmission of the PUSCH from the UE carrying the LPP Provide Location Information message. | [0.5-1, 12.5] | 0.5ms | - PUCCH latency for positioning reporting - Transmission of the PUCCH from the UE carrying measurement/location message. Which value less than the DL/UL data transmission time, including Tprocess time_pucch, Talign time andTPUCCH and smaller than 0.5ms. |
| End trigger | 10ms |  |  |

Observation 35

- Positioning requesting/ positioning reporting doesn’t need to consider the RRC process time if low layer triggered Positioning requesting/ positioning reporting is supported, and the physical layer latency is reduced by nearly 20 ms.

### 3.2.4 Combination of the above enhancement schemes

According to the analysis of 3.2.1~3.2.3, it is observed that every scheme only works for some parts of the whole positioning procedure. For case 11, we analyze all the above schemes to evaluate the minimum latency with NR enhancements. And the latency components with corresponding range of values of NR enhancement are listed in Table 25 and the PHY procedure in Figure 13.

Figure 13 The PHY layer latency for NR enhancements

Table 25 The latency component and values range for NR enhancements

| [Case 11], [IIoT/ Commercial], [Frequency Band], [DL-TDOA/AoD] Source [UE, Network]/Destination [UE,Network] Positioning technique [DL-TDOA/AoD], type [DL], mode [UE-A], Initial and Final RRC States [IDLE, INACTIVE, CONNECTED] |  |  |
| --- | --- | --- |
| Latency Component | Value Range (ms) | Description of Latency Component |
| Start trigger | 0.5ms | - Dynamic Scheduling latency for a positioning request - Transmission of the PDCCH from the gNB carrying Positioning requesting message. Which value less than the DL/UL data transmission time, including Tprocess time_pdcch, Talign time andTPDCCH and smaller than 0.5ms |
| 1: Transmission of the PDCCH from the gNB carrying activate aperiodic/on-demand MG/Positioning BWP switch message | 0.5 ms/1.5ms | - Dynamic Scheduling latency for activating aperiodic/on-demand MG/Positioning BWP switch - Transmission of the PDCCH from the gNB carrying activate aperiodic/on-demand MG/Positioning BWP switch message. Which value less than the DL/UL data transmission time, including Tprocess time_pdcch, Talign time andTPDCCH and smaller than 0.5ms. - BWP switching latency: 1ms |
| 2.DL measurement &process delay. | 2.5-34.5 | - Dynamic Scheduling latency for activating aperiodic/on-demand PRS - Transmission of the PDCCH from the gNB carrying activate aperiodic/on-demand PRS message. Which value less than the DL/UL data transmission time, including Tprocess time_pdcch, Talign time andTPDCCH and smaller than 0.5ms. - Scheduling offset(TSlot offset) - The slot offset can be 0-32 slots if the scheduling offset of SRS and PRS are consistent - Process delay is assumed as 2ms |
| End trigger | 0.5 ms | - PUCCH latency for positioning reporting - Transmission of the PUCCH from the UE carrying measurement/location message. Which value less than the DL/UL data transmission time, including Tprocess time_pucch, Talign time andTPUCCH and smaller than 0.5ms. |
| Total values | 5 ms~ | The minimum total value is 5 ms if only consider the minimum value for every step. |

### 3.2.5 Observations on NR positioning latency enhancements

Table 26 NR positioning enhancements - physical layer latency performance summary [X]

| Description Evaluation Case | L1 Latency for UE-A | Gain over Rel-16 | Commercial requirements [100]ms are met Yes/No. If no, provide performance gaps | IIoT requirements of [10]ms are met Yes/No. If no, provide performance gaps | IIoT requirements of [100]ms are met Yes/No. If no, provide performance gaps |
| --- | --- | --- | --- | --- | --- |
| [Case 7], [on-demand/aperiodic PRS] | 44.5ms~ | 19.5ms~ |  | 34.5ms~ |  |
| [Case 8], [on-demand/aperiodic MG] | 27.5ms~ | 36.5ms~ |  | 17.5ms~ |  |
| [Case 9], [Positioning BWP] | 28.5ms~ | 35.5ms~ |  | 18.5ms~ |  |
| [Case 10], [physical layer triggered] | 44ms~ | 20ms~ |  | 32ms~ |  |
| [Case 11], [combination scheme] | 5ms~ | 59ms~ |  | Yes |  |

According to the above analysis and evaluation result, the different technique of case7-case 11 can reduce the physical layer latency,

Observation 36

- 10ms physical layer latency can be reached with the combination of on-demand/aperiodic PRS, on-demand/aperiodic MG, Positioning BWP, and physical layer triggered request/report.

Proposal 4

- On-demand/aperiodic PRS, on-demand/aperiodic MG, Positioning BWP, and physical layer triggered need to be studied in Rel-17 for reducing latency.

## 3.3 The latency of Idle/ inactive to the connected mode

In the current specification, the positioning is only performed when UE in the connection mode. So if UE in the idle or inactive, the latency for position estimation of UE is the sum latency of initial access and positioning. Firstly, the latency of the initial access as below:

| Initial connection procedure | The latency |
| --- | --- |
| SSB | 20ms |
| SIB | 20ms |
| PRACH | 10ms-160ms |
| RAR | 10ms |
| Msg3 | 5ms+∆ |
| Msg4 | 5ms+∆ |
| Msg5 | 5ms+∆ |

It can be observed that the initial access nearly needs 40ms~200ms. So if the UE in the idle/ inactive state, nearly 40ms~200ms is needed to come to the connected state before positioning.

Observation 37

- Additional latency of 40~200ms will be introduced if the UE switches to connected state from idle state for positioning measurement and report.

- Network and UE efficiency

## 4.1 The evaluation of the network and UE efficiency for Rel.16 solutions

In the subsection, we firstly analyze the network and UE efficiency from the overhead aspect for Rel-16 solutions. For network efficiency, it is defined as the percentage of PRS in total transmission and the detailed in our contribution [2]. For UE efficiency, it is defined as the percentage of the MGL in the MG periodicity. And the MGL is chosen from{1.5, 3, 3.5, 4, 5, 5.5, 6, 10,20}ms, which includes the measurement time and the gap switching time.

In general, the FR2 needs more beams and symbols than FR1 even though the number of TRP is the same. For example, network and UE efficiency in terms of the percentage of PRS in the total transmission being analyzed as Table 28 based on the below assumptions listed in Table 27.

Table 27 The parameter assumption for evaluation

| Parameter | Configuration |
| --- | --- |
| Reference Signal Transmission Bandwidth | 100 MHz for FR1 400 MHz for FR2 |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) | Rel-16 PRS within a slot (comb-6 frequency structure, 6 symbols within a slot) |
| Number of sites | 18 |
| Number of beams | 1 for FR1 64 for FR2 |

Table 28 The network and UE efficiency for different physical layer latency

| u | TPRS (ms) | Tmeas Gap ms | Physical layer Latency (ms) (case 3 in 3.1.3) | MGL ms For max{TPRS, Tmeasuement gap} | Network efficiency (percentage) | UE efficiency |
| --- | --- | --- | --- | --- | --- | --- |
| 30K (FR1) | 4 | 20 | 55.5 | 3ms | 16.07% | 15.00% |
|  | 5 | 20 | 55.5 | 3ms | 12.85% | 15.00% |
|  | 8 | 40 | 75.5 | 3ms | 8.03% | 7.50% |
|  | 10 | 20 | 55.5 | 3ms | 6.42% | 15.00% |
|  | 16 | 80 | 115.5 | 3ms | 4.01% | 3.75% |
|  | 20 | 20 | 55.5 | 3ms | 3.21% | 15.00% |
|  | 32 | 160 | 195.5 | 3ms | 2.0% | 1.88% |
|  | 40 | 40 | 75.5 | 3ms | 1.6% | 7.50% |
|  | 64 | 160 | 355.5 | 3ms | 1.0% | 1.88% |
|  | 80 | 80 | 115.5 | 3ms | 0.8% | 3.75% |
|  | 160 | 160 | 195.5 | 3ms | 0.4% | 1.88% |
|  | 320 | 160 | 355.5 | 3ms | 0.2% | 1.88% |
|  | 640 | 160 | 675.5 | 3ms | 0.1% | 1.88% |
|  | 1280 | 160 | 1315.5 | 3ms | 0.05% | 1.88% |
|  | 5120 | 160 | 5155.5 | 3ms | 0.01% | 1.88% |
|  | 10240 | 160 | 10275.5 | 3ms | 0.006% | 1.88% |
| 120K (FR2) | 4 | 20(160Note) | 55.5 | 20ms | 257.14% | 100.00% |
|  | 5 | 20(160) | 55.5 | 20ms | 205.71% | 100.00% |
|  | 8 | 40(160) | 75.5 | 20ms | 128.57% | 50.00% |
|  | 10 | 20(160) | 55.5 | 20ms | 102.85% | 50.00% |
|  | 16 | 80(160) | 115.5 | 20ms | 64.28% | 25% |
|  | 20 | 20(160) | 55.5 | 20ms | 51.42% | 100.00% |
|  | 32 | 160(160) | 195.5 | 20ms | 32.14% | 12.5% |
|  | 40 | 40(160) | 75.5 | 20ms | 25.71% | 50.00% |
|  | 64 | 160(160) | 355.5 | 20ms | 16.07% | 12.5% |
|  | 80 | 80(160) | 115.5 | 20ms | 12.85% | 12.50% |
|  | 160 | 160(160) | 195.5 | 20ms | 6.42% | 12.5% |
|  | 320 | 160(160) | 355.5 | 20ms | 3.21% | 12.5% |
|  | 640 | 160(160) | 675.5 | 20ms | 1.60% | 12.5% |
|  | 1280 | 160(160) | 1315.5 | 20ms | 0.80% | 12.5% |
|  | 5120 | 160(160) | 5155.5 | 20ms | 0.20% | 12.5% |
|  | 10240 | 160(160) | 10275.5 | 20ms | 0.10% | 12.5% |

Note: In practice, 20ms MGL is only for 160ms periodicity. So the real PHY latency is longer than 160ms considering 20ms MGL is needed for FR2. But in this table, the required network and UE efficiency are evaluated assuming it meets the corresponding Physical layer Latency in the table.

For FR1, MGL is the minimum length of MGL that is longer than TRPNum×NumSymbolsCombSizeN symbolNumOfms+2×gap switching time=1.6ms , that is 3ms, while FR2 is 20 ms.

For FR1

Network efficiency = TRPNum×NumSymbolsCombSizeNPRS periodicity×symbolNumOfms×100%= 18*6/6PRS periodicity×28×100%

UE efficiency=MGLMG periodicity×100%=MGLMG periodicity×100%

For FR2, the number of beams in the calculation of the overhead in FR2 is 64, so

Network efficiency = TRPNum×NumSymbolsCombSizeN×BeamNumPRS periodicity×symbolNumOfms×100%= 18×6/6×64PRS periodicity×112×100%

UE efficiency=MGLMG periodicity×100%=MGLMG periodicity×100%

Recall that there’s an agreement in previous RAN1 meeting regarding measurement gap for Rel-16 UE.

Agreement:

- For the purpose of DL PRS processing capability with measurement gap, the maximum value of X = MGL/MGRP supported in specification should not exceed 30%

Based on the evaluation and the above agreement, it is observed that the highlighted line with red color in Table 28 can’t be achieved, due to the network efficiency exceeds 100%, or the MGL/MGRP(UE efficiency) exceed 30%.

Observation 38

- The network efficiency exceeds 100% in some FR2 cases.

- The MGL/MGRP (UE efficiency) exceeds 30% in some FR2 cases.

Proposal 5

- The network efficiency and UE efficiency for low latency positioning needs to be enhanced.

## 4.2 The evaluation of the network and UE efficiency for NR enhancement

### 4.2.1 On-demand PRS for network and UE efficiency

Agreement: - Semi-persistent and a-periodic transmission and reception of DL PRS will be investigated in Rel-17. - FFS: the details on when and how to enable semi-persistent and a-periodic DL PRS - FFS: to be supported for which positioning methods, e.g., - UE-assisted and/or UE-based positioning - DL positioning and/or Multi-RTT - On-demand transmission and reception of DL PRS will be investigated in Rel-17. - FFS: the details on when and how to enable on-demand DL PRS - FFS: to be supported for which positioning methods, e.g., - UE-assisted and/or UE-based positioning - DL positioning and/or Multi-RTT - Notes: - Semi-persistent means MAC-CE triggered - Aperiodic would correspond to DCI-triggered - On-demand corresponds to the UE-initiated or network-initiated request of PRS and/or SRS. So, it is NOT the same as whether PRS is DCI-triggered or MAC-CE triggered. It is about UE or LMF request/suggesting/recommending specific PRS pattern, ON/OFF, periodicity, BW, etc.

In this section, we analyze the performance of on-demand PRS for network and UE efficiency, where on-demand PRS only considers the specific PRS pattern, ON/OFF, periodicity. The network and device efficiency between the on-demand PRS and periodic PRS are compared as below in Table 29.

Table 29 The network and device efficiency of the on-demand PRS and periodic PRS in FR2

| Option | Periodic PRS |  | On-demand PRS |
| --- | --- | --- | --- |
| Configuration | Period 10ms | Period 160ms | Period 10ms, repetition :16 Muting and MG for on-demand service |
| PHY Latency (minimum) | Latencymin= 53.5ms Where DL measurement &process delay equals MGL(20ms). |  | Latencymin=37.5ms Where DL measurement &process delay equals MGL for minimum TRP(such as 4ms). |
| PHY Latency (maximum) (case 3 in 3.1.3) | Latencymax=55.5ms | #### Latencymax=195.5ms | Latencymax=55.5ms |
| Network efficiency | TRPNum×NumSymbolsCombSizeN×beamNumPRS periodicity×symbolNumOfms =51.42% | 6.42% | i=015Boolean(Yi>0)* TRPNum* NumSymbols / CombSizeN160 =0~51.4% |
| UE efficiency | MGLMG periodicity #### =100% | 12.5% | i=015Boolean(Yi>0)*MGL160 =0~4ms*16/160 =0%~40% |

Note 1: the parameter assumption is summarized in Annex.

In this table, 160ms is the evaluation period. For periodic PRS, whether UE needs or not, the PRS needs transmitting based on the period, such as 10ms or 160ms. It is observed that the PHY latency of period 10ms is smaller and the network and UE efficiency is higher than period 160ms. The on-demand PRS can adjust the configuration of PRS based on the requirement. For example, on-demand PRS can determine which repetition and TRP transmitting if assumed 16 repetitions in 160ms for every TRP. The minimum Network and UE efficiency of on-demand PRS is 0% if no positioning requesting in 160ms or no positioning requesting for the UE. It is observed that the network and device efficiency will be reduced by on-demand PRS within the same level latency.

Observation 39

- The network and device efficiency will be reduced by on-demand PRS within the same level latency compared to periodic PRS.

### 4.2.2 Aperiodic PRS for network and UE efficiency

The network and UE efficiency of aperiodic PRS are affected by the number of activations within units time (such as 160ms). It can be observed the network and UE efficiency of aperiodic PRS less than the Periodic PRS with 160ms periodicity if the number of activations less than 1 and less than the Periodic PRS with 10ms periodicity if the number of activations less than 10. Meanwhile, the latency can be reduced by nearly 20ms.

It is noted that the on-demand PRS has the same characteristics as the aperiodic PRS if the resource type of on-demand PRS is aperiodic.

Table 30 The network and device efficiency of the aperiodic PRS and periodic PRS in FR2

| Option | Periodic PRS |  | A-PRS |
| --- | --- | --- | --- |
| Configuration | Period 10ms | Period 160ms | SlotOffset INTEGER (1..32) |
| PHY Latency (minimum) | Latencymin= 53.5ms Where DL measurement &process delay equals MGL(20ms). |  | Latencymin=36ms Case 7 in 3.2.1,where DL measurement &process delay equals 2.5ms |
| PHY Latency (maximum) | Latencymax=55.5ms (case 3 in 3.1.3) | Latencymax=195.5ms (case 3 in 3.1.3 where TPRS=160ms) | Latencymax=39.5ms Case 7 in 3.2.1, where DL measurement &process delay equals 6ms if scs is 120kHZ |
| Network efficiency | TRPNum×NumSymbolsCombSizeN×beamNumPRS periodicity×symbolNumOfms =51.42% | 6.42% | NumActivation×TRPNum×NumSymbolsCombSizeN×beamNum160ms×symbolNumOfms = NumActivation×6.42% |
| UE efficiency | MGLMG periodicity =100% | 12.5% | = NumActivation×12.5% |

Observation 40

- The network and device efficiency of aperiodic PRS is multiple of the number of activations.

## 4.3 The evaluation of power consumption for NR enhancement

In this section, we present our evaluation results based on our power consumption model in our companion contribution [2].

### 4.3.1 Baseline evaluation of UE power consumption for positioning

In this subsection, we evaluate baseline UE power consumption for the procedure of PRS measurement in idle state and connected state.

Idle state power consumption (baseline)

We model the positioning measurements in idle state with following power components:

- PRS occasion

- For simplicity, UE only performs intra-frequency PRS measurements every I-DRX cycle (1280ms)

- 4ms PRS length, 1PRS occasion

- 20MHz

- Relative power: 96

- Paging occasion

- 4ms length

- PDCCH only or PDCCH+PDSCH

- Relative power: 50 for PDCCH only; 120 for PDCCH+PDSCH

- Note1: for PDCCH-only, no PDSCH and same-slot scheduling; this includes time for PDCCH decoding and any micro-sleep within the slot;  for PDCCH+PDSCH, it is used when the UE decodes the PDCCH for paging and obtains the need to decode the corresponding PDSCH

- Note2: PDCCH only is assumed as baseline in idle state

- SSB burst for inter-frequency RRM measurement

- 5ms window after paging occasion, 1 SSB burst

- UE performs inter-frequency RRM every I-DRX cycle (1280ms)

- Relative power: 60

- SSB burst for serving/intra-frequency RRM measurement

- 2ms window before paging occasion, 1 SSB burst

- UE performs intra-frequency RRM every I-DRX cycle (1280ms)

- SSB burst period: 20ms

- Relative power: 60

- SSB burst for fine time-frequency sync. and RSRP measurement of serving/camping cell

- 2ms window before paging occasion

- 3 SSB bursts: 2 SSB burst is used standalone, 1 SSB burst reuse SSB burst for serving/intra-frequency RRM measurement

- Relative power: 50

- Sleep

- Sleep type is determined by duration between two power conponients above

- Deep sleep: duration>20ms

- Relative power: 1, transition energy: 450

- Light sleep: duration=6~20ms

- Relative power: 20, transition energy: 100

- Micro sleep: duration<6ms

- Relative power: 45, transition energy: 0

A procedure of PRS measurement in idle state is shown in below Figure 14 .

Figure 14  Procedure of PRS measurements in idle state

Then we analyze the time of multiple power components and calculate the average energy consumption, the specific content can refer to the following Table 31.

Table 31  Power components analysis for PRS measurement in idle state

| Power state | Relative power | Duration(ms) |
| --- | --- | --- |
| Deep sleep | 1 | 1215 |
| Light sleep | 20 | 41.5 |
| Micro sleep | 45 | 4.5 |
| SSB for Inter-frequency measurement | 60 | 5 |
| SSB for Intra-frequency measurement | 60 | 2 |
| SSB Proc. | 50 | 2+2 |
| Single positioning frequency layer measurement | 96 | 4 |
| Paging Occasion | 50 | 4 |
| Sleep transition type | Transition energy | Transition times |
| Deep sleep transition | 450 | 1 |
| Light sleep transition | 100 | 4 |
| Average power | 3.3605 |  |

Connected state power consumption (baseline)

We also model the  positioning measurements in connected model with following power components:

- PRS occasion

- For simplicity, UE only performs intra-frequency PRS measurements every C-DRX cycle (160ms)

- 4ms PRS length, 1PRS occasion

- 100MHz

- Relative power: 610

- Measurement gap for PRS measurement

- Length: 5ms

- Period: 160ms

- Total switching time: 1ms

- Relative power for switching: 45

- PDCCH only monitoring during active time

- For simplicity, only performs PDCCH decoding, traffic model is not introduced

- 4ms length

- Relative power: 100

- Note:  For PDCCH-only, no PDSCH and same-slot scheduling; this includes time for PDCCH decoding and any micro-sleep within the slot.

- SSB burst for inter-frequency RRM measurement

- 5ms window after PDCCH monitoring, 1 SSB burst

- UE performs inter-frequency RRM every C-DRX cycle (160ms)

- Relative power: 150

- SSB burst for serving/intra-frequency RRM measurement

- 2ms window before PDCCH monitoring, 1 SSB burst

- UE performs intra-frequency RRM every C-DRX cycle (160ms)

- SSB burst period: 20ms

- Relative power: 150

- SSB burst for fine time-frequency sync. and RSRP measurement of serving/camping cell

- 2ms window before PDCCH monitoring

- 2 SSB bursts: 1 SSB burst is used standalone, 1 SSB burst reuse SSB burst for serving/intra-frequency RRM measurement

- Relative power: 100

- Sleep

A procedure for PRS measurement in connected state is shown in Figure 15 below. Based on the above assumptions and analysis, Table 32 shows the power consumption for PRS measurement in connected state.

Figure 15  Procedure of PRS measurements in connected state

Table 32  Power components analysis for PRS measurement in connected state

| Power state | Relative power | Duration(ms) |
| --- | --- | --- |
| Deep sleep | 1 | 115 |
| Light sleep | 20 | 23 |
| Micro sleep | 45 | 4 |
| SSB for Inter-frequency measurement | 150 | 5 |
| SSB for Intra-frequency measurement | 150 | 2 |
| SSB Proc. | 100 | 2 |
| Single positioning frequency layer measurement | 610 | 4 |
| Gap switching | 45 | 1 |
| PDCCH-only monitoring | 100 | 4 |
| Sleep transition type | Transition energy | Transition times |
| Deep sleep transition | 450 | 1 |
| Light sleep transition | 100 | 3 |
| Average power | 35.2500 |  |

The above cases for both idle and connected mode are typical examples for PRS measurement power evaluation. These examples can be used as a baseline for subsequent evaluation in next 2 subsections.

### 4.3.2 Evaluation for different PRS configurations

In this subsection, we evaluated the power consumption under different PRS configurations and focus on power consumption in connected state.

Extending PRS period

As we all know, increasing the period of PRS can reduce the power consumption of PRS measurement. Therefore, we extend PRS period 2 times (320ms). The procedure and power components analysis of PRS measurements  are shown in Figure 16 and Table 33 respectively.

Figure 16 Procedure of PRS measurements in DRX cycle with PRS and DRX cycle without PRS in connected state

Table 33  Power components analysis for PRS measurement by extending PRS period in connected state

| Power state | Relative power | Duration(ms) |  |
| --- | --- | --- | --- |
|  |  | PRS period(320ms) | PRS period(640ms) |
| Deep sleep | 1 | 230 | 460 |
| Light sleep | 20 | 55 | 119 |
| Micro sleep | 45 | 4 | 4 |
| SSB for Inter-frequency measurement | 150 | 10 | 20 |
| SSB for Intra-frequency measurement | 150 | 4 | 8 |
| SSB Proc. | 100 | 4 | 8 |
| Single positioning frequency layer measurement | 610 | 4 | 4 |
| Gap switching | 45 | 1 | 1 |
| PDCCH-only monitoring | 100 | 8 | 16 |
| Sleep transition type | Transition energy | Transition times | Transition times |
| Deep sleep transition | 450 | 2 | 4 |
| Light sleep transition | 100 | 6 | 12 |
| Calculation |  |  |  |
| DRX cycle | - | 160 | 160 |
| Average power | - | 27.4844 | 23.6016 |
| Power saving gain | - | 22.03% | 33.05% |

It can be observed that,

- By extending the PRS period to 2 times(160ms to 320ms), 22.03% power saving gain is shown ,comparing with the baseline assumption.

- By extending the PRS period to 4 times(160ms to 640ms), 33.05 % power saving gain is shown ,comparing with the baseline assumption.

Therefore, for stationary or low mobility UEs, properly increasing the PRS period can obtain power saving gains.

Observation 41

- By extending the PRS period to 2 times(160ms to 320ms), 22.03% power saving gain is shown ,comparing with the baseline assumption

- By extending the PRS period to 4 times(160ms to 640ms), 33.05 % power saving gain is shown ,comparing with the baseline assumption

Concentrated PRS distribution

In this subsection, we compare two PRS distribution methods, namely concentrated PRS (baseline, as in Figure 15) and distributed PRS (as in Figure 17), which is based on our power consumption model, method and assumptions. The duration of concentrated PRS distribution is 5ms with 4ms PRS length and 1ms MG switching time. While for distributed PRS, we divide the concentrated PRS occasion of 4ms (baseline) into 4 PRS occasions with 1ms, and the adjacent PRS occasions are separated by 40ms. In this case, 4 measurement gap occasion with 2ms duration are assumed. Then we compare the power consumption performance of the 2 cases in one DRX cycle as shown in Table 34.

Figure 17 Procedure of distributed PRS measurements in one DRX cycle

Table 34 Power components analysis for concentrated and distributed PRS measurement

| Power state | Relative power | Duration(ms) |  |
| --- | --- | --- | --- |
|  |  | Concentrated PRS measurement (baseline) | Distributed PRS measurement |
| Deep sleep | 1 | 115 | 103 |
| Light sleep | 20 | 23 | 36 |
| Micro sleep | 45 | 4 | 0 |
| SSB for Inter-frequency measurement | 150 | 5 | 5 |
| SSB for Intra-frequency measurement | 150 | 2 | 2 |
| SSB Proc. | 100 | 2 | 2 |
| Single positioning frequency layer measurement | 610 | 4 | 4 |
| Gap switching | 45 | 1 | 4 |
| PDCCH-only monitoring | 100 | 4 | 4 |
| Sleep transition type | Transition energy | Transition times | Transition times |
| Deep sleep transition | 450 | 1 | 3 |
| Light sleep transition | 100 | 3 | 5 |
| Calculation |  |  |  |
| DRX cycle | - | 160 | 160 |
| Average power | - | 35.2500 | 43.3937 |
| Power saving gain | - | 18.77% | 0 |

It can be seen that when configuring concentrated PRS measurement (1 concentrated PRS occasion every 160ms), 18.77% power saving gain is shown, comparing with the distributed PRS measurement (4 distributed PRS occasion every 160ms). Concentrated PRS measurement can mainly reduce the power consumption of deep sleep transition, which is the main part of power saving gain.

Observation 42

- When configuring concentrated PRS measurement(1 concentrated PRS occasion every 160ms), 18.77% power saving gain is shown, comparing with the distributed PRS measurement (4 distributed PRS occasion every 160ms)

Defining PRS measurement window(PRS-MTC)

In Rel-15, SMTC has been defined. SSB-MTC is used to configure measurement timing configurations and includes duration which is the duration of the measurement window in which to receive SS/PBCH blocks. When SMTC is configured, the UE is not expected to measure SSB outside the SMTC window which reduces unnecessary power consumption. For PRS measurement, a similar window within a constrained duration (which can be called PRS-MTC) may be introduced in Rel-17 for UE power efficient positioning. PRS-MTC can limit the processing for measurement within a constrained time window as shown in Figure 18. In this case, the UE is only required to measure PRS of 2ms length when configuring 2ms PRS MTC window.

Figure 18  Procedure of PRS measurements with PRS MTC (2ms) in the DRX cycle

In this subsection, we compare power consumption between PRS measurement with 2ms PRS-MTC window and without PRS MTC window (baseline). The corresponding power analysis is shown below in Table 35.

Table 35 Power components analysis for PRS measurement by adding PRS MTC window in connected state

| Power state | Relative power | Duration(ms) |  |
| --- | --- | --- | --- |
|  |  | Without PRS-MTC(baseline) | PRS MTC(2ms) |
| Deep sleep 23 | 1 | 115 | 115 |
| Light sleep | 20 | 23 | 24 |
| Micro sleep | 45 | 4 | 5 |
| SSB for Inter-frequency measurement | 150 | 5 | 5 |
| SSB for Intra-frequency measurement | 150 | 2 | 2 |
| SSB Proc. | 100 | 2 | 2 |
| Single positioning frequency layer measurement | 610 | 4 | 2 |
| Gap switching | 45 | 1 | 1 |
| PDCCH-only monitoring | 100 | 4 | 4 |
| Sleep transition type | Transition energy | Transition times | Transition times |
| Deep sleep transition | 450 | 1 | 1 |
| Light sleep transition | 100 | 3 | 3 |
| Calculation |  |  |  |
| DRX cycle | - | 160 | 160 |
| Average power | - | 35.2500 | 28.0313 |
| Power saving gain | - | 0 | 20.48% |

It can be observed that, by adding the PRS MTC window to limit PRS measurement in 2ms (from 4ms to 2ms), 20.48% power saving gain is shown ,comparing with PRS measurement without PRS-MTC (the baseline assumption).

Observation 43

- By adding the PRS MTC window to limit PRS measurement in 2ms (from 4ms to 2ms), 20.48% power saving gain is shown ,comparing with PRS measurement without PRS-MTC .

Reducing number of TRPs to be measured

When reducing the number of TRPs to be measured, the power component analysis is shown in Table 36.

Table 36 Power components analysis for PRS measurement for 4 TRPs in connected state

| Power state | Relative power | Duration(ms) |
| --- | --- | --- |
| Deep sleep | 1 | 115 |
| Light sleep | 20 | 23 |
| Micro sleep | 45 | 4 |
| SSB for Inter-frequency measurement | 150 | 5 |
| SSB for Intra-frequency measurement | 150 | 2 |
| SSB Proc. | 100 | 2 |
| Single positioning frequency layer measurement | 490 | 4 |
| Gap switching | 45 | 1 |
| PDCCH-only monitoring | 100 | 4 |
| Sleep transition type | Transition energy | Transition times |
| Deep sleep transition | 450 | 1 |
| Light sleep transition | 100 | 3 |
| Average power | 32.2500 |  |
| Power saving gain | 8.51% |  |

It can be observed that,

- By reducing the number of TRPs for PRS measurement (from 8 TRPs to 4 TRPs),  8.51% power saving gain is shown ,comparing with the baseline assumption.

Observation 44

- By reducing the number of TRPs for PRS measurement (from 8 TRPs to 4 TRPs),  8.51% power saving gain is shown ,comparing with the baseline assumption.

Reducing number of positioning frequency layers to be measured

To simplify the evaluation, we only list the power component analysis for a single freqeuency layer in the baseline. In this subsection, more analysis are shown for multiple positioning frequency layers as shown in Figure 19, Figure 20 and Table 37.

Figure 19 Procedure of PRS measurements of 2 frequency layers in the DRX cycle in connected state

Figure 20  Procedure of PRS measurements of 4 frequency layers in the DRX cycle in connected state

Table 37 Power components analysis for multiple frequency layers PRS measurement in connected state

| Power state | Relative power | Duration(ms) |  |
| --- | --- | --- | --- |
|  |  | 2 frequency layers | 4 frequency layers |
| Deep sleep | 1 | 115 | 115 |
| Light sleep | 20 | 14 | 0 |
| Micro sleep | 45 | 8 | 12 |
| SSB for Inter-frequency measurement | 150 | 5 | 5 |
| SSB for Intra-frequency measurement | 150 | 2 | 2 |
| SSB Proc. | 100 | 2 | 2 |
| Multiple positioning frequency layers measurement | 610 | 8 | 16 |
| Gap switching | 45 | 2 | 4 |
| PDCCH-only monitoring | 100 | 4 | 4 |
| Sleep transition type | Transition energy | Transition times | Transition times |
| Deep sleep transition | 450 | 1 | 1 |
| Light sleep transition | 100 | 2 | 0 |
| Average power | - | 52.0313 | 82.4688 |
| Power consmuption gain | - | 47.61% | 133.95% |

It can be observed that,

- By increasing the number of frequency layers to 2 (from 1 to 2), 47.61% power consumption gain is shown, comparing with the baseline assumption.

- By increasing the number of frequency layers to 4 (from 1 to 4), 133.95% power consumption gain is shown, comparing with the baseline assumption.

Correspondingly, if we take the case of 4 frequency layers as the baseline, then by reducing the number of frequency layer to 2 (from 4 to 2), 36.91% power saving gain is shown; by  reducing number of frequency layer to 1 (from 4 to 1), 57.26% power saving gain is shown.

Therefore, by reducing the number of positioning frequency layers to be measured, the power saving is largely increased.

Observation 45

- By reducing the number of frequency layer to 2 (from 4 to 2), 36.91% power saving gain is shown; by  reducing number of frequency layer to 1 (from 4 to 1), 57.26% power saving gain is shown.

Observation 46: For PRS measurements, the following approaches are benefit for power saving.

- Extending PRS period

- Defining positioning measurement window

- Concentrated PRS distribution

- Reducing the number of TRPs  to be measured

- Reducing the number of positioning frequency layers to be measured

### 4.3.3 Evaluation for idle state positioning

In this section, we evaluate power consumption for idle state positioning.

Idle state measurement and report

Figure 21  Procedure of PRS measurements and report in idle state

In this subsection, we discuss power consumption for positioning measurement and report in idle states. For positioning report in idle state, let’s take the positioning information reported by EDT as an example. If the UE and NG-RAN node both support EDT, the UE sends an RRCEarlyDataRequest message to the NG-RAN node and includes a NAS control plane service request. The NAS control plane service request includes an positioning report message. From the perspective from physical layer, this message is carried in Msg3. Furthermore, the UE can receive RRCEarlyDataComplete containing positioning report completion information which is also included in Msg4.

The average power for the components introduced by positioning report in idle mode is listed below in Table 38.

Table 38 Average power for the components introduced by positioning report in idle mode

| Power State | Characteristics | Relative Power |
| --- | --- | --- |
| Coreset0+SIB1 | Equivalent to PDCCH + PDSCH. For idle mode, scaling factor is assume as 0.4 compared with connected states. | 120 |
| RAR | Equivalent to PDCCH + PDSCH. | 120 |
| Msg4 | Equivalent to PDCCH + PDSCH. Msg4 contains contention resolution information and RRCEarlyDataComplete information, doesn’t contain additional RRC information to establish RRC connection. | 120 |
| Msg3 | Equivalent to PUSCH. For idle mode, scaling factor is assume as 0.4. Considering positionnig report carried in Msg3, the length of Msg3 is assumed as 4ms. | 280 |
| PRACH | Sequence length is 839. SCS is 1.25kHz. | 175 |
| Paging occasion | Equivalent to PDCCH + PDSCH. | 120 |

Therefore, the power components analysis for measurement and report in idle mode is below in Table 39. The power calculation cycle is 1280ms.

Table 39 Power components analysis for positioning measurement and report in idle state

| Power state | Relative power | Duration(ms) |
| --- | --- | --- |
| Deep sleep | 1 | 1177 |
| Light sleep | 20 | 57.5 |
| Micro sleep | 45 | 16.5 |
| SSB for Inter-frequency measurement | 60 | 5 |
| SSB for Intra-frequency measurement | 60 | 2 |
| SSB Proc. | 50 | 6 |
| Single positioning frequency layer measurement | 96 | 4 |
| Paging Occasion | 120 | 4 |
| Coreset0+SIB1 | 120 | 1 |
| PRACH | 175 | 1 |
| RAR | 120 | 1 |
| Msg3(carrying positioning report) | 280 | 4 |
| Msg4 | 120 | 1 |
| Sleep transition type | Transition energy | Transition times |
| Deep sleep transition | 450 | 1 |
| Light sleep transition | 100 | 6 |
| Average power | 5.7488 |  |

Idle state measurement and connected state report

In this subsection, we discuss power consumption for positioning measurement in idle state and report in connected state. For simplicity, it is assumed that the only thing the UE does in the connected state is to report positioning information. The related procedure is shown in Figure 22.

Figure 22  Procedure of PRS measurements in idle state and report in connected state

The average power for the components introduced by positioning report in connected state and RRC state switching is listed below in Table 40.

Table 40 Average power for the components introduced by positioning report in connected state and RRC state switching

| Power State |  | Characteristics | Relative Power |
| --- | --- | --- | --- |
| RRC state switching | Coreset0+SIB1 | Equivalent to PDCCH + PDSCH. For idle mode, scaling factor is assume as 0.4 compared with connected states. | 120 |
|  | RAR | Equivalent to PDCCH + PDSCH. | 120 |
|  | Msg4 | Equivalent to PDCCH + PDSCH. Msg4 contains contention resolution information and RRC setup information to establish RRC connection. | 120 |
|  | Msg3 | Equivalent to PUSCH. For idle mode, scaling factor is assume as 0.4. | 280 |
|  | PRACH | Sequence length is 839. SCS is 1.25kHz. | 175 |
|  | Paging occasion | Equivalent to PDCCH + PDSCH. | 120 |
| Positioning report in connected state | SSB burst in connected state | SSB can be used for fine time-frequency sync. | 100 |
|  | Msg5 | Equivalent to PUSCH. For connected state, scaling factor is assume as 1. | 700 |
|  | Uplink grant | Equivalent to PDCCH only. | 100 |
|  | PUCCH SR | Equivalent to short PUCCH. Short PUCCH power = 0.3 x uplink power. | 210 |
|  | Pos report | Equivalent to PUSCH. | 700 |
|  | RRC release | Equivalent to PDSCH only. RRC connected state switches to idle state. | 280 |

Therefore, the power components analysis for measurement in idle mode and report in connected states is below in Table 41. The power calculation cycle is 1280ms.

Table 41 Power components analysis for positioning measurement and report in idle state

| Power state | Relative power | Duration(ms) |
| --- | --- | --- |
| Sleep |  |  |
| Deep sleep | 1 | 1129 |
| Light sleep | 20 | 69.5 |
| Micro sleep | 45 | 35.5 |
| Idle states |  |  |
| SSB for Inter-frequency measurement | 60 | 5 |
| SSB for Intra-frequency measurement | 60 | 2 |
| SSB Proc. | 50 | 8 |
| Single positioning frequency layer measurement | 96 | 4 |
| Paging Occasion | 120 | 4 |
| Coreset0+SIB1 | 120 | 1 |
| PRACH | 175 | 1 |
| RAR | 120 | 1 |
| Msg3 | 280 | 1 |
| Msg4 | 120 | 5 |
| Connected states |  |  |
| Uplink grant | 100 | 2 |
| SSB Proc. | 100 | 4 |
| Msg5 | 700 | 2 |
| PUCCH SR | 210 | 1 |
| Positioning report | 700 | 3 |
| RRC release | 280 | 2 |
| Sleep transition type | Transition energy | Transition times |
| Deep sleep transition | 450 | 1 |
| Light sleep transition | 100 | 8 |
| Average power | 10.3246 |  |

It can be observed that

- compared to reporting in the idle state, the power consumption of reporting in the connected state increases 79.6%.

- correspondingly, if we take the case of reporting in the connected state as the baseline, for reporting in the idle state , 44.32% power saving gain is shown.

Observation 47: Under the premise of idle state measurement, positioning report in the idle state can obtain 44.32% power saving gain compared to report in connected state.

Connected state measurement and report

In this subsection, we evaluate the case for connected states measurement and report. In order to be comparable with the above subsections, it is assumed that the UE still starts positioning from idle state. Furthermore, for simplicity, some additional assumptions is below.

- In connected state, LPP procedures including capability transfer,  positioning assistant data transfer, some request signalings between the UE and the LMF are ignored.

- One shot positioning measurement and report related procedures are considered in connected state.

- Procedures related to measurement gap configuration are ignored.

- Once the positioning report is completed, the RRC connection is released.

- Measurement and report cycle is equal to idle state (1280ms).

- PRS bandwidth in connected state is equal to PRS bandwidth in idle state.

- Paging occasion power is equivalent to ‘PDCCH+PDSCH’, considering it may lead to RRC state transition.

Therefore, related procedure is shown in Figure 23.

Figure 23  Procedure of PRS measurements and report in connected state

The average power for the components introduced by positioning measurement/report in connected state and RRC state switching is listed below in Table 42.

Table 42 Average power for the components introduced by positioning measurement/report in connected state and RRC state switching

| Power State |  | Characteristics | Relative Power |
| --- | --- | --- | --- |
| RRC state switching | Coreset0+SIB1 | Equivalent to PDCCH + PDSCH. For idle mode, scaling factor is assume as 0.4 compared with connected states. | 120 |
|  | RAR | Equivalent to PDCCH + PDSCH. | 120 |
|  | Msg4 | Equivalent to PDCCH + PDSCH. Msg4 contains contention resolution information and RRC setup information to establish RRC connection. | 120 |
|  | Msg3 | Equivalent to PUSCH. For idle mode, scaling factor is assume as 0.4. | 280 |
|  | PRACH | Sequence length is 839. SCS is 1.25kHz. | 175 |
|  | Paging occasion | Equivalent to PDCCH + PDSCH. | 120 |
| Positioning measurement and report in connected state | SSB burst in connected state | SSB can be used for fine time-frequency sync. | 100 |
|  | Msg5 | Equivalent to PUSCH. For connected state, scaling factor is assume as 1. | 700 |
|  | Uplink grant | Equivalent to PDCCH only. | 100 |
|  | PUCCH SR | Equivalent to short PUCCH. Short PUCCH power = 0.3 x uplink power. | 210 |
|  | Pos report | Equivalent to PUSCH. | 700 |
|  | RRC release | Equivalent to PDSCH only. RRC connected state switches to idle state. | 280 |
|  | PRS measurement | PRS bandwidth is equal to 20M. | 240 |

Therefore, the power components analysis for measurement and report in connected states is below in Table 43. The power calculation cycle is 1280ms.

Table 43 Power components analysis for positioning measurement and report in connected state

| Power state | Relative power | Duration(ms) |
| --- | --- | --- |
| Sleep |  |  |
| Deep sleep | 1 | 1115 |
| Light sleep | 20 | 84 |
| Micro sleep | 45 | 32 |
| Idle states |  |  |
| SSB for Inter-frequency measurement | 60 | 5 |
| SSB for Intra-frequency measurement | 60 | 2 |
| SSB Proc. | 50 | 8 |
| Paging Occasion | 120 | 4 |
| Coreset0+SIB1 | 120 | 1 |
| PRACH | 175 | 1 |
| RAR | 120 | 1 |
| Msg3 | 280 | 1 |
| Msg4 | 120 | 5 |
| Connected states |  |  |
| Single positioning frequency layer measurement | 240 | 4 |
| Gap switching | 45 | 1 |
| Uplink grant | 100 | 2 |
| SSB Proc. | 100 | 6 |
| Msg5 | 700 | 2 |
| PUCCH SR | 210 | 1 |
| Positioning report | 700 | 3 |
| RRC release | 280 | 2 |
| Sleep transition type | Transition energy | Transition times |
| Deep sleep transition | 450 | 1 |
| Light sleep transition | 100 | 9 |
| Average power | 11.1367 |  |

It can be observed that

- compared to reporting in the idle state, the power consumption of reporting in the connected state increases 93.72%.

- correspondingly, if we take the case of reporting in the connected state as the baseline, for reporting in the idle state , 48.38% power saving gain is shown.

Furthermore, considering that we make some assumptions to simplify power consumption evaluation, such as ignoring complicated steps for LPP procedures, aligning the bandwidth and period with idle state measurement, it will consume more power when positioning in the connected state in general.

Observation 48: Compared to positioning measurement and report all in the connected state, positioning measurement and report in the idle state can obtain at least 48.38% power saving gain.

Conclusion

In this contribution, we present our evaluations of NR positioning performance. We have the following observations and proposals:

Observation 1

- For Rel.16 DL-TDOA positioning, the performance target [0.2m 90%] can be achieved in InF-SH with perfect synchronization for convex UEs, and in InF-SH with perfect synchronization for all UEs for FR2, but cannot be achieved in other cases in InF-SH.

Observation 2

- For Rel.16 DL-TDOA positioning, the performance target [0.2m 90%] can be achieved in InF-DH with perfect synchronization for convex UEs for FR2, but cannot be achieved in other cases in InF-DH.

Observation 3

- The positioning performance of DL-TDOA degraded significantly with 50ns synchronization error.

Observation 4

- For DL-TDOA positioning with the DH clutter parameter {0.6,6,2}, the performance target [0.2m 90%] cannot be achieved and the performance gap is nearly 18 m.

Observation 5

- For Rel.16 UL-TDOA positioning, the performance target [0.2m 90%] can be achieved in InF-SH with perfect synchronization for convex UEs, and in InF-SH with perfect synchronization for all UEs for FR2, but cannot be achieved in other cases in InF-SH.

Observation 6

- For Rel.16 UL-TDOA positioning, the performance target [0.2m 90%] can be achieved in InF-DH with perfect synchronization for convex UEs for FR2, but cannot be achieved in other cases in InF-DH.

Observation 7

- For Rel.16 UL-AOA positioning, the performance target [0.2m 90%] cannot be achieved in InF-SH and InF-DH.

Observation 8

- Compared to DL-TDOA or UL-TDOA, synchronization error has little effect on positioning performance on UL-AOA.

Observation 9

- For Rel.16 UL-TDOA+AOA positioning, the performance target [0.2m 90%] cannot be achieved in InF-SH and InF-DH.

Observation 10

Compared to UL-AOA, UL-TDOA+AOA can improve the positioning accuracy to some extent.

Observation 11

- For Rel.16 Multi-RTT positioning, the performance target [0.2m 90%] can be achieved in InF-SH for convex UEs, and in InF-SH for all UEs for FR2, but cannot be achieved in InF-SH in other cases.

Observation 12

- For Rel.16 Multi-RTT positioning, the performance target [0.2m 90%] can be achieved in InF-DH for convex UEs for FR2, but cannot be achieved in other cases in InF-DH.

Observation 13

- Compared to DL-TDOA and UL-TDOA, Multi-RTT can mitigate the positioning error caused by synchronization error between network nodes.

Observation 14

- For vertical evaluation with DL-TDOA，the performance target [1m 90%] can be achieved in InF-SH and InF-DH scenarios for FR1 with baseline assumptions.

Observation 15

- For vertical evaluation with AOA+ZOA technique，the performance target [1m 90%] can be achieved in InF-SH scenario but not in InF-DH scenario for FR1 with baseline assumptions.

Observation 16

- For IOO scenario，the performance target [1m 90%] can be achieved with perfect synchronization.

Observation 17

- For DL-TDOA positioning，the performance target [0.2m 90%] can be achieved in InF-SH and InF-DH with perfect synchronization for convex UEs with enhancement.

Observation 18

- For DL-TDOA positioning，the performance target [0.2m 90%] can be achieved in InF-SH for FR2 with perfect synchronization for all UEs with enhancement, but cannot be achieved in other cases for all UEs.

Observation 19

- For UL-TDOA positioning，the performance target [0.2m 90%] can be achieved in InF-SH and InF-DH with perfect synchronization for convex UEs with enhancement.

Observation 20

- For UL-TDOA positioning，the performance target [0.2m 90%] can be achieved in InF-SH for FR2 with perfect synchronization for all UEs with enhancement, but cannot be achieved in other cases for all UEs.

Observation 21

- For Multi-RTT positioning，the performance target [0.2m 90%] can be achieved in InF-SH and InF-DH for convex UEs with enhancement.

Observation 22

- For Multi-RTT positioning，the performance target [0.2m 90%] can be achieved in InF-SH and InF-DH for FR2 for all UEs with enhancement, but cannot be achieved in other cases for all UEs.

Observation 23

- Outlier determination/rejection techniques have better positioning accuracy performance than LOS detection.

- The positioning performance of LOS/NLOS detection method degrades as LOS detection error probability increases.

Observation 24

- The minimum Rel.16 granularity of timing measurement reports is enough to avoid degradation in IIoT scenarios and meet positioning requirements.

Observation 25

- With the TX/RX timing error 0.5ns, the performance target [0.2m 90%] cannot be achieved in both InF-SH and InF-DH.

Observation 26

- For DL-TDOA, the UE timing error has little impact on positioning accuracy, while the BS timing error causes some degradation on positioning performance.

Observation 27

- For Multi-RTT, the UE timing error and BS timing error both cause degradation in accuracy.

Observation 28

- For ideal aggregation of multiple DL positioning frequency 50M+50M, performance target [0.2m 90%] cannot be achieved in both InF-SH and InF-DH.

Observation 29

- For ideal aggregation of multiple DL positioning frequency 50M+50M, the performance is worse than 100M but better than 50M.

Observation 30

- The longer period of positioning reference signal and measurement gap, the greater the latency.

Observation 31

- 10ms physical layer latency cannot be reached with Rel-16  solutions.

Observation 32

- DL measurement &process delay will be reduced by Aperiodic PRS, and the physical layer latency is reduced by nearly 20ms.

Observation 33

- DL measurement delay is impacted by PRS periodicity instead of the least common multiple of PRS and MG periodicity if on-demand /aperiodic MG is introduced, and the physical layer latency is reduced by nearly 16 ms for DL measurement.

- The latency of measurement requesting and measurement configuration can also be reduced by on-demand /aperiodic MG. The physical layer latency is reduced by nearly 20ms for measurement requesting and measurement configuration if on-demand /aperiodic MG is introduced.

Observation 34

- DL measurement delay is impacted by PRS periodicity instead of the least common multiple of PRS and MG periodicity if Positioning BWP is introduced, and the physical layer latency is reduced by nearly 16 ms for DL measurement.

- The latency of measurement requesting and measurement configuration can also be reduced by Positioning BWP. The physical layer latency is reduced by nearly 20ms for measurement requesting and measurement configuration if Positioning BWP is introduced.

Observation 35

- Positioning requesting/ positioning reporting doesn’t need to consider the RRC process time if low layer triggered Positioning requesting/ positioning reporting is supported, and the physical layer latency is reduced by nearly 20 ms.

Observation 36

- 10ms physical layer latency can be reached with the combination of on-demand/aperiodic PRS, on-demand/aperiodic MG, Positioning BWP, and physical layer triggered request/report.

Observation 37

- Additional latency of 40~200ms will be introduced if the UE switches to connected state from idle state for positioning measurement and report.

Observation 38

- The network efficiency exceeds 100% in some FR2 cases.

- The MGL/MGRP (UE efficiency) exceeds 30% in some FR2 cases.

Observation 39

- The network and device efficiency will be reduced by on-demand PRS within the same level latency compared to periodic PRS.

Observation 40

- The network and device efficiency of aperiodic PRS is multiple of the number of activations.

Observation 41

- By extending the PRS period to 2 times(160ms to 320ms), 22.03% power saving gain is shown, comparing with the baseline assumption

- By extending the PRS period to 4 times(160ms to 640ms), 33.05 % power saving gain is shown ,comparing with the baseline assumption

Observation 42

- When configuring concentrated PRS measurement(1 concentrated PRS occasion every 160ms), 18.77% power saving gain is shown, comparing with the distributed PRS measurement (4 distributed PRS occasion every 160ms)

Observation 43

- By adding the PRS MTC window to limit PRS measurement in 2ms (from 4ms to 2ms), 20.48% power saving gain is shown ,comparing with PRS measurement without PRS-MTC .

Observation 44

- By reducing the number of TRPs for PRS measurement (from 8 TRPs to 4 TRPs),  8.51% power saving gain is shown ,comparing with the baseline assumption.

Observation 45

- By reducing the number of frequency layer to 2 (from 4 to 2), 36.91% power saving gain is shown; by  reducing number of frequency layer to 1 (from 4 to 1), 57.26% power saving gain is shown.

Observation 46: For PRS measurements, the following approaches are benefit for power saving.

- Extending PRS period

- Defining positioning measurement window

- Concentrated PRS distribution

- Reducing the number of TRPs  to be measured

- Reducing the number of positioning frequency layers to be measured

Observation 47: Under the premise of idle state measurement, positioning report in the idle state can obtain 44.32% power saving gain compared to report in connected state.

Observation 48: Compared to positioning measurement and report all in the connected state, positioning measurement and report in the idle state can obtain at least 48.38% power saving gain.

Proposal 1

- LOS/NLOS detection/identification should not be considered in Rel-17.

Proposal 2

- The performance impact of non-ideal aggregation of multiple DL positioning frequency should be further studied.

Proposal 3

- Physical layer latency needs to be reduced in Rel-17.

Proposal 4

- On-demand/aperiodic PRS, on-demand/aperiodic MG, Positioning BWP, and physical layer triggered need to be studied in Rel-17 for reducing latency.

Proposal 5

- The network efficiency and UE efficiency for low latency positioning needs to be enhanced.

- References

- Chairman’s notes for 3GPP TSG RAN WG1 Meeting 102, e-Meeting, August 17th – 28th, 2020.

- R1-2007664, Discussion on additional scenarios for NR positioning evaluation,vivo, October, 2020.

- R1-2007666, Discussion on potential positioning enhancements, vivo, October, 2020.

- Evaluation scenarios and parameters

Table A.1: Rel.16 NR positioning - evaluation scenarios and parameters

| Parameter | [Case 1-18] [DL-TDOA] [InF] | [Case 19-42] [UL-TDOA/ UL-AOA/UL-TDOA+UL-AOA] [InF] | [Case 43-58] [Multi-RTT] [InF] |
| --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | baseline | baseline | baseline |
| Carrier frequency | FR1:3.5GHz FR2:28GHz | FR1:3.5GHz FR2:28GHz | FR1:3.5GHz FR2:28GHz |
| Subcarrier spacing | FR1:30kHz FR2:120kHz | FR1:30kHz FR2:120kHz | FR1:30kHz FR2:120kHz |
| Reference Signal Transmission Bandwidth | FR1:100MHz FR2:400kHz | FR1:100MHz FR2:400kHz | FR1:100MHz FR2:400kHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | R16 PRS within a slot (comb-6 frequency structure, 6 symbols within a slot) | SRS comb-4 frequency structure 4 symbols within a slot No sequence/group/frequency hopping | DL: R16 PRS within a slot (comb-6 frequency structure, 6 symbols within a slot) UL: SRS comb-4 frequency structure 4 symbols within a slot |
| Reference signal (type of sequence, number of ports, …) | 1 port, QPSK-PN sequence | 1 port, ZC sequence | DL: 1 port, QPSK-PN sequence UL: 1 port, ZC sequence |
| Number of sites | 18 (4 sites are used for positioning) | 18 (4 sites are used for positioning) | 18 (4 sites are used for positioning) |
| Number of symbols used per occasion | 1 | 1 | 1 |
| number of occasions used per positioning estimate | 1 | 1 | 1 |
| Power-boosting level | 7.78dB | 6dB | DL:7.78dB UL:6dB |
| Uplink power control (applied/not applied) | not applied | not applied | not applied |
| interference modelling (ideal muting, or other) | ideal muting | ideal muting | ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | super resolution | super resolution | super resolution |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | taylor series | UL-TDOA: taylor series UL-AOA: LS | Multi-RTT: taylor series |
| Network synchronization assumptions | Perfect sync/sync error 50ns | Perfect sync/sync error 50ns | Perfect sync/sync error 50ns |
| UE/gNB Tx/Rx Calibration Error | 0 | 0 | 0 |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | alignment assumptions at the tx and rx sides | alignment assumptions at the tx and rx sides | alignment assumptions at the tx and rx sides |

Table A.2: Rel.16 NR positioning - evaluation scenarios and parameters

| Parameter | [Case 59-60] [DL-TDOA] [InF] | [Case 61-62] [UL-AOA+ZOA] [InF] |
| --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | baseline | baseline |
| Carrier frequency | FR1:3.5GHz FR2:28GHz | FR1:3.5GHz FR2:28GHz |
| Subcarrier spacing | FR1:30kHz FR2:120kHz | FR1:30kHz FR2:120kHz |
| Reference Signal Transmission Bandwidth | FR1:100MHz FR2:400kHz | FR1:100MHz FR2:400kHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | R16 PRS within a slot (comb-6 frequency structure, 6 symbols within a slot) | SRS comb-4 frequency structure 4 symbols within a slot No sequence/group/frequency hopping |
| Reference signal (type of sequence, number of ports, …) | 1 port, QPSK-PN sequence | 1 port, ZC sequence |
| Number of sites | 18 (4 sites are used for positioning) | 18 (4 sites are used for positioning) |
| Number of symbols used per occasion | 1 | 1 |
| number of occasions used per positioning estimate | 1 | 1 |
| Power-boosting level | 7.78dB | 6dB |
| Uplink power control (applied/not applied) | not applied | not applied |
| interference modelling (ideal muting, or other) | ideal muting | ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | super resolution | super resolution |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | taylor series first/median peak+RAIM | UL-TDOA: taylor series first/median peak+RAIM UL-AOA: LS |
| Network synchronization assumptions | Perfect sync/sync error 50ns | Perfect sync/sync error 50ns |
| UE/gNB Tx/Rx Calibration Error | 0 | 0 |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | alignment assumptions at the tx and rx sides | alignment assumptions at the tx and rx sides |

Table A.3: Rel.16 NR positioning - evaluation scenarios and parameters

| Parameter | [Case 63-64] [DL-TDOA] [IOO] | [Case 65-66] [UL-TDOA] [IOO] | [Case 67-68] [Multi-RTT] [IOO] |
| --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | baseline | baseline | baseline |
| Carrier frequency | FR1:4GHz FR2:30GHz | FR1:4GHz FR2:30GHz | FR1:4GHz FR2:30GHz |
| Subcarrier spacing | FR1:30kHz FR2:120kHz | FR1:30kHz FR2:120kHz | FR1:30kHz FR2:120kHz |
| Reference Signal Transmission Bandwidth | FR1:100MHz FR2:400kHz | FR1:100MHz FR2:400kHz | FR1:100MHz FR2:400kHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | R16 PRS within a slot (comb-6 frequency structure, 6 symbols within a slot) | SRS comb-4 frequency structure 4 symbols within a slot No sequence/group/frequency hopping | DL: R16 PRS within a slot (comb-6 frequency structure, 6 symbols within a slot) UL: SRS comb-4 frequency structure 4 symbols within a slot |
| Reference signal (type of sequence, number of ports, …) | 1 port, QPSK-PN sequence | 1 port, ZC sequence | DL: 1 port, QPSK-PN sequence UL: 1 port, ZC sequence |
| Number of sites | 18 (4 sites are used for positioning) | 18 (4 sites are used for positioning) | 18 (4 sites are used for positioning) |
| Number of symbols used per occasion | 1 | 1 | 1 |
| number of occasions used per positioning estimate | 1 | 1 | 1 |
| Power-boosting level | 7.78dB | 6dB | DL:7.78dB UL:6dB |
| Uplink power control (applied/not applied) | not applied | not applied | not applied |
| interference modelling (ideal muting, or other) | ideal muting | ideal muting | ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | super resolution | super resolution | super resolution |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | taylor series first/median peak+RAIM | taylor series first/median peak+RAIM | taylor series first/median peak+RAIM |
| Network synchronization assumptions | Perfect sync/sync error 50ns | Perfect sync/sync error 50ns | Perfect sync/sync error 50ns |
| UE/gNB Tx/Rx Calibration Error | 0 | 0 | 0 |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | alignment assumptions at the tx and rx sides | alignment assumptions at the tx and rx sides | alignment assumptions at the tx and rx sides |

Table A.4: NR positioning enhancements - evaluation scenarios and parameters

| Parameter | [Case E1-E16] [DL-TDOA] [InF] | [Case E17-E32] [UL-TDOA] [InF] | [Case E33-E48] [Multi-RTT] [InF] |
| --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | baseline | baseline | baseline |
| Carrier frequency | FR1:3.5GHz FR2:28GHz | FR1:3.5GHz FR2:28GHz | FR1:3.5GHz FR2:28GHz |
| Subcarrier spacing | FR1:30kHz FR2:120kHz | FR1:30kHz FR2:120kHz | FR1:30kHz FR2:120kHz |
| Reference Signal Transmission Bandwidth | FR1:100MHz FR2:400kHz | FR1:100MHz FR2:400kHz | FR1:100MHz FR2:400kHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | R16 PRS within a slot (comb-6 frequency structure, 6 symbols within a slot) | SRS comb-4 frequency structure 4 symbols within a slot No sequence/group/frequency hopping | DL: R16 PRS within a slot (comb-6 frequency structure, 6 symbols within a slot) UL: SRS comb-4 frequency structure 4 symbols within a slot |
| Reference signal (type of sequence, number of ports, …) | 1 port, QPSK-PN sequence | 1 port, ZC sequence | DL: 1 port, QPSK-PN sequence UL: 1 port, ZC sequence |
| Number of sites | 18 (4 sites are used for positioning) | 18 (4 sites are used for positioning) | 18 (4 sites are used for positioning) |
| Number of symbols used per occasion | 1 | 1 | 1 |
| number of occasions used per positioning estimate | 1 | 1 | 1 |
| Power-boosting level | 7.78dB | 6dB | DL:7.78dB UL:6dB |
| Uplink power control (applied/not applied) | not applied | not applied | not applied |
| interference modelling (ideal muting, or other) | ideal muting | ideal muting | ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | super resolution RAIM | super resolution RAIM | super resolution RAIM |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | taylor series | taylor series | taylor series |
| Network synchronization assumptions | Perfect sync/sync error 50ns | Perfect sync/sync error 50ns | Perfect sync/sync error 50ns |
| UE/gNB Tx/Rx Calibration Error | 0 | 0 | 0 |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | alignment assumptions at the tx and rx sides | alignment assumptions at the tx and rx sides | alignment assumptions at the tx and rx sides |

Table A.5: NR positioning enhancements - evaluation scenarios and parameters

| Parameter | [Case E49] [Case E55] [DL-TDOA] [InF] | [Case E51] [Case E57] [DL-TDOA] [InF] | [Case E50] [Case E52-E54] [Case E56] [Case E58-E60] [DL-TDOA] [InF] |
| --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | baseline | baseline | baseline |
| Carrier frequency | FR1:3.5GHz | FR1:3.5GHz | FR1:3.5GHz |
| Subcarrier spacing | FR1:30kHz | FR1:30kHz | FR1:30kHz |
| Reference Signal Transmission Bandwidth | FR1:100MHz | FR1:100MHz | FR1:100MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | R16 PRS within a slot (comb-6 frequency structure, 6 symbols within a slot) | R16 PRS within a slot (comb-6 frequency structure, 6 symbols within a slot) | R16 PRS within a slot (comb-6 frequency structure, 6 symbols within a slot) |
| Reference signal (type of sequence, number of ports, …) | 1 port, QPSK-PN sequence | 1 port, QPSK-PN sequence | 1 port, QPSK-PN sequence |
| Number of sites | 18 (4 sites are used for positioning) | 18 (4 sites are used for positioning) | 18 (4 sites are used for positioning) |
| Number of symbols used per occasion | 1 | 1 | 1 |
| number of occasions used per positioning estimate | 1 | 1 | 1 |
| Power-boosting level | 7.78dB | 7.78dB | 7.78dB |
| Uplink power control (applied/not applied) | not applied | not applied | not applied |
| interference modelling (ideal muting, or other) | ideal muting | ideal muting | ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | super resolution | super resolution | super resolution |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | taylor series RAIM | taylor series RAIM+LOS detection | taylor series LOS detection |
| Network synchronization assumptions | Perfect sync | Perfect sync | Perfect sync |
| UE/gNB Tx/Rx Calibration Error | 0 | 0 | 0 |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | alignment assumptions at the tx and rx sides | alignment assumptions at the tx and rx sides | alignment assumptions at the tx and rx sides |

Table A.6:NR positioning enhancements - evaluation scenarios and parameters

| Parameter | [Case E61-E66] [DL-TDOA] | [Case E67-E84] [DL-TDOA] | [Case E85-E102] [Multi-RTT] |
| --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | baseline | baseline | baseline |
| Carrier frequency | FR1:3.5GHz | FR1:3.5GHz | FR1:3.5GHz |
| Subcarrier spacing | FR1:30kHz | FR1:30kHz | FR1:30kHz |
| Reference Signal Transmission Bandwidth | FR1:100MHz | FR1:100MHz | FR1:100MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | R16 PRS within a slot (comb-6 frequency structure, 6 symbols within a slot) | R16 PRS within a slot (comb-6 frequency structure, 6 symbols within a slot) | SRS comb-4 frequency structure 4 symbols within a slot No sequence/group/frequency hopping |
| Reference signal (type of sequence, number of ports, …) | 1 port, QPSK-PN sequence | 1 port, QPSK-PN sequence | 1 port, ZC sequence |
| Number of sites | 18 (4 sites are used for positioning) | 18 (4 sites are used for positioning) | 18 (4 sites are used for positioning) |
| Number of symbols used per occasion | 1 | 1 | 1 |
| number of occasions used per positioning estimate | 1 | 1 | 1 |
| Power-boosting level | 7.78dB | 7.78dB | 6dB |
| Uplink power control (applied/not applied) | not applied | not applied | not applied |
| interference modelling (ideal muting, or other) | ideal muting | ideal muting | ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | super resolution | super resolution | super resolution |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | taylor series first/median peak+RAIM | taylor series first/median peak+RAIM | taylor series first/median peak+RAIM |
| Network synchronization assumptions | Perfect sync | Perfect sync | Perfect sync |
| UE/gNB Tx/Rx Calibration Error | 0 | 0.5-5ns | 0.5-5ns |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | alignment assumptions at the tx and rx sides | alignment assumptions at the tx and rx sides | alignment assumptions at the tx and rx sides |

Table A.7: Rel.16 NR positioning - evaluation scenarios and parameters

| Parameter | [Case E103] [Case E106] [DL-TDOA] | [Case E104] [Case E107] [DL-TDOA] | [Case E105] [Case E108] [DL-TDOA] |
| --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | baseline | baseline | baseline |
| Carrier frequency | FR1:3.5GHz | FR1:3.5GHz | FR1:3.5GHz |
| Subcarrier spacing | FR1:30kHz | FR1:30kHz | FR1:30kHz |
| Reference Signal Transmission Bandwidth | 50M | 100M | 50M+50M |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | R16 PRS within a slot (comb-6 frequency structure, 6 symbols within a slot) | R16 PRS within a slot (comb-6 frequency structure, 6 symbols within a slot) | R16 PRS within a slot (comb-6 frequency structure, 6 symbols within a slot) |
| Reference signal (type of sequence, number of ports, …) | 1 port, QPSK-PN sequence | 1 port, QPSK-PN sequence | 1 port, QPSK-PN sequence |
| Number of sites | 18 (4 sites are used for positioning) | 18 (4 sites are used for positioning) | 18 (4 sites are used for positioning) |
| Number of symbols used per occasion | 1 | 1 | 1 |
| number of occasions used per positioning estimate | 1 | 1 | 1 |
| Power-boosting level | 7.78dB | 7.78dB | 7.78dB |
| Uplink power control (applied/not applied) | not applied | not applied | not applied |
| interference modelling (ideal muting, or other) | ideal muting | ideal muting | ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | super resolution | super resolution | super resolution |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | taylor series first/median peak+RAIM | taylor series first/median peak+RAIM | taylor series first/median peak+RAIM |
| Network synchronization assumptions | Perfect sync | Perfect sync | Perfect sync |
| UE/gNB Tx/Rx Calibration Error | 0 | 0 | 0 |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | alignment assumptions at the tx and rx sides | alignment assumptions at the tx and rx sides | alignment assumptions at the tx and rx sides |

- Performance analysis of Rel-16 positioning solutions

B.1 Horizontal accuracy evaluation

B.1.1 Downlink only evaluations

Figure 24 Accuracy with DL-TDOA in SH for convex UEs                   Figure 25 Accuracy with DL-TDOA in SH for all UEs

Figure 26 Accuracy with DL-TDOA in DH for convex UEs            Figure 27 Accuracy with DL-TDOA in DH for all UEs

Figure 28 Accuracy with DL-TDOA in SH for convex UEs          Figure 29 Accuracy with DL-TDOA in SH for all UEs

Figure 30 Accuracy with DL-TDOA in DH for convex UEs       Figure 31 Accuracy with DL-TDOA in DH for all UEs

B.1.2 Uplink only evaluations

Figure 32 Accuracy with UL-TDOA in SH for convex UEs             Figure 33 Accuracy with UL-TDOA in SH for all UEs

Figure 34 Accuracy with UL-TDOA in DH for convex UEs            Figure 35 Accuracy with UL-TDOA in DH for all UEs

Figure 36 Accuracy with UL-TDOA in SH for convex UEs                 Figure 37 Accuracy with UL-TDOA in SH for all UEs

Figure 38 Accuracy with UL-TDOA in DH for convex UEs          Figure 39 Accuracy with UL-TDOA in DH for all UEs

Figure 40 Accuracy with UL-AOA in SH and DH for convex UEs       Figure 41 Accuracy with UL-AOA in SH and DH for all UEs

Figure 42 Accuracy with UL-TDOA+UL-AOA for convex UEs           Figure 43 Accuracy with UL-TDOA+UL-AOA for all UEs

B.1.3 Uplink and downlink evaluations

Figure 44 Accuracy with Multi-RTT in SH for convex UEs          Figure 45 Accuracy with Multi-RTT in SH for all UEs

Figure 46 Accuracy with Multi-RTT in DH for convex UEs          Figure 47 Accuracy with Multi-RTT in DH for all UEs

Figure 48 Accuracy with Multi-RTT in SH for convex UEs              Figure 49 Accuracy with Multi-RTT in SH for all UEs

Figure 50 Accuracy with Multi-RTT in DH for convex UEs             Figure 51 Accuracy with Multi-RTT in DH for all UEs

B.2 Vertical accuracy evaluation

Figure 52 Vertical accuracy in SH and DH for convex UEs                 Figure 53 Vertical accuracy in SH and DH for all UEs

B.3 horizontal accuracy evaluation for IOO scenario

Figure 54 Horizontal accuracy in IOO for all UEs

- Performance of NR positioning enhancements

C.1 Horizontal accuracy evaluation

C.1.1 Downlink evaluations

Figure 55 Accuracy with DL-TDOA in SH for convex UEs                Figure 56 Accuracy with DL-TDOA in SH for all UEs

Figure 57 Accuracy with DL-TDOA in DH for convex UEs              Figure 58 Accuracy with DL-TDOA in DH for all UEs

Figure 59 Accuracy with DL-TDOA in SH for convex UEs             Figure 60 Accuracy with DL-TDOA in SH for all UEs

Figure 61 Accuracy with DL-TDOA in DH for convex UEs                Figure 62 Accuracy with DL-TDOA in DH for all UEs

C.1.2 Uplink evaluations

Figure 63 Accuracy with UL-TDOA in SH for convex UEs                 Figure 64 Accuracy with UL-TDOA in SH for all UEs

Figure 65 Accuracy with UL-TDOA in DH for convex UEs               Figure 66 Accuracy with UL-TDOA in DH for all UEs

Figure 67 Accuracy with UL-TDOA in SH for convex UEs                  Figure 68 Accuracy with UL-TDOA in SH for all UEs

Figure 69 Accuracy with UL-TDOA in DH for convex UEs                  Figure 70 Accuracy with UL-TDOA in DH for all UEs

C.1.3 Downlink and uplink evaluations

Figure 71 Accuracy with Multi-RTT in SH for convex UEs             Figure 72 Accuracy with Multi-RTT in SH for all UEs

Figure 73 Accuracy with Multi-RTT in DH for convex UEs              Figure 74 Accuracy with Multi-RTT in DH for all UEs

Figure 75 Accuracy with Multi-RTT in SH for convex UEs                  Figure 76 Accuracy with Multi-RTT in SH for all UEs

Figure 77 Accuracy with Multi-RTT in DH for convex UEs                Figure 78 Accuracy with Multi-RTT in SH for all UEs

C.2 RAIM and LOS detection

Figure 79 Accuracy with DL-TDOA in SH for convex UEs            Figure 80 Accuracy with DL-TDOA in DH for convex UEs

C.3 Timing measurement reporting granularity

Figure 81 Accuracy with DL-TDOA in SH and DH for convex UEs

C.4 Tx/Rx timing error

Figure 82 Accuracy with DL-TDOA in SH (fixed BS timing error 0.5ns)                   Figure 83 Accuracy with DL-TDOA in SH (fixed UE timing error 0.5ns)

Figure 84 Accuracy with DL-TDOA in SH (fixed BS timing error 0.5ns)                   Figure 85 Accuracy with DL-TDOA in DH (fixed UE timing error 0.5ns)

Figure 86 Accuracy with Multi-RTT in SH (fixed BS timing error 0.5ns)                 Figure 87 Accuracy with Multi-RTT in SH (fixed UE timing error 0.5ns)

Figure 88 Accuracy with Multi-RTT in DH (fixed BS timing error 0.5ns)              Figure 89 Accuracy with Multi-RTT in DH (fixed UE timing error 0.5ns)

C.5 Aggregation  of DL positioning frequency layers

Figure 90 Accuracy with DL-TDOA for convex UEs                        Figure 91 Accuracy with DL-TDOA for all UEs