3GPP TSG RAN WG1 #103-e                                        R1-2007692

e-Meeting, October 26th – November 13th, 2020

Source:	vivo

Title:	Discussion on mechanisms to improve reliability for RRC_CONNECTED UEs

Agenda Item:	8.12.2

Document for:  Discussion and Decision

- Introduction

In RAN#86 meeting, the work item of NR support of Multicast and Broadcast Services has been approved [1]. The followings have been identified as the objectives related with physical layer of the work item.

- Specify RAN basic functions for broadcast/multicast for UEs in RRC_CONNECTED state [RAN1, RAN2, RAN3]:

  - Specify a group scheduling mechanism to allow UEs to receive Broadcast/Multicast service [RAN1, RAN2]

    - This objective includes specifying necessary enhancements that are required to enable simultaneous operation with unicast reception.

  - Specify required changes to improve reliability of Broadcast/Multicast service, e.g. by UL feedback. The level of reliability should be based on the requirements of the application/service provided. [RAN1, RAN2]

- Specify RAN basic functions for broadcast/multicast for UEs in RRC_IDLE/ RRC_INACTIVE states [RAN2, RAN1]:

  - Specify required changes to enable the reception of Point to Multipoint transmissions by UEs in RRC_IDLE/ RRC_INACTIVE states, with the aim of keeping maximum commonality between RRC_CONNECTED state and RRC_IDLE/RRC_INACTIVE state for the configuration of PTM reception. [RAN2, RAN1].

Note: the possibility of receiving Point to Multipoint transmissions by UEs in RRC_IDLE/ RRC_INACTIVE states, without the need for those UEs to get the configuration of the PTM bearer carrying the Broadcast/Multicast service while in RRC CONNECTED state beforehand, is subject to verification of service subscription and authorization assumptions during the WI.

In last e-meeting, the following agreements was made regarding the mechanisms to improve reliability for RRC_CONNECTED UEs [2].

Agreements:

- For RRC_CONNECTED UEs, HARQ-ACK feedback is supported for multicast and no additional evaluation is needed to justify this.

  - FFS: The detailed HARQ-ACK feedback solutions, e.g., ACK/NACK based, NACK-only based.

  - FFS: HARQ-ACK feedback can be optionally disabled and/or enabled.

Agreements:

- For RRC_CONNECTED UEs, at least support slot-level repetition for group-common PDSCH.

  - FFS: whether enhancement is needed

Agreements:

- For RRC_CONNECTED UEs, existing CSI feedback can be used for multicast transmission.

  - FFS: whether enhancement is needed

In this contribution, we will make discussions on the mechanisms to improve reliability for RRC_CONNECTED UEs.

- Discussion

PDSCH repetition

In NR, PDSCH transmission with repetition is supported to improve reliability, where the number of PDSCH repetitions can be semi-statically configured or dynamically indicated by corresponding DCI. For MBS, to improve reliability of Broadcast/Multicast service, it was agreed in the last e-meeting that for RRC_CONNECTED UEs, at least support slot-level repetition for group-common PDSCH. Considering that differnet MBS services which may be assocated with different group common RNTIs can have different reliablity requirements, it is natural to support seperate configurations for PDSCH repetition factor for different MBS services.

Proposal 1: For PDSCH repetition of group-common PDSCH, the PDSCH repetition factors for different MBS services should be separately configured.

HARQ-ACK feedback

In the last e-meeting, it was agreed that for RRC_CONNECTED UEs, HARQ-ACK feedback is supported for multicast. Considering that the level of reliability should be based on the requirements of the application/service provided, whether to transmit HARQ-ACK feedback should be RRC configurable so that a UE can transmit HARQ-ACK for a PDSCH flexibly based on the QoS requirement of multicast service.

Proposal 1: HARQ-ACK feedback for multicast should be RRC configurable.

Regarding HARQ-ACK for multicast, the followings have been identified as some issues to be discussed/decided.

- ACK/NACK feedback or NACK only feedback

Currently, there are two supported HARQ-ACK feedback modes which can be considered for MBS service, i.e., ACK/NACK feedback and NACK only feedback.

- Option 1: dedicated resource for ACK/NACK feedback

  - For the ACK/NACK feedback, a UE will generate ACK or NACK for a TB based on the TB decoding result, and the PUCCH resources for HARQ-ACK feedback needs to be orthogonal among different UEs. gNB can fully get the PDSCH transmission states for each UE. Therefore, it is natural that MBS PDSCH retransmission can be based on unicast PDSCH per UE basis.

- Option 2: common resource for NACK only feedback

  - For the NACK only mode, only when a TB is decoding failed, a UE needs to transmit a NACK. For this feedback mode, the PUCCH resources for HARQ-ACK feedback among different UEs can be group common. Thus, it can be benefit from PUCCH resource consumption. However, gNB is unable to tell which UEs or how many UEs are transmitting NACK due to the group common resource. Thus, it is not possible to support MBS PDSCH retransmission through unicast PDSCH. gNB can’t tell it is ACK or DTX even when the feedback resource is UE-specific. In addition, it will bring some additional issues when considering multiplexing between NACK for multicast PDSCH and other UCI, such as HARQ-ACK for unicast PDSCH, CSI, SR or multiplexing with PUSCH.

Some comparisons of these two HARQ-ACK feedback modes are summarized in the following Table 1.

Table 1 Comparisons of two different HARQ-ACK feedback modes

| HARQ-ACK feedback mode | ACK/NACK feedback | NACK only feedback |
| --- | --- | --- |
| PUCCH resource(s) | Orthogonal among UEs in an MBS group | Same PUCCH resource can be shared among UEs in an MBS group |
| HARQ-ACK information reception for gNB | Full HARQ-ACK information, i.e., ACK, NACK or DTX can be got from each UE. | gNB can determine based on simple energy detection on PUCCH, but gNB can’t be told how many UEs report NACK if the PUCCH resource is group common; gNB can’t determine whether the HARQ of one UE is ACK or DTX if PUCCH resource is UE-specific. |
| MBS PDSCH retransmission | Can be retransmitted by multicast or unicast PDSCH | Can only be retransmitted by multicast PDSCH |
| Multiplexing/prioritization rule of HARQ-ACK for MBS and HARQ-ACK for unicast, CSI, SR or PUSCH | Can be multiplexed with HARQ-ACK for unicast PDSCH and/or other types of UCI, as well as PUSCH transmission. | It’s hard to support multiplexing of HARQ-ACK for MBS and HARQ-ACK for unicast PDSCH and/or other types of UCI, as well as PUSCH transmission. |

To make more comparisons of these two HARQ-ACK feedback modes, system-level simulation was conducted. The evaluation results in terms of spectral efficiency (SE) and resource utilization (RU) for the two HARQ-ACK feedback modes are shown in Table 2. The video traffic with 720p and 30fps is evaluated. UEs belong to same beam generate a UE group. The retransmission methods for ACK/NACK feedback mode and NACK only feedback modes are unicast and multicast respectively. Other detailed simulation assumptions are provided in Appendix.

Table 2 Evaluation results for HARQ-ACK feedback mode

| Feedback mode | ACK/NACK feedback | NACK only feedback |
| --- | --- | --- |
| Cell average spectral efficiency (bps/Hz/TRP) | 1.6372 | 1.6420 |
| Resource utilization | 0.5248 | 0.5535 |

From the simulation results in Table 2, it can be observed that:

- The ACK/NACK feedback mode outperforms with about 5.18% gains in term of RU than the NACK only feedback mode. Compared to the retransmission by group common PDSCH in NACK only feedback mode, the MCS levels of retransmission in ACK/NACK feedback mode are higher due to the UE-specific CSI, which saves more resources, resulting in the lower resource utilization.

- For the cell spectral efficiency, the performances of the two HARQ feedback mode are similar with slight gain of the NACK only feedback scheme mode. The reason of similar results is that both feedback modes can complete the packet transfer in packet interval of generation time, and the gain in SE largely results from the fact that the retransmission MCS of the NACK only feedback mode is lower than that of the ACK/NACK mode, resulting in a lower packet drop probability than that of the ACK/NACK feedback mode.

Observation 1: There is about 5.18% gain in term of RU of the ACK/NACK feedback mode than the NACK only feedback mode.

Observation 2: The NACK only feedback mode has slight gain in terms of SE compared with the ACK/NACK feedback mode.

Based on the pros and cons of these two HARQ-ACK feedback modes, the following proposal is made.

Proposal 2: For RRC_CONNECTED UE, both NACK only and ACK/NACK feedback are supported for multicast.

- HARQ-ACK feedback is per PDSCH or is in aggregated manner.

In NR, for unicast PDSCH transmission, 1-bit/2-bit HAR-ACK information for each unicast PDSCH is transmitted, which depends on the number of TBs of each PDSCH and whether the UE is configured with spatial bundling. Noting that, for MBS PDSCH transmission, the level of reliability is based on the requirements of the application/service provided. In other words, it may not be necessary to support PDSCH retransmission for a certain MBS service and HARQ-ACK feedback for link adaption is enough in that case. If multiple HARQ-ACK transmissions are in the same slot, the HARQ-ACK feedback payload may be very large. In order to save HARQ-ACK payload, aggregated HARQ-ACK feedback, i.e., 1-bit HARQ-ACK for multiple PDSCHs associated with the same PUCCH, can be considered for HARQ-ACK payload offload and power consumption reduction.

- Whether HARQ-ACK for multicast PDSCH and unicast PDSCH be multiplexed in one HARQ-ACK codebook or not

In NR, a UE will construct one HARQ-ACK codebook in one slot if multiple unicast PDSCHs are scheduled to be transmitted in the same slot, and the UE would transmit the HARQ-ACK codebook on one PUCCH determined based on the HARQ-ACK payload and PRI indicated by the last DCI. One issue needs to be discussed is whether HARQ-ACK for multicast PDSCH and unicast PDSCH can be multiplexed in one HARQ-ACK codebook or not. Some potential issues as follows are identified if multiplexing is supported.

- For type 1 HARQ-ACK codebook, a UE will construct the HARQ-ACK codebook based on the TDRA table of PDSCH transmission, if different TDRA tables are used for multicast PDSCH and unicast PDSCH, when determining candidate PDSCH reception occasions, it may need to take the union of TDRA tables for multicast and unicast PDSCH.

- In addition, type 1 codebook can’t support HARQ-ACK feedback for FDMed PDSCHs, then separate construction of HAREQ-ACK codebooks for multicast PDSCH and unicast PDSCH may be needed if they are transmitted in FDMed manner.

- For type 2 HARQ-ACK codebook, DAI is needed to guarantee same understanding on HARQ-ACK payload and order between gNB and UE. If multiplexing of HARQ-ACK for multicast PDSCH and unicast PDSCH in one HARQ-ACK codebook is supported, separate DAI counting may be needed since a UE can be interested in one or multiple MBS services.

- For the determination of PUCCH resource, it is possible that the DCI for multicast PDSCH is the last DCI, so the determination of PUCCH resource may need to be discussed.

Proposal 3: Regarding HARQ-ACK for multicast PDSCH, the followings need to be discussed/decided.

- Aggregated HARQ-ACK feedback, i.e., 1-bit HARQ-ACK for multiple PDSCHs can be considered

- In case of simultaneous multicast and unicast traffic for the same UE, whether HARQ-ACK for multicast PDSCH and unicast PDSCH can be multiplexed in one HARQ-ACK CB

- Conclusion

In this contribution, we make discussions on the mechanisms to improve reliability for RRC_CONNECTED UEs, and have the following observations and proposals.

Observation 1: The ACK/NACK feedback mode gains about 5.18% in terms of RU compared with the NACK only feedback mode.

Observation 2: The NACK only feedback mode has slight gain in terms of SE compared with the ACK/NACK feedback mode.

Proposal 1: For PDSCH repetition of group-common PDSCH, the PDSCH repetition factors for different MBS services should be separately configured.

Proposal 2: For RRC_CONNECTED UE, both NACK only and ACK/NACK feedback are supported for multicast.

Proposal 3: Regarding HARQ-ACK for multicast PDSCH, the followings need to be discussed/decided.

- Aggregated HARQ-ACK feedback, i.e., 1-bit HARQ-ACK for multiple PDSCHs can be considered

- In case of simultaneous multicast and unicast traffic for the same UE, whether HARQ-ACK for multicast PDSCH and unicast PDSCH can be multiplexed in one HARQ-ACK CB

- References

- RP-193248, ‘New Work Item on NR support of Multicast and Broadcast Services’, Huawei

- RAN WG1 #102 e-meeting Chairman’s Notes

- Appendix

Table 3 Simulation assumptions for evaluation of multicast

| Parameters | Value |
| --- | --- |
| Layout | UMa in TR38.901 |
| Inter-BS distance | 500 m |
| Carrier frequency | 4GHz |
| Simulation bandwidth | 20 MHz, 8.2% Guard Band |
| Frame structure | DDDDDDDSUU, 10:2:2 for S |
| SCS / Cyclic Prefix | 30 KHz / NCP |
| Channel model | UMa in TR 38.901 |
| BS antenna configuration | (M, N, P, Mg, Ng; Mp, Np) = (8, 8, 2, 1, 1; 1, 8) (dH,dV) = (0.5, 0.8)λ +45°, -45° polarization |
| UE antenna configuration | (M, N, P, Mg, Ng; Mp, Np) = (1, 2, 2, 1, 1; 1, 2) (dH,dV) = (0.5, N/A)λ 0°,90° polarization |
| BS beam set | For direction of BS analog beam steering (in LCS): Azimuth angle φi = [0] Zenith angle θj = [90 95 100 105] degree NOTE: (azimuth, zenith)=(0, pi/2) is the direction perpendicular to the array. |
| Traffic model | Traffic mode: video Traffic frequency: 30fps Traffic Image resolution: 720*1280p |
| Transmit power | 49 dBm at BS |
| Antenna height | asign with TR38.901 |
| Antenna gain | 8 dBi at BS and 0 dBi at UE |
| Noise figure | 9 dB at UE |
| UE distribution | 80% indoor, with 3km/h 20% outdoor, with 3 km/h |
| Number of UEs per cell | 10 |
| Schedule method | Beam round robin and TDM scheduled |
| Schedule granularity | Unicast: subband; Multicast: wideband |
| PMI CQI feedback granularity | Unicast: subband; Multicast: wideband |
| Max retransmission time | 4 |
| MCS table | EMBB_CP_OFDM_64QAM |
| Receiver | MMSE-IRC |