3GPP TSG RAN WG1 #103-e                                      R1-2007694

e-Meeting, October 26th – November 13th, 2020

Source:	vivo

Title:	Other issues for Rel-17 MBS

Agenda Item:	8.12.4

Document for:  Discussion and Decision

Introduction

According to the WID [1] of the NR MBS, the MBS service of NR is to be provided to both the IDLE/INACTIVE UE and the CONNECTED UE. As the UE could be configured to provide the UL feedbacks (e.g. HARQ feedback), the UE would be required to support the HARQ process for the MBS reception. In this contributions, we provide our analysis on some of the remaining issues on supporting the HARQ operation for the MBS.

Discussion

HARQ process allocation

According to the LTE MBMS/SC-PTM, the MBS PDSCH does not support HARQ. However, the L2 buffer of a UE is shared between the unicast reception and the MBS reception according to 3GPP TS 36.306 [2], as this can save the UE buffer cost and make more efficient use of the UE buffer. Then if the NR MBS is to support the soft buffer of the HARQ, the soft buffer of the HARQ process(es) should be shared between the unicast (i.e. unicast PDSCH) and the MBS (i.e. groupcast PDSCH). Thus we consider that the same HARQ process should be shared between the unicast and the MBS.

Proposal 1: The MBS transmission (i.e. groupcast PDSCH) and the unicast transmission (i.e. unicast PDSCH) share the same HARQ process pool.

If the HARQ processes are shared between the MBS and the unicast, one may consider that we could have the following HARQ process allocation schemes to avoid the HARQ process collision between the MBS and the unicast:

- Option 1: semi-static HARQ-process sharing

- Option 2: Dynamic HARQ-process sharing

For the IDLE/INACTIVE UE MBS reception, the UE can use all HARQ processes as no HARQ process is used for the unicast transmission, and there is no HARQ process collision between the MBS and the unicast. Given that the P-RNTI PDSCH (i.e. paging) does not support HARQ and the SI-RNTI PDSCH (i.e. SI) uses a dedicated broadcast HARQ process [3].

For the CONNNECTED UE MBS reception, as the gNB can know the UE’s MBS reception via the RRC message of the UE’s MBS interest indication, the gNB can avoid the HARQ process collision between the unicast and the MBS. Then the gNB can simply indicate the HARQ process via the g-RNTI DCI.

Proposal 2: For the dynamic scheduling of MBS, the HARQ process used for the MBS transmission is indicated via the g-RNTI DCI.

If RAN1 agreed to support the semi-static resource allocation (e.g. alike SPS) for the MBS transmission, the RRC could configure the HARQ process used for the semi-static resource of the MBS, alike the current SPS configuration. Alternatively, the HARQ-process for SPS-like MBS transmission can be implicitly derived based on the slot index, similarly as the SPS HARQ-process ID determination in Rel-15/16

Proposal 3: For the semi-static resource allocation of MBS (i.e. SPS alike), the HARQ-process ID for MBS transmission can be either RRC-configured, or implicitly derived based on slot index.

HARQ new/re-transmission

As discussed in [4], either group PDCCH (i.e. g-RNTI based) or unicast PDCCH (i.e. C-RNTI based) can be used to schedule the transmission of MBS PDSCH.

Regarding the HARQ retransmission of the MBS service, as the gNB by implementation can avoid the HARQ process collision between the unicast service and the MBS service according to the analysis given in section 2.1, then the gNB can indicate the HARQ retransmission of MBS service via either C-RNTI or g-RNTI. Furthermore, using the unicast PDCCH for the MBS retransmission would also improve the transmission reliability of the MBS service due to UE specific beamforming and save the power of the other UEs which have already received an MBS TB successfully. However, one may consider that using the g-RNTI based PDCCH for scheduling of HARQ retransmission (i.e. without sending the MBS TB to each UE independently) would also benefit the resource utilization. Thus, it would also be possible that the gNB by implementation can dynamically select whether to use the C-RNTI or the g-RNTI for the scheduling of a MBS HARQ retransmission, so as to achieve a good balance between the resource utilization and the transmission reliability.

To evaluate the performance of different retransmission schemes, i.e. with C-RNTI, g-RNTI, or C-RNTI/g-RNTI dynamically selected by gNB implementation, system-level simulation was conducted. The evaluation results in terms of spectral efficiency (SE) and resource utilization (RU) are shown in Table 1. The video traffic with 720p and 30fps is evaluated. UEs belong to same beam generate a UE group. For retransmission scheme of C-RNTI/g-RNTI dynamically selected by gNB implementation, C-RNTI is selected when the UEs number needing retransmission belong to same beam is more than one, otherwise the g-RNTI is used. Other detailed simulation assumptions are provided in Appendix.

Table 1 Evaluation results for MBS HARQ retransmission identity

| HARQ re-transmission scheme | g-RNTI only | C-RNTI only | C-RNTI/g-RNTI |
| --- | --- | --- | --- |
| Cell average spectral efficiency (bps/Hz/TRP) | 1.6420 | 1.6372 | 1.6427 |
| Resource utilization | 0.5535 | 0.5248 | 0.5190 |

Based on the results shown above, it can be observed that,

- The retransmission scheme with C-RNTI/g-RNTI dynamically selected has about 6.23% and 1.11% gains in term of RU than the g-RNTI only and the C-RNTI only scheme respectively. Compared with C-RNTI, the slight gains of C-RNTI/g-RNTI can be attributed to the group common retransmission when there are multiple UEs belong to same beam need retransmission, which spends only one slot for retransmission while multi slots are required in C-RNTI only scheme. Compared with g-RNTI, the reason for the gain obtained in C-RNTI/g-RNTI is that the MCS levels of retransmission for C-RNTI are higher than that of C-RNTI due to UE-specific CSI, which saves more resources, resulting in the lower resource utilization.

- From the point of view of cell spectral efficiency, the performances of the three kinds of MBS HARQ retransmission identities are similar. The main reason for the differences of the three retransmission identities in cell spectral efficiency is that MBS HARQ retransmission identity involves C-RNTI, which makes the MCS level of initial transmission and interference conditions different from g-RNTI due to the UE specific CSI.

Observation 1: The retransmission scheme with dynamically selected C-RNTI/g-RNTI brings about 6.23% and 1.11% gain in term of RU compared to the g-RNTI only and C-RNTI retransmission scheme respectively.

Observation 2: For the cell spectral efficiency, the performances of the three kinds of MBS HARQ retransmission schemes are similar.

Proposal 4: If g-RNTI scheduled MBS transmission is supported, the HARQ re-transmission can be scheduled via either g-RNTI or C-RNTI.

Conclusions

In this contribution, we make discussions on the other issues for Rel-17 MBS, and we have the following proposals:

Observation 1: The retransmission scheme with dynamically selected C-RNTI/g-RNTI brings about 6.23% and 1.11% gain in term of RU compared to the g-RNTI only and C-RNTI retransmission scheme respectively.

Observation 2: For the cell spectral efficiency, the performances of the three kinds of MBS HARQ retransmission schemes are similar.

Proposal 1: The MBS transmission (i.e. PDSCH scheduled by C-RNTI) and the unicast transmission (i.e. PDSCH scheduled by MBS-RNTI) share the same HARQ process pool.

Proposal 2: For the dynamic scheduling of MBS, the HARQ process used for the MBS transmission is indicated via the g-RNTI DCI.

Proposal 3: For the semi-static resource allocation of MBS (i.e. SPS alike), the HARQ-process ID for MBS transmission can be either RRC-configured, or implicitly derived based on slot index.

Proposal 4: If g-RNTI scheduled MBS transmission is supported, the HARQ re-transmission can be scheduled via either g-RNTI or C-RNTI.

Reference

[1] RP-201038, Huawei, “WID revision: NR Multicast and Broadcast Services”.

[2] 3GPP TS 36.306, “Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio access capabilities”.

[3] 3GPP TS 36.321, “NR; Medium Access Control (MAC) protocol specification”.

- Appendix

Table 2  Simulation assumptions for evaluation of MBS service

| Parameters | Value |
| --- | --- |
| Layout | UMa in TR38.901 |
| Inter-BS distance | 500 m |
| Carrier frequency | 4GHz |
| Simulation bandwidth | 20 MHz, 8.2% Guard Band |
| Frame structure | DDDDDDDSUU, 10:2:2 for S |
| SCS / Cyclic Prefix | 30 KHz / NCP |
| Channel model | UMa in TR 38.901 |
| BS antenna configuration | (M, N, P, Mg, Ng; Mp, Np) = (8, 8, 2, 1, 1; 1, 8) (dH,dV) = (0.5, 0.8)λ +45°, -45° polarization |
| UE antenna configuration | (M, N, P, Mg, Ng; Mp, Np) = (1, 2, 2, 1, 1; 1, 2) (dH,dV) = (0.5, N/A)λ 0°,90° polarization |
| BS beam set | For direction of BS analog beam steering (in LCS): Azimuth angle φi = [0] Zenith angle θj = [90 95 100 105] degree NOTE: (azimuth, zenith)=(0, pi/2) is the direction perpendicular to the array. |
| Traffic model | Traffic mode: video Traffic frequency: 30fps Traffic Image resolution: 720*1280p |
| Transmit power | 49 dBm at BS |
| Antenna height | assign with TR38.901 |
| Antenna gain | 8 dBi at BS and 0 dBi at UE |
| Noise figure | 9 dB at UE |
| UE distribution | 80% indoor, with 3km/h 20% outdoor, with 3 km/h |
| Number of UEs per cell | 10 |
| Schedule method | Beam round robin and TDM scheduled |
| Schedule granularity | Unicast: subband; Groupcast: wideband |
| PMI CQI feedback granularity | Unicast: subband; Groupcast: wideband |
| Max retransmission time | 4 |
| MCS table | EMBB_CP_OFDM_64QAM |
| Receiver | MMSE-IRC |