3GPP TSG RAN WG1 #103-e                                             R1-2007696

e-Meeting, October 26th – November 13th, 2020

Source:	vivo

Title:	Discussion on joint scheduling

Agenda Item:	8.13.2

Document for:	Discussion and Decision

- Introduction

Regarding the study of scheduling PDSCH on multiple cells using a single DCI in R17 DSS WID [1], the following agreements have been made[2]:

Agreements: - For the study on single DCI scheduling PDSCH on two cells - Consider the following scenarios as baseline for evaluation - UE configured with Inter-band CA with PCell and an SCell - PCell for the UE is operated on a DSS carrier (i.e., same carrier is also used for serving LTE users) - Case 1: Different SCS for PCell and SCell - Case 2: Same SCS for PCell and Scell - Additional scenarios can also be evaluated, e.g. as below - Intra-band CA case with multiple serving cells having same SCS (all cells operated on non DSS carriers) - Inter-band CA case with PCell and more than one SCell (at least the SCells are operated on non DSS carriers) - Note: other combinations not precluded - Note: Further details of evaluation framework (including carrier BW, slot format etc.) to be discussed in next stage

In the contribution, we provide our views and simulation results on scheduling PDSCH on two cells with a single DCI.

- Scenario and assumptions

Dynamic spectrum sharing (DSS) provides a very useful migration path from LTE to NR by allowing LTE and NR to operate on the same frequency, it is important to ensure sufficient scheduling capacity for NR UEs on the carriers shared with LTE. However, some operators are operating several narrow spectrums scattered across different frequencies. For example, Table 1 from [3] has listed some narrow channel bandwidths that have been requested so far. With such narrow bandwidth, PDCCH capacity becomes a major bottleneck on DSS carriers to achieve the best performance.

Table 1. Requested narrow channel bandwidth

| Band (s) | Channel Bandwidth(s) |
| --- | --- |
| n5 | 7, 11 MHz |
| n12 | 12 MHz |
| n26 | 7 MHz |
| n28 | 13, 33 MHz |
| n29 | 6, 7, 8, 11 MHz |

Compared to single scheduling, multi-cell scheduling is expected to allow gNB to provide the same or more scheduling opportunities with limited PDCCH resources, thereby hopefully improving resource efficiency. Before evaluating the performance of multi-cell scheduling, we need to identify the scenario for performance comparison.

It was agreed that the baseline for evaluation of multi-cell scheduling is that Pcell is on the DSS carrier. A typical deployment concerning DSS is that the PDCCHs are always located on the DSS carrier to provide sufficient DL coverage, in which case the PDCCHs capacity on the Pcell may be problematic and be prone to PDCCH blocking. Therefore, multi-cell scheduling should be compared with single scheduling with enabled cross-carrier scheduling to identify if multi-cell scheduling can alleviate the PDCCH blocking issue, the following scenario should be investigated as a baseline.

- Scenario of single scheduling for comparison:

  - Pcell schedules Pcell and cross-carrier-schedules Scell by single scheduling

- Scenario of multi-cell scheduling for evaluation:

  - Joint DCI on Pcell schedules Pcell and Scell simultaneously

In addition to scenarios, detailed assumptions such as bandwidth and DCI size should be aligned so that the evaluation results can be fairly compared. Regarding the perspective of bandwidth, at least some typical NR bandwidth configurations should be considered for evaluation. Currently, the representative bandwidths in NR are 20MHz and 100MHz, which can be suggested as the basic simulation assumptions for the bandwidth of Pcell and Scell, respectively. Multi-cell scheduling can be proved to be generalizable enhancement only if it brings gain under these assumptions.

Joint DCI size depends on the specific design, such as which domains can be shared by two cells. It may be too early to conclude the detailed field design of the joint DCI at this stage; however, we can determine a preliminary but reasonable DCI size range that yields acceptable gain according to simulations under different size assumptions, and then identify whether support of multi-cell scheduling is necessary by analyzing how much flexibility could be sacrificed to meet this size limit and whether it is feasible to achieve the size limit.

Figure 1. scenarios for comparison between single scheduling and multi-cell scheduling

Proposal 1. Consider the following scenarios and assumptions for performance evaluation and comparison: 
-  Scenario
  -  multi-cell scheduling: Pcell on a DSS carrier performs self-scheduling of Pcell and cross-carrier scheduling of a Scell with a joint DCI
  -  single scheduling: Pcell on a DSS carrier performs self-scheduling of Pcell and cross-carrier scheduling of a Scell with different single DCIs
-  Assumptions of bandwidth: Pcell(20MHz)+Scell(100MHz)
-  Assumptions of DCI size excluding CRC: single DCI(60 bits), joint DCI(72/84/96/108 bits)

On the other hand, several aspects are not clear according to the current WID and need further clarification.

First of all, if a single DCI schedules PDSCH on multiple cells, there can be different understandings of the number of TBs to be transmitted in the scheduled PDSCHs. We would like to clarify that PDSCHs jointly scheduled by a single DCI should correspond to different TBs, there would be a significant impact on RAN2 otherwise. Specifically, if the PDSCHs correspond to single TB, in addition to the necessary work in the physical layer that needs to be done, high layer spec also needs to be heavily modified to support this feature. For example, RAN2 may need to change their protocol architecture to allow sharing a HARQ entity between two cells as well as joint buffer maintenance of two cells, which is complicated and may not be feasible.

Proposal 2. The study focuses on the case that the two PDSCH jointly scheduled by a single DCI correspond to different TBs.

Secondly, it is noted that there are a lot of commonalities between the design of UL grant and DL grant in R15 and R16. However, the WID only mentions multi-cell DL scheduling for enhancement. Thus, it is beneficial to clarify whether or not multi-cell UL scheduling needs to be studied.

Proposal 3. Clarify whether PUSCH multi-cell scheduling should be studied.

- Evaluation and analysis

To evaluate the potential gain brought by multi-cell scheduling for both DSS case and general CA case, we performed SLS and LLS based on the simulation assumptions @ 2.6GHz /4GHz as given in the Appendix.

- Scenario#1 of multi-cell scheduling for DSS case

  - Join DCI is transmitted on Pcell at 2.6GHz

- Scenario#2 of multi-cell scheduling for general CA case

  - Join DCI is transmitted on Pcell at 4GHz

- Scenario of single scheduling for comparison

  - Pcell schedules Pcell and cross-carrier-schedules Scell by single scheduling

In this section, several aspects including CCE saving, PDCCH blocking, and scheduling flexibility are analyzed. In the LLS simulation, we assumed that the payload size of a non-fallback DCI which schedules a single serving cell is 60bits (excluding 24bits CRC), and the payload size of joint DCI is 72/84/96/108 bits (excluding 24bits CRC) corresponding to a 40/30/20/10% DCI information bits compression rate compared with that for 2 single DCIs.

- CCE saving

Table 2 and Table 3 provide the required SNR to achieve 1% BLER as well as the corresponding distribution of CCE with different ALs. Based on the SNR distribution CDF provided in Table 8 and Table 9 of the appendix, the distribution of UEs for each AL in the evaluated deployment is also provided. On top of it, the average number of CCEs occupied by a single DCI or a joint DCI can be derived. It can be observed that when the size of a joint DCI (excluding CRC) is no larger than 60% of the total size of the two single DCIs, it saves more than 44.87% or 44.51% CCE compared with the case where two single DCIs are transmitted to schedule two PDSCH at 2.6GHz and 4GHz respectively. When the compression ratio of joint DCI becomes 10%, the amount of CCE saved drops and becomes less than 32%.

Observation 1. Compared with using single DCI, a joint DCI scheduling two PDSCHs on two cells brings around 44.87% and 44.51% CCE saving gain if the joint DCI size (excluding CRC) is less than 60% of the total size of two single DCI at 2.6GHz and 4GHz respectively, and the gain becomes less significant if the compression rate of joint DCI size increases.

Table 2. Required SNR and the distribution of each AL @ 2.6 GHz

| 2.6GHz | Single DCI excluding CRC |  | Joint DCI excluding CRC |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 60 bits |  | 72 bits (40% compression) |  | 84 bits (30% compression) |  | 96 bits (20% compression) |  | 108 bits (10% compression) |  |
| AL | SNR | % | SNR | % | SNR | % | SNR | % | SNR | % |
| AL=1 | 12.45 | 23 | 14.13 | 20 | 17.82 | 15 | / | / | / | / |
| AL=2 | 5.047 | 24 | 6.25 | 23 | 7.229 | 24 | 8.11 | 36 | 8.887 | 33 |
| AL=4 | 0.5421 | 25 | 1.087 | 25 | 1.596 | 26 | 2.106 | 26 | 2.492 | 26 |
| AL=8 | -2.842 | 19 | -2.378 | 21 | -1.854 | 21 | -1.491 | 22 | -1.114 | 23 |
| AL=16 | -5.787 | 6 | -5.395 | 8 | -4.978 | 10 | -4.607 | 12 | -4.273 | 13 |
| Average number of CCEs | 4.32 |  | 4.76 |  | 5.16 |  | 5.67 |  | 5.92 |  |
| Saving ratio | / |  | 44.87% |  | 40.32% |  | 34.41% |  | 31.52% |  |

Table 3. Required SNR and the distribution of each AL @ 4 GHz

| 4GHz | Single DCI excluding CRC |  | Joint DCI excluding CRC |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 60 bits |  | 72 bits (40% compression) |  | 84 bits (30% compression) |  | 96 bits (20% compression) |  | 108 bits (10% compression) |  |
| AL | SNR | % | SNR | % | SNR | % | SNR | % | SNR | % |
| AL=1 | 12.23 | 24 | 14.21 | 21 | 18.41 | 14 | / | / | / | / |
| AL=2 | 4.977 | 22 | 6.02 | 21 | 6.849 | 25 | 7.724 | 36 | 8.387 | 33 |
| AL=4 | 0.8775 | 20 | 1.429 | 21 | 1.872 | 22 | 2.365 | 22 | 2.919 | 22 |
| AL=8 | -2.514 | 21 | -2.103 | 21 | -1.615 | 20 | -1.254 | 21 | -0.9002 | 22 |
| AL=16 | -5.132 | 7 | -4.743 | 10 | -4.358 | 12 | -4.063 | 13 | -3.75 | 15 |
| Average number of CCEs | 4.55 |  | 5.05 |  | 5.42 |  | 5.83 |  | 6.20 |  |
| Saving ratio | / |  | 44.51% |  | 40.49% |  | 36.02% |  | 31.96% |  |

While joint DCI may result in less CCE overhead as discussed above, the overhead may be saved and reused for PDSCH for higher spectrum efficiency. Assuming a CORESET configuration of 2 symbols in time and a system-wide BW, the total PDCCH overhead in a slot is approximately 14.29%. If PDSCH is rate matched around PDCCH resources, the 44.87% CCE savings gain can be repurposed as additional PDSCH resources to provide up to 6.41% throughput improvement, which is not very attractive. Alternatively, if the PDSCH rate matches around the entire CORESET, the 44.87% CCE savings gain cannot bring throughput gain because it is not sufficient to reduce the number of symbols required for a CORESET and the resources allocated for control and data would remain the same as the case without CCE saving. Note that here we assume all the UEs in the system are scheduled by joint DCI so the 44.87% CCE saving gain and 6.41% throughput gain can be understood as the upper limit of CCE saving and throughput improvement respectively.

Observation 2. CCE saving by joint DCI scheduling can bring up to around 6.41% throughput gain if the PDSCH can rate match around PDCCH.

- PDCCH blocking

When the number of UE in the system increases, conflicts inevitably occur between the PDCCHs of different UEs, resulting in some UEs not being able to be successfully scheduled and thus some potential PDSCH resources not being fully utilized. Thus, we evaluated the PDCCH blocking rate based on the above AL distribution.

It can be observed from Figure 2 that when the number of UE using joint DCI is 5, the average blocking rate is reduced by about 8.55%-13.57% at 4 GHz and 8.35%-13.95% at 2.6 GHz with joint DCI. More specifically, When the joint DCI size is 108 bits, it brings about 8.35% or 8.55% reduction in PDCCH blocking rate for 2.6GHz and 4GHz respectively. Compressing the DCI payload size can further reduce the PDCCH blocking rate. For example, when joint DCI size is reduced to 72 bits, the reduction rate increases to 13.95% or 13.57% for 2.6GHz and 4GHz respectively.

Observation 3. Compared with using single DCI, a joint DCI scheduling two PDSCHs on two cells brings around 13.95% or 13.57% reduction in PDCCH blocking rate if the joint DCI size (excluding CRC) is less than 60% of the total size of two single DCI at 2.6GHz and 4GHz respectively, and the reduction rate becomes less significant if the compression rate of joint DCI size increases.

Figure 2. Blocking rate (No. of PDCCH candidate for AL {1 2 4 8 16} = {6 6 2 2 2})

To estimate the PDCCH blocking rate reduction in a practical scenario, we also perform system level evaluation assuming the traffic model in Table 10 to see how many UEs are scheduled in one CORESET as shown in Figure 3. It can be observed that the number of scheduled UEs in one CORESET is 1 in most instances, in which case joint DCI won’t bring PDCCH blocking rate reduction. Taking 4GHz as an example, considering the PDCCH blocking rate per UE number in Figure 2 and the distribution for the number of scheduled UEs in Figure 3, the average PDCCH blocking rates for the practical scenario in 4GHz are given in Table 4. It can be observed that the PDCCH blocking rate reduction brought by joint DCI with one UE is 0, and the average PDCCH blocking rate reduction with joint DCI is no larger than 3.31%.

In another aspect, the contribution to throughput improvement by joint DCI should be even smaller than the PDCCH blocking reduction rate. For example, when the number of scheduled UEs is 2 and the PDCCH of the second UE is blocked, the first UE can still be scheduled and the throughput benefit brought by PDCCH blocking reduction from joint DCI only contains the second UE’s scheduled PRBs.

Observation 4. PDCCH blocking rate reduction by joint DCI scheduling can only bring a marginal <=3.31% throughput gain in practical scenarios at 4GHz.

Table 4. Average PDCCH blocking rate for practical scenario

|  | Joint DCI size | PDCCH blocking rate reduction per UE number (1,2,3,4) with joint DCI | Ratio for No. scheduled UEs (1,2,3,4) | Average PDCCH blocking rate reduction |
| --- | --- | --- | --- | --- |
| 4GHz | 72 | (0, 9.24%, 12.01%, 13.28%) | (66.73%, 24.99%, 6.91%, 1.29%) | 3.31% |
|  | 84 | (0, 9.24%, 11.25%, 11.60%) |  | 3.24% |
|  | 96 | (0, 9.24%, 10.39%, 10.57%) |  | 3.16% |
|  | 108 | (0, 9.24%, 9.31%, 8.60%) |  | 3.06% |

Note: PDCCH blocking rate of 2 single DCI per UE number (1,2,3,4)=(0, 9.24%, 17.36%, 24.41%)

Figure 3. No of scheduled UEs @ 2.6 GHz and @ 4 GHz

- Scheduling flexibility

According to the simulation results above, to achieve more CCE saving and PDCCH blocking rate reduction, the joint DCI payload size must be compressed to around or less than 96 bits, which means that many of the fields such as resource allocation related fields in the joint DCI must be shared by two cells. However, field sharing reduces scheduling flexibility and thus affects spectrum efficiency adversely. For example, if the FDRA is shared between the two scheduled cells to save DCI payload size, gNB may not be able to schedule the most appropriate UE for some sub-bands of one cell due to the restrictions or considerations of the other cell, which may instead lead to throughput loss.

Observation 5. The scheduling flexibility will be restricted if a large DCI compression rate is assumed, which would degrade the spectrum efficiency.

In summary, multi-cell scheduling could bring throughput gain in terms of CCE saving and PDCCH blocking rate reduction. On one hand, the gain is not much significant in practical scenario based on the above evaluation and analysis. On the other hand, the benefit is obtained by scarifying the scheduling flexibility which would degrade the spectrum efficiency. How to make the trade-off to achieve maximum throughput gain in a typical scenario should be studied before deciding on whether or not to specify joint DCI for multiple cell scheduling.

Proposal 4. For joint DCI with DCI payload compression, study the trade-off between system capacity improvement due to CCE saving/PDCCH blocking rate reduction and the spectrum efficiency loss due to degraded scheduling flexibility.

- Potential issues

As shown in the above evaluation, DCI size compression is important to achieve resource-saving and thus system throughput improvement. However, how to compress the DCI payload size for multi-cell scheduling of different TBs is a challenging task. On one side, in R15 and R16, most of the information fields in a DCI depend on the configurations of the scheduled cell, so an information field cannot be directly shared between the cells, especially if there is a big difference between the configurations of the two cells. An example is in Figure 4, CC#1 and CC#2 have different SCS, TDD configuration, and bandwidth, it would be hard for the two cells to share all fields. On the other side, if each functionality has two separate information fields for the cells in the DCI of multi-cell scheduling, the DCI payload size, as well as the number of CCE consumed, may double, which does not reduce the PDCCH overhead overall and PDCCH detection reliability is also impacted.

Figure 4. Example of multi-cell scheduling

It is preferable not to share the fields that heavily depend on the configuration of the scheduled cell and the fields that directly affect the transmission mechanism such as MCS. The fields that do not depend on the scheduled cells, such as MIMO related information and PUCCH-related information that is mainly associated with PUCCH Pcell, can be considered to be shared between two cells.

In the following table, we listed some major information fields and provides a preliminary analysis of the field type. The single DCI is 60 bits while the joint DCI based on Table 5 is 107 bits. In this case, the compression ratio of joint DCI is around 10% which means it achieves less than 8.35% PDCCH blocking gain according to our simulation. Thereby, some CC-dependent fields other than MIMO and PUCCH related fields should be further jointly coded for the scheduled carries to reduce DCI size.

Table 5. DCI information field type

| Information field | Separate field or shared field | Number of bits |
| --- | --- | --- |
| Identifier for DCI formats – 1 bit | Shared field | 1 |
| Carrier indicator – 0 or 3 bits | Shared field | 3 |
| Bandwidth part indicator – 0, 1 or 2 bits | Shared field | 2 |
| PRB bundling size indicator – 0 or 1 bit | Separate field | 2 |
| Rate matching indicator – 0, 1, or 2 bits | Separate field | 4 |
| ZP CSI-RS trigger – 0, 1, or 2 bits | Separate field | 4 |
| FDRA – 13 bits for 106 RB in 20MHz | Separate field | 26 |
| TDRA – 0, 1, 2, 3 or 4 bits | Separate field | 8 |
| Modulation and coding scheme – 5 bits | Separate field | 10 |
| New data indicator – 1 bit | Separate field | 2 |
| Redundancy version – 2 bits | Separate field | 4 |
| HARQ process number – 4 bits | Separate field | 8 |
| VRB-to-PRB mapping – 1 bit | Shared field | 1 |
| Downlink assignment index – 0,2,4 bits | Shared field | 4 |
| TPC command for scheduled PUCCH – 2 bits | Shared field | 2 |
| PUCCH resource indicator – 3 bits | Shared field | 3 |
| PDSCH-to-HARQ feedback timing indicator – 3 bits | Shared field | 3 |
| Antenna port(s) – 4, 5, or 6 bits | Separate field | 12 |
| SRS request – 2, 3 bits | Separate field | 6 |
| DMRS sequence initialization – 1 bit. | Separate field | 2 |
| Joint DCI size |  | 107 bits (excluding CRC) |

Proposal 5. The fields type (shared or cell-specific) in joint DCI needs to be investigated.

In addition to the field design of joint DCI mentioned above, the following issues need to be resolved as well.

- The framework of multi-cell scheduling

In cross-carrier scheduling, UE is indicated by RRC IE CrossCarrierSchedulingConfig that the data on this cell is scheduled either by PDCCH in its own cell (i.e. ‘own’ in schedulingCellInfo) or PDCCH on another cell (i.e. ‘other’ in schedulingCellInfo). It is not clear if a cell that can be jointly scheduled is allowed to perform self-scheduling, e.g., UL scheduling. If yes, the design of multi-cell scheduling to some extent is similar to type3 CSS. If not, it should be further clarified that whether the UL grant for the jointly scheduled cell can be located on a cell other than the one on which joint DCI is received. We need to clarify whether the support of multi-cell scheduling may reuse the current cross-carrier scheduling framework with some changes.

- Whether multi-cell scheduling reuses the framework of type3 CSS or cross-carrier scheduling

- Whether a cell jointly scheduled by another cell can be configured with PDCCH resources and can perform self-scheduling, e.g., for UL transmission

- Whether a joint DCI monitored for a cell and a single DCI monitored for the same cell should be located on the same serving cell

- Configuration of cell pairs for multi-cell scheduling

It is necessary to further clarify that how to configure the cell pairs for multi-cell scheduling and whether there are any restrictions on choosing cell pairs for multi-cell scheduling. For example, the following aspects need to be clarified:

- Whether there can be more than one cell configured with a joint DCI

- Whether a Scell can be configured with a joint DCI

- Whether a cell can jointly schedule other two cells with a joint DCI

- Whether Pcell can be scheduled by a Scell with a joint DCI

- SS configuration and DCI format for multi-cell scheduling

Another aspect that needs to study is the DCI format for multi-cell scheduling. If an existing DCI format such as DCI format 1_1 is used for multi-cell scheduling purpose, the DCI for single scheduling case should always perform zero-padding to keep alignment with the DCI size of the case where multi-cell scheduling is enabled, which implies that a large number of redundant bits will be inserted into the DCI format even for single scheduling. In this case, it not only leads to a higher DCI code rate but also leads to worse coverage and worse performance for single scheduling. By contrast, introducing a new DCI format for multi-cell scheduling can ensure a high degree of scheduling flexibility while avoiding negative impacts on the performance of the existing DCI. However, it raises some other issues, such as how to maintain the DCI size budget.

- PDCCH BD budget maintenance for multi-cell scheduling

In R15, the PDCCH BD budget is maintained for each scheduled cell and there is only one scheduled cell corresponding to a single DCI. However, two scheduled cells are corresponding to a joint DCI in the multi-cell scheduling case, it should be discussed how to maintain the PDCCH BD budget across the scheduled cell pair. For example, whether the two scheduled cells share the same budget, or if they should be handled individually.

- HARQ-ACK reporting for multi-cell scheduling

For single TB on every PDSCH occasion involved in a HARQ-ACK codebook in R15, there is only 1 HARQ-ACK bit generated in response to the corresponding DCI with an associated PUCCH. However, if multi-cell scheduling is supported, there would be more than 1 TB scheduled by a DCI, which may have different outcomes of PDSCH decoding. Thus, some discussions such as whether HARQ-ACK bits corresponding to the jointly scheduled PDSCHs should be multiplexed in the same codebook and how to determine the HARQ-ACK in response to a joint DCI, as well as some enhancements for the HARQ-ACK codebook construction, i.e., how to guarantee the reliability of HARQ codebook in case of missed joint DCI, are needed.

Observation 6. To support multi-cell scheduling, the following issues need to be resolved
-  DCI field design
-  Any restrictions on the scheduled cells to be paired for multi-cell scheduling
-  Framework of multi-cell scheduling
-  Whether to introduce a new DCI format 
-  PDCCH BD budget maintenance if multi-cell scheduling is enabled
-  HARQ-ACK codebook determination if multi-cell scheduling is enabled

- Conclusion

In this contribution, we discuss some issues on multi-cell scheduling and have the following observation and proposals:

Observation 1. Compared with using single DCI, a joint DCI scheduling two PDSCHs on two cells brings around 44.87% and 44.51% CCE saving gain if the joint DCI size (excluding CRC) is less than 60% of the total size of two single DCI at 2.6GHz and 4GHz respectively, and the gain becomes less significant if the compression rate of joint DCI size increases.

Observation 2. CCE saving by joint DCI scheduling can bring up to around 6.41% throughput gain if the PDSCH can rate match around PDCCH.

Observation 3. Compared with using single DCI, a joint DCI scheduling two PDSCHs on two cells brings around 13.95% or 13.57% reduction in PDCCH blocking rate if the joint DCI size (excluding CRC) is less than 60% of the total size of two single DCI at 2.6GHz and 4GHz respectively, and the reduction rate becomes less significant if the compression rate of joint DCI size increases.

Observation 4. PDCCH blocking rate reduction by joint DCI scheduling can only bring a marginal <=3.31% throughput gain in practical scenarios at 4GHz.

Observation 5. The scheduling flexibility will be restricted if a large DCI compression rate is assumed, which would degrade the spectrum efficiency.

Observation 6. To support multi-cell scheduling, the following issues need to be resolved
-  DCI field design
-  Any restrictions on the scheduled cells to be paired for multi-cell scheduling
-  Framework of multi-cell scheduling
-  Whether to introduce a new DCI format 
-  PDCCH BD budget maintenance if multi-cell scheduling is enabled
-  HARQ-ACK codebook determination if multi-cell scheduling is enabled

Proposal 1. Consider the following scenarios and assumptions for performance evaluation and comparison: 
-  Scenario
  -  multi-cell scheduling: Pcell on a DSS carrier performs self-scheduling of Pcell and cross-carrier scheduling of a Scell with a joint DCI
  -  single scheduling: Pcell on a DSS carrier performs self-scheduling of Pcell and cross-carrier scheduling of a Scell with different single DCIs
-  Assumptions of bandwidth: Pcell(20MHz)+Scell(100MHz)
-  Assumptions of DCI size excluding CRC: single DCI(60 bits), joint DCI(72/84/96/108 bits)

Proposal 2. The study focuses on the case that the two PDSCH jointly scheduled by a single DCI correspond to different TBs.

Proposal 3. Clarify whether PUSCH multi-cell scheduling should be studied.

Proposal 4. For joint DCI with DCI payload compression, study the trade-off between system capacity improvement due to CCE saving/PDCCH blocking rate reduction and the spectrum efficiency loss due to degraded scheduling flexibility.

Proposal 5. The fields type (shared or cell-specific) in joint DCI needs to be investigated.

- Reference

- RP-193260, “New WID on NR Dynamic spectrum sharing (DSS)” , Ericsson, RAN #86, Dec 2019.

- RAN1#102, e-Meeting, Chairman note

- RP-201352, “New SID: Efficient utilization of licensed spectrum that is not aligned with existing NR channel bandwidths”, T-Mobile USA, Ericsson, RAN#88-E, Jun 2020.

- Appendix

Table 6. LLS Simulation assumptions for PDCCH

| Parameters | Values |
| --- | --- |
| Carrier frequency | 2.6GHz 4GHz |
| Subcarrier spacing | 2.6GHz:15kHz 4GHz:30kHz |
| Simulation bandwidth/No. of PRBs | 2.6GHz:20MHz/106 RB 4GHz:100MHz/273RB |
| Link-level Channel model | TDL-C |
| TDL correlation level | Low |
| Delay Spread | 300ns |
| UE speed | 3 km/h |
| Antenna configuration at BS | 2T |
| Antenna configuration at UE | 2R |
| CORESET duration | 2 |
| CORESET RBs | 96 |
| Aggregation level | 1/2/4/8/16 |
| CCE-to-REG mapping | interleaved |
| REG bundle size | 6 |
| Interleaver size | 2 |
| Tx Diversity | One port precoder cycling |
| Target BLER | 1% |
| Single DCI size and AL for baseline | 60+24 bits CRC, AL={1,2,4,8,16} |
| Joint DCI size and AL | 72+24 bits CRC, AL={1,2,4,8,16} |
| Joint DCI size and AL | 84+24 bits CRC, AL={1,2,4,8,16} |
| Joint DCI size and AL | 96+24 bits CRC, AL={2,4,8,16} |
| Joint DCI size and AL | 108+24 bits CRC, AL={2,4,8,16} |

Table 7. SLS Simulation assumptions

| Parameters | Values |
| --- | --- |
| Scenario | Urban Macro |
| Frequency | 2.6GHz 4GHz |
| BS Tx power | 49dBm for 20MHz 56dBm for 100MHz (Follow TR38.901) |
| Indoor UE Penetration Loss | 20% high loss, 80% low loss |
| ISD | 500m |
| TRxP antenna configurations | (M, N, P, Mg, Ng;Mp,Np) = (8, 8, 2, 1, 1; 1, 1). (dH, dV) = (0.5, 0.8)λ （Follow TR38.802） |
| UE antenna configurations | 2.6/4GHz (M,N,P,Mg,Ng;Mp,Np)= (1,2,2,1,1; 1,2) (dH, dV)=(0.5, N/A)λ |
| BS antenna pattern | Three-sector antenna |
| UE antenna pattern | omni |
| BS antenna down-tilt | 102 |
| UE distribution | 80% indoor, 20% outdoor Even distribution |
| BS receiver noise figure | 5dB |
| UE receiver noise figure | 9dB（Follow TR38.901 and TR 38.802） |
| Noise | -174 dBm/Hz |
| UE density | 10 UE per TRxP |
| Digital codebook | SVD |
| Subcarrier spacing | 2.6GHz:15kHz 4GHz:30kHz |

Table 8. SNR CDF table @ 2.6GHz

| Outage [%] | SNR [dB] | Outage [%] | SNR [dB] | Outage [%] | SNR [dB] | Outage [%] | SNR [dB] | Outage [%] | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | -14.14 | 20 | -0.69 | 40 | 2.46 | 60 | 7.08 | 80 | 14.32 |
| 1 | -8.11 | 21 | -0.46 | 41 | 2.63 | 61 | 7.38 | 81 | 14.99 |
| 2 | -6.01 | 22 | -0.36 | 42 | 2.87 | 62 | 7.54 | 82 | 15.61 |
| 3 | -5.11 | 23 | -0.27 | 43 | 3.03 | 63 | 7.90 | 83 | 16.29 |
| 4 | -4.43 | 24 | -0.05 | 44 | 3.14 | 64 | 8.15 | 84 | 17.29 |
| 5 | -3.98 | 25 | 0.12 | 45 | 3.34 | 65 | 8.41 | 85 | 18.20 |
| 6 | -3.74 | 26 | 0.27 | 46 | 3.60 | 66 | 8.69 | 86 | 18.95 |
| 7 | -3.36 | 27 | 0.42 | 47 | 3.79 | 67 | 9.04 | 87 | 20.02 |
| 8 | -3.14 | 28 | 0.61 | 48 | 4.05 | 68 | 9.38 | 88 | 20.86 |
| 9 | -2.80 | 29 | 0.77 | 49 | 4.26 | 69 | 9.62 | 89 | 21.66 |
| 10 | -2.54 | 30 | 0.89 | 50 | 4.44 | 70 | 9.84 | 90 | 22.69 |
| 11 | -2.30 | 31 | 1.04 | 51 | 4.72 | 71 | 10.23 | 91 | 23.78 |
| 12 | -2.07 | 32 | 1.20 | 52 | 4.97 | 72 | 10.65 | 92 | 25.05 |
| 13 | -1.92 | 33 | 1.32 | 53 | 5.19 | 73 | 10.98 | 93 | 26.06 |
| 14 | -1.77 | 34 | 1.49 | 54 | 5.36 | 74 | 11.36 | 94 | 27.84 |
| 15 | -1.60 | 35 | 1.65 | 55 | 5.57 | 75 | 11.74 | 95 | 29.02 |
| 16 | -1.44 | 36 | 1.89 | 56 | 5.92 | 76 | 12.03 | 96 | 30.45 |
| 17 | -1.23 | 37 | 2.01 | 57 | 6.28 | 77 | 12.69 | 97 | 32.37 |
| 18 | -1.02 | 38 | 2.16 | 58 | 6.62 | 78 | 13.27 | 98 | 34.87 |
| 19 | -0.83 | 39 | 2.33 | 59 | 6.89 | 79 | 13.86 | 99 | 38.79 |
|  |  |  |  |  |  |  |  | 100 | 43.74 |

Table 9. SNR CDF table @ 4GHz

| Outage [%] | SNR [dB] | Outage [%] | SNR [dB] | Outage [%] | SNR [dB] | Outage [%] | SNR [dB] | Outage [%] | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | -25.80 | 20 | -1.34 | 40 | 2.08 | 60 | 6.82 | 80 | 14.81 |
| 1 | -14.52 | 21 | -1.21 | 41 | 2.31 | 61 | 7.03 | 81 | 15.33 |
| 2 | -9.88 | 22 | -0.99 | 42 | 2.47 | 62 | 7.37 | 82 | 16.05 |
| 3 | -6.59 | 23 | -0.83 | 43 | 2.72 | 63 | 7.62 | 83 | 16.71 |
| 4 | -5.72 | 24 | -0.64 | 44 | 2.92 | 64 | 7.89 | 84 | 17.32 |
| 5 | -5.24 | 25 | -0.46 | 45 | 3.05 | 65 | 8.23 | 85 | 18.08 |
| 6 | -4.63 | 26 | -0.31 | 46 | 3.24 | 66 | 8.38 | 86 | 18.61 |
| 7 | -4.12 | 27 | -0.17 | 47 | 3.48 | 67 | 8.62 | 87 | 19.08 |
| 8 | -3.66 | 28 | 0.00 | 48 | 3.74 | 68 | 8.95 | 88 | 20.02 |
| 9 | -3.43 | 29 | 0.14 | 49 | 3.90 | 69 | 9.21 | 89 | 21.01 |
| 10 | -3.13 | 30 | 0.29 | 50 | 4.12 | 70 | 9.61 | 90 | 22.05 |
| 11 | -2.96 | 31 | 0.46 | 51 | 4.35 | 71 | 9.92 | 91 | 23.28 |
| 12 | -2.70 | 32 | 0.66 | 52 | 4.59 | 72 | 10.54 | 92 | 23.98 |
| 13 | -2.46 | 33 | 0.83 | 53 | 4.92 | 73 | 10.87 | 93 | 25.03 |
| 14 | -2.29 | 34 | 0.98 | 54 | 5.19 | 74 | 11.48 | 94 | 26.80 |
| 15 | -2.15 | 35 | 1.15 | 55 | 5.43 | 75 | 12.16 | 95 | 28.51 |
| 16 | -1.99 | 36 | 1.37 | 56 | 5.61 | 76 | 12.69 | 96 | 30.08 |
| 17 | -1.82 | 37 | 1.52 | 57 | 5.87 | 77 | 13.37 | 97 | 31.62 |
| 18 | -1.66 | 38 | 1.70 | 58 | 6.19 | 78 | 13.89 | 98 | 34.97 |
| 19 | -1.46 | 39 | 1.88 | 59 | 6.47 | 79 | 14.36 | 99 | 39.45 |
|  |  |  |  |  |  |  |  | 100 | 44.33 |

Table 10. Traffic model and configuration

| Duplex | 2.6GHz: DDDDDDDSUU(S: 6D:4G:4U) 4GHz: DDDSUDDSUU (S: 10D:2G:2U) |
| --- | --- |
| UE number | 2.6G:2~5 4G:5,10 |
| Traffic model | FTP3 Packet size:0.5 Mbytes Mean inter-arrival time:200 ms |